IEEE P802.19
TV Whitespace Coexistence Method

| Comment Resolution regarding Event Indication to another CM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Gwangzeen Ko | ETRI |  |  |  |
| Myung-Sun Song | ETRI |  |  |  |
| Jae-Ick Choi | ETRI |  |  |  |

Abstract

This document provides proposed resolutions for sub-clause 6.3.1.10 Event Indication to another CM related to CID 71 received in the 2nd WG letter ballot re-circulation of Draft P802.19.1/DF3.05.

- 6.3.1.10  Event Indication to another CM

To indicate event to another CM, a CM shall perform the sending event indication from CM to CM procedure as shown in clause 5.2.11.2.

First the CM shall generate and send the EventIndication message to another CM and wait for the EventConfirm message from another CM.

When generating the EventIndication message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | eventIndication |

The CM shall set the parameters of the eventIndication payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| ceID | CxID | CE ID |
| eventParams | EventParams | This parameter contains a list of event parameters. (SINR threshold reached or QoS degradation) |

After another CM has received the eventIndication message from CM, another CM shall generate and send the EventConfirm message to the CM.