IEEE P802.19
Wireless Coexistence

| Update on data types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Chen Sun | SONY |  |  | chen.sun@sony.com.cn |

Abstract

This document is a submission to IEEE 802.19 TG1 proposing update on the data types of DF3.05 as shown in the track changes.

Annex A (normative) Data types

IEEE802191DataType DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-----------------------------------------------------------

--Exported data types

-----------------------------------------------------------

--Exported data types

EXPORTS

--Coexistence protocol entity ID

CxID,

Status,

--CxMedia status

CxMediaStatus,

--Coexistence service

CoexistenceService,

--Subscribed service

SubscribedService,

--Network technology

NetworkTechnology,    --Network type

NetworkType,

--Discovery information

DiscoveryInformation,

--Geolocation

Geolocation,

--Coverage area

CoverageArea,

--Installation parameters

InstallationParameters,

--Frequency range

FrequencyRange,

--List of available frequencies

ListOfAvailableFrequencies,

--List of operating frequencies

ListOfOperatingFrequencies,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--Operation code for registration

OperationCode,

--Measurement capability

MeasurementCapability,

--List of available frequencies of the subject WSO

ListOfSubjectWSOAvailableFrequencies,

--Transmission schedule

TxSchedule,

--CM registration

CMRegistration,

--CE registration

CERegistration,

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--List of subject CEs

ListOfSubjectCEs,

--List of neighbor CMs transport information

ListOfNeighborCMsTransport,

--List of neighbor CM

ListOfNeighborCM,

--List of neighbor CM WSOs

ListOfNeighborCMWSOs,

--List of CEs for reconfiguration

ReconfigListOfCEs,

--Channel classification information

ChClassInfo,

--Channel classification information list

ChClassInfoList,

--Failed parameters

FailedParameters,

--Event parameters

EventParams,

--Required information description

ReqInfoDescr,

-- Requested information value

ReqInfoValue,

-- Negotiation status

NegotiationStatus,

-- Negotiation information

NegotiationInformation

-- Winner CM ID list

ListOfWinnerCMID,

-- Slot time position list

ListOfSlotTimePosition

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult,

--Mobility Information

MobilityInformation,

--Entity profile

EntityProfile;

-----------------------------------------------------------

--Coexistence protocol entity ID

-----------------------------------------------------------

--Coexistence protocol entity type

CxType ::= ENUMERATED {

--Coexistence enabler

ce,

--Coexistence manager

cm,

--Coexistence discovery and information server

cdis}

--Coexistence protocol entity ID

CxID ::= SEQUENCE {

--Entity type

type    CxType,

--Entity ID

id    OCTET STRING}

-----------------------------------------------------------

--Status

-----------------------------------------------------------

--Status

Status ::= ENUMERATED {

--Primitive/message is successfully processed

noError,

--Primitive/message is rejected due to security reasons

rejected,

--Primitive/message cannot be successfully processed because according to the current entity status different primitive/message is expected

invalidEntityStatus,

--Primitive/message cannot be successfully processed because of invalid values of parameters

invalidArgument,

--Primitive/message cannot be successfully processed because of the process error in the receiving entity

processFailure,

--Primitive/message cannot be successfully processed because of the connection error

networkFailure}

--CxMedia status

CxMediaStatus ::= ENUMERATED {

noErrorAccepted,

noErrorRejected,

errorInvalidEntityStatus,

errorInvalidArgument,

errorProcessFailure,

errorNetworkFailure,

errorUnknown

}

-----------------------------------------------------------

--Coexistence service

-----------------------------------------------------------

--Coexistence service

CoexistenceService ::= ENUMERATED {

--Information service

information,

--Management service

management,

--No service

noService}

SubscribedService::= ENUMERATED {

information,

management,

interCMCoexistenceSetElementsNeighbors,

allCoexistenceSetElementsNeighbors

}

-----------------------------------------------------------

--Network technology

-----------------------------------------------------------

NetworkTechnology ::= ENUMERATED {

--IEEE 802.11af

ieee802dot11af,

--IEEE 802.22

ieee802dot22,

--Radio microphone

radioMic,

--Area broadcast

areaBroadcast,

--ECMA 392

ecma392}

-----------------------------------------------------------

--Network type

-----------------------------------------------------------

NetworkType ::= ENUMERATED {

fixed,

mode1,

mode2,

…

}

-----------------------------------------------------------

--Discovery information

-----------------------------------------------------------

DiscoveryInformation ::= SEQUENCE {

coordinateX    REAL,

coordinateY    REAL,

coordinateZ    REAL,

maxTxPower    REAL,

rxSensitivity    REAL,

antennaGain    REAL,

minReqSNR    REAL,

antennaHeight    REAL,

…

}

-----------------------------------------------------------

--Location

-----------------------------------------------------------

--Location

Geolocation ::= CHOICE {

--Place name or ID

placeID    OCTET STRING,

--Coordinates of the master station

coordinates    SEQUENCE {

--Latitude

latitude    REAL,

--Longitude

longitude    REAL,

--Altitude

altitude    REAL    OPTIONAL}}

-----------------------------------------------------------

--Coverage area

-----------------------------------------------------------

--Coverage area

CoverageArea ::= SEQUENCE {

--Coverage radius

radius    REAL,

--Reference central frequency

refFrequency    REAL,

--Reference height of master station

refMasterHeight    REAL,

--Reference height of slave station

refSlaveHeight    REAL,

--Reference transmission power

refTxPower    REAL}

-----------------------------------------------------------

--Installation parameters

-----------------------------------------------------------

--Installation parameters

InstallationParameters ::= SEQUENCE {

--Operating height of master station

opMasterHeight    REAL    OPTIONAL,

--Operating height of slave station

opSlaveHeight    REAL    OPTIONAL,

--Operating transmission power

opTxPower    REAL    OPTIONAL,

--Adjacent channel selectivity of the WSO

aCS REAL OPTIONAL,

--Adjacent channel leakage ratio of the WSO

aCLR REAL OPTIONAL,

--Guaranteed QoS of backhaul connection of the WSO

guaranteedQoSOfBackhaulConnection GuaranteedQoSOfBackhaulConnection OPTIONAL

}

-----------------------------------------------------------

--Guaranteed QoS of backhaul connection related data types

----------------------------------------------------------

GuaranteedQoSOfWiredConnection:: = SEQUENCE{

--Backhaul type ID

backhaulTypeID ENUMERATED{

xDSL,

opticalFibre,

…

},

--Guaranteed minimum bit rates of backhaul connection

GuaranteedMinimumBitRates	REAL OPTIONAL,

…

}

-----------------------------------------------------------

--Frequency range related data types

-----------------------------------------------------------

--Frequency range

FrequencyRange ::= SEQUENCE {

--Start frequency

startFreq    REAL,

--Stop frequency

stopFreq    REAL}

--List of available frequencies

ListOfAvailableFrequencies ::= SEQUENCE OF SEQUENCE {

--Frequency range

frequencyRange    FrequencyRange,

--Transmission power limit

txPowerLimit    REAL    OPTIONAL,

--Start time when this frequency range is available

availableStartTime    GeneralizedTime    OPTIONAL,

--Duration during which this frequency range is available

availableDuration    REAL    OPTIONAL,

--Aggregated interference control parameters

aggInterfCtrolParam AggregatedInterferenceControlParameters OPTIONAL}

--List of supported frequencies

ListOfSupportedFrequencies ::= SEQUENCE OF FrequencyRange

--List of operating frequencies

ListOfOperatingFrequencies ::= SEQUENCE OF SEQUENCE {

--Frequency range

frequencyRange    FrequencyRange,

--Occupancy if known

occupancy    REAL    OPTIONAL}

-----------------------------------------------------------

--Aggregated interference control parameters

-----------------------------------------------------------

AggregatedInterferferenceControlParameters :: =  SEQUENCE{

--Reference point ID to be protected in controlling aggregated interference from the other WSO(s)

referencePointID INTEGER,

--Geolocation information of the reference point ID

geolocation	Geolocation,

--Adjacent channel selectivity of the WSO

aCS REAL,

--Adjacent channel leakage ratio of the WSO

aCLR REAL,

--Potential antenna height of the reception to be protected in the reference point

antennaHeight REAL,

--Potential antenna gain of the reception to be protected in the reference point

antennaGain	 REAL,

--Protection ratio of the reception to be protected at the reference point for the frequency

protectionRatio REAL,

…

}

-----------------------------------------------------------

--Available channel numbers

-----------------------------------------------------------

ConstOfChUseID :: = ENUMERATED {

regulationMaxTxPower,

regulationMaxAntGain,

regulationMaxAntHeight,

regulationTVDBUpdateTime,

outOfBandEmissionLimit,

…

}

ConstOfChUseValue ::= CHOICE {

regulationMaxTxPower    REAL,

regulationMaxAntMaxGain    REAL,

regulationAntMaxHeight    REAL,

regulationTVDBUpdateTime    REAL,

outOfBandEmissionLimit    REAL,

…

}

ConstOfChUses ::= SEQUENCE OF SEQUENCE {

constOfChUseID    ConstOfChUseID,

constOfChUseValue    ConstOfChUseValue

}

ListOfAvailableChNumbers ::= SEQUENCE OF SEQUENCE {

chNumber    INTEGER,

availableStartTime    GeneralizedTime,

availableDuration    REAL,

constOfChUses    ConstOfChUses

}

-----------------------------------------------------------

--Operating channel numbers

-----------------------------------------------------------

ListOfOperatingChNumbers ::= SEQUENCE OF SEQUENCE {

chNumber    INTEGER,

occupancy    REAL

}

-----------------------------------------------------------

--Required resource

-----------------------------------------------------------

--Required resource

RequiredResource ::= SEQUENCE OF SEQUENCE {

--Required bandwidth

requiredBandwidth    REAL,

--Expected occupancy if known

occupancy    REAL    OPTIONAL}

-----------------------------------------------------------

--Operation code for registration

-----------------------------------------------------------

--Operation code for registration

OperationCode ::= ENUMERATED {

--New registration

new,

--Update of registration information

update,

--Deregistration

delete}

-----------------------------------------------------------

--Measurement capability

-----------------------------------------------------------

MeasurementCapability ::= ENUMERATED {

energyDetection,

featureDetection,

…

}

-----------------------------------------------------------

--Reconfiguration related data types

-----------------------------------------------------------

--Transmission schedule

TxSchedule ::= SEQUENCE {

--Schedule start time

scheduleStartTime    GeneralizedTime,

--Schedule frame duration

scheduleFrameDuration    REAL,

--Number of schedule frames

numberOfFrames    INTEGER,

--Transmission start time within a schedule frame

transmissionStartTime    REAL,

--Transmission duration within a schedule frame

transmissionDuration    REAL}

-----------------------------------------------------------

--CM registration

-----------------------------------------------------------

--CM registration

CMRegistration ::= SEQUENCE {

--CM IP address

ipAddress    OCTET STRING,

--CM port number

portNumber    INTEGER}

--List of WSO for registration

ListOfWSORegistrations ::= SEQUENCE OF SEQUENCE {

--New registration, registration update or deregistration

operationCode    OperationCode,

--WSO ID

wsoID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology    OPTIONAL,    --Location

geolocation    Geolocation    OPTIONAL,

--Coverage area

coverageArea    CoverageArea    OPTIONAL,

--Installation parameters

installationParameters    InstallationParameters    OPTIONAL,

--List of available frequencies

listOfAvailableFrequencies    ListOfAvailableFrequencies    OPTIONAL,

--Maximum number of controllable WSO

maximumNumberOfControllableWSO   INTEGER OPTIONAL

}

--CE registration

CERegistration ::= SEQUENCE OF SEQUENCE{

--CE ID

ceID    CxID,

-- List of WSO registration

listOfWSORegistration    ListOfWSORegistrations}

-----------------------------------------------------------

--Coexistence report

-----------------------------------------------------------

CoexistenceReport ::= SEQUENCE OF SEQUENCE {

networkID    OCTET STRING,

networkTechnology    NetworkTechnology,

listOfOperatingChNumbers    ListOfOperatingChNumbers

}

ChannelPriority ::= SEQUENCE OF SEQUENCE {

channelNumber    INTEGER,

priority    INTEGER

}

-----------------------------------------------------------

--Coexistence set information related data types

-----------------------------------------------------------

--Interference direction

InterferenceDirection ::= ENUMERATED {

--Subject WSO creates interference to neighbor WSO

source,

--Neighbor WSO creates interference to subject WSO

victim,

--Both subject WSO and neighbor WSO create interference to each other

mutual}

--Network geometry class

NetworkGeometryClass  ::= ENUMERATED {

--	Class#1 network geometry

class_1,

--	Class#2 network geometry

class_2,

--	Class#3 network geometry

class_3,

--	Class#4 network geometry

class_4

}

--List of neighbor WSOs

ListOfNeighborWSOs ::= SEQUENCE OF SEQUENCE {

--Neighbor WSO ID

wsoID    OCTET STRING,

--Neighbor WSO network technology

networkTechnology    NetworkTechnology,

--Interference direction

interferenceDirection    InterferenceDirection,

--Network geometry classification

networkGeometryClass	NetworkGeometryClass OPTIONAL

--Distance to subject WSO

distance    REAL

--List of operating frequencies

--Not used in CoexistenceSetInformationAnnouncement

--Used in CoexistenceReportAnnouncement and CxMediaCoexistenceReportIndication

listOfOperatingFrequencies    ListOfOperatingFrequencies    OPTIONAL}

--List of neighbor CEs

ListOfNeighborCEs ::= SEQUENCE OF SEQUENCE {

--Neihgbor CE ID

ceID    CxID,

--List of neighbor WSOs

listOfNeighborWSOs    ListOfNeighborWSOs}

--List of neighbor CMs

ListOfNeighborCMs ::= SEQUENCE OF SEQUENCE {

--Neighbor CM ID

cmID    CxID,

--List of neighbor CEs

listOfNeighborCEs    ListOfNeighborCEs}

--List of available frequencies of the subject WSO

ListOfSubjectWSOAvailableFrequencies ::= SEQUENCE OF SEQUENCE {

--Frequency range

frequencyRange    FrequencyRange,

--List of neighbor CMs

listOfNeighborCMs    ListOfNeighborCMs}

--List of subject WSOs

ListOfSubjectWSOs ::= SEQUENCE OF SEQUENCE {

--Subject WSO ID

wsoID    OCTET STRING,

--List of available frequencies of the subject WSO

listOfSubjectWSOAvailableFrequencies    ListOfSubjectWSOAvailableFrequencies}

--List of subject CEs

ListOfSubjectCEs ::= SEQUENCE OF SEQUENCE {

--Subject CE ID

ceID    CxID,

--List of subject WSOs

listOfSubjectWSOs    ListOfSubjectWSOs}

--List of neighbor CMs transport information

ListOfNeighborCMsTransport ::= SEQUENCE OF SEQUENCE {

--Neighbor CM ID

cmID    CxID,

-- Neighbor CM IP address

ipAddress    OCTET STRING,

-- Neighbor CM port number

portNumber    INTEGER}

-----------------------------------------------------------

--Coexistence set information

-----------------------------------------------------------

ListOfCoexSetElement ::= SEQUENCE OF SEQUENCE {

networkID    OCTET STRING,

networkTechnology    NetworkTechnology

}

ListOfNeighborCM ::= SEQUENCE OF SEQUENCE {

neighborCMID    CxID,

listOfCoexSetElement    ListOfCoexSetElement

}

-----------------------------------------------------------

--Coexistence set element information related data types

-----------------------------------------------------------

--List of neighbor CM WSOs

ListOfNeighborCMWSOs ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING,

--List of available frequencies

listOfAvailableFrequencies    ListOfAvailableFrequencies    OPTIONAL,

--List of operating frequencies

listOfOperatingFrequencies    ListOfOperatingFrequencies    OPTIONAL,

--Network geometry classification

networkGeometryClass	NetworkGeometryClass OPTIONAL

}

-----------------------------------------------------------

--Coexistence set element reconfiguration related data types

-----------------------------------------------------------

--List of WSOs

ReconfigListOfWSOs ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING,

--Potential new operating frequency

newOperatingFrequency    FrequencyRange,

--Additionally operable network technology

addNetworkTechnology NetworkTechnology OPTIONAL

}

--List of CEs

ReconfigListOfCEs ::= SEQUENCE OF SEQUENCE {

--CE ID

ceID    CxID,

--List of WSOs

reconfigListOfWSOs    ReconfigListOfWSOs

}

-----------------------------------------------------------

--Channel classification

-----------------------------------------------------------

OperatingChannelInfo ::= SEQUENCE {

operatingChannelNumber    INTEGER,

listOfNetworkID    SEQUENCE OF OCTET STRING,

…

}

ChClassInfo ::= SEQUENCE {

availableChannelList    SEQUENCE OF INTEGER,

restrictedChannelList    SEQUENCE OF INTEGER,

protectedChannelList    SEQUENCE OF INTEGER,

unclassifiedChannelList    SEQUENCE OF INTEGER,

operatingChannelList    SEQUENCE OF OperatingChannelInfo,

coexistenceChannelList    SEQUENCE OF OperatingChannelInfo,

…

}

ChClassInfoList ::= SEQUENCE OF SEQUENCE {

networkID    OCTET STRING,

chClassInfo    ChClassInfo

}

-----------------------------------------------------------

--Failed parameters

-----------------------------------------------------------

FailedParameterID ::= ENUMERATED {

listOfoperatingChNumber,

txPowerLimit,

channelIsShared,

txSchedule

}

FailedParameterValue ::= CHOICE {

listOfoperatingChNumber    SEQUENCE OF INTEGER,

txPowerLimit    REAL,

channelIsShared    BOOLEAN,

txSchedule    TxSchedule

}

FailedParameters ::= SEQUENCE OF SEQUENCE {

failedParameterID    FailedParameterID,

failedParameterValue    FailedParameterValue

}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

EventDescr ::= ENUMERATED{

sinrThresholdReached,

qosDegradation,

…

}

EventParams ::= SEQUENCE {

eventDescr 	EventDescr

}

-----------------------------------------------------------

--Information Acquiring

-----------------------------------------------------------

ReqInfoDescr ::= SEQUENCE OF ENUMERATED {

sinr,

desiredBandwidth,

desiredOccupancy,

desiredQoS,

desiredCoverage,

channelNumber,

subscribedService,

interferenceLevel,

fairness,

threshold,

...

}

ReqInfoValue ::= SEQUENCE OF SEQUENCE {

reqInfoDescr    ReqInfoDescr,

reqInfoValue    CHOICE {

sinrValue    REAL,

desiredBandwidthValue    REAL,

desiredOccupancyValue    REAL,

desiredQoSValue    REAL,

desiredCoverageValue    REAL,

channelNumberValue    REAL,

subscribedServiceValue    SubscribedService

interferenceLevelValue    REAL,

fairnessValue    REAL,

thresholdValue    REAL,

otherValue    ANY

}

}

-----------------------------------------------------------

--Negotiation

-----------------------------------------------------------

NegotiationStatus :: = SEQUENCE {

negotiationSuccess    BOOLEAN,

negotiationFailure    BOOLEAN,

underNegotiation    BOOLEAN,

…

}

StartEndTime :: = SEQUENCE {

startTime    REAL,

endTime    REAL

}

TimeSharingUnitInfo ::= SEQUENCE {

referenceTime    REAL,

windowTime    StartEndTime,

slotTime    StartEndTime,

…

}

NegotiationInformation :: = SEQUENCE {

mode    BOOLEAN,

listOfChNumber    SEQUENCE OF INTEGER,

timeSharingUnitInfo    TimeSharingUnitInfo,

slotTimePosition    StartEndTime,

numberOfSlots    INTEGER,

disallowedSlotTimePosition    StartEndTime,

listOfContentionNumbers    SEQUENCE OF REAL,

…

}

ListOfWinnerCMID ::= SEQUENCE OF CxID

ListOfSlotTimePosition ::= SEQUENCE OF REAL

-----------------------------------------------------------

--Measurement

-----------------------------------------------------------

MeasurementSchedule ::= SEQUENCE {

measStartTime    REAL,

numberOfMeasurements    INTEGER,

timeBetweenMeasurements    REAL

}

MeasurementFreq ::= SEQUENCE OF INTEGER

MeasurementType ::= ENUMERATED {

interferenceLevel

}

MeasurementDescription ::= SEQUENCE {

measType    MeasurementType,

measSchedule    MeasurementSchedule,

measFreq    MeasurementFreq

}

MeasurementReport ::= CHOICE {

interferenceLevelValue    REAL,

…

}

MeasurementResult ::= SEQUENCE OF SEQUENCE {

measurementDescription    MeasurementDescription,

measurementReport    MeasurementReport

}

END