IEEE P802.19
Wireless Coexistence

| Update on primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Chen Sun | SONY |  |  | chen.sun@sony.com.cn |

Abstract

This document is a submission to IEEE 802.19 TG1 proposing update on the primitives of DF3.05 as shown in the track changes.

B1. Transport SAP

IEEE80219TRSAPPrimitive DEFINITIONS AUTOMATIC TAGS::= BEGIN

-----------------------------------------------------------

--Imported data types

-----------------------------------------------------------

--Imported data types

IMPORTS

--Coexistence protocol entity ID

CxID,

--Status

Status

FROM IEEE802191DataType;

-----------------------------------------------------------

--Transport service configuration

-----------------------------------------------------------

--Transport service configuration request

TrConfigurationRequest ::= SEQUENCE {

--Source ID

sourceID CxID,

--Destination ID

destinationID CxID,

--Destination IP address

destinationIPAddress    OCTET STRING,

--Destination port number

destinationPortNumber    INTEGER}

--Transport service configuration response

TrConfigurationResponse ::= SEQUENCE {

--Destination ID

destinationID CxID,

--Configuration status

status Status}

-----------------------------------------------------------

--Connection establishment

-----------------------------------------------------------

--Request for connection

TrConnectionRequest ::= SEQUENCE {

--Source ID

sourceID CxID,

--Destination ID

destinationID CxID}

--Indication of the received connection request

TrConnectionIndication ::= SEQUENCE {

--Source ID

sourceID CxID}

--Response to the received connection request

TrConnectionResponse ::= SEQUENCE {

--Destination ID

destinationID CxID,

--Connection status

status Status}

--Confirmation to the connection request

TrConnectionConfirm ::= SEQUENCE {

--Source ID

sourceID CxID,

--Connection status

status Status}

-----------------------------------------------------------

--Connection termination

-----------------------------------------------------------

--Connection termination request

TrDisconnectionRequest ::= SEQUENCE {

--Destination ID

destinationID CxID}

--Connection termination indication

TrDisconnectionIndication ::= SEQUENCE {

--Source ID

sourceID CxID}

-----------------------------------------------------------

--Sending a coexistence protocol message

-----------------------------------------------------------

--Request to send coexistence protocol message

TrSendRequest ::= SEQUENCE {

--Destination ID

destinationID CxID,

--Encoded coexistence protocol message

cxMessage OCTET STRING}

--Confirmation of sending coexistence protocol message

TrSendConfirm ::= SEQUENCE {

--Destination ID

destinationID CxID,

--Status of sending coexistence protocol message

status Status}

-----------------------------------------------------------

--Receiving a coexistence protocol message

-----------------------------------------------------------

--Indication of received coexistence protocol message

TrReceiveIndication ::= SEQUENCE {

--Source ID

sourceID CxID,

--Encoded coexistence protocol message

cxMessage OCTET STRING}

END

- B.2 Coexistence Media SAP

IEEE80219MEDIASAPPrimitive DEFINITIONS AUTOMATIC TAGS::= BEGIN

-----------------------------------------------------------

--Imported data types

-----------------------------------------------------------

--Imported data types

IMPORTS

--Status

Status,

--CxMedia status

CxMediaStatus,

--Coexistence service

CoexistenceService,

--Subscribed service

SubscribedService,

--Network technology

NetworkTechnology,

--Network type

NetworkType,

--Geolocation

Geolocation,

--Discovery information

DiscoveryInformation,

--Coverage area

CoverageArea,

--Installation parameters

InstallationParameters,

--List of available frequencies

ListOfAvailableFrequencies,

--List of operating frequencies

ListOfOperatingFrequencies,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--List of subject WSO available frequencies

ListOfSubjectWSOAvailableFrequencies,

--Transmission schedule

TxSchedule,

--Coexistence protocol entity ID

CxID,

--Frequency range

FrequencyRange,

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--Measurement capability

MeasurementCapability,

-- Operation code

OperationCode,

--Failed parameters

FailedParameters,

--Channel classification information list

ChClassInfoList,

--Event parameters

EventParams,

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult

FROM IEEE802191DataType;

-----------------------------------------------------------

--WSO subscription

-----------------------------------------------------------

--Request for subscription information

CxMediaSubscriptionRequest ::= SEQUENCE {}

--Subscription information

CxMediaSubscriptionResponse ::= SEQUENCE {

--WSO subscription ID

clientID    IA5String    OPTIONAL,

--WSO subscription password

clientPassword    IA5String    OPTIONAL,

--List of serving CMs

listOfCMs    SEQUENCE OF SEQUENCE {

--CM ID

cmID    CxID,

--CM IP address

cmIPAddress    OCTET STRING,

--CM port number

cmPortNumber    INTEGER,

--CM subscription ID

serverID    IA5String,

--CM subscription password

serverPassword    IA5String}    OPTIONAL,

--Coexistence service to which WSO is subscribed

coexistenceService    CoexistenceService    OPTIONAL,

--Coexistence service to which WSO is subscribed

subscribedService    SubscribedService    OPTIONAL,

--Status

status    CxMediaStatus    OPTIONAL}

--Request to change subscription

CxMediaSubscriptionIndication ::= SEQUENCE {

--Coexistence service to which WSO is subscribed

coexistenceService    CoexistenceService    OPTIONAL,

--Coexistence service to which WSO is subscribed

subscribedService    SubscribedService    OPTIONAL}

--Subscription status

CxMediaSubscriptionConfirm ::= SEQUENCE {

--Subscription status

--noError means subscription is confirmed

--rejected means subscription is not confirmed

status    Status    OPTIONAL,

--Status

cxMediaStatus    CxMediaStatus    OPTIONAL}

--Request to change subscription

CxMediaChangeSubscriptionRequest ::= SEQUENCE {

--Coexistence service to which WSO is subscribed

subscribedService    SubscribedService}

--Response for subscription change

CxMediaChangeSubscriptionResponse :: = SEQUENCE {

--Status

Status    CxMediaStatus}

-----------------------------------------------------------

--WSO registration

-----------------------------------------------------------

--Request for registration information

CxMediaRegistrationRequest ::= SEQUENCE {}

--Registration information

CxMediaRegistrationResponse ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING    OPTIONAL,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType    NetworkType    OPTIONAL,

--Geolocation

geolocation    Geolocation    OPTIONAL,

--Discovery information

discoveryInformation    DiscoveryInformation    OPTIONAL,

--Coverage area

coverageArea    CoverageArea    OPTIONAL,

--Installation parameters

installationParameters    InstallationParameters    OPTIONAL,

--List of available frequencies

listOfAvailableFrequencies  ListOfAvailableFrequencies  OPTIONAL,

--List of operating frequencies

listOfOperatingFrequencies  ListOfOperatingFrequencies  OPTIONAL,

--List of available channel number

listOfAvailableChNumbers    ListOfAvailableChNumbers    OPTIONAL,

--List of supported channel number

listOfSupportedChNumbers    SEQUENCE OF INTEGER    OPTIONAL,

--List of operating channel number

listOfOperatingChNumbers    ListOfOperatingChNumbers    OPTIONAL,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN    OPTIONAL,

--Measurement capability

measurementCapability    MeasurementCapability    OPTIONAL,

--Required resource

requiredResource    RequiredResource}

--Updated registration information

CxMediaRegistrationIndication ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING    OPTIONAL,

--Network ID

networkID    OCTET STRING    OPTIONAL,

--Network technology

networkTechnology    NetworkTechnology    OPTIONAL,    --Network type

networkType    NetworkType    OPTIONAL,

--Indication of WSO stop operation

wsoStopOperation    BOOLEAN    OPTIONAL,

--List of available frequencies

listOfAvailableFrequencies  ListOfAvailableFrequencies  OPTIONAL,

--List of operating frequencies

listOfOperatingFrequencies  ListOfOperatingFrequencies  OPTIONAL,

--Required resource

requiredResource    RequiredResource    OPTIONAL,

--Discovery information

discoveryInformation    DiscoveryInformation    OPTIONAL,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN    OPTIONAL,

--List of available channel number

listOfAvailableChNumbers    ListOfAvailableChNumbers    OPTIONAL,

--List of supported channel number

listOfSupportedChNumbers    SEQUENCE OF INTEGER    OPTIONAL,

--List of operating channel number

listOfOperatingChNumbers    ListOfOperatingChNumbers    OPTIONAL,

--Measurement capability

measurementCapability    MeasurementCapability    OPTIONAL}

--Registration confirmation

CxMediaRegistrationConfirm ::= SEQUENCE {

--Registration status

status    Status    OPTIONAL,

--CxMedis status

cxMediastatus    CxMediaStatus    OPTIONAL}

-----------------------------------------------------------

--WSO reconfiguration

-----------------------------------------------------------

--Reconfiguration request

CxMediaReconfigurationRequest ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING    OPTIONAL,

--Operating frequency

operatingFrequency    FrequencyRange    OPTIONAL,

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER    OPTIONAL,

--Transmission power limit

txPowerLimit    REAL    OPTIONAL,

--Indication whether the channel is shared

channelIsShared    BOOLEAN,

--Transmission schedule

txSchedule    TxSchedule    OPTIONAL,

-- Channel classification information

chClassInfo    ChClassInfo    OPTIONAL},

--Additionally operable network technology

addNetworkTechnology CHOICE { NetworkTechnology } OPTIONAL

--Reconfiguration response

CxMediaReconfigurationResponse ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING    OPTIONAL,

--Reconfiguration status

status    Status    OPTIONAL,

--CxMedia status

cxMediaStatus    CxMediaStatus    OPTIONAL,

--Failed parameters

failedParameters    FailedParameters    OPTIONAL}

-----------------------------------------------------------

--CM stop operation

-----------------------------------------------------------

--Stop operation indication

CxMediaStopOperationIndication ::= SEQUENCE {}

--Stop operation confirm

CxMediaStopOperationConfirm ::= SEQUENCE {

--Stop operation status

status    Status}

-----------------------------------------------------------

--Coexistence report

-----------------------------------------------------------

-- Coexistence report indication

CxMediaCoexistenceReportAnnouncement ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING,

--List of subject WSO available frequencies

listOfSubjectWSOAvailableFrequencies    ListOfSubjectWSOAvailableFrequencies}

-- Coexistence report confirm

CxMediaCoexistenceReportConfirm ::= SEQUENCE {

--Coexistence report status

status    Status}

--Request for coexistence report

CxMediaCoexistenceReportRequest ::= SEQUENCE {}

--Response for coexistence report

CxMediaCoexistenceReportResponse ::= SEQUENCE {

--Coexistence report information

coexistenceReport    CoexistenceReport,

--Channel priority information

channelPriority    ChannelPriority}

-----------------------------------------------------------

--Channel classification requested by CE

-----------------------------------------------------------

--Channel classification request

CxMediaChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID    SEQUENCE OF OCTET STRING}

-- Channel classification response

CxMediaChannelClassificationResponse ::= SEQUENCE {

--List of channel classification information

chClassInfoList    ChClassInfoList}

-----------------------------------------------------------

--Channel classification update

-----------------------------------------------------------

--Channel classification update

CxMediaChannelClassificationIndication ::= SEQUENCE {

--List of channel classification information

chClassInfoList    ChClassInfoList}

-----------------------------------------------------------

--Available channel list from WSO

-----------------------------------------------------------

-- Available channel List request

CxMediaAvailableChannelListRequest ::= SEQUENCE {}

-- Available channel list response

CxMediaAvailableChannelListResponse ::= SEQUENCE {

--Available channel list information

listOfAvailableChNumbers    ListOfAvailableChNumbers}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

-- Event indication

CxMediaEventIndication ::= SEQUENCE {

-- Event indication information

eventParams    EventParams}

-----------------------------------------------------------

--Measurement Request

-----------------------------------------------------------

-- Measurement request

CxMediaGetMeasurementRequest ::= SEQUENCE {

-- Measurement request information

measurementDescription    MeasurementDescription}

-----------------------------------------------------------

-- Measurement results

-----------------------------------------------------------

-- Measurement response

CxMediaGetMeasurementResponse ::= SEQUENCE OF SEQUENCE{

-- Measurement results

measurementResult    MeasurementResult}

-----------------------------------------------------------

-- WSO Deregistration

-----------------------------------------------------------

--Deregistration request

CxMediaPerformDeregistrationRequest ::= SEQUENCE {

--List of operating channel number

wsoDeregistration    BOOLEAN}

-- Deregistration response

CxMediaPerformReconfigurationResponse ::= SEQUENCE {

--CxMedia status

cxMediaStatus    CxMediaStatus}

END