IEEE P802.19
Wireless Coexistence

| Update on messages |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Chen Sun | SONY |  |  | chen.sun@sony.com.cn |

Abstract

This document is a submission to IEEE 802.19 TG1 proposing update on the messages of DF3.05 as shown in the track changes.

Annex C (normative) Messages

IEEE802191Message DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-----------------------------------------------------------

--Imported data types

-----------------------------------------------------------

--Imported data types

IMPORTS

--Coexistence protocol entity ID

CxID,

--Status

Status,

--CxMedia status

CxMediaStatus,

--Coexistence service

CoexistenceService,

--Subscribed service

SubscribedService,

--Network technology

NetworkTechnology,

--Network type

NetworkType,

--Location

Geolocation,

--Discovery information

DiscoveryInformation,

--Coverage area

CoverageArea,

--Installation parameters

InstallationParameters,

--List of available frequencies

ListOfAvailableFrequencies,

--List of operating frequencies

ListOfOperatingFrequencies,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--Operation code for registration

OperationCode,

--List of available frequencies of the subject WSO

ListOfSubjectWSOAvailableFrequencies,

--Transmission schedule

TxSchedule,

--Frequency range

FrequencyRange,

--CM registration

CMRegistration,

--CE registration

CERegistration,

--List of subject CEs

ListOfSubjectCEs,

--List of neighbor CMs transport information

ListOfNeighborCMsTransport,

--List of neighbor CM WSOs

ListOfNeighborCMWSOs,

--List of CEs for reconfiguration

ReconfigListOfCEs

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--Measurement capability

MeasurementCapability,

--List of neighbor CM

ListOfNeighborCM,

--Failed parameters

FailedParameters,

--Channel classification information

ChClassInfo,

--Required information description

ReqInfoDescr,

--Requested information value

ReqInfoValue,

--Event parameters

EventParams,

--Negotiation status

NegotiationStatus,

--Negotiation information

NegotiationInformation

--Winner CM ID list

ListOfWinnerCMID,

-- Slot time position list

ListOfSlotTimePosition

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult

FROM IEEE802191DataType;

-----------------------------------------------------------

--Message structure, header structure, and payload types

-----------------------------------------------------------

--Message structure

CxMessage ::= SEQUENCE {

-- Message header

header CxHeader,

-- Message payload

payload CxPayload}

--Header

CxHeader ::= CHOICE {

--For announcement

none NULL,

--For request or single response

requestID INTEGER (0..2147483647),

--For multiple responses

multipleResponse SEQUENCE {

--Original request ID

requestID INTEGER (0..2147483647),

--Response number

sequenceNumber INTEGER (0..2147483647),

--True is the response is last

isLastResponse BOOLEAN}}

--Payload types

CxPayload ::= CHOICE {

--WSO subscription request

subscriptionRequest    SubscriptionRequest,

--WSO subscription response

subscriptionResponse    SubscriptionResponse,

--WSO subscription change request

subscriptionChangeRequest    SubscriptionChangeRequest,

--WSO subscription change response

subscriptionChangeResponse    SubscriptionChangeResponse,

--CE registration request

ceRegistrationRequest    CERegistrationRequest,

--Registration response

registrationResponse    RegistrationResponse,

--Reconfiguration request

reconfigurationRequest    ReconfigurationRequest,

--Reconfiguration response

reconfigurationResponse    ReconfigurationResponse,

--Stop operation announcement

stopOperationAnnouncement    StopOperationAnnouncement,

--Stop operation confirm

stopOperationConfirm    StopOperationConfirm,

--Coexistence report announcement

coexistenceReportAnnouncement    CoexistenceReportAnnouncement,

--Coexistence report confirm

coexistenceReportConfirm    CoexistenceReportConfirm,

--Coexistence report request

coexistenceReportRequest    CoexistenceReportRequest,

--Coexistence report response

coexistenceReportResponse    CoexistenceReportResponse,

--CM registration request

cmRegistrationRequest    CMRegistrationRequest,

--Coexistence set information announcement

coexistenceSetInformationAnnouncement    CoexistenceSetInformationAnnouncement,

--Coexistence set information confirm

coexistenceSetInformationConfirm    CoexistenceSetInformationAnnouncement,

--Coexistence set information request

coexistenceSetInformationRequest     CoexistenceSetInformationRequest,

--Coexistence set information response

coexistenceSetInformationResponse    CoexistenceSetInformationResponse,

--Coexistence set element information announcement

coexistenceSetElementInformationAnnouncement    CoexistenceSetElementInformationAnnouncement,

--Coexistence set element information confirm

coexistenceSetElementInformationConfirm    CoexistenceSetElementInformationConfirm,

--Coexistence set element information request

coexistenceSetElementInformationRequest    CoexistenceSetElementInformationRequest,

--Coexistence set element information response

coexistenceSetElementInformationResponse    CoexistenceSetElementInformationResponse,

--Coexistence set element reconfiguration request

coexistenceSetElementReconfigurationRequest    CoexistenceSetElementReconfigurationRequest,

--Coexistence set element reconfiguration response

coexistenceSetElementReconfigurationResponse    CoexistenceSetElementReconfigurationResponse,

--CM Reconfiguration request

cmReconfigurationRequest    CMReconfigurationRequest,

--CM Reconfiguration response

cmReconfigurationResponse    CMReconfigurationResponse,

--WSO channel classification request

channelClassificationRequest     ChannelClassificationRequest,

-- WSO channel classification response

channelClassificationResponse   ChannelClassificationResponse,

--CM channel classification request

cmChannelClassificationRequest     CMChannelClassificationRequest,

-- CM channel classification response

cmChannelClassificationResponse   CMChannelClassificationResponse,

-- WSO channel classification update

channelClassificationAnnouncement     ChannelClassificationAnnouncement,

--Available channel list request from WSO

availableChannelsRequest      AvailableChannelsRequest,

-- Available channel list response from WSO

availableChannelsResponse     AvailableChannelsResponse,

--Information acquiring request

infoAcquiringRequest    InforAcquiringRequest,

--Information acquiring response

infoAcquiringResponse    InforAcquiringResponse,

-- Event indication

eventIndication     EventIndication,

-- Event confirm

eventConfirm     EventConfirm,

--WSO measurement request

measurementRequest      MeasurementRequest,

--WSO measurement response

measurementResponse      MeasurementResponse,

--WSO measurement confirm

measurementConfirm      MeasurementConfirm,

--Master/Slave CM request

masterCMRequest      MasterCMRequest,

--Master/Slave CM response

masterCMResponse      MasterCMResponse,

--Master/Slave CM configuration request

masterSlaveCMconfigurationRequest      MasterSlaveCMconfigurationRequest,

--Master/Slave CM configuration response

masterSlaveCMconfigurationResponse      MasterSlaveCMconfigurationResponse,

--Negotiation request

negotiationRequest             NegotiationRequest,

--Negotiation announcement

negotiationAnnouncement        NegotiationAnnouncement,

--Deregistration request

wsoDeregistrationRequest      WsoDeregistrationRequest,

-- Deregistration response

wsoDeregistrationResponse      WsoDeregistrationResponse

}

-----------------------------------------------------------

--WSO and CM subscription

-----------------------------------------------------------

--WSO subscription request

SubscriptionRequest ::= SEQUENCE {

--WSO subscription ID

clientID    IA5String    OPTIONAL,

--WSO subscription password

clientPassword    IA5String    OPTIONAL,

--Coexistence service to which WSO is subscribed

coexistenceService    CoexistenceService    OPTIONAL,

--Coexistence service to which WSO is subscribed

subscribedService    SubscribedService    OPTIONAL}

--WSO subscription response

SubscriptionResponse ::= SEQUENCE {

--CM subscription ID

serverID    IA5String    OPTIONAL,

--CM subscription password

serverPassword    IA5String    OPTIONAL,

--Subscription status

--noError means subscription is confirmed

--rejected means subscription is not confirmed

status    Status    OPTIONAL,

--Status

status   CxMediaStatus    OPTIONAL}

-----------------------------------------------------------

--WSO subscription change

-----------------------------------------------------------

--Request to change subscription

SubscriptionChangeRequest ::= SEQUENCE {

--Coexistence service to which WSO is subscribed

subscribedService   SubscribedService}

--Response for subscription change

SubscriptionChangeResponse :: = SEQUENCE {

--Status

status   CxMediaStatus}

-----------------------------------------------------------

--WSO registration

-----------------------------------------------------------

CERegistrationRequest ::= SEQUENCE OF SEQUENCE {

--New registration, registration update or deregistration

operationCode    OperationCode,

--WSO ID

wsoID    OCTET STRING,

--Network ID

networkID    OCTET STRING    OPTIONAL,

--Network technology

networkTechnology    NetworkTechnology    OPTIONAL,

--Network type

networkType    NetworkType    OPTIONAL,

--Location

geolocation    Geolocation    OPTIONAL,

--Discovery information

discoveryInformation   DiscoveryInformation    OPTIONAL,

--Coverage area

coverageArea    CoverageArea    OPTIONAL,

--Installation parameters

installationParameters    InstallationParameters    OPTIONAL,

--List of available frequencies

listOfAvailableFrequencies  ListOfAvailableFrequencies  OPTIONAL,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN    OPTIONAL,

--List of operating frequencies

listOfOperatingFrequencies  ListOfOperatingFrequencies  OPTIONAL,

--List of available channel number

listOfAvailableChNumbers    ListOfAvailableChNumbers    OPTIONAL,

--List of supported channel number

listOfSupportedChNumbers    SEQUENCE OF INTEGER    OPTIONAL,

--List of operating channel number

listOfOperatingChNumbers    ListOfOperatingChNumbers    OPTIONAL,

--Required resource

requiredResource    RequiredResource    OPTIONAL,

--Measurement capability

measurementCapability    MeasurementCapability    OPTIONAL}

--Registration response

RegistrationResponse ::= SEQUENCE {

--Registration status

status    Status    OPTIONAL}

-----------------------------------------------------------

--WSO reconfiguration

-----------------------------------------------------------

--Reconfiguration request

ReconfigurationRequest ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING    OPTIONAL,

--Operating frequency

operatingFrequency    FrequencyRange    OPTIONAL,

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER    OPTIONAL,

--Transmission power limit

txPowerLimit    REAL    OPTIONAL,

--Indication whether the channel is shared

channelIsShared    BOOLEAN    OPTIONAL,

--Transmission schedule

txSchedule    TxSchedule    OPTIONAL,

-- Channel classification information

chClassInfo    ChClassInfo    OPTIONAL,

-- Mobility information

mobilityInformation MobilityInformation OPTIONAL

--Additionally operable network technology

addNetworkTechnology NetworkTechnology OPTIONAL

}

--Reconfiguration response

ReconfigurationResponse ::= SEQUENCE OF SEQUENCE {

--WSO ID

wsoID    OCTET STRING    OPTIONAL,

--Reconfiguration status

status    Status    OPTIONAL,

--Status

status    BOOLEAN    OPTIONAL,

--Failed parameters

failedParameters    FailedParameters    OPTIONAL}

-----------------------------------------------------------

--Stop operation

-----------------------------------------------------------

--Stop operation announcement

StopOperationAnnouncement ::= SEQUENCE {}

--Stop operation confirm

StopOperationConfirm ::= SEQUENCE {

--Stop operation status

status    Status}

-----------------------------------------------------------

--Coexistence report

-----------------------------------------------------------

--Coexistence report announcement

CoexistenceReportAnnouncement ::= SEQUENCE OF SEQUENCE {

--Subject WSO ID

wsoID    OCTET STRING,

--List of subject WSO available frequencies

listOfSubjectWSOAvailableFrequencies    ListOfSubjectWSOAvailableFrequencies}

--Coexistence report confirm

CoexistenceReportConfirm ::= SEQUENCE {

--Coexistence report status

status    Status}

--Request for coexistence report

CoexistenceReportRequest ::= SEQUENCE {}

--Response for coexistence report

CoexistenceReportResponse ::= SEQUENCE {

--Coexistence report information

coexistenceReport     CoexistenceReport,

--Channel priority information

channelPriority     ChannelPriority}

-----------------------------------------------------------

--CM registration

-----------------------------------------------------------

--CM registration request

CMRegistrationRequest ::= SEQUENCE {

--CM profile

cmProfile EntityProfile OPTIONAL,

--CM registration

cmRegistration    CMRegistration    OPTIONAL,

--CE registration

ceRegistration    CERegistration    OPTIONAL,

--Operationg code

operationCode    OperationCode    OPTIONAL,

--CE ID

ceID    CxID    OPTIONAL,

--Network ID

networkID    OCTET STRING    OPTIONAL,

--Network technology

networkTechnology    NetworkTechnology    OPTIONAL,

--Network type

networkType    NetworkType    OPTIONAL,

--Discovery information

discoveryInformation    DiscoveryInformation    OPTIONAL,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER    OPTIONAL,

-- List of supported frequencies

listOfSuppFrequencies ListOfSupportedFrequencies OPTIONAL},

--Maximum number of controllable WSO

maximumNumberOfControllableWSO   INTEGER OPTIONAL

}

-----------------------------------------------------------

--Coexistence set information

-----------------------------------------------------------

--Coexistence set information announcement

CoexistenceSetInformationAnnouncement ::= SEQUENCE {

--List of subject CEs

listOfSubjectCEs    ListOfSubjectCEs,

--List of neighbor CMs transport information mentioned inside the list of subject CEs

listOfNeighborCMsTransport    ListOfNeighborCMsTransport}

--Coexistence set information confirm

CoexistenceSetInformationConfirm ::= SEQUENCE {

--Coexistence set information status

status    Status}

--Request for coexistence set information

CoexistenceSetInformationRequest ::= SEQUENCE {

listOfNetworkID      SEQUENCE OF OCTET STRING}

--Response for coexistence set information

CoexistenceSetInformationResponse ::= SEQUENCE {

--Network ID

coexistenceReport     CoexistenceReport,

--List of neighbor CM

listOfneighborCM      ListOfneighborCM}

-----------------------------------------------------------

--Coexistence set element information

-----------------------------------------------------------

--Coexistence set element information announcement

CoexistenceSetElementInformationAnnouncement ::= SEQUENCE OF SEQUENCE{

--CE ID

ceID    CxID,

--Coexistence service

coexistenceService    CoexistenceService    OPTIONAL,

--List of neighbor CM WSOs

listOfNeighborCMWSOs    ListOfNeighborCMWSOs    OPTIONAL

}

--Coexistence set element information confirm

CoexistenceSetElementInformationConfirm ::= SEQUENCE {

--Coexistence set element information status

status    Status}

--Coexistence set element information request

CoexistenceSetElementInformationRequest ::= SEQUENCE OF SEQUENCE{

--CE ID

ceID    CxID,

--List of neighbor CM WSOs

listOfNeighborCMWSOs    ListOfNeighborCMWSOs

}

--Coexistence set element information response

CoexistenceSetElementInformationResponse ::= SEQUENCE OF SEQUENCE{

--CE ID

ceID    CxID,

--Coexistence service

coexistenceService    CoexistenceService,

--List of neighbor CM WSOs

listOfNeighborCMWSOs    ListOfNeighborCMWSOs

}

-----------------------------------------------------------

--Coexistence set element reconfiguration

-----------------------------------------------------------

--Coexistence set element reconfiguration request

CoexistenceSetElementReconfigurationRequest ::= SEQUENCE{

--List of subject CEs

reconfigListOfSubjectCEs    ReconfigListOfCEs,

--List of neighbor CEs

reconfigListOfNeighborCEs    ReconfigListOfCEs

}

--Coexistence set element reconfiguration response

CoexistenceSetElementReconfigurationResponse ::= SEQUENCE{

--Indicates whether the suggested reconfiguration is accepted or not

requestIsAccepted    BOOLEAN

}

-----------------------------------------------------------

--WSO reconfiguration for another CM

-----------------------------------------------------------

--Reconfiguration request

CMReconfigurationRequest ::= SEQUENCE {

--Indication for CE to be reconfigured

reconfigTarget    CxID,

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER,

--Transmission power limitation

txPowerLimit    REAL,

--Indication whether the channel is shared

channelIsShared    BOOLEAN,

--Transmission schedule

txSchedule    TxSchedule ,

-- Channel classification information

chClassInfo    ChClassInfo,

--Additionally operable network technology

newNetworkTechnology NetworkTechnology OPTIONAL

--

}

--Reconfiguration response

ReconfigurationResponse ::= SEQUENCE {

--Indication for CE to be reconfigured

reconfigTarget    CxID,

--Status

status   BOOLEAN

--Failed parameters

failedParameters    FailedParameters}

-----------------------------------------------------------

--Channel classification

-----------------------------------------------------------

--Channel classification request

ChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID     SEQUENCE OF OCTET STRING}

-- Channel classification response

ChannelClassificationResponse ::= SEQUENCE  OF SEQUENCE{

-- Network ID

networkID    OCTET STRING

--Channel classification information

chClassInfo      ChClassInfo}

--CM Channel classification request

CMChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID     SEQUENCE OF OCTET STRING}

-- CM Channel classification response

CMChannelClassificationResponse ::= SEQUENCE  OF SEQUENCE{

-- Network ID

networkID    OCTET STRING

--Channel classification information

chClassInfo      ChClassInfo}

-----------------------------------------------------------

--Channel classification update

-----------------------------------------------------------

--Channel classification update

ChannelClassificationAnnouncement ::= SEQUENCE OF SEQUENCE {

-- Network ID

networkID    OCTET STRING

--Channel classification information

chClassInfo      ChClassInfo}

-----------------------------------------------------------

--Information acquiring from another CM

-----------------------------------------------------------

-- Information acquiring request

InfoAcquringRequest ::= SEQUENCE {

ceID    CxID,

listOfReqInfoDescr    SEQUENCE OF ReqInfoDescr}

-- Information acquiring response

InfoAcquringResponse ::= SEQUENCE {

ceID    CxID,

reqInfoValue    ReqInfoValue}

-----------------------------------------------------------

--Available channel list from WSO

-----------------------------------------------------------

-- Available channel request

AvailableChannelsRequest ::= SEQUENCE {}

-- Available channel response

AvailableChannelsResponse ::= SEQUENCE {

--Available channel list information

listOfAvailableChNumbers      ListOfAvailableChNumbers}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

-- Event indication

EventIndication ::= SEQUENCE {

-- Event indication information

eventParams    EventParams}

-- Event confirm

EventConfirm ::= SEQUENCE {}

-----------------------------------------------------------

--Measurement Request

-----------------------------------------------------------

-- Measurement request

MeasurementRequest ::= SEQUENCE {

-- Measurement request information

measurementDescription    MeasurementDescription}

-----------------------------------------------------------

-- Measurement results

-----------------------------------------------------------

-- Measurement response

MeasurementResponse ::= SEQUENCE OF SEQUENCE{

-- Measurement results

measurementResult    MeasurementResult}

-----------------------------------------------------------

-- Negotiation

-----------------------------------------------------------

-- Negotiation request

NegotiationRequest ::= SEQUENCE {

-- Negotiation status

negotiationStatus             NegotiationStatus,

-- Negotiation information

negotiationInformation    NegotiationInformation}

-- Negotiation announcement

NegotiationAnnouncement ::= SEQUENCE {

-- Winner CM ID list

listOfWinnerCMID         ListOfWinnerCMID,

-- Slot time position list

listOfSlotTimePosition    ListOfSlotTimePosition}

-----------------------------------------------------------

--Master/Slave CM selection

-----------------------------------------------------------

--Master/Slave CM selection request

MasterCMRequest ::= SEQUENCE {

--List of CEs managed by CM that intends to become slave CM

listOfCEs    SEQUENCE OF CxID}

-- Master/Slave CM selection response

MasterCMResponse ::= SEQUENCE {

--Status

status    CxMediaStatus}

-----------------------------------------------------------

--Master/Slave CM configuration

-----------------------------------------------------------

--Master/Slave CM configuration request

MasterSlaveCMconfigurationRequest ::= SEQUENCE {

-- List of CEs managed by CM

listOfCEs    SEQUENCE OF CxID}

--Master/Slave CM configuration response

MasterSlaveCMconfigurationResponse ::= SEQUENCE {

--Operationg code

operationCode    OperationCode,

--Slave CE ID

slaveCeID    CxID,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType

--Discovery information

discoveryInformation   DiscoveryInformation,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN,

--List of available channel number

listOfAvailableChNumbers      ListOfAvailableChNumbers,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

--List of operating channel number

listOfOperatingChNumbers     ListOfOperatingChNumbers,

--Required resource

requiredResource    RequiredResource

--Measurement capability

measurementCapability      MeasurementCapability}

-----------------------------------------------------------

--WSO deregistration

-----------------------------------------------------------

--Deregistration request

WsoDeregistrationRequest ::= SEQUENCE {

--Flag of wso deregistration

wsoDeregistration    BOOLEAN}

--Deregistration response

WsoDeregistrationResponse ::= SEQUENCE {

--Status

status    CxMediaStatus}

END