IEEE P802.19
TV Whitespace Coexistence Method

| Proposed resolutions regarding comment i-42 and i-76 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Gwangzeen Ko | ETRI |  |  |  |
| Myung-Sun Song | ETRI |  |  |  |
| Jae-Ick Choi | ETRI |  |  |  |

Abstract

This document provides proposed resolutions regarding comment i-42 and i-76 received in WG sponsor ballot of Draft P802.19.1/DF3.06.

| Comment # | Page | Subclause | Line # | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| i-42 | 270 | D.2 | 11 | In this use case, the WSO registers to the CM using the CE. How is this done in practice. Does the WSO use Ethernet or USB to make the initial connection to the CE? | The use case should state how an initial connection is made to the CE, when the TVWS channel list is not known. |
| i-76 | 50 | 6.2.2.1 | 36 | "What is a ""being-engagement request""? This occurs multiple places in the text, but is not shown on flowcharts or interaction diagrams." | Describe the term, and use it consistently throughout. Could be less awkward term, too. |

Proposed resolution to comment i-42:

A WSO uses transport SAP operation to get connected to the CE as described in subclause 6.1.1. D.2 describes the WSO registration to CM after connection establishment between the WSO and the CE is done.

Proposed resolution to comment i-76:

“Being engagement request” is used to check whether the connection between the CM and the CDIS is still valid or not. Connection establishment between two entities is described in subclause 6.1.1. Flow chart describes CDIS operation after connection establishment between the CDIS and the CM is done, i.e., after the CDIS entered the engaged state.