IEEE P802.19.1a
Wireless Coexistence

| Profile 1 based CM Operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Ae-Soon Park | ETRI |  |  |  |
| Seung Chan Bang | ETRI |  |  |  |

Abstract

This document provides CM operation for profile 1 on entities operation in order to resolve CID 167 in 1st TG review. Related procedures, messages, primitives, and data types are also represented.

Proposal

The text below is proposed for CM operation for profile 1

    - Profile 1

      - General description

The entity operation for the CM shall consist of the following 6 states:

- Inactive

- Active

- Waiting engagement

- Engaged

- Request sent

- Request received

Each state shall represent as followings:

- Inactive: the state where it is impossible to set up any interfaces with other 802.19.1 entities. The inactive state goes to the active state when initialization process is finished.

- Active: the state where it is possible to set up interface with other 802.19.1 entities. The Active state goes to the inactive state when shutdown process is finished.

- Waiting engagement: the state where one entity has requested for interface setup to the other entity, and is waiting for response from it.

- Engaged: the state where two entities are ready to communicate with each other by exchanging messages or primitives.

- Request sent: the state where the request has been sent from home entity to remote entity, and the home entity is waiting for response from the remote entity.

- Request received: the state where the request has been received from remote entity to home entity, and home entity is waiting for the completion of the request.

State transition shall be occurred as follows:

- The active state goes to the waiting engagement state if CM sends connection request to CDIS/another CM.

- The waiting engagement state goes to the engaged state if CM receives connection response from CDIS/another CM within time limit. If not, it goes back to the active state.

- The engaged state goes to the request received state if CM receives request from CE/CDIS/another CM. After CM sends corresponding response to CE/CDIS/another CM it goes back to the engaged state.

- CM sends corresponding event response to CE/another CM at the engaged state without receiving event request from CE/another CM if CM received the event request from CE/another CM in advance.

- The engaged state goes to the request sent state if CM sends request to CE/CDIS/another CM. After CM receives corresponding response from CE/CDIS/another CM it goes back to the engaged state.

- CM receives corresponding event response from CE/another CM at the engaged state without sending event request to CE/another CM if CM sent the event request to CE/another CM in advance.

- CM stays the engaged state if it periodically sends being-engagement request to CDIS/another CM.

- The engaged state goes to the active state if CM receives disconnection request from CDIS/another CM, or fails to receive being-engagement request from CE/another CM periodically.

- The engaged state goes to the active state if CM sends disconnection request to CDIS/another CM.

Flow chart of CM operation is depicted in Figure v1andFigure v2.

- ・CM operation (part 1).

- ・CM operation (part 2).

After CM authentication is successfully done, a CM shall perform CM subscription procedure and shall switch to the engaged state, where it shall check for incoming messages from CE/CM/CDIS/another CM. Authentication is out of scope of this standard

If the CM received subscription information from a CE, the CM shall perform GCO subscription procedure and GCO registration procedure in sequence, and back to the engaged state.

If the CM received event indication from CE, CM shall perform sending event indication from CE to CM procedure and back to the engaged state.

If the CM received event indication from another CM, CM shall perform sending event indication from CM to another CM procedure and back to the engaged state.

If the CM requires available channel list from GCO, CM shall perform obtaining available channel list from GCO procedure and back to the engaged state.

If the CM received channel classification request from CE, CM shall perform obtaining channel classification information by CE procedure and back to the engaged state.

If channel classification of GCO is updated, CM shall perform announcing channel classification information update to CE procedure and back to the engaged state.

If channel classification of CM is updated, CM shall perform announcing channel classification information update to CM procedure and back to the engaged state.

If the CM requires channel classification of another CM, CM shall perform obtaining channel classification information by CM procedure and back to the engaged state.

If the CM requires measurement result from GCO, CM shall perform requesting measurement procedure and back to the engaged state.

If the CM received measurement result from GCO and the CM requested one-time measurement, CM shall perform obtaining one-time measurement procedure and back to the engaged state.

If the CE received measurement result from GCO and the CM requested scheduled measurement, CM shall perform obtaining scheduled measurement procedure and back to the engaged state.

If the CM is subscribed for discovery service of inter-CM coexistence discovery only, the CM shall execute a coexistence discovery for intra-CM domain and the CM shall perform obtaining coexistence set information. Otherwise, the CM shall perform obtaining coexistence set information without execution of coexistence discovery by itself.

If the GCO subscribed for information service and the CM received coexistence report request from the CE, the CM shall perform obtaining coexistence report procedure and back to the engaged state.

If the GCO subscribed for management service, CM shall perform one of coexistence decision makings; autonomous, centralized, and distributed.

- If CM operates autonomous decision making, CM shall perform its decision making independently from its neighbor CM.

- If CM operates centralized decision making, master CM shall perform both its decision making and slave CM’s decision making.

- If CM operates distributed decision making, CM shall perform its decision making through negotiation among neighbor CMs.

If the CM operates centralized decision making, i.e., master CM or slave CM is required, the CM shall perform master/slave CM selection procedure.

- If master/slave CM selection procedure is succeeded to select the master and the slave CM, the CM shall perform master/slave CM configuration procedure.

- If CM is selected as the slave CM, and received reconfiguration request from the master CM, the CM shall perform sending reconfiguration request from CM to another CM procedure. After that, the CM shall perform GCO reconfiguration procedure and back to the engaged state.

- If CM is selected as the master CM, the CM shall perform coexistence decision making for both GCOs of slave CM and its registered GCOs. After that, the CM shall perform sending reconfiguration request from CM to another CM procedure and shall perform GCO reconfiguration procedure and back to the engaged state.

- If master/slave CM selection procedure is failed to select the master and the slave CM, the CM shall determine it operates distributed decision making or autonomous decision making.

If the CM operates distributed decision making, i.e., negotiation between CMs is required, the CM shall perform negotiation between CMs procedure.

- If negotiation is succeeded, based on results on negotiation, if coexistence decision making for registered GCOs is finished, CM shall perform GCO reconfiguration procedure and back to the engaged state.

- If negotiation is failed, the CM shall determine it operates centralized decision making or autonomous decision making.

If the CM operates autonomous decision making and coexistence decision making for registered GCOs is finished, the CM shall perform GCO reconfiguration procedure and back to the engaged state.

If the CM received reconfiguration failure response from registered GCO, the CM shall perform GCO deregistration procedure and back to the engaged state.

If the CM received updated registration information from CE, CM shall perform GCO registration update procedure and back to the engaged state.

If the CM received subscription update request from CE, CM shall perform GCO subscription update procedure and back to the engaged state.

If the CM requires subscription change of GCO, CM shall perform GCO subscription change procedure and back to the engaged state.

If the CM received deregistration request from CE, a CM shall perform GCO registration update procedure and back to the engaged state.

If the CM requires subscription change for the type of discovery service from the CDIS, CM shall perform CM subscription update procedure and back to the engaged state.

If the CM received new subscription information from new CE, the CM shall perform GCO subscription procedure and GCO registration procedure in sequence, and back to the engaged state.

If there is no CE to serve for coexistence service, the CM shall switch to the active state.

Related procedures, messages, and data types are listed in the following subsections.

      - CM subscription

To subscribe discovery service, a CM shall perform the CM subscription procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the SubscriptionRequest message to CDIS and wait for the SubscriptionResponse message from the CDIS.

When generating the SubscriptionRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionRequest |

Table below shows the parameters in the subscriptionRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | The subscribedService parameter (interCMCoexistenceSetElements or allCoexistenceSetElements) |

Table below shows the parameters of the CxMessage in the SubscriptionResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | BOOLEAN | status |

      - CM subscription update

To do the subscription update, a CM shall perform the CM subscription update procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the SubscriptionRequest message to the CDIS that serves this CM and wait for the SubscriptionResponse message from the CDIS.

When generating the SubscriptionRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionRequest |

Table below shows the parameters in the subscriptionRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | The subscribedService parameter (interCMCoexistenceSetElements or allCoexistenceSetElements) |

Table below shows the parameters of the CxMessage in the SubscriptionResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | BOOLEAN | status |

      - GCO subscription

To subscribe GCO for coexistence service provided by the CM, a CM shall perform the GCO subscription procedure as shown in clause Error: Reference source not found.

After CM has received the SubscriptionRequest message from the CE, the CM shall generate and send the SubscriptionResponse message to the CE.

Table below shows the parameters of the CxMessage in the SubscriptionResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

      - GCO subscription update

To do the subscription update for GCO, a CM shall perform the GCO subscription update procedure as shown in clause Error: Reference source not found.

After CM has received the SubscriptionRequest message from the CE, the CM shall generate and send the SubscriptionResponse message to the CE.

When generating the SubscriptionResponse message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

      - GCO subscription change

To do the subscription change for GCO, a CM shall perform the GCO subscription change procedure as shown in clause Error: Reference source not found. The CM shall perform this procedure when the GCO/CE is not suitable for the current coexistence service from the CM; information service or management service. After the CM initiates this procedure, the GCO/CE shall change its coexistence service subscription from one service to the other service.

First the CM shall generate and send the SubscriptionChangeRequest message to the CE, and wait for the SubscriptionChangeResponse message from the CE.

When generating the SubscriptionChangeRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionChangeRequest |

Table below shows the parameters in the subscriptionChangeRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| newSubscribedService | SubscribedService | Proposed new subscribed coexistence service (management or information). |

Table below shows the parameters of the CxMessage in the SubscriptionChangeResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionChangeResponse |

Table below shows the parameters in the subscriptionChangeResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

      - GCO registration

To register GCO to CDIS, a CM shall perform the GCO registration procedure as shown in clause Error: Reference source not found.

After the CM has sent the RegistrationResponse message to the CE, the CM shall generate and send the CMRegistrationRequest message to the CDIS that serves this CM and wait for the RegistrationResponse message from the CDIS.

When generating the CMRegistrationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmRegistrationRequest |

Table below shows the parameters in the cmRegistrationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| ceID | CxID | CE ID |
| networkID | OCTET STRING | Identifier of the network which the GCO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the GCO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the GCO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the GCO. |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |

Table below shows the parameters of the CxMessage in the RegistrationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | RegistrationResponse |

      - GCO registration update

To do the registration update, a CM shall perform the GCO registration update procedure as shown in clause Error: Reference source not found.

After the CM has sent the RegistrationResponse message to the CE, the CM shall generate and send the CMRegistrationRequest message to the CDIS that serves this CM and wait for the RegistrationResponse message from the CDIS.

When generating the CMRegistrationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmRegistrationRequest |

Table below shows the parameters in the cmRegistrationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| ceID | CxID | CE ID |
| networkID | OCTET STRING | Identifier of the network which the GCO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the GCO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the GCO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the GCO. |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |

Table below shows the parameters of the CxMessage in the RegistrationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | RegistrationResponse |

      - Event indication to CM

To indicate event to CM, a CE shall perform the sending event indication from CE to CM procedure as shown in clause Error: Reference source not found.

After the CM has received the EventIndication message from the CE, the CM shall generate and send the EventConfirm message to the CE that serves the GCO.

Table below shows the parameters of the CxMessage in the EventIndication message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | eventIndication |

Table below shows the parameters in the eventIndication payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| eventParams | EventParams | This parameter contains a list of event parameters. (SINR threshold reached or QoS degradation) |

When generating the EventConfirm message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | eventConfirm |

      - Event indication to another CM

To indicate event to another CM, a CM shall perform the sending event indication from CM to CM procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the EventIndication message to another CM and wait for the EventConfirm message from another CM.

When generating the EventIndication message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | eventIndication |

The CM shall set the parameters of the eventIndication payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| ceID | CxID | CE ID |
| eventParams | EventParams | This parameter contains a list of event parameters. (SINR threshold reached or QoS degradation) |

After another CM has received the eventIndication message from CM, another CM shall generate and send the EventConfirm message to the CM.

      - Available channel list from GCO

To obtain available channel list from GCO, a CM shall perform the obtaining available channel list from GCO procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the AvailableChannelsRequest message to the CE that serves the GCO and wait for the AvailableChannelsResponse message from the CE.

When generating the AvailableChannelsRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | availableChannelsRequest |

Table below shows the parameters of the CxMessage in the AvailableChannelsResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | availableChannelsResponse |

Table below shows the parameters in the availableChannelsResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | List of available channel number from the received AvailableChannelListResponse primitive. |

Table below shows the data type of the each element of the sequence in the listOfAvailableChNumbers parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| channelNumber | INTEGER | Available channel number |
| availableStartTime | GeneralizedTime | Available start time of the available channel number if applicable. |
| availableDuration | REAL | Available duration of the available channel number if applicable. |
| constOfChUses | ConstOfChUses | If constraint is not known, this parameter is not used. If they are know, this parameter is used. |

      - Channel classification request by CE

To provide channel classification information for GCO, the CM shall perform the obtaining channel classification information by CE procedure, as shown in clause Error: Reference source not found.

After the CM has received the ChannelClassificationRequest message from the CE, the CM shall generate and send the ChannelClassificationResponse message to the CE that serves the GCO.

Table below shows the parameters of the CxMessage in the the ChannelClassificationRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationRequest |

Table below shows the parameters of the channelClassificationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | - listOfNetworkID parameter from the - received ChannelClassificationRequest - primitive. |

When generating the ChannelClassificationResponse message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationResponse |

The CM shall set the parameters of the each element of the sequence in the ChannelClassificationResponse payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID. |
| chClassInfo | ChClassInfo | Channel classification information |

Table below shows the data type of the chClassInfo parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| availableChannelList | SEQUENCE OF INTEGER | Available channel list |
| restrictedChannelList | SEQUENCE OF INTEGER | Restricted channel list |
| protectedChannelList | SEQUENCE OF INTEGER | Protected channel list |
| unclassifiedChannelList | SEQUENCE OF INTEGER | Unclassified channel list |
| operatingChannelList | SEQUENCE OF OperatingChannelInfo | Operating channel list |
| coexistenceChannelList | SEQUENCE OF OperatingChannelInfo | Coexistence channel list |

      - Channel classification update to CE

To provide updated channel classification information, the CM shall perform the announcing channel classification information update to CE procedure, as shown in clause Error: Reference source not found.

When channel classification information is updated, the CM shall generate and send the ChannelClassificationAnnouncement message to the CE.

When generating the ChannelClassificationAnnouncement message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationAnnouncement |

Table below shows the parameters of the each element of the sequence in the channelClassificationAnnouncement payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID. |
| chClassInfo | ChClassInfo | Channel classification information |

After the CE has received the ChannelClassificationAnnouncement message from the CM, the CE shall generate and send the ChannelClassificationIndication primitive to the GCO.

The CE shall set the parameters of the each element of the sequence in the ChannelClassificationIndication primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| chClassInfoList | ChClassInfoList | Channel classification information from the received ChannelClassificationAnnouncement message. (see below) |

Table below shows the data type of the chClassInfoList parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID |
| chClassInfo | ChClassInfo | Channel classification information |

      - Channel classification request by CM

To obtain channel classification information from another CM, the CM shall perform the obtaining channel classification information by CM procedure, as shown in clause Error: Reference source not found.

First the CM shall generate and send the CMChannelClassificationRequest message to another CM and wait for the CMChannelClassificationResponse message from another CM.

When generating the CMChannelClassificationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmChannelClassificationRequest |

Table below shows the parameters of the cmChannelClassificationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | - List of network ID |

Table below shows the parameters of the CxMessage in the CMChannelClassificationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmChannelClassificationResponse |

Table below shows the parameters of cmChannelClassificationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID. |
| chClassInfo | ChClassInfo | Channel classification information |

      - Channel classification update to another CM

To provide updated channel classification information to another CM, the CM shall perform the announcing channel classification information update to CM procedure, as shown in clause Error: Reference source not found.

When channel classification information is updated, the CM has sent the ChannelClassificationAnnouncement message to another CM.

When generating the ChannelClassificationAnnouncement message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationAnnouncement |

Table below shows the parameters of the each element of the sequence in the channelClassificationAnnouncement payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID. |
| chClassInfo | ChClassInfo | Channel classification information |

      - Measurement request

To request measurement to the GCO, the CM shall perform the requesting measurements procedure, as shown in clause Error: Reference source not found.

First the CM shall generate and send the MeasurementRequest message to CE and wait for the MeasurementConfirm message from CE.

When generating the MeasurementRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementRequest |

Table below shows the parameters of the MeasurementRequest message.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementDescription | MeasurementDescription | Measurement description. |

Table below shows the parameters of the CxMessage in the MeasurementConfirm message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementConfirm |

      - One-time measurement

To provide one-time measurement results from GCO to CM, a CE shall perform the providing one-time measurement procedure, as shown in clause Error: Reference source not found.

After the CM has received the MeasurementResponse message from the CE, the CM shall generate and send the MeasurementConfirm message to the CE.

When generating the MeasurementResponse message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementResponse |

The CM shall set the parameters of the measurementResponse payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementResult | MeasurementResult | Measurement results received from the received MeasurementResult primitive. |

When generating the MeasurementConfirm message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementConfirm |

      - Scheduled measurement

To provide scheduled measurement results from GCO to CM, a CE shall perform the providing scheduled measurements procedure, as shown in clause Error: Reference source not found.

After the CM has received the MeasurementResponse message from the CE, the CM shall generate and send the MeasurementConfirm message to the CE.

When generating the MeasurementResponse message, the CE shall set the parameters of the each element of the sequence in the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementResponse |

Table below shows the parameters of the each element of the sequence in the measurementResponse payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementResult | MeasurementResult | Measurement results received from the received MeasurementResult primitive. |

When generating the MeasurementConfirm message, the CM shall set the parameters of the each element of the sequence in the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementConfirm |

      - Information from another CM

To get the information from another CM, a CM shall perform the obtaining information from another CM procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the InfoAcquringRequest message to another CM and wait for the InfoAcquringResponse message from another CM.

When generating the InfoAcquringRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | infoAcquringRequest |

Table below shows the parameters in the infoAcquringRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| ceID | CxID | CE ID |
| listOfReqInfoDescr | SEQUENCE OF ReqInfoDescr | List of requested information description. (SINR, desired bandwidth, desired occupancy, desired QoS, interference level, fairness index, fairness threshold, subscribed service) |

Table below shows the parameters of the CxMessage in the InfoAcquringResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | infoAcquringResponse |

Table below shows the parameters in the infoAcquringResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| ceID | CxID | CE ID |
| reqInfoValue | ReqInfoValue | Requested information value. |

      - Coexistence report for information service

To provide coexistence information to GCO, a CM shall perform the obtaining coexistence report procedure as shown in clause Error: Reference source not found.

After the CM has received the CoexistenceReportRequest message from CE, the CM shall generate and send the CoexistenceReportResponse message to the CE.

When generating the CoexistenceReportRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceReportRequest |

When generating the CoexistenceReportResponse message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceReportResponse |

Table below shows the parameters in the coexistenceReportResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| coexistenceReport | CoexistenceReport | Information about coexistence set elements of the GCO in form of a coexistence report |
| channelPriority | ChannelPriority | Channel priority. |

Table below shows the data type of the each element of the sequence in the CoexistenceReport parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Neighbor network ID. |
| networkTechnology | NetworkTechnology | This parameter shall be set to the neighbor GCO network technology. |
| listOfOperatingChNumbers | ListOfOperatingChNumbers | The neighbor operating channel numbers. |

Table below shows the data type of the each element of the sequence in the ChannelPriority parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| channelNumber | INTEGER | Channel number. |
| priority | INTEGER | Channel priority order. |

      - Coexistence set information

To obtain coexistence set information from CDIS, a CM shall perform the obtaining coexistence set information procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the CoexistenceSetInformationRequest message to the CDIS, and wait for the CoexistnceSetInformationResponse message from the CDIS.

When generating the CoexistenceSetInformationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceSetInformationRequest |

Table below shows the parameters in the coexistenceSetInformationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | Network ID list of GCO for which coexistence set information is requested |

Table below shows the parameters of the CxMessage in the CoexistenceSetInformationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceSetInformationResponse |

Table below shows the parameters of the each element of the sequence in the coexistenceSetInformationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | GCO for which coexistence set information is requested |
| listOfneighborCM | ListOfNeighborCM | Indicates the list of neighbor CM (see below) |

Table below shows the data type of the each element of the sequence in the listOfneighborCM parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| neighorCMID | CxID | Neighbor CM ID |
| listOfCoexSetElement | ListOfCoexSetElement | Indicates the list of neighbor GCO (see below) |

Table below shows the data type of the each element of the sequence in the listOfCoexSetElement parameter.

| networkID | OCTET STRING | Neighbor network ID. |
| --- | --- | --- |
| networkTechnology | NetworkTechnology | Neighbor GCO network technology. |

      - Negotiation between CMs

To negotiate with other CMs for coexistence decision making, a CM shall perform the negotiation between CMs procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the NegotiationRequest message to another CM and wait for the NegotiationRequest message from another CM. Both etiquette mode and round-robin mode of negotiation as shown in reference shall be covered by the NegotiationRequest message.

If Competition mode of negotiation negotiation as shown in reference is needed, the CM shall generate and send NegotiationAnnouncement message to another CM.

When generating the NegotiationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | negotiationRequest |

Table below shows the parameters in the negotiationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| negotiationStatus | NegotiationStatus | Negotiation status (success, failure, under negotiation) |
| negotiationInformation | NegotiationInformation | Negotiation information |

Table below shows the data type of the NegotiationStatus.

| Parameter | Data type | Description |
| --- | --- | --- |
| neogtiationSuccess | Boolean | Negotiation success |
| neogtiationFailure | Boolean | Negotiation failure |
| underNeogtiation | Boolean | Under negotiation |

Table below shows the data type of the NegotiationInformation.

| Parameter | Data type | Description |
| --- | --- | --- |
| mode | Boolean | Negotiation success |
| listOfChNumber | SEQUENCE OF INTEGER | Negotiation failure |
| timeSharingUnitInfo | TimeSharingUnitInfo | Under negotiation |
| slotTimePosition | StartEndTime | Allowed slot time position |
| numberOfSlots | INTEGER | The number of slots |
| DisallowedSlotTimePosition | StartEedTime | Disallowed slot time position |
| listOfContentionNumbers | SEQUENCE OF REAL | List of contention number |

When generating the NegotiationAnnouncement message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | negotiationAnnouncement |

Table below shows the parameters in the negotiationAnnouncement payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfWinnerCMID | ListOfWinnerCMID | List of winner CM ID |
| listOfSlotTimePosition | ListOfSlotTimePosition | List of slot time position |

      - GCO reconfiguration for management service

To request GCO reconfiguration, a CM shall perform the GCO reconfiguration procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the ReconfigurationRequest message to CE and wait for the ReconfigurationResponse message from CE.

When generating the ReconfigurationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | reconfigurationRequest |

Table below shows the parameters of the each element of the sequence in the reconfigurationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| OperatingChNumbers | SEQUENCE OF INTEGER | Operating channel list for the GCO. |
| txPowerLimit | REAL | Transmission power limitation |
| channelIsShared | BOOLEAN | TRUE, if the frequency will be shared with other GCOs, FALSE otherwise. |
| txSchedule | txSchedule | Transmission schedule |
| chClassInfo | ChClassInfo | Channel classification information |

Table below shows the data type of the chClassInfo parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| availableChannelList | SEQUENCE OF INTEGER | Available channel list |
| restrictedChannelList | SEQUENCE OF INTEGER | Restricted channel list |
| protectedChannelList | SEQUENCE OF INTEGER | Protected channel list |
| unclassifiedChannelList | SEQUENCE OF INTEGER | Unclassified channel list |
| operatingChannelList | SEQUENCE OF OperatingChannelInfo | Operating channel list |
| coexistenceChannelList | SEQUENCE OF OperatingChannelInfo | Coexistence channel list |

Table below shows the parameters in the ReconfigurationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | reconfigurationResponse |

Table below shows the parameters in the ReconfigurationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | BOOLEAN | Reconfiguration parameter from the received PerformReconfigurationResponse primitive. |
| failedParameters | FailedParameters | Failed parameters from the received PerformReconfigurationResponse primitive. |

      - Sending reconfiguration request from CM to another CM

To request GCO reconfiguration registered to another CM, a CM shall perform the sending reconfiguration request from CM to another CM procedure as shown in clause Error: Reference source not found.

First the CM shall generate and send the CMReconfigurationRequest message to another CM and wait for the CMReconfigurationResponse message from another CM.

When generating the CMReconfigurationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmReconfigurationRequest |

Table below shows the parameters of the each element of the sequence in the cmRreconfigurationRequest payload. The CM shall generate this payload for each CE that is registered to another CM. The total number of payload shall be equal to the number of CEs that is registered to another CM but managed by the CM.

| Parameter | Data type | Description |
| --- | --- | --- |
| reconfigTarget | CxID | Indication for CE to be reconfigured |
| OperatingChNumbers | SEQUENCE OF INTEGER | Operating channel list for the GCO. |
| txPowerLimit | REAL | Transmission power limitation |
| channelIsShared | BOOLEAN | TRUE, if the frequency will be shared with other GCOs, FALSE otherwise. |
| txSchedule | txSchedule | Transmission schedule |
| chClassInfo | ChClassInfo | Channel classification information |

Table below shows the data type of the chClassInfo parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| availableChannelList | SEQUENCE OF INTEGER | Available channel list |
| restrictedChannelList | SEQUENCE OF INTEGER | Restricted channel list |
| protectedChannelList | SEQUENCE OF INTEGER | Protected channel list |
| unclassifiedChannelList | SEQUENCE OF INTEGER | Unclassified channel list |
| operatingChannelList | SEQUENCE OF OperatingChannelInfo | Operating channel list |
| coexistenceChannelList | SEQUENCE OF OperatingChannelInfo | Coexistence channel list |

Table below shows the parameters in the CMReconfigurationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmReconfigurationResponse |

Table below shows the parameters in the cmReconfigurationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| reconfigTarget | CxID | Indication for CE to be reconfigured |
| status | BOOLEAN | Reconfiguration parameter from the received PerformReconfigurationResponse primitive. |
| failedParameters | FailedParameters | Failed parameters from the received PerformReconfigurationResponse primitive. |

      - Master/slave CM selection

To select master and slave CM for centralized coexistence decision making, a CM shall perform the master/slave CM selection procedure as shown in clause Error: Reference source not found. The CM intended to become slave CM shall perform this procedure for number of candidate CM for the master CM, one by one. The CM intended to become slave CM shall determine list of GCO/CE that supposed to be managed by the master CM.

First the CM shall generate and send the MasterCMRequest message to another CM informing that the CM intends to be slave CM, and wait for the MasterCMResponse message from another CM.

When generating the MasterCMRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | masterCMRequest |

Table below shows the parameters in the masterCMRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfCEs | SEQUENCE OF CxID | List of CEs managed by CM that intends to become slave CM |

After another CM has received the MasterCMRequest message from the CM, another CM shall generate and send the MasterCMResponse message to the CM.

When generating the MasterCMResponse message, another CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | masterCMResponse |

Table below shows the parameters in the masterCMResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

      - Master/slave CM configuration

To configure master and slave CM for centralized coexistence decision making, a CM shall perform the master/slave CM configuration procedure as shown in clause Error: Reference source not found.

First the master CM shall generate and send the MasterSlaveCMConfigurationRequest message to the slave CM, and wait for the MasterSlaveCMConfigurationResponse message from the slave CM.

When generating the MasterSlaveCMConfigurationRequest message, the master CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | masterSlaveCMConfigurationRequest |

Table below shows the parameters in the masterSlaveCMConfigurationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfCEs | SEQUENCE OF CxID | List of CEs managed by the master CM |

After the slave CM has received the MasterSlaveCMConfigurationRequest message from the master CM, the slave CM shall generate and send the MasterSlaveCMConfigurationResponse message to the master CM.

When generating the MasterSlaveCMConfigurationResponse message, the slave CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | masterSlaveCMConfigurationResponse |

Table below shows the parameters in the masterSlaveCMConfigurationResponse payload. The slave CM shall generate this payload for each CE managed by the master CM. The total number of payload shall be equal to the number of CEs that is registered to the slave CM but managed by the master CM.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| slaveCeID | CxID | Slave CE ID |
| networkID | OCTET STRING | Identifier of the network which the GCO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the GCO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the GCO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the GCO. |
| txScheduleSupported | BOOLEAN | Indicates whether scheduled transmission is supported or not |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | Information about available channels (see table below). |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |
| listOfOperatingChNumbers | ListOfOperatingChNumbers | The GCO operating channel numbers (see table below). |
| requiredResource | RequiredResource | Resource required for the GCO operation (see table below). |
| measurementCapability | MeasurementCapability | Measurement capability of the GCO (Energy detection or feature detection) |

Table below shows the data type of the each element of the sequence in the listOfAvailableChNumbers parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| channelNumber | INTEGER | Available channel number |
| availableStartTime | GeneralizedTime | Available start time of the available channel number if applicable. |
| availableDuration | REAL | Available duration of the available channel number if applicable. |
| constOfChUses | ConstOfChUses | If constraint is not known, this parameter is not used. If they are know, this parameter is used. |

Table below shows the data type of the each element of the sequence in the listOfOperatingChNumbers parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| ChannelNumber | INTEGER | operating channel number |
| occupancy | REAL | If occupancy is not known, this parameter is not used. If it is known, this parameter is set to its value. Range is from 0 to 1. |

Table below shows the data type of the requiredResource parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| requiredBandwidth | REAL | Required bandwidth for the GCO operation. |
| occupancy | REAL | If expected occupancy is not known, this parameter is not used. If it is known, this parameter is set to its value. Range is from 0 to 1. |

      - GCO deregistration

To deregister GCO from CM, a CM shall perform the GCO deregistration procedure as shown in clause Error: Reference source not found. The CM shall perform this procedure when the GCO/CE subscribed for the management service not accept reconfiguration request from the CM. After the CM completes this procedure, the GCO/CE shall perform one of the followings;

- To switch its coexistence service from the management service to the information service

- To subscribe its coexistence service to new CM and register to the CM

First the CM shall generate and send the DeregistrationRequest message to the CE, and wait for the DeregistrationResponse message from the CE.

When generating the DeregistrationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | deregistrationRequest |

Table below shows the parameters in the PerformDeregistrationRequest primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| GCODeregistration | BOOLEAN | TRUE |

Table below shows the parameters of the CxMessage in the DeregistrationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | deregistrationResponse |

Table below shows the parameters in the deregistrationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

After the CM has received the DeregistrationResponse message from the CE, the CM shall generate and send the CMRegistrationRequest message to the CDIS that serves this CM and wait for the RegistrationResponse message from the CDIS.

When generating the CMRegistrationRequest message, the CM shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmRegistrationRequest |

Table below shows the parameters in the cmRegistrationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) is selected as remove |
| ceID | CxID | CE ID |
| networkID | OCTET STRING | This parameter is not used. |
| networkTechnology | NetworkTechnology | This parameter is not used. |
| networkType | NetworkType | This parameter is not used. |
| discoveryInformation | DiscoveryInformation | This parameter is not used. |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | This parameter is not used. |

Table below shows the parameters of the CxMessage in the RegistrationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| ayload | CxPayload | RegistrationResponse |