IEEE P802.19.1a
Wireless Coexistence

| Profile 1 based CDIS Operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Ae-Soon Park | ETRI |  |  |  |
| Seung Chan Bang | ETRI |  |  |  |

Abstract

This document provides CDIS operation for profile 1 on entities operation in order to resolve CID 169 in 1st TG review. Related procedures, messages and data types are also represented.

Proposal

The text below is proposed for CDIS operation for profile 1

    - Profile 1

      - General description

The entity operation for the CDIS consists of the following 5 states:

- Inactive

- Active

- Engaged

- Request sent

- Request received

Each state represents as followings:

- Inactive: the state where it is impossible to set up any interfaces with other 802.19.1 entities. The inactive state goes to the active state when initialization process is finished.

- Active: the state where it is possible to set up interface with other 802.19.1 entities. The Active state goes to the inactive state when shutdown process is finished.

- Engaged: the state where two entities are ready to communicate with each other by exchanging messages.

- Request sent: the state where the request has been sent from home entity to remote entity, and the home entity is waiting for response from the remote entity.

- Request received: the state where the request has been received from remote entity to home entity, and home entity is waiting for the completion of the request.

State transition shall be occurred as follows:

- The active state goes to the engaged state if CDIS receives connection request from CM and sends connection response to CM. Connection request and response are for connection establishement between two entities as described in subclause Error: Reference source not found.

- The engaged state goes to the request sent state if CDIS sends request to CM. After CDIS receives corresponding response from CM it goes back to the engaged state.

- CDIS receives corresponding event response from CM at the engaged state without sending event request to CM if CDIS sent the event request to CM in advance.

- The engaged state goes to the request received state if CDIS receives request from CM. After CDIS sends corresponding response to CM it goes back to the engaged state.

- CDIS sends corresponding event response to CM at the engaged state without receiving event request from CDIS if CDIS received the event request from CM in advance.

- CDIS stays the engaged state if it periodically receives being-engagement request from CM.

- The engaged state goes to the active state if CDIS receives disconnection request from CM, or fails to receive being-engagement request from CM periodically.

- The engaged state goes to the active state if CDIS sends disconnection request to CM.

Flow chart of CDIS operation is shown inFigure v1..

- ・CDIS operation.

After CM authentication is successfully done, the CDIS shall switch to the engaged state, where it shall check for incoming messages from CM. Authentication is out of scope of this standard.

If the CDIS received a subscription request from a CM, a CDIS shall perform CM subscription procedure, and back to the engaged state.

If the CDIS received a registration request from a CM, a CDIS shall perform GCO registration procedure, and back to the engaged state.

After GCO registration, if the CM is subscribed for discovery service of inter-CM coexistence discovery only, the CDIS shall execute a coexistence discovery for inter-CM domain only. Otherwise, the CDIS shall execute a coexistence discovery for both intra-CM and inter-CM domain.

If the CDIS received updated registration information from registered CMs, the CDIS shall perform GCO registration update procedure, and back to the engaged state.  After GCO registration update, the CDIS shall execute coexistence discovery based on the type of discovery service of the CM.

If the CDIS received subscription request from new CM, the CDIS shall perform CM subscription procedure, and back to the engaged state.

If the CDIS received coexistence set information request from the CM, CDIS shall perform obtaining coexistence set information procedure providing coexistence set information based on the type of discovery service the CM subscribed, and back to the engaged state.

If the CDIS received subscription change for the type of discovery service from the CM, CDIS shall perform CM subscription update procedure and back to the engaged state.

If there is no subscribed CM for discovery service, the CDIS shall switch to the active state.

Related procedures, messages, and data types are listed in the following subsections.

      - CM subscription

To subscribe discovery service, a CM shall perform the CM subscription procedure as shown in clause Error: Reference source not found.

After the CDIS received the SubscriptionRequest message from CM, CDIS shall generate and send the SubscriptionResponse message to CM.

Table below shows the parameters of the CxMessage in the SubscriptionRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionRequest |

Table below shows the parameters in the subscriptionRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | The subscribedService parameter (interCMCoexistenceSetElements or allCoexistenceSetElements) |

When generating the SubscriptionResponse message, the CDIS shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

      - CM subscription update

To do the subscription update, a CM shall perform the CM subscription update procedure as shown in clause Error: Reference source not found.

After the CDIS received the SubscriptionRequest message from CM, CDIS shall generate and send the SubscriptionResponse message to CM.

Table below shows the parameters of the CxMessage in the SubscriptionRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionRequest |

Table below shows the parameters in the subscriptionRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | The subscribedService parameter (interCMCoexistenceSetElements or allCoexistenceSetElements) |

When generating the SubscriptionResponse message, the CDIS shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

      - GCO registration

To register GCO to CDIS, a CM shall perform the GCO registration procedure as shown in clause Error: Reference source not found.

After the CDIS received the CMRegistreationRequest message from CM, CDIS shall generate and send the RegistreationResponse message to CM.

Table below shows the parameters of the CxMessage in the CMRegistrationRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmRegistrationRequest |

Table below shows the parameters in the cmRegistrationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| ceID | CxID | CE ID |
| networkID | OCTET STRING | Identifier of the network which the GCO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the GCO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the GCO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the WSO. |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |

When generating the RegistrationResponse message, the CDIS shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | registrationResponse |

      - GCO registration update

To do the registration update, a CM shall perform the GCO registration update procedure as shown in clause Error: Reference source not found.

After CDIS received the CMRegistreationRequest message from CM, CDIS shall generate and send the RegistreationResponse message to CM.

Table below shows the parameters of the CxMessage in the CMRegistrationRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | cmRegistrationRequest |

Table below shows the parameters in the cmRegistrationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| ceID | CxID | CE ID |
| networkID | OCTET STRING | Identifier of the network which the GCO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the GCO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the GCO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the GCO. |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |

When generating the RegistrationResponse message, the CDIS shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | registrationResponse |

      - Coexistence set information

To obtain coexistence set information from CDIS, a CM shall perform the obtaining coexistence set information procedure as shown in clause Error: Reference source not found.

After the CDIS received the CoexistenceSetInformationRequest message from CM, CDIS shall generate and send the CoexistnceSetInformationResponse message to CM.

Table below shows the parameters of the CxMessage in the CoexistenceSetInformationRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceSetInformationRequest |

Table below shows the parameters in the coexistenceSetInformationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | Network ID list of GCO for which coexistence set information is requested |

When generating the CoexistenceSetInformationResponse message, the CDIS shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceSetInformationResponse |

Table below shows the parameters of the each element of the sequence in the coexistenceSetInformationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | GCO for which coexistence set information is requested |
| listOfneighborCM | ListOfNeighborCM | Indicates the list of neighbor CM (see below) |

Table below shows the data type of the each element of the sequence in the listOfneighborCM parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| neighorCMID | CxID | Neighbor CM ID |
| listOfCoexSetElement | ListOfCoexSetElement | Indicates the list of neighbor GCO (see below) |

Table below shows the data type of the each element of the sequence in the listOfCoexSetElement parameter.

| networkID | OCTET STRING | Neighbor network ID. |
| --- | --- | --- |
| networkTechnology | NetworkTechnology | Neighbor GCO network technology. |