IEEE P802.19.1a
Wireless Coexistence

| Messages for profile 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Ae-Soon Park | ETRI |  |  |  |
| Seung Chan Bang | ETRI |  |  |  |

Abstract

This document provides messages for profile 1 in order to resolve CID 172 in 1st TG review.

Proposal

The text below is proposed for normative annex C.2. based on profile 1

Normative Annex Messages Definition

C.2.x Messages for IEEE802.19.1a [profile 1]

IEEE802191aProfile1Message DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-----------------------------------------------------------

--Imported data types

-----------------------------------------------------------

--Imported data types

IMPORTS

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--Network technology

NetworkTechnology,

--Network type

NetworkType,

--Discovery information

DiscoveryInformation,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--Measurement capability

MeasurementCapability,

--List of neighbor CM

ListOfNeighborCM

-- Operation code

OperationCode,

--Transmission schedule

TxSchedule,

--Failed parameters

FailedParameters,

--Subscribed service

SubscribedService,

--Status

CxMediaStatus,

--Coexistence protocol entity ID

CxID,

--Channel classification information

ChClassInfo,

--Required information description

ReqInfoDescr,

- Requested information value

ReqInfoValue,

--Event parameters

EventParams,

-- Negotiation status

NegotiationStatus,

-- Negotiation information

NegotiationInformation

-- Winner CM ID list

ListOfWinnerCMID,

-- Slot time position list

ListOfSlotTimePosition

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult

FROM IEEE802191aProfile1DataType;

-----------------------------------------------------------

--Message structure, header structure, and payload types

-----------------------------------------------------------

--Message structure

CxMessage ::= SEQUENCE {

-- Message header

header CxHeader,

-- Message payload

payload CxPayload

}

--Header

CxHeader ::= CHOICE {

--For announcement

none NULL,

--For request or single response

requestID INTEGER (0..2147483647),

--For multiple responses

multipleResponse SEQUENCE {

--Original request ID

requestID INTEGER (0..2147483647),

--Response number

sequenceNumber INTEGER (0..2147483647),

--True is the response is last

isLastResponse BOOLEAN}

}

--Payload types

CxPayload ::= CHOICE {

--GCO subscription request

subscriptionRequest    SubscriptionRequest,

--GCO subscription response

subscriptionResponse    SubscriptionResponse,

--GCO subscription change request

subscriptionChangeRequest    SubscriptionChangeRequest,

--GCO subscription change response

subscriptionChangeResponse    SubscriptionChangeResponse,

--CE registration request

ceRegistrationRequest    CERegistrationRequest,

--Registration response

registrationResponse    RegistrationResponse,

--CM registration request

cmRegistrationRequest    CMRegistrationRequest,

--Coexistence set information request

coexistenceSetInformationRequest     CoexistenceSetInformationRequest,

--Coexistence set information response

coexistenceSetInformationResponse    CoexistenceSetInformationResponse,

--Coexistence report request

coexistenceReportRequest    CoexistenceReportRequest,

--Coexistence report response

coexistenceReportResponse    CoexistenceReportResponse,

--Reconfiguration request

reconfigurationRequest    ReconfigurationRequest,

--Reconfiguration response

reconfigurationResponse    ReconfigurationResponse,

--CM Reconfiguration request

cmReconfigurationRequest    CMReconfigurationRequest,

--CM Reconfiguration response

cmReconfigurationResponse    CMReconfigurationResponse,

--GCO channel classification request

channelClassificationRequest     ChannelClassificationRequest,

-- GCO channel classification response

channelClassificationResponse   ChannelClassificationResponse,

--CM channel classification request

cmChannelClassificationRequest     CMChannelClassificationRequest,

-- CM channel classification response

cmChannelClassificationResponse   CMChannelClassificationResponse,

-- GCO channel classification update

channelClassificationAnnouncement     ChannelClassificationAnnouncement,

--Available channel list request from GCO

availableChannelsRequest      AvailableChannelsRequest,

-- Available channel list response from GCO

availableChannelsResponse     AvailableChannelsResponse,

--Information acquiring request

infoAcquiringRequest		InforAcquiringRequest,

--Information acquiring response

infoAcquiringResponse		InforAcquiringResponse,

-- Event indication

eventIndication     EventIndication,

-- Event confirm

eventConfirm     EventConfirm,

--GCO measurement request

measurementRequest      MeasurementRequest,

--GCO measurement response

measurementResponse      MeasurementResponse,

--GCO measurement confirm

measurementConfirm      MeasurementConfirm,

--Master/Slave CM request

masterCMRequest      MasterCMRequest,

--Master/Slave CM response

masterCMResponse      MasterCMResponse,

--Master/Slave CM configuration request

masterSlaveCMconfigurationRequest      MasterSlaveCMconfigurationRequest,

--Master/Slave CM configuration response

masterSlaveCMconfigurationResponse      MasterSlaveCMconfigurationResponse,

--Negotiation request

negotiationRequest             NegotiationRequest,

--Negotiation announcement

negotiationAnnouncement        NegotiationAnnouncement,

--Deregistration request

gcoDeregistrationRequest      GcoDeregistrationRequest,

-- Deregistration response

gcoDeregistrationResponse      GcoDeregistrationResponse,

}

-----------------------------------------------------------

--GCO & CM subscription  & update

-----------------------------------------------------------

--Subscription request

SubscriptionRequest ::= SEQUENCE {

--Coexistence service to which GCO is subscribed

subscribedService   SubscribedService

}

-- Subscription response

SubscriptionResponse ::= SEQUENCE {

--Status

status   Status

}

-----------------------------------------------------------

--GCO subscription change

-----------------------------------------------------------

--Request to change subscription

SubscriptionChangeRequest ::= SEQUENCE {

--Coexistence service to which GCO is subscribed

subscribedService   SubscribedService

}

--Response for subscription change

SubscriptionChangeResponse :: = SEQUENCE {

--Status

status   Status

}

-----------------------------------------------------------

--GCO registration & update

-----------------------------------------------------------

--CE registration request

CERegistrationRequest ::= SEQUENCE {

--Operationg code

operationCode    OperationCode,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType,

--Discovery information

discoveryInformation   DiscoveryInformation,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN,

--List of available channel number

listOfAvailableChNumbers      ListOfAvailableChNumbers,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

--List of operating channel number

listOfOperatingChNumbers     ListOfOperatingChNumbers,

--Required resource

requiredResource    RequiredResource,

--Measurement capability

measurementCapability      MeasurementCapability

}

--CM registration request

CMRegistrationRequest ::= SEQUENCE {

--Operationg code

operationCode    OperationCode,

--CE ID

ceID    CxID,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType,

--Discovery information

discoveryInformation   DiscoveryInformation,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

}

-- Registration response

RegistrationResponse ::= SEQUENCE {}

-----------------------------------------------------------

--Coexistence set information

-----------------------------------------------------------

--Request for coexistence set information

CoexistenceSetInformationRequest ::= SEQUENCE {

listOfNetworkID      SEQUENCE OF OCTET STRING

}

--Response for coexistence set information

CoexistenceSetInformationResponse ::= SEQUENCE {

--Network ID

coexistenceReport     CoexistenceReport,

--List of neighbor CM

listOfneighborCM      ListOfneighborCM

}

-----------------------------------------------------------

--Coexistence report

-----------------------------------------------------------

--Request for coexistence report

CoexistenceReportRequest ::= SEQUENCE {}

--Response for coexistence report

CoexistenceReportResponse ::= SEQUENCE {

--Coexistence report information

coexistenceReport     CoexistenceReport,

--Channel priority information

channelPriority     ChannelPriority

}

-----------------------------------------------------------

--GCO reconfiguration

-----------------------------------------------------------

--Reconfiguration request

ReconfigurationRequest ::= SEQUENCE {

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER,

--Transmission power limitation

txPowerLimit    SEQUENCE OF REAL,

--Indication whether the channel is shared

channelIsShared SEQUENCE OF BOOLEAN,

--Transmission schedule

txSchedule    TxSchedule ,

-- Channel classification information

chClassInfo    ChClassInfo

}

--Reconfiguration response

ReconfigurationResponse ::= SEQUENCE {

--Status

status   Status,

--Failed parameters

failedParameters    FailedParameters

}

-----------------------------------------------------------

--GCO reconfiguration for another CM

-----------------------------------------------------------

--Reconfiguration request

CMReconfigurationRequest ::= SEQUENCE {

--Indication for CE to be reconfigured

reconfigTarget    CxID,

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER,

--Transmission power limitation

txPowerLimit    SEQUENCE OF REAL,

--Indication whether the channel is shared

channelIsShared    SEQUENCE OF BOOLEAN,

--Transmission schedule

txSchedule    TxSchedule,

-- Channel classification information

chClassInfo    ChClassInfo

}

--Reconfiguration response

ReconfigurationResponse ::= SEQUENCE {

--Indication for CE to be reconfigured

reconfigTarget    CxID,

--Status

status   Status,

--Failed parameters

failedParameters    FailedParameters

}

-----------------------------------------------------------

--Channel classification

-----------------------------------------------------------

--Channel classification request

ChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID     SEQUENCE OF OCTET STRING

}

-- Channel classification response

ChannelClassificationResponse ::= SEQUENCE  OF SEQUENCE{

-- Network ID

networkID    OCTET STRING

--Channel classification information

chClassInfo      ChClassInfo

}

--CM Channel classification request

CMChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID     SEQUENCE OF OCTET STRING

}

-- CM Channel classification response

CMChannelClassificationResponse ::= SEQUENCE  OF SEQUENCE{

-- Network ID

networkID    OCTET STRING,

--Channel classification information

chClassInfo      ChClassInfo

}

-----------------------------------------------------------

--Channel classification update

-----------------------------------------------------------

--Channel classification update

ChannelClassificationAnnouncement ::= SEQUENCE OF SEQUENCE {

-- Network ID

networkID    OCTET STRING,

--Channel classification information

chClassInfo      ChClassInfo

}

-----------------------------------------------------------

--Information acquiring from another CM

-----------------------------------------------------------

-- Information acquiring request

InfoAcquringRequest ::= SEQUENCE {

ceID 			CxID,

listOfReqInfoDescr 	SEQUENCE OF ReqInfoDescr

}

-- Information acquiring response

InfoAcquringResponse ::= SEQUENCE {

ceID 			CxID,

reqInfoValue	 	ReqInfoValue

}

-----------------------------------------------------------

--Available channel list from GCO

-----------------------------------------------------------

-- Available channel request

AvailableChannelsRequest ::= SEQUENCE {}

-- Available channel response

AvailableChannelsResponse ::= SEQUENCE {

--Available channel list information

listOfAvailableChNumbers      ListOfAvailableChNumbers

}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

-- Event indication

EventIndication ::= SEQUENCE {

-- Event indication information

eventParams	EventParams

}

-- Event confirm

EventConfirm ::= SEQUENCE {}

-----------------------------------------------------------

--Measurement Request

-----------------------------------------------------------

-- Measurement request

MeasurementRequest ::= SEQUENCE {

-- Measurement request information

measurementDescription	MeasurementDescription

}

-----------------------------------------------------------

-- Measurement results

-----------------------------------------------------------

-- Measurement response

MeasurementResponse ::= SEQUENCE OF SEQUENCE{

-- Measurement results

measurementResult	MeasurementResult

}

-----------------------------------------------------------

-- Negotiation

-----------------------------------------------------------

-- Negotiation request

NegotiationRequest ::= SEQUENCE {

-- Negotiation status

negotiationStatus             NegotiationStatus,

-- Negotiation information

negotiationInformation    NegotiationInformation

}

-- Negotiation announcement

NegotiationAnnouncement ::= SEQUENCE {

-- Winner CM ID list

listOfWinnerCMID         ListOfWinnerCMID,

-- Slot time position list

listOfSlotTimePosition    ListOfSlotTimePosition

}

-----------------------------------------------------------

--Master/Slave CM selection

-----------------------------------------------------------

--Master/Slave CM selection request

MasterCMRequest ::= SEQUENCE {

--List of CEs managed by CM that intends to become slave CM

listOfCEs    SEQUENCE OF CxID,

}

-- Master/Slave CM selection response

MasterCMResponse ::= SEQUENCE {

--Status

status Status

}

-----------------------------------------------------------

--Master/Slave CM configuration

-----------------------------------------------------------

--Master/Slave CM configuration request

MasterSlaveCMconfigurationRequest ::= SEQUENCE {

-- List of CEs managed by CM

listOfCEs    SEQUENCE OF CxID,

}

--Master/Slave CM configuration response

MasterSlaveCMconfigurationResponse ::= SEQUENCE {

--Operationg code

operationCode    OperationCode,

--Slave CE ID

slaveCeID    CxID,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType,

--Discovery information

discoveryInformation   DiscoveryInformation,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN,

--List of available channel number

listOfAvailableChNumbers      ListOfAvailableChNumbers,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

--List of operating channel number

listOfOperatingChNumbers     ListOfOperatingChNumbers,

--Required resource

requiredResource    RequiredResource,

--Measurement capability

measurementCapability      MeasurementCapability

}

-----------------------------------------------------------

--GCO deregistration

-----------------------------------------------------------

--Deregistration request

GcoDeregistrationRequest ::= SEQUENCE {

--Flag of gco deregistration

gcoDeregistration    BOOLEAN,

}

--Deregistration response

GcoDeregistrationResponse ::= SEQUENCE {

--Status

status Status

}

END