IEEE P802.19.1a
Wireless Coexistence

| Primitives for profile 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Ae-Soon Park | ETRI |  |  |  |
| Seung Chan Bang | ETRI |  |  |  |

Abstract

This document provides primitives for profile 1 in order to resolve CID 171 in 1st TG review.

Proposal

The text below is proposed for normative annex B.2. based on profile 1

Normative Annex Primitives Definition

B.2.x Primitives for IEEE802.19.1a [profile 1]

IEEE802191aProfile1MEDIASAPPrimitive DEFINITIONS AUTOMATIC TAGS::= BEGIN

-----------------------------------------------------------

--Imported data types

-----------------------------------------------------------

--Imported data types

IMPORTS

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--Network technology

NetworkTechnology,

--Network type

NetworkType,

--Discovery information

DiscoveryInformation,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--Measurement capability

MeasurementCapability,

-- Operation code

OperationCode,

--Transmission schedule

TxSchedule,

--Failed parameters

FailedParameters,

--Subscribed service

SubscribedService,

--Status

CxMediaStatus,

--Coexistence protocol entity ID

CxID,

--Channel classification information list

ChClassInfoList,

--Event parameters

EventParams,

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult,

FROM IEEE802191aProfile1DataType;

-----------------------------------------------------------

--GCO subscription

-----------------------------------------------------------

--Request for subscription information

GetServiceSubscriptionRequest ::= SEQUENCE {}

--Subscription information

GetServiceSubscriptionResponse ::= SEQUENCE {

--Coexistence service to which GCO is subscribed

subscribedService   SubscribedService,

--Status

cxMediaStatus CxMediaStatus

}

--Confirm for subscription

GetServiceSubscriptionConfirm :: = SEQUENCE {

--CxMediaStatus

cxMediaStatus CxMediaStatus

}

-----------------------------------------------------------

--GCO subscription update

-----------------------------------------------------------

--Request to update subscription

NewServiceSubscriptionIndication ::= SEQUENCE {

--Coexistence service to which GCO is subscribed

subscribedService   SubscribedService

}

--Confirm for subscription update

GetServiceSubscriptionConfirm :: = SEQUENCE {

--CxMediaStatus

cxMediaStatus CxMediaStatus

}

-----------------------------------------------------------

--GCO subscription change

-----------------------------------------------------------

--Request to change subscription

ChangeSubscriptionRequest ::= SEQUENCE {

--Coexistence service to which GCO is subscribed

subscribedService   SubscribedService

}

--Response for subscription change

ChangeSubscriptionResponse ::= SEQUENCE {

--CxMediaStatus

cxMediaStatus CxMediaStatus

}

-----------------------------------------------------------

--GCO registration

-----------------------------------------------------------

--Request for registration information

GetRegInfoRequest ::= SEQUENCE {}

--Registration information

GetRegInfoResponse ::= SEQUENCE {

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType,

--Discovery information

discoveryInformation   DiscoveryInformation,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN,

--List of available channel number

listOfAvailableChNumbers      ListOfAvailableChNumbers,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

--List of operating channel number

listOfOperatingChNumbers     ListOfOperatingChNumbers,

--Required resource

requiredResource    RequiredResource,

--Measurement capability

measurementCapability      MeasurementCapability

}

--Confirm for registration

GetRegInfoConfirm :: = SEQUENCE {

--CxMediaStatus

cxMediaStatus CxMediaStatus

}

-----------------------------------------------------------

--GCO registration update

-----------------------------------------------------------

--Updated registration information

NewRegInfoIndication ::= SEQUENCE {

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType,

--Discovery information

discoveryInformation   DiscoveryInformation,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN,

--List of available channel number

listOfAvailableChNumbers      ListOfAvailableChNumbers,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

--List of operating channel number

listOfOperatingChNumbers     ListOfOperatingChNumbers,

--Required resource

requiredResource    RequiredResource,

--Measurement capability

measurementCapability      MeasurementCapability

}

--Confirm for registration

NewRegInfoConfirm :: = SEQUENCE {

--CxMediaStatus

cxMediaStatus CxMediaStatus

}

-----------------------------------------------------------

--Coexistence report for information service

-----------------------------------------------------------

--Request for coexistence report

CoexistenceReportRequest ::= SEQUENCE {}

--Response for coexistence report

CoexistenceReportResponse ::= SEQUENCE {

--Coexistence report information

coexistenceReport     CoexistenceReport,

--Channel priority information

channelPriority     ChannelPriority

}

-----------------------------------------------------------

--GCO reconfiguration for management service

-----------------------------------------------------------

--Reconfiguration request

PerformReconfigurationRequest ::= SEQUENCE {

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER,

--Transmission power limitation

txPowerLimit    SEQUENCE OF REAL,

--Indication whether the channel is shared

channelIsShared    SEQUENCE OF BOOLEAN,

--Transmission schedule

txSchedule    TxSchedule,

-- Channel classification information

chClassInfo    ChClassInfo

}

--Reconfiguration response

PerformReconfigurationResponse ::= SEQUENCE {

--Status

status   Status,

--Failed parameters

failedParameters    FailedParameters

}

-----------------------------------------------------------

--Channel classification requested by CE

-----------------------------------------------------------

--Channel classification request

ChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID     SEQUENCE OF OCTET STRING

}

-- Channel classification response

ChannelClassificationResponse ::= SEQUENCE {

--List of channel classification information

chClassInfoList      ChClassInfoList

}

-----------------------------------------------------------

--Channel classification update

-----------------------------------------------------------

--Channel classification update

ChannelClassificationIndication ::= SEQUENCE {

--List of channel classification information

chClassInfoList      ChClassInfoList

}

-----------------------------------------------------------

--Available channel list from GCO

-----------------------------------------------------------

-- Available channel List request

AvailableChannelListRequest ::= SEQUENCE {}

-- Available channel list response

AvailableChannelListResponse ::= SEQUENCE {

--Available channel list information

listOfAvailableChNumbers      ListOfAvailableChNumbers

}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

-- Event indication

EventIndication ::= SEQUENCE {

-- Event indication information

eventParams	EventParams

}

-----------------------------------------------------------

--Measurement Request

-----------------------------------------------------------

-- Measurement request

GetMeasurementRequest ::= SEQUENCE {

-- Measurement request information

measurementDescription	MeasurementDescription

}

-----------------------------------------------------------

-- Measurement results

-----------------------------------------------------------

-- Measurement response

GetMeasurementResponse ::= SEQUENCE OF SEQUENCE{

-- Measurement results

measurementResult	MeasurementResult

}

-----------------------------------------------------------

-- GCO Deregistration

-----------------------------------------------------------

--Deregistration request

PerformDeregistrationRequest ::= SEQUENCE {

--

Der,

}

-- Deregistration response

PerformReconfigurationResponse ::= SEQUENCE {

--Status

status

}

END