IEEE P802.19.1a
Wireless Coexistence

| A coexistence discovery algorithm for multi-band operating GCOs under inaccurate geo-location information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Ae-Soon Park | ETRI |  |  |  |
| Seung Chan Bang | ETRI |  |  |  |

Abstract

This document provides a coexistence discovery algorithm for multi-band operating GCOs under inaccurate geo-location information in order to resolve CID 165 in 1st TG review.

Proposal

The text below is proposed for a coexistence discovery algorithm for multi-band operating GCOs under inaccurate geo-location information.

- 7.2.1.yy Algorithm for multi-band operating GCOs under geo-location uncertainty

This algorithm provides a method for a hidden master GCO discovery among master GCOs using multi-bands. It is preferentially executed in a channel in which a sub channel exists based on a channel width extension scheme by channel (width) information and a master GCO like AP in WLAN included in a primary channel when the multi-bands are used in each master GCO and slave GCO to enable an inter-adjacent master GCO discovery using the multi-bands to be effectively performed. In addition, this algorithm provides a method in which when the master GCOs using the multi-bands discover each other through direct discovery using a beacon signal or channel detection using an slave GCO positioned in an overlapping area in one band, a discovery of another band is not separately performed and the discovery of another band is performed based on information obtained in one band to minimize overhead of a channel detection time required for a discovery in the multi-bands.

The adjacent master GCOs shall classified into the following four types and when an interference situation occurs, a priority of a target to be preferentially avoided shall be determined.

- Single-band single-channel adjacent master GCO (e.g., priority 4)

- Single-band multi-channel adjacent master GCO (e.g., priority 3

- Multi-band single-channel adjacent master GCO (e.g., priority 2)

- Multi-band multi-channel adjacent master GCO (e.g., priority 1)

Intuitionally, the adjacent master GCO using the multi-bands and the multi-channels shall have the highest priority in the interference avoidance and the adjacent master GCO using the single-band and the single-channel shall have the lowest priority.

It is assumed that one CM and one CE are embeded in each master GCO which is a GCO. A coexisetence discovery algorithm shall include the following steps as depcited Figure 1:

- P#1: CE collects multi-band multi channel scanning results performed by each master GCO and slave GCO

- P#2: CE processes master GCO association performed by each slave GCO associati

- P#3: Each master GCO collects scanning result from associated slave GCO

- BC#1 and P#4: Until all its associated slave GCOs take into consideration, each CM compares scanning results thereof and scanning result of associated slave GCO to perform hidden master GCO discovery and generte/update neighbor master GCO list

- BC#2: If hidden master GCOs are exist,

- P#5: CM selects slave GCO that will transmit MasterGCODiscovery_indication message among slave GCOs associated therewith

- P#6: CM transmits masterGCODiscovery_indication message to selected slave GCO among associated slave GCOs

- P#7: CE of selected slave GCO broadcasts master GCODiscovery_indication message received from associated master GCO so that adjacent master GCO receives MasterGCODiscovery_indication message

- P#8: Cooresponding CM that receives masterGCODiscovery_indication message updates neighbor master GCO list

- BC#2: If hidded master GCOs are not exist,

- P#8: Cooresponding master GCO that receives masterGCODiscovery_indication message updates neighbor master GCO list

igure 1   Flow chart of hidden master GCO disocovery for multi-band operating master GCOs

Figure 2 depicts CM operation for multi-band operating master GCOs hidden AP discoveryahall search the channel in the band of A (i.e., 2.4 GHz, and the like) or B (i.e., 5 GHz, and the like) with respect to the adjacent master GCO based on the beacon signals from one or more adjacent master GCOs and when an operating channel is detected, it shall be determined whether the detected operating channel is the primary channel.

If it is the primary channel, each master GCO shall acquire channel information regarding the primary channel, and shall verify whether the master GCO operating channel exists in the sub channel corresponding to the primary channel according to channel arrangement information based on the acquired channel information, and shall acquire information on the corresponding operating channel which exists in the sub channel of the corresponding adjacent master GCO to store and manage the operating channel information including the acquired primary channel and sub channel information.

Channel detection between adjacent master GCOs shall be preferentially executed in a channel in which the sub channel exists as compared with a channel in which the sub channel does not exist according to a channel arrangement scheme based on information included in the primary channel to more rapidly recognize the interference between the adjacent master GCOs depending on use of the multi-bands and the multi-channels and reflect the recognized interference to the determination of the interference avoidance in real time whenever the interference is recognized.

The operating channel detecting process for the adjacent master GCO is repeated until the operating channel information is acquired with respect to all bands and all channels of the adjacent master GCO.

When acquiring the operating channel information (primary channel and sub channel information) for all bands of the adjacent master GCO which is detected, is completed, each master GCO shall divide the adjacent master GCOs into four adjacent master GCO types of a single-band single-channel adjacent master GCO, a single-band multi-channel adjacent master GCO, a multi-band single-channel adjacent master GCO, and a multi-band multi-channel adjacent master GCO according to whether multi-channels overlap with the adjacent master GCO and whether multi-channels overlap with the adjacent master GCO to store and manage the four adjacent master GCO types.

When a channel scanning process for the adjacent master GCO is completed as described above, the channel scanning process shall be repeated with respect to all other adjacent master GCOs which are detectable to acquire channel scanning information.

Figure 2   Flow chart of CM operation for multi-band operating master GCOs

Figure 3 depicts an example of hidden master GCOs situation for multi-band operating master GCOs. In this case, master GCO1 has hidden neighbors, master GCO2 and master GCO3. Master GCO2 has a hidden neighbor, master GCO1. master GCO3 has a hidden neighbor, master GCO1. After the algorithm is processed, each master GCO shall discover the followings;

- Master GCO1 detects master GCO2 as a single band multi-channel adjacent master GCO

- Master GCO1 also detects master GCO3 as a multi-band multi channel adjacent master GCO

- Master GCO2 detects master GCO1 as a single band mult-channel adjacent master GCO

- Master GCO2 also detects master GCO3 as a multi-band single channel adjacent master GCO

- Master GCO3 detects master GCO1 as a multi-band multi-channel adjacent master GCO

- Master GCO3 also detects master GCO2 as a multi-band single channel adjacent master GCO

Figure 3 An example of hidden master GCO situation for multi-band operating master GCOs