| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER |  |
| Date Submitted | May, 2005 |  |
| Source(s) | Joint Harmonized Contribution |  |
| Re: | 21-05-xxxx-00-0000-One_Proposal_Draft_Text |  |
| Abstract | Draft Text for MEDIA INDEPENDENT HANDOVER Specification |  |
| Purpose | Proposal for 802.21 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

List of Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Mani Mahalingam | Avaya | 1001 Murphy Ranch Road, Milpitas, CA 95035 | 408 321 4840 |  | mmani@avaya.com |
| Michael HogHooghi Jeff Keating Natraj Ekambaram Michael Lee Daniel Orozco-Perez | Freescale | 7700 W. Parmer Lane, Mailstop PL28, Austin, Texas, 78729 | 512-996-5392 | 512-996-7191 | jeff.keating@freescale.com |
| Eric Njedjou Mathieu Peresse | France Telecom | France Telecom R&D Rennes 4 rue du Clos Courtel, BP 91226, 35512 CESSON SEVIGNE, FRANCE | +33 2 99 12 48 78 | +33 2 99 12 40 98 | eric.njedjou@francetelecom.com mperesse.ext@rd.francetelecom.com |
| Yao Zhonghui | Huawei | Banxuegang Industrial Park, Buji Longgang, Shenzhen 518129, P.R.CHINA | +86 755 89650502 | +86 755 89650543 | yaoth@huawei.com |
| Vivek Gupta Prakash Iyer | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503 712 1754 503 264 1815 | 503 264 4230 | vivek.g.gupta@intel.com prakash.iyer@intel.com |
| Alan Carlton Ulises Olvera Juan Carlos Zuniga Marian Rudolf Guang Lu Maged Zaki Akbar Rahman | Interdigital | Two Huntington Quadrangle Third Floor, Melville, NY 11747, USA | 631 622 4338 514 904 6282 514 904 6251 514 904 6258 514 904 6262 514 904 6250 514 904 6270 |  | alan.carlton@interdigital.com ulises.olvera-hernandez@interdigital.com juancarlos.zuniga@interdigital.com marian.rudolf@interdigital.com guang.lu@interdigital.com maged.zaki@interdigital.com akbar.rahman@interdigital.com |
| Padraig Moran Peter Larsson | ipUnplugged | ipUnplugged AB, Arenavägen 23 , SE-121 28 Stockholm Sweden |  |  | padraig@ipunplugged.com |
| Ronny Kim Yongwon Kwak Jin Lee | LG Electronics Inc. | 533, Hogye1-dong, Dongan-Gu, Anyang-shi, Kyoungki-do, Korea, 431-749 | +82-31-450-2945 | +82-31-450-7912 | ronnykim@lge.com yongwon@lge.com jins978@lge.com |
| Peretz Feder Andrea Francini Fang Hao Ray Miller Ajay Rajkumar Sampath Rangarajan Sameer Sharma Yousif Targali | Lucent Technologies | 67 Whippany Rd , Whippany, NJ 07981, USA | 919 932 4369 |  | pfeder@lucent.com francini@lucent.com fangh@lucent.com rbmiller@lucent.com ajayrajkumar@lucent.com sampath@lucent.com svsharma@lucent.com targali@lucent.com |
| Yogesh Bhatt Qiaobing Xie Alistair Buttar Les Eastwood Nat Natarajan Hong Yon Lach Ajoy Singh | Motorola | 600 North U.S. Highway 45, Libertyville, IL 60048 U.S.A | 847-523-0126 | 847 523 5660 | yogeshbhatt@motorola.com |
| Stefano Faccin | Nokia | NH3:400, 6000 Connection Drive, Irving, Texas 75039 | 9728944994 | 972 894 4589 | stefano.faccin@nokia.com |
| Cheng Hong Benjamin Koh | Panasonic | BLK1022 TaiSeng Ave #06-3530, Singapore 534415 | (65)6550 5477 (65)6550 5481 | (65)6550 5459 | hcheng@psl.com.sg benjamin@psl.com.sg |
| Uri Goldstein | PCTEL |  |  |  | uri.goldstein@pctel.com |
| Xiaoyu Liu Youn-Hee Han Sungjin Lee Soohong Park Chihyun Park | Samsung | Mt.14-1,Nongseo-Ri, Giheung-Eup, Yongin-Si,Gyeonggi-Do,Korea 449-712 | +82-31-280-9615 +82-31-280-9585 +82-31-279-5248 +82-31-200-4508 +82-31-280-9657 | +82-31-280-9569 | xiaoyu.liu@samsung.com yh21.han@samsung.com steve.lee@samsung.com soohong.park@samsung.com ch02.park@samsung.com |
| Reijo Salminen | Seesta |  |  |  | reijo.salminen@seesta.com |
| Stephen McCann Eleanor Hepworth | Siemens Roke Manor | Roke Manor, Romsey, Hampshire, SO51 0ZN, UK | +44 1794-833341 | +44 1794 833434 | stephen.mccann@roke.co.uk eleanor.hepworth@roke.co.uk |
| Wolfgang Gröting Stefan Berg Kalyan Koora | Siemens AG | Frankenstr. 2, 46395 Bocholt, Germany | +49 2871 91 2142 | +49 2871 91 3387 | wolfgang.groeting@siemens.com stefan.berg@siemens.com kalyan.koora@siemens.com |
| Yoshiro Ohba Kenichi Taniuchi Raziq Yaqub Victor Fajardo | Toshiba America Research Inc. | P.O. Box 429, Piscataway, NJ 08854-4151 | 732-699-5305 | 732-336-3578 | yohba@tari.toshiba.com |
| Subir Das Farooq Anjum, Ashutosh Dutta Sunil Madhani Tao Zhang | Telcordia Technologies Inc | One Telcordia Drive, RRC-1B229, Piscataway, NJ 08854-4157 | 732 699 2483 | 732 336 7026 | subir@research.telcordia.com |
| Prasad Govindarajan | Varaha | Varaha Systems Inc. 2650, Valley View Ln. Bldg 1, Suite 200, Dallas, TX - 75234 | 972-333-3325 |  | prasad@varaha.com |

1 Overview of Specification	7

1.1 Scope	7

1.2 Purpose	7

1.3 Assumptions	8

1.4 Media Independence	9

1.5 Media Dependence	9

1.6 Interoperability and Compliance	9

2 References	10

2.1 Normative References	10

2.2 Other References	10

3 Definitions	10

4 Acronyms and Abbreviations	13

5 General Architecture	15

5.1 Introduction	15

5.1.1 Service Continuity	15

5.1.2 Application Class	15

5.1.3 Quality of Service (QoS)	15

5.1.4 Network Discovery	15

5.1.5 Network Selection	16

5.1.6 Security	16

5.1.7 Power Management	16

5.1.8 Handovers due to Mobile Terminal Movement	16

5.1.9 Handover Policy	16

5.2 General Design Principles	16

5.3 Media Independent Handover Reference Architecture	17

5.3.1 Logical Network Reference Model	17

5.3.2 Relation between MIH Function in Terminal and MIH Function in Network	18

5.3.3 MIHF- MIHF Communication in the Network	19

5.4 MIH Function Services	19

5.4.1 Media Independent Event Service	20

5.4.2 Media Independent Command Service	20

5.4.3 Media Independent Information Service	21

5.5 MIH Reference Models for Access Networks	22

5.5.1 MIH Reference Model for 802.3	22

5.5.2 MIH Reference Model for 802.11	23

5.5.3 MIH Reference Model for 802.16	24

5.5.4 MIH Reference Model for 3GPP	25

5.5.5 MIH Reference Model for 3GPP2	27

5.6 Service Access Points (SAPs)	28

5.6.1 MIH_MGMT_SAP	29

5.6.2 MIH_SME_SAP	29

5.6.3 MIH_SAP	30

5.6.4 MIH_MAC_SAP	30

5.6.5 MIH_PHY_SAP	30

5.6.6 MIH_LLC_SAP	30

5.6.7 MIH_RRC_SAP	30

5.6.8 MIH_PPP_SAP	30

5.6.9 MIH_LAC_SAP	30

5.7 Media Independence	30

5.7.1 MIH Protocol	30

5.8 Media Dependence	31

5.8.1 802 Amendments	31

5.8.2 3GPP Amendments	31

5.8.3 3GPP2 Amendments	31

5.8.4 Upper layer Amendments	32

6 MIH Function Services	33

6.1 Media Independent Event Service	33

6.1.1 Introduction	33

6.1.2 Types of Events	34

6.1.3 Event Flow Model	35

6.1.4 Event Registration	36

6.1.5 Primitives for Link Layer Events	37

6.1.6 Link Layer Events	37

6.2 Media Independent Command Service	38

6.2.1 Introduction	38

6.2.2 MIH Commands	38

6.2.3 Link Commands	39

6.2.4 Command Service Flow Model	39

6.2.5 Command List	41

6.3 Media Independent Information Service	42

6.3.1 Introduction	42

6.3.2 Information Service Elements	43

6.3.3 Information Service Schema	48

6.3.4 Information Flow	54

7 MIH SAPs and Primitives	56

7.1 Introduction	56

7.2 MIH_MGMT_SAP Primitives	56

7.2.1 Link_Event_Discover.request	56

7.2.2 Link_Event_Discover.confirm	57

7.2.3 Link_Event_Register.request	58

7.2.4 Link_Event_Register.confirm	58

7.2.5 Link_Event_Deregister.request	59

7.2.6 Link_Event_Deregister.confirm	60

7.2.7 Link_Up.indication	61

7.2.8 Link_Down.indication	62

7.2.9 Link_Going_Down.indication	63

7.2.10 Link_Event_Rollback.indication	64

7.2.11 Link_Detected.indication	65

7.2.12 Link_Parameters_Change.indication	66

7.2.13 Link_SDU_Transmit_Success.indication	67

7.2.14 Link_SDU_Transmit_Failure.indication	68

7.2.15 Link_Handoff_Imminent. Indication	69

7.2.16 Link_ Handoff_Proceeding.Indication	69

7.2.17 Link_Handoff_Complete.Indication	70

7.3 MIH_SME_SAP Primitives	71

7.4 MIH_SAP Primitives	72

7.4.1 MIH Capability Discover	72

7.4.2 MIH Event Register	72

7.4.3 MIH Event Deregister	73

7.4.4 MIH Event Configure	73

7.4.5 MIH Events	73

7.4.6 MIH Poll	73

7.4.7 MIH Switch	75

7.4.8 MIH Configure	77

7.4.9 MIH Scan	80

7.4.10 MIH Handover Initiate	81

7.4.11 MIH Handover Prepare	85

7.4.12 MIH_Handover_Complete	87

7.4.13 MIH_Network_Address_Information	88

7.4.14 MIH Information	92

8 Media Independent Handover Protocol	97

8.1 Introduction	97

8.2 MIH Protocol Transport	97

8.2.1 L2 Data Frames Packet Format	97

8.2.2 L2 Management Frame Packet Format	98

8.2.3 L3 Frame Packet Format	98

8.3 Description	98

8.3.1 MIH General Packet Format	99

8.3.2 MIH Function Messages	101

8.4 MIH Function Messages	102

8.4.1 MIH Capability Discovery Messages	102

8.4.2 MIH Remote Registration Messages	103

8.4.3 MIH Event Service Packet Format	104

8.4.4 MIH Command Service Packet Format	106

8.4.5 MIH Information Service Packet Format	107

8.5 Protocol Flow	108

8.5.1 MIH Capability Discovery	108

9 Appendix	109

9.1 Handover Procedures	109

9.1.1 MIP based Handover Sequence Diagrams	109

9.1.2 SIP based Handover Sequence Diagrams	109

9.1.3 MIH Service Selection based Handover Sequence Diagrams	109

9.1.4 Network Initiated Handover Sequence Diagrams	109

9.2 Media Independent Neighbor Graph (MING): An Example	110

9.2.1 Media Independent Neighbor Graph	110

9.2.2 Management of Neighboring Network Information	111

9.2.3 MING_POA Report	112

9.2.4 MING_POA_Report.request	112

9.2.5 MING_POA_Report.confirm	113

9.2.6 MING_POA_Report.indication	114

9.2.7 MING_POA_Report.response	115

9.2.8 Media Independent Neighbor Graph (MING) Query	115

9.2.9 Neighbor Information IE	116

9.3 Schema Representation	117

9.3.1 Extended Schema Representation in RDF/XML: An Example	117

9.3.2 ASN.1 Representation of Extended Information Elements (An Example)	127

9.4 Media specific Amendments	138

9.4.1 802.3 MAC/PHY Amendments	138

9.4.2 802.11 MAC/PHY Amendments	138

9.4.3 802.16 MAC/PHY Amendments	139

9.4.4 Cellular specific Amendments	144

# Overview of Specification

## Scope

The scope of the IEEE 802.21 (Media Independent Handover) standard is to develop a specification that provides link layer intelligence and other related network information to upper layers to optimize handovers between heterogeneous media. This includes links specified by 3GPP, 3GPP2 and both wired and wireless media in the IEEE 802 family of specifications.

## Purpose

The purpose of the IEEE 802.21 standard is to enhance user experience of mobile devices by supporting handovers between heterogeneous networks.

This document describes a proposal to satisfy the requirements as outlined in [14]. The proposal can support handovers for both mobile and stationary users. For the mobile users handovers may occur due to a change in wireless link conditions. Alternatively, handovers may occur due to a gap in radio coverage as a result of terminal movement.  For the stationary user handovers may become imminent when the environment around the user changes making one network more attractive than another. The user may choose an application which requires handover to a higher data rate channel, for example to download a large data file. Handovers should maximize service continuity, such as making a network transition during the pause in a voice call so as to minimize any perceptible interruption in service.

The IEEE802.21 standard supports cooperative use of both mobile terminals and network infrastructure. The mobile terminal is well-placed to detect available networks, and the infrastructure is in a position to store overall network information, such as neighborhood cell lists and the location of mobile devices. In general, both the terminals and the network point of attachments such as base stations or access points can be multimode, i.e. supporting different radio standards, and in some cases being capable of transmission on more than one interface simultaneously.

The network can have both micro cells (IEEE 802.11 or IEEE 802.15 coverage) and macro cells (3GPP, 3GPP2 or IEEE 802.16) and these will in general intersect. The handover process is typically based on measurements and triggers supplied from link layers on the terminal. These measurements may include signal quality measurements, synchronization time differences, transmission error rates, etc. and are some of the metrics used in handover algorithms.

Specifically the proposal covers the following.

- An architecture that enables transparent service continuity while a mobile node (MN) switches between heterogeneous link-layer technologies. The architecture relies on the identification of a mobility-management protocol stack within the network elements that support the handover. The architectural description does not address implementation details and does not provide indication of preferred implementations of the IEEE 802.21 standard.The architecture describes the MIH Reference model for different networks.

- A set of handover-enabling functions within the mobility-management protocol stacks of the network elements and the creation therein of a new entity called the MIH Function. A set of media independent Service Access Points (SAPs) and associated primitives are defined which provide access to the services of MIH Function. The MIH Function provides the following services.

  - Media Independent Event service which provides a set of events and triggers from local as well as remote interfaces.

  - Media independent Command service provides set of commands that enables MIH users to issue commands to control link behavior relevant to handovers.

  - Media Independent Information service which provides information model and an information repository to make more effective handover decisions. The moble terminal accesses information from this repository using it’s current network point of attachments.

- Definition of additional MAC layer SAPs and associated primitives for each specific access technology. These primitives help in collecting link information and controlling link behavior during handovers. These MAC level SAPs will be recommended as amendments to specifications of respective access technology standards.

Lower Layers (L2 and below)

MIH  Function

802.3

802.11

802.16

3GPP

3GPP2

Upper Layers (L3 and above)

SIP

MIPv4

MIPv6

HIP

……

MIH Events

Link Events

Link Commands

MIH Commands

Information Service

Information Service

Figure 1: MIH Function Location and Key Services

Error: Reference source not found shows the placement of the MIH Function within the mobility-management protocol stack for handovers associated with heterogeneous link switches. The MIH Function provides services to the upper layers through a single technology-independent interface (SAP) and obtains services from the lower layers through a variety of technology-dependent interfaces (SAPs).

## Assumptions

The following assumptions have been made in the development of the proposal.

- The mobile node is capable of supporting multiple interfaces, which can be both wireless and wired.

- The MIH Function is a logical entity, whose definition has no implications on the way the MIH functionality is implemented either in the mobile node or in the network.

- The MIH Function on the mobile node continuously receives information about the performance of access networks around the MN. This information typically originates at lower layers of the mobility management protocol stack, within the MN or other network elements.

- When the information originates at a remote network element, the mobile MIH module obtains it through MIH message exchanges with a peer MIH entity that resides in the remote network element.

- When the information originates at a lower layer of the protocol stack within the MN, the mobile MIH module obtains it through local primitives exposed by the service access points (SAPs) that define the interface between the lower layers and the MIH Function.

## Media Independence

The intent of the IEEE802.21 specification is to provide as much generic link layer intelligence as possible without being tied into the features or specifics of particular terminals or radio networks. As such the IEEE 802.21 specification is intended to be a media independent specification applying to different media specified by 3GPP, 3GPP2 and IEEE 802 family of specifications.

## Media Dependence

The IEEE802.21 specification shall define generic SAPs and other primitives that provide generic link layer intelligence. Individual media specific technologies thereafter need to provide corresponding media specific SAPs and primitives which can satisfy these generic abstractions as specified by IEEE 802.21 specification. As such suitable amendments may be required to link layer (MAC/PHY) specifications of different media specific technologies such as IEEE 802.11, IEEE 802.16, 3GPP, 3GPP2, etc. to satisfy the requirements of generic link layer intelligence identified by 802.21. This process may be carried out once the core elements of the IEEE 802.21 specification are developed.

## Interoperability and Compliance

The following compliance clauses shall be observed:

- An implementation must allow the services described in the MIH_SAP to be provided to and accessed by an MIH user. The MIH_SAP service primitives do not specify how they are to be implemented. However the formats and semantics of the service parameters shall be implemented as per 802.21 and are subject to standards compliance.

- The protocol specified in 802.21, including message exchanges, protocol data units, and the state machine, shall be implemented accordingly and is subject to standards compliance. Various classes of interoperability and implementation compliance will be specified based on the set of mandatory and optional features in the specification.

# References

## Normative References

- Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- J. Manner, M. Kojo, “Mobility Related Terminology”, RFC 3753

- Part 16: Air Interface for Fixed Broadband Wireless Access Systems

- Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems  (Amendment for Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands)

- IEEE Std 802.3-2002 Carrier Sense Multiple Access with Collision Detection|(CSMA/CD) Access Method and Physical Layer Specifications

- 3GPP TS 23.002,"Network Architecture."

- 3GPP TS 23.060, "General Packet Radio Service (GPRS); Service Description; Stage 2."

- 3GPP TS 23.234, “3GPP system to Wireless Local Area Network (WLAN) inter-working; System description.”

- 3GPP TR 43.901 “Feasibility Study on Generic Access to A/Gb Interface.”

- W3C Recommendation, “RDF Vocabulary Description Language 1.0: RDF Schema”, http://www.w3.org/TR/rdf-schema/.

- W3C Recommendation, “SPARQL Query Language for RDF”, http://www.w3.org/TR/2004/WD-rdf-sparql-query-20041012/.

- W3C Recommendation, “Resource Description Framework (RDF) – Concepts and Abstract Syntax”, http://www.w3.org/TR/rdf-concepts/.

- W3C Recommendation, “RDF/XML Syntax Specification”, http://www.w3.org/TR/rdf-syntax-grammar/.

## Other References

- 21-04-0087-12-0000 IEEE P802.21 Media Independent Handover Service, Draft Technical Requirements, Sept. 21, 2004. Available at: http://www.ieee802.org/21/sept04_meeting_docs/ 21-04-0087-12-0000-Draft_Technical_Requirements.doc

# Definitions

Handover – The process by which a mobile node obtains the preservation of facilities for supporting traffic flows upon occurrence of a link-switch event. The mechanisms and protocol layers involved in the handover may vary with the type of the link-switch event (i.e., with the type of the serving and target point of attachment and the respective subnet associations). Different types of handover are defined based on the way facilities for supporting traffic flows are preserved:

Hard Handover – Handover where facilities for supporting traffic flows are subject to complete unavailability between their disruption on the serving link and their restoration on the target link (break-before-make).

Soft Handover – Handover where facilities for supporting traffic flows are continuously available while the mobile-node link-layer connection transfers from the serving point of attachment to the target point of attachment. The network allocates transport facilities to the target point of attachment prior to the occurrence of the link-switch event (make-before-break).

Seamless Handover – Handover associated with a link switch between heterogeneous interfaces, where the mobile node either experiences no degradation in service quality, security, and capabilities, or experiences some degradation in service parameters that is mutually acceptable to the mobile subscriber and to the network that serves the newly connected interface.

Handover Policies – A set of rules that contribute to shaping the handover decision for a mobile node.

Link – A communication facility or medium over which network nodes can communicate. Each link is associated with a minimum of two endpoints. Each link endpoint has a unique link-layer identifier.

Link Layer – Conceptual layer of control or processing logic that is responsible for maintaining control of the data link. The data link layer functions provide an interface between the higher-layer logic and the data link.

Link Indication – Information provided by the link layer to higher layers regarding the state of a link.

Link Switch – The process by which a mobile node changes the link that connects it to the network. Changing link implies changing the remote link endpoint and therefore the point of attachment of the mobile node.

Message – The sequence of contiguous octets delivered as a unit by the N-user to the N-SAP and destined for a remote N-user.

Mobile Node – Network node that can change its point of attachment from one link to another at any time. The mobile node can either be in close proximity of or coincide with the mobile terminal.

Mobile Node Association: The term association is used to describe the state where the mobile node is ready to exchange user data (like TCP/UDP packets) with the network point of attachment.

Mobile Terminal – User-facing network node that includes protocol stack layers above the link layer. The mobile terminal can either be in close proximity of or coincide with the mobile node.

N-entity – Abstraction of a functional component of a network element, whose control resides within the N-layer. See also peer N-entity.

N-event – A change of state in an entity whose control resides within the N-layer (N-entity). An N-event can be notified to an N-user through an indication service primitive, or stimulated by an N-user through a request service primitive.

N-layer – A subdivision of the layered protocol architecture, constituted by subsystems of the same rank (N).

N-trigger – N-event notification to an N-user, typically conveyed by an indication service primitive. See also remote N-trigger.

N-user – An (N+1)-layer entity that uses the services of the N-layer and lower layers to communicate with another (N+1)-layer entity.

Network Detection – The process by which a mobile node collects information on its neighboring networks, identifies its current point of attachment, and ascertains the validity of its link-layer configuration.

Network Point of Attachment – This is a location within the network where a mobile node may be aconnected. This point of attachment is associated with a unique link-layer identifier and can be characterized by its address prefix and next hop routing information. The points of attachment involved in a link-switch event are qualified as follows:

Serving Point of Attachment – The point of attachment of the mobile node prior to the link-switch event.

Target Point of Attachment – The point of attachment of the mobile node that results from the link-switch event.

Network Selection – The process by which a mobile node collects policy information on its detected neighboring networks and selects a link (possibly out of many available) to establish network-layer connectivity.

Network Selector – The entity that undertakes Network Selection decision that may lead to a handover.

Peer N-entity – N-entity residing in a network element, defined with reference to its N-layer communication (messaging) with another N-entity in a different network element. See also N-entity.

Protocol Data Unit – The sequence of contiguous octets delivered as a unit by the N-user to the N-SAP and destined for a remote N-user.

Remote N-trigger – Notification to an N-user of an N-event affecting a remote N-entity, typically conveyed by the sequence of an N-PDU transmitted from the remote N-entity to the local N-entity and an indication service primitive passed from the local N-entity to the destination N-user. See also N-trigger.

Service Access Point (SAP) – The point in a protocol stack where the services of a lower layer are available to its next higher layer.

Service Continuity – Transparent maintenance of an active service while the mobile node transitions across wireless or wire-line coverage.

Service Data Unit – The sequence of contiguous octets transmitted as a unit from one N-SAP to another N-SAP.

Service Primitive – Conceptual abstraction describing the information transfer the occurs between N-user and N-layer in the provision of a service. The abstraction resides in the exclusive specification of the service provided rather than the means by which the service is provided.

# Acronyms and Abbreviations

3G				Third Generation

3GPP				3G Partnership Project

3GPP2				3G Partnership Project 2

AAA				Authentication, Authorization, and Accounting

ACK				Acknowledge

AN				Access Network

ANI				Access Network Information

AP				Access Point

AR				Access Router

ASN.1				Abstract Syntax Notation One

BS				Base Station

BSC				Base Station Controller

BSSID				Basic Service Set Identifier

BTS				Base Transceiver Station

ESS				Extended Service Set

CN				Core Network

CS				Convergence Sub-layer

CSME				Convergence Sub-layer Management Entity

FA				Foreign Agent

FMIP				Fast Handover Mobile IP

GGSN				Gateway GPRS Support Node

GMII				Gigabit Media Independent Interface

GMM				GPRS Mobility Management

GPRS				General Packet Radio Service

GSM				Global System for Mobile Communication

HA				Home Agent

HIP				Host Identity Protocol

HLSI				Higher Layer Service Information

HMIP				Hierarchical Mobile IP

IEEE				Institute of Electrical and Electronics Engineers

IETF				Internet Engineering Task Force

ICMP				Internet Control Message Protocol

IP				Internet Protocol

ISP				Internet Service Provider

ITU				International Telecommunications Union

L3MP				Layer 3 Mobility Management Protocol

L1				Physical Layer (PHY)

L2				Medium Access Control (MAC)

LAN				Local Area Network

LLC				Logical Link Control

LLI				Link Layer Information

MAC				Medium Access Control

MDI				Medium dependent Interface

MICS				Media Independent Command Services

MIES				Media Independent Event Services

MII 				Media Independent Interface

MIH				Media Independent Handover

MIHO				Media Independent Handover

MIHF				Media Independent Handover Function

MIHS				Media Independent Handover Services

MIIS				Media Independent Information Service

MIP				Mobile IP

MLME				MAC Layer Management Entity

MMS				Multimédia Message Service

MN				Mobile Node

MS				Mobile Station

MSS				Mobile Subscriber Station

MSDU				Medium Access Control (MAC) Service Data Unit

MSME				MAC Common Part Sub-layer Management Entity

MT				Mobile Terminal

NAK				Negative Acknowledge

NMS				Network management system

PCS				Physical Coding Sublayer

PHY				Physical Layer

PLMN				Public Land Mobile Network

PLME				PHY Layer Management Entity

PMA				Physical Medium Attachment

PMD				Physical Medium Dependent

PoA				Point of Attachment

QoS				Quality of Service

RFC				Request for Comment

RLC				Radio Link Control

RNC				Radio Network Controller

RRC				Radio Resource Control

SAP				Service Access Point

SDU				Service Data Unit

SGSN				Serving GPRS Support Node

SM				Session Management

SME				Station Management Entity

SNDCP				Sub-Network  Data Convergence Protocol

SNR				Signal Noise Ratio

SS				Subscriber Station

STA				Station

TCP				Transmission Control Protocol

TLV				Type-Length-Value message

UDP				User Datagram Protocol

UMTS				Universal Mobile Telecommunications System

VPN				Virtual Private Network

WLAN				Wireless Local Area Network

WPAN				Wireless Personal Area Network

WMAN				Wireless Metropolitan Area Network

XML				Extensible Mark-up Language

# General Architecture

## Introduction

The IEEE 802.21 specification is intended to facilitate a variety of handover methods. Such methods are generally classified as 'hard' or 'soft', depending on whether the handover procedure is “break before make” or “make before break” with respect to the data transport facilities that support the exchange of data packets between the mobile node and the network.

In general handover involves cooperative use of both mobile terminal and network infrastructure in order to satisfy network operator and end user needs. Handover control, handover policies and other algorithms involved in handover decision making are generally handled by communication system elements which do not fall within the scope of the IEEE 802.21 standard. However it is beneficial to describe certain aspects of the overall handover procedure so that the role and purpose of MIH Event Service, MIH Command Service, MIH Information Service and MIH Function in overall handover process is clear.

The following subsections give an overview of how the different factors that may affect handovers are addressed within the current proposal.

### Service Continuity

Handovers may occur either between two different access networks or between two different points of attachment of a single access network. In such cases Service continuity is defined as the continuation of the service during and after the handover while minimizing aspects such as data loss and break time during the handover without requiring any user intervention. The change of access network may or may not be noticeable to the end user, but there should be no need for the user to re-establish the service. There may be a change in service quality as a consequence of the transition between different networks due to the varying capabilities and characteristics of the access networks. For example if the QoS supported by new access network is unacceptable, higher layer entities may decide not to handover or may terminate the current session after the handover based on applicable policies. This specification specifies essential elements which enable service continuity.

### Application Class

Various applications have different tolerance characteristic for delay and data loss. Application aware handover decisions can be enabled by making a provision for such characteristics. For example when a network transition due to impending handover is made during the pause phase of conversation in an active voice call, the perceptible interruption in service is minimized.

### Quality of Service (QoS)

QoS is an important factor to consider in the handover decision making. The preference is to select the network with the most appropriate QoS support level for handover in an effort to maintain the level of user experience. Hence to enable seamless handover operations, it is necessary that the 802.21 standard specify the means by which QoS information can be obtained for each of the supported access networks and be made available to the upper layers involved in handover decision making.

### Network Discovery

Network discovery is essential to provide new possibilities for network selection to suit the need of the applications and mobility. Thus it is necessary that 802.21 defines the network information and specifies the means by which such information can be obtained for supported access networks and made available to the upper layers. The network information could include information about link type, link identifier, link availability, link quality, etc.

### Network Selection

Network selection is the continuous process of selecting the most appropriate network for any user operation at any given time. The selection can be based on various criteria such as required QoS, cost, user preferences, policies, etc. If the selected network is not the currently used network, then a handover to the preferred network may be required. The 802.21 standard may specify the means for such information to be made available to the upper layers to enable effective network selection.

### Security

Events, commands and information messages carried between a MT (Mobile Terminal) and a network PoA (Point of Attachment) cannot be secured until the MT is securely associated with the network PoA. This association can be achieved either via lower or higher layers security mechanisms. Once such a secure association has been established between the MT and the network PoA, any messages exchanged between two MIH Function entities should retain integrity and be replay protected over a secure transport. Otherwise the exchanged MIH messages are prone to integrity, replay and man-in-the-middle attacks.  The 802.21 standard may specify the means for security information to be made available to the upper layers to setup secure connections.

### Power Management

This specification provides for information that helps to preserve battery life. For example efficient 'sleep modes' can be managed based on real time link status, efficient scanning is achieved using neighbor maps of different networks and readily available reports of optimum link layer parameters.

### Handovers due to Mobile Terminal Movement

Handovers due to the mobile station speeds relative to the base station or access point are facilitated by providing real time link conditions and timely information about overlay micro-cells and macro-cells.

### Handover Policy

The primary role of the MIH Function is to facilitate handoffs and provide intelligence to the network selector entiity or the mobility management entity responsible for handover decision as described by other standards or proprietary implementations. The MIH Function aids the network selector entity with the help of Event service, Command service and Information service. The network selector entity and the handover policies that control handovers are outside the scope of MIH Function. Description of specific handover policies and the details of network selector entity are outside the scope of 802.21 standards as well.

The IEEE 802.21 specification defines services that enhance handovers between heterogeneous access links. This is achieved through facilitating handover process by providing link layer intelligence relevant in handover detection, handover initiation and candidate link selection by MIH user.

- A Media Independent Event Service (MIES) which provides event classification, event filtering and event reporting corresponding to dynamic changes in link characteristics, link status, and link quality.

- A Media independent Command Service (MICS) which enables MIH user to manage and control link behavior relevant to handovers and mobility.

- Media Independent Information Service (MIIS) which provides details on the charateristics and services provided by the serving and surrounding networks. The information enables effective system access and effective handover decisions.

- The above services are supported by the Media Independent Handover Function (MIHF) to facilitate a MIH user in mobility management and handover process. The MIH Function provides convergence of link-layer state information from multiple heterogeneous access technologies into a unified presentation to the upper layers of the mobility-management protocol stack.

## General Design Principles

The 802.21 standard is based on the following general design principles.

- The Media Independent Handover (MIH) Function is logically defined as a shim layer in the mobility-management protocol stack of both the mobile node and the network elements that provide mobility support. MIH is a helper and facilitator function which helps in handover decision making. Upper layers make handover decisions and link selection based on inputs and context from MIH. Facilitating the recognition that a handover should take place is one of the key goals of MIH Function. Discovery of information on how to make effective handover decisions is also a key component.

- MIH Function provides abstracted services to higher layers. From that perspective MIH offers a unified interface to the upper layers. The service primitives exposed by this unified interface are independent of the technology specific protocol entities of the different access networks. The MIH Function communicates with the lower layers of the mobility-management protocol stack through technology-specific interfaces. The specification of the MIH interfaces with the lower layers generally does not fall within the scope of this standard. Such interfaces are already specified as service access points (SAPs) within the standards that pertain to the respective access technologies, such as IEEE 802.1, IEEE 802.3, IEEE 802.11, IEEE 802.16, 3GPP and 3GPP2. This standard may contain recommendations to amend the existing access technology specific standards when modifications of the lower-layer interfaces may enable or enhance MIH functionality.

- Handover signaling (as part of handover execution and subsequent updates) may not be part of the standard. Different networks support different intra-roaming handover mechanisms (mobile initiated, network initiated, etc.). Handover initiation trigger may be useful in heterogeneous handovers when not done as per the homogeneous scheme.

- MIH Function may do further processing on MAC/PHY triggers and other related events. Definition of this processing is outside the scope of the standard. The standard shall provide support for Remote Events as well. Some of these events may encapsulate messages related to network selection (such as Link Going Down that may be exchanged between different MIH entities. Events are advisory in nature. The decision to cause a handover or not based on these events is outside the scope of the standard.

- The standard shall specify mechanisms to support both network-initiated and MN-initiated handovers.

- The standard should support transparent inter-working with legacy equipment. Thus 802.21compatible equipment should be able to co-exist with legacy non-802.21 compliant equipment.

## Media Independent Handover Reference Architecture

The following sections describe the key salient points with regards to communication between different MIH entities in the client device and the network.

### Logical Network Reference Model

The network reference model shown in Figure 2 includes an MIH-capable mobile node (or UE, for User Equipment) that supports multiple access-technology options. Each access technology either advertises its MIH capability or responds to MIH terminal queries.

On the network side, the location of the MIH-capable nodes is not deterministic and may vary even within the same access technology. The MIH functionality may reside in the network point of attachment such as an AP or BS, but alternatively even deeper inside the access or core portions of the technology-specific network. Event-service triggers can originate at both sides of an active link. However, the UE is typically the first node to react to link-state change events. The overall mobility management architecture may include Mobile IP infrastructure (client, FA, HA, H-AAA), SIP mobility infrastructure (client, SIP servers) or reflect other mobility schemes.

Figure 2: Logical Network Reference Model

### Relation between MIH Function in Terminal and MIH Function in Network

- The relation between different peer MIH Function entities depends on the scenario and the MIH service under consideration. For MICS and MIES, the MIH Function in the terminal communicates with one and only one MIH Function for a single access network. This MIH Function in the access network may be termed as the front end MIH Function for that access network. This is true for both L2 and L3 mode of transport when the mobile terminal may be connected through one single access network. The front-end MIH Function may communicate with other MIH Functions in the network to provide CS and ES. However this communication is not visible to the MIH Function within the terminal. MIH Function in the terminal talks to one and only one MIH Function in the network for each of the given access networks. The MIH Function in the network may talk to other MIH Functions in the network.

In case of multiple active links supported through multiple access networks, the MIH Function in the terminal may communicate with one MIH Function for each access network (or a single MIH Function when applicable, e.g. L3 transport in tight coupling scenarios where one MIH Function serves multiple accesses in a domain).

### MIHF- MIHF Communication in the Network

The communication between MIH Functional entities on the network side also depends on where the MIH Function entities reside and the type of scenario. When MIH Function is in the access network (e.g. MIHF logically located at AP or BS), L2 or L3 transport can be used over access. For network controlled or coordinated handovers (i.e. cases where network decides where/when to handover), the source MIHF can send messages to destination MIHF to inform of imminent handovers to allow the destination MIHF to begin “preparation”. Examples of such preparation may include checking availability of appropriate resources including acceptable QoS levels at new PoA, procuring an IP address for the mobile station, etc.

Also possible are scenarios where the MIH Function (e.g. in AP) communicates with MIH Function e.g. in an AR. This type of communication includes events such as Link Up, Link Down, etc. This is in line with the architectural view described in the various Media Independent Handover Reference Models. Thus the MIH Command Service, Information Service and Event Service messages can be exchanged remotely only between two MIH functions. This following section describes the impact of IEEE 802.21 on reference model of different media specific technologies.

## MIH Function Services

The MIH Function provides asynchronous and synchronous services through well defined SAPs for lower layers and upper layers. In the case of a system with multiple network interfaces of arbitrary type, the L3MP can use the Event service, Command service and Information service provided by MIH to manage, determine, and control the state of the underlying interfaces.

These services provided by MIH help the L3MP and other protocols in maintaining service continuity, service adaptation to varying quality of service, battery life conservation, and network discovery and link selection. In a system containing heterogeneous network interfaces of 802 types and cellular 3GPP, 3GPP2 types, the Media Independent Handover Function can help the L3MP to implement effective procedures to couple services across heterogeneous network interfaces. Such L3MP procedures may utilize services provided by the MIH Function across different entities to query and reserve resources required for a handover operation between heterogeneous networks.

MIH services in mobile devices facilitate seamless handover between heterogeneous networks. A higher layer mobility protocol such as Mobile IP could be supported for handover and seamless session continuity. This shall not preclude other protocols in addition to Mobile IP and even other upper layers from making use of MIH services to optimize handovers.

Mobile terminals employing the MIH services would receive indications from lower layers for asynchronous operations like Event service. The interaction with Command service and Information service will be through synchronous query and response type of mechanisms.

The MIH Function would also provide the functionality for the exchange of information between the network and host entities of the same media type. Note, if mechanism for such information exchange already exists with a given type of media (such as with some cellular media types), the MIH Function will make use of the existing mechanism whenever possible.

### Media Independent Event Service

The Media Independent Event service supports local events as well as remote events if the remote node(s) is of the same media type. In case of local events information typically propagates upwards from the L2 data link (MAC, Radio Link etc) to the MIH or layer 3 mobility protocol (L3MP) within a local stack. In case of remote events information may propagate from MIH or a L3MP in one stack to MIH or a L3MP in another stack. The two stacks may be present at the two ends of a link and thus may be connected directly through a medium (wired or wireless). One of the above local stacks may be present in a client or mobile terminal type of device while the other may be present in a network point of attachment type of device such as an AP or a BS. Note, remote events between two stacks of different media types (e.g., between an 802.11 mobile terminal and a UMST BS) are not supported since a link between such two stacks are impossible. Events can indicate changes in state and transmission behavior of the L2 data links (functions such as MAC, Radio resource management etc depending upon the L2 access link technology) and or predict state changes of the data link layers. Events can also be used to indicate management actions or commands on part of network or some such management entity.

The event service will typically be used to facilitate handover detection within the L3MP. For example an indication that the link will cease to carry MAC SDUs at some point in the near future may be used by the L3MP to prepare a new point of attachment ahead of the current point of attachment ceasing to carry frames. This has the potential to reduce the time taken to hand over between attachment points.

Events defined include Link Up, Link Down, Link Parameters Change,  Link Going Down, L2SDU Transmission Status, Link Event Rollback, Pre trigger (L2 Handoff Imminent),etc.

Events can originate from the L2 data link (MAC, functions such as MAC, Radio resource management etc depending upon the L2 access link technology) the PHY or the MIH Function. The destination of any event may be the MIH Function within the local stack, the MIH Function of the remote stack at the other end of the link, or both. The destination of events is established dynamically with a registration mechanism that enables an endpoint to register its interest in particular event types.

Events may carry additional context data. For example a Link Up event may carry information including but not limited to by any means about layer 2 & layer 3 identification, identifier of serving entities where these identifiers can be acquired, or IP address renewal indication which can inform the MIH or L3MP whether L3MP needs to initiate handover. This enables a mobile station to determine whether or not a handover has necessitated a layer 3 handover, immediately at the time of attachment. This can serve to improve the performance of layer 3 handovers.

### Media Independent Command Service

The command service refers to the commands sent from the higher layers to the lower layers in the reference model. It includes the commands from upper layer to MIH (e.g. upper layer mobility protocol to MIH, or policy engine to MIH, etc), and from MIH to lower layer (e.g. MIH to MAC, or MIH to PHY). Commands can also be sent from a local MIH entity to a peer MIH entity. These commands mainly carry the upper layer decisions to the lower layers on local device entity or at remote entity, and thus control the behavior of lower layers.

The full set of the commands that could be supported from upper layer to MIH shall be defined in this specification. A minimum set of command services that are provided by the L2 data link (MAC, functions such as MAC, Radio Resource Management etc. depending upon the L2 access link technology) and PHY should be defined in this specification.

### Media Independent Information Service

Media Independent Information Service (MIIS) provides a framework and corresponding mechanisms   by which a MIHF (Media Independent Handover Function) entity can discover and obtain network information existing within a geographical area to facilitate the handovers.  MIIS primarily provides a set of information elements (IEs), the information structure and its representation and a query/response type of mechanism for information transfer. This contrasts with the asynchronous push model of information transfer for the event service. The information may be stored within the MIH functional (MIHF) entity or maybe present in some information server from where the MIH in the station can access it. The definition of the information server and the mechanism to access it are out of scope.

The information can be made available via both lower as well as higher layers.  Information can be made available at L2 through both a secure and an insecure port. Information available through insecure port would typically be less sensitive and would allow the mobile terminal to make a quick handover decision before incurring the overhead of authentication and establishing a secure L2 connection with the network. In certain scenarios L2 information may not be available or sufficient to make intelligent handover decision. In such cases, higher layer information service is required. The capability of such network information service can be obtained via schema access. The structure and definition of a schema can be represented in a high level language such as XML. However the transport and security of this information are outside the scope of IEEE 802.21.

The Information service also provides access to static information such as neighbor reports. This information helps in network discovery. The service may also provide access to dynamic information which may optimize link layer connectivity with different networks. This could include link layer parameters such as channel information, MAC addresses, security information, etc. Information about available higher layer services in a network may also help in more effective handover decision making before the mobile terminal actually attaches to any particular network.

While selecting for an appropriate syntax for information exchange it is desired that:

- the information exchange should be done fast

- the overhead in exchange of information should be as less as possible

- the extraction of the information at the terminal should not include high computational complexity

The information provided by Information service conforms to structure and semantics specified within 802.21. The Media Independent Information service specifies a common (or media independent) way of representing this information across different technologies by using a standardized format such as XML or ASN.1

MIIS provides the ability to access this information about all heterogeneous networks in a geographical area from any single L2 network, depending on how the 802.21 MIIS service is implemented. MIIS either relies on existing access media specific transports and security mechanisms or L3 transport and L3 security mechanisms to provide access to the information. How this information is developed and deployed in a given L2 network is outside the scope of the standard. A media independent neighbor graph may be abstracted obtaining the neighbor reports from media specific technology, however, the generation and maintenance of this media independent neighbor graph is out of scope of 802.21. Typically, in a heterogeneous network composed of multiple media types, it is the handover decision module or higher layer mobility management will collect information from different media types and assemble a consolidated view to facilitate its inter-media handover decision.

Some networks such as the cellular networks already have an existing means of detecting a list of neighborhood base stations within the vicinity of an area via the broadcast control channel. Other IEEE groups define similar means and supports clients in detecting a list of neighborhood access points within the vicinity of an area via either beaconing [1] or via broadcast of MAC management messages [4]. The Media Independent Information Service (MIIS) provides a unified framework to help the higher layer mobility protocols (HLMP) across the heterogeneous network environment to facilitate the client’s discovery and selection of multiple types of networks existing within a geographical area. In the larger scope, the macro objective is to help the higher layer mobility protocol to acquire a global view of the heterogeneous networks to facilitate seamless handover when roaming across these networks.

## MIH Reference Models for Access Networks

MIH provides asynchronous and synchronous services through well defined SAPs for lower layers and upper layers.

Figure 3: General MIH Reference Model

### MIH Reference Model for 802.3

Figure 4: MIH Reference Model for 802.3

The MIH reference model for 802.3 is illustrated above. The IEEE 802.21 standard supports the Media Independent Event service, Media Independent Command service and Media Independent Information service. A management protocol is required for the exchange of information between MIH entities within a terminal and a network. However, as shown in the above figure, there are no peer management facilities in CSMA/CD. Thus, if the MIH services are supported over wired ethernet networks, the payloads of the MIH services can only be carried in data frames.

In order to carry the above payload over normal ethernet data frames, a L2 protocol with a new ethertype is necessary. The MIH ethertype is encoded in the SNAP header. The IEEE 802.21 standard should define the packet format and payloads in media independent manner in standard TLV format. Thereafter these packets can be encapsulated in a L2 MIH Protocol using MIH ethertype when the payload needs to be sent over normal data frames as in case of ethernet. In other cases the TLV based messages and payload can be directly encapsulated in media specific management frames.

### MIH Reference Model for 802.11

Figure 5: MIH Reference Model for 802.11

The above figure shows the MIH functions for 802.11 stations and network PoA (APs). The MIH_MAC_SAP defines the MIH interface to the data plane and can encapsulate MIH protocol packets in data packets. However since 802.11 does not currently support Class 1 data frames, traffic can be sent over the data plane only when the client is associated with the AP. The MIH_MGMT_SAP provides interface with the management plane (MLME) and allows MIH protocol packets to be encapsulated in management frames (such as action frames). Thus MIH_MGMT_SAP is use to transfer packets after a station associates with an AP, whereas the MIH_MAC_SAP canbe used to transfer packets before establishing an association with AP.

The MIH_SAP shows the interface of MIH Function with other higher layer entities such as Transport, Handover policy, L3 Mobility protocol, etc. The MIH Function may interface with the OS or the system using the MIH_ME_SAP.

### MIH Reference Model for 802.16

The figure below shows the MIH function for 802.16 based system. The MIH_MGMT_SAP in this case provides the MIH functionality over management plane and help with transporting MIH protocol messages across peer MIH Function entities. The MIH_MGMT_SAP uses a set of appropriate connection identifiers to transfer the messages.  The MIH_SAP shows the interface of MIH Function with other higher layer entities such as Transport, Handover policy, L3 Mobility protocol, etc. The MIH Function may interface with the OS or the system using the MIH_ME_SAP.

Figure 6: MIH Reference Model for 802.16

### MIH Reference Model for 3GPP

Figure 7: MIH Reference Model for 3GPP enabled Mobile Stations

A potential realization of MIH functions for 3GPP enabled Mobile Stations is illustrated above. The MIH_RRC_SAP defines the MIH interface to the 3GPP RRC layer. The MIH_MGMT_SAP defines the MIH interface to the 3GPP GMM/SM. Most of the MIH services may utilize the information that is already defined in the RRC layer and GMM/SM. The desired information from MIH Event Services, Command Services and Information Services will be provided to MIH_RRC_SAP and MIH_MGMT_SAP which will be further defined by 3GPP SDO. It is possible that in the future, GMM/SM may be enhanced by utilizing the new functionality provided by the MIH_MGMT_SAP. In the case of MIH in the Mobile Stations, the PHY and MAC SAPs would be used to communicate link-state information to the RRC. The exchange between the 3GPP RRC and PHY or MAC layers is left for the 3GPP organization to define and there will be no direct interaction between the PHY and MAC layers with the MIH. No new interfaces and primitives need to be defined for these SAPs.

In the case of MIH in the network, the first MIH reachable can be very remote w.r.t. the network whose features the Mobile Station is trying to discover. The architecture placement of MIH function shall be eventually figured out by 3GPP. Because the PHY and MAC would be at the Node_B, and the RRC would be at the RNC, some mechanisms may be required for the information exchanges between Node B, RNC and the MIH entity in the network side. These mechanisms are out of the scope of this document. The figure above is for illustrative purposes only and shall not constrain implementations.

### MIH Reference Model for 3GPP2

Figure 8: MIH Reference Model for 3GPP2 enabled Mobile Stations

A potential realization of MIH functions for 3GPP2 enabled Mobile Stations is illustrated above. The MIH_MGMT_SAP defines the MIH interface to the 3GPP2 L3 Signaling. The MIH_PPP_SAP and MIH_LAC_SAP define the MIH interface to the PPP layer and LAC layer. Most of the MIH services may utilize the information that is already defined in the PPP layer, LAC layer and L3 Signaling. The desired information from MIH Services will be provided to MIH_MGMT_SAP, MIH_PPP_SAP and MIH_LAC_SAP which will be further defined by 3GPP2 SDO.

The MIH_LAC_SAP would communicate with the PHY and MAC layers using PHY and MAC SAPs currently defined within 3GPP2. The LAC would communicate with the MIH entity using the MIH_LAC_SAP. There would be no direct interaction between the PHY and MAC layers and the MIH. The PPP layer would use the MIH_PPP_SAP to communicate with the MIH layer. Moreover, it is possible that in the future cdma2000 L3 may be enhanced by utilizing the new functionality provided by the MIH_SAP. The figure above is for illustrative purposes only and shall not constrain implementations. The figure below shows how the MIH Function spans across different media specific technologies and provides a common abstraction of handover services to higher layers through MIH_SAP.

Figure 9: MIH Reference Model for Mobile Stations with Multiple Protocol Stacks

## Service Access Points (SAPs)

The MIH Function exchanges messages with other layers and functional planes using Service Access Points (SAPs). Each SAP consists of a set of service primitives that specify the information to be exchanged and the format of the information exchanges.

The specification of the MIH Function includes the definition of SAPs that are media-independent (i.e., insensitive to the interface types that the mobile terminal can support) and recommendations to define or extend other SAPs that are media-dependent. Media-independent SAPs allow the MIH Function to provide services to the upper layers of the mobility-management protocol stack, the network management plane, and the data bearer plane. The MIH_SAP and associated primitives provide the interface from MIH_Function to the upper layers of the mobility-management stack. Upper layers need to register with MIH Function as MIHF users to receive MIHF-generated events and also for link layer events that originate at layers below the MIH Function but can be passed on to upper layers through MIH Function. Upper layers can directly send commands to MIH Function. Similarly MIH Function entities can also send commands to other remote (peer) MIH Function entities. Primitives corresponding to all these services described above are within the scope of MIH_SAP.

Media-dependent SAPs allow the MIH Function to use services from the lower layers of the mobility-management protocol stack and their management planes. All inputs (including the events) from the lower layers of the mobility-management stack into the MIH Function are provided through existing media-specific SAPs such as MAC SAPs, PHY SAPs, and LLC SAPs. Link Commands generated by the MIH Function to control the PHY and MAC layers during the handover are part of the media specific MAC/PHY SAPs and are already defined elsewhere.

Figure below shows the key MIH related SAPs for different networks. The key MIH related SAPs are as follows:

Figure 10: Relationship between different MIH SAPs

### MIH_MGMT_SAP

This defines the interface between the MIH Function layer and the management plane of different networks. This SAP can be used for sending MIH messages between MIH Function and local link layer entities as well as between peer MIH Function entities. Messages based on management frames can be sent in the unauthenticated state as well. The MIH_MGMT_SAP defines primitives for MIES, MICS and MIIS.

### MIH_SME_SAP

This SAP defines the interface between MIH Function layer and the Station Management Entity or the Network Management System. This SAP defines the MIH interface with the sstem and is used for MIH configuration and operation.

### MIH_SAP

This SAP defines the interface between MIH and a L3MP, or a Handover Function which might reside at higher layer or a higher layer Transport as well. The interface between MIH and higher layers is a local interface which may depend on the choice of L3MP as well. The methods and other messages defined by this SAP result in local communication between peer layer entities. Definition of this SAP is required to define the scope and functionality of MIH Function.

### MIH_MAC_SAP

This SAP defines the interface between MIH Functon and media specific MAC. The interfaces defined by this SAP are primarily used to transfer data (MSDUs) between peer entities. No new interfaces and primitives need to be defined for this SAP. The interfaces defined by this SAP can be used to deliver MIH protocol based payloads to peer MIH entities usng the data plane.

### MIH_PHY_SAP

This SAP defines the direct interface between MIH Function and the media specific PHY. The MIH Function shall communicate with different media specific PHYs using the media specific MACs. As such this SAP does not need to be defined.

### MIH_LLC_SAP

This defines the LLC interface to MIH. The MIH can directly use the existing LLC primitives to initiate connections and communicate with peer LLC entities and thus establish data path for sending MSDUs across different links. As such this SAP does not need to be defined.

### MIH_RRC_SAP

This defines the interface between the MIH Function and RRC.

### MIH_PPP_SAP

This defines the interface between the MIH Function and PPP.

### MIH_LAC_SAP

This defines the interface between the MIH Function and LAC.

## Media Independence

### MIH Protocol

The IEEE 802.21 standard supports the Media Independent Event service, Media Independent Command service and Media Independent Information service. The MIH protocol defines the format of the messages that are exchanged between remote MIH entities and the transport mechanisms that support the delivery of the messages. The selection of the transport mechanism is dependent on the access technology that connects the MN to the network. The packet payload for these services may be carried over L2 management frames, L2 data frames or other higher layer protocols. Wireless networks such as 802.11 and 802.16 have a management plane and support management frames which could be suitably enhanced for carrying the above payloads. However the wired Ethernet network does not have management plane and can carry the above payloads only in data frames.

The IEEE 802.21 standard should define the packet format and payloads in media independent manner in standard TLV format. Thereafter these packets can be encapsulated in a L2 MIH Protocol using MIH ethertype when the payload needs to be sent over normal data frames as in case of ethernet. In other cases the TLV based messages and payload can be directly encapsulated in media specific management frames.

The IEEE 802.21 standard defines the format of MIH protocol data unit (PDU) header and payload. Standard TLV format provides media-independent representation for the PDU payload contents. The MIH PDU’s are encapsulated in data frames with MIH EtherType over 802 links. For 802.11 and 802.16 links extensions of media-specific management frames are recommended for carrying MIH messages. No assumptions are made in this proposal regarding the transport of MIH messages over 3GPP and 3GPP2 access links at L2.

## Media Dependence

The MIH Function aggregates disparate interfaces with respective media-dependent lower-layer instances (media-dependent service access points) into a single interface with the upper layers (the MIH SAP), making the inter-media differences fully transparent to the upper layers.

The MIH Function features media-dependent interfaces with IEEE 802 link-layer technologies (802.2, 802.3, 802.11, and 802.16) and cellular technologies (3GPP and 3GPP2). The MIH Function for the most part uses existing primitives and functionality provided by different access technology standards. Amendments to existing standards are recommended only when deemed necessary to fulfill the MIH Function capabilities.

The sections below list general amendments recommended to different underlying access technology standards due to the enhanced heterogeneous handover capability provided by MIH Function.

### 802 Amendments

The MIH Function interacts with the MAC and PHY layers in the IEEE 802 family of standards to monitor the status of each interface. In general, the IEEE 802 standards provide both control, data plane Service Access Points (SAPs) and management plane SAPs for the MAC and PHY layers. The MIH Function utilizes these existing SAPs and primitives wherever possible. Extensions of existing IEEE 802 standard primitives are proposed for link layer events and indications and additional information elements are identified which can help with effective heterogeneous handovers. Mechanisms to discover MIH capability through broadcast and specific query and response type mechanisms are also proposed.

These recommendations should be handled by specific IEEE 802 working groups, e.g. IEEE802.11 and IEEE802.16.  For the better understanding of this specification, some examples are provided as informative recommendations in the Appendix.

### 3GPP Amendments

The existing 3GPP specifications identify the PHY and MAC SAPs as the means to exchange events to and from the RRC layer in the mobility-management (control) plane. Following amendments are proposed as recommendations to 3GPP:

- Possible extending or reusing existing primitive fields as defined in 3GPP Standards.

### 3GPP2 Amendments

Within 3GPP2, the Physical (PHY), Medium Access Control (MAC), and Point-to-Point Protocol (PPP) signaling layers are distinct sources of link-state triggers and events. 3GPP2 designates the Link Access Control (LAC) layer as the recipient of the PHY- and MAC-generated triggers. 3GPP2 has no designated recipient for PPP-generated triggers. This proposal defines requirements for the delivery of triggers and events from the LAC and PPP layers to the MIH Function. Support of these triggers implies the following recommendations to 3GPP2:

- The specification of a new LAC SAP

- The specification of a new PPP SAP.

- The specification of a new management SAP for L3 signaling.

### Upper layer Amendments

Existing mobility mechanism based on different upper layer protocols such as MIP, SIP etc. are based on layer 3 procedures that are unaware of the operational state of the underlying link that supports the communication channel. This constraint affects the delivery of service that require low delay and low latency, as packets are dropped or delayed when the terminal is required to connect to a new network point of attachment.

The MIH Function helps to mitigate this problem by providing link-level and other information that can be used by a mobility management entity to characterize the conditions of both the existing communication link and potential candidates. Upper layers can interface with the MIH SAP to effectively use these services.

# MIH Function Services

The MIH Function provides Media Independent Event Service, Media Independent Command Service and Media Independent information Service which facilitate handovers across heterogeneous networks. This section provides a general description of these services.

## Media Independent Event Service

### Introduction

In general handoffs can be initiated either by the mobile terminal or by the network. Events that can initiate handoff may originate from MAC, PHY or MIH Function either at the mobile node or at the network point of attachment. This could be due to user or terminal mobility, state change in the environment or because of some management function on part of the network. Thus the source of these events can be either local or remote. A transport protocol is needed for supporting remote events. Security is another important consideration in such transport protocols.

Multiple higher layer entities may be interested in these events at the same time. Thus these events may need to have multiple destinations. Higher layer entities can register to receive event notifications from a particular event source. The MIH Function can help in dispatching these events to multiple destinations.

These events are treated as discrete events. As such there is no general event state machine. However in certain cases a particular event may have state information associated with it, such as the Link_Going_Down event discussed below. In such cases the event may be assigned an identifier and other related events may be associated with the corresponding event using this identifier.

From the recipient’s perspective these events are mostly “advisory” in nature and not “mandatory”. Layer 3 and above entities may also need to deal with reliability and robustness issues associated with these events. Higher layer protocols and other entities may prefer to take a more “defensive” approach when events originate remotely as opposed to when they originate locally.

The Event Service may be broadly divided into two categories, Link Events and MIH Events.  Both Link and MIH Events typically traverse from a lower to higher layer.  Link Events are defined as events that originate from event source entities below the MIH Function and typically terminate at the MIH Function.  Entities generating Link Events include but arenot restricted to various IEEE802-defined, 3GPP-defined and 3GPP2-defined interfaces.  Within the MIH Function, Link Events may be further propagated, with or without additional processing, to upper layer entities that have registered for the specific event.  Events that are propagated by the MIH to the upper layers are defined as MIH Events. This relationship is shown in Figure 11.

Figure 11: Link Events and MIH Events

### Types of Events

The Media Independent Event service can support several types of events.

- MAC and PHY State Change events

These events correspond to changes in MAC and PHY state. These events correspond to definite changes in state. For example Link_Up event is a case of a state change event.

- Predictive events

Predictive events express the likelihood of change in properties in future based on past and present conditions. For example, decay in signal strength of WLAN network may indicate loss of link connectivity in near future. Since they attempt to predict the future, they may be incorrect and hence there is a need to retract predictive events. Predictive events may carry predictive information including a time bound, specifying the time interval in which the event is expected to occur and a level of confidence that the event shall occur in the specified time bound.

- Link Synchronous events

The timing of link layer handover events controls the latency involved in detecting handover at the network layer and thus affects the delay in updating the route tables. A link layer event delivered to a link layer element must be conveyed to the corresponding network layer element in order for the event to have relevance. This coupling between Link Events and Network Events is enabled by L2 handoff event type.

- Link Transmission events

The occurrence of a handoff of a Mobile Node, from one access network (AN) to another results in the reestablishment of a link layer connection to the target AN. The reestablishment of a new link layer connection with target AN causes the link layers of source AN as well as the mobile node to flush out the content of outstanding transmit and re-transmit queues. The flushing of outstanding radio frames introduces noticeable impact on the performance of end-to-end application especially for real time applications such as VoIP. Link Transmission events enable lossless inter-technology handovers.

- Local and Remote Events

Local Events are defined as being propagated upwards from the event source entity to the local MIH Function or from the MIH Function to local upper layer entities.  Should an event be required to be communicated from the MIH Function to another MIH Function not within the local stack, such events are defined as remote events.  Remote Link Events traverse originate from a remote event source entity to the remote MIH Function and is then propagated from the remote MIH Function to the local MIH Function as shown in Figure 12.

Figure 12: Remote Link Events

Remote MIH Events traverse from the remote MIH Function to the local MIH Function and then from the local MIH Function to the local upper layer entities as shown in Figure 13.  This is with the assumption that the local upper layer entities has subscribed for the remote event.

Figure 13: Remote MIH Events

### Event Flow Model

The figure below shows the event flow model for link events and MIH events.

Decision to request Event Registration from peer STA

MIH

MAC

MAC

MIH

STA (Client)

Network PoA

Link_Event_Register.request

Event Registration REQUEST FRAME

REMOTE EVENTS

Link_Event_Register.confirm

Link_EVENT.indication

Actual Event Indication from local or remote MAC

Decision to request Event Deregistration from peer STA

Link_Event_Deregister.request

Event Deregistration REQUEST FRAME

Link_Event_Deregister.confirm

Event Registration RESPONSE FRAME

Event Deregistration RESPONSE FRAME

Link-EVENT.request

Link-EVENT.confirm

Remote Event Generation

LOCAL EVENTS

Figure 14: Link Events Flow Model

Decision to request Event Registration from peer STA

Upper Layers

Local MIH

Remote MIH

STA (Client)

AP

MIH_Event_Register.request

Event Registration REQUEST FRAME

REMOTE EVENTS

MIH_Event_Register.confirm

MIH_EVENT.indication

Actual Event Indication from local or remote MIH

Decision to request Event Deregistration from peer STA

MIH_Event_Deregister.request

Event Deregistration REQUEST FRAME

MIH_Event_Deregister.confirm

Event Registration RESPONSE FRAME

Event Deregistration RESPONSE FRAME

LOCAL EVENTS

Figure 15: MIH Events Flow Model

### Event Registration

Event registration provides a mechanism for upper layer entities to selectively receive events.  Registration may be divided into Link Events Registration and MIH Events Registration.  Link Events Registration is performed by the MIH Function with the event source entities in order to determine the events that each event source (link) is able to provide.  MIH Event Registration is performed by upper layer entities with the MIH Function to select the events to receive.  It is possible for upper layer entities to register for all existing events or notifications that are provided by the event source entity even if no additional processing of the event is done by the MIH Function.

#### Link Events Registration

It may be possible during initialization that the MIH Function actively searches for pre-existing interfaces, devices and modules that serve as link event sources in the Event service.  In addition to the link event source entities that are present during the bootstrapping stage, allowances must be made for devices such as hot-plugged interfaces or an external module.  The exact description and implementation of such mechanisms are out of scope of this document. The MIH Function may then register individually with each of these link layers based on user preferences.

#### MIH Events Registration

Upper layer entities that utilise the Event Service must register themselves with the MIH Function.  After registration, the upper layer entity may specify a list of events for which they want to receive from the MIH Function.  The specified events may include additional parameters in order to control the behaviour of the Event Service.  Examples include threshold values, request to receive events as a bundle and so forth.  The description of the exact mechanism by which the MIH Function uses to achieve this is out of scope of this document.  Upper layer entities may also query on the availability of events from the MIH Function without prior registration.  The query may have additional parameters that affect the returned results.  An example would be a request to retrieve a list of all events for IEEE802.11 defined interfaces only.

### Primitives for Link Layer Events

The generic model of Event Service primitives specifies the format of events that are passed from the link layer to the MIH Function and also from the MIH Function to the upper layer entity.  The Event primitive is defined as:

EventXX. indication (EventSource, EventInformationSet) where,

- EventSource: is used to identify the event source entity where the event originated

- EventInformationSet: is an extensible parameter that may contain implementation specific information and may be different across different events. EventInformationSet completely describes the event that occurred.

Event primitives typically do not receive a response.

### Link Layer Events

The following set of link layer events are defined.

| Event Identifier | Event Type | Event Name | Description |
| --- | --- | --- | --- |
| 1 | State Change | Link Up |  |
| 2 | State Change | Link Down |  |
| 3 | Predictive | Link Going Down |  |
| 4 | State Change | Link Detected |  |
| 5 | State Change | Link Parameters Change |  |
| 6 | Administrative | Link Event Rollback |  |
| 7 | Link Transmission | Link SDU Transmit Success |  |
| 8 | Link Transmission | Link SDU Transmit Failure |  |
| 9 | Link Synchronous | Link Handoff Imminent |  |
| 10 | Link Synchronous | Link Handoff Proceeding |  |
| 11 | Link Synchronous | Link Handoff Complete |  |
| ….. | …… | …… |  |
| 11+ | Reserved | Reserved |  |

## Media Independent Command Service

### Introduction

Media Independent Command Service refers to the commands sent from the higher layers to the lower layers in the reference model. Upper layers and other MIH users may utilize command services to determine the status of links and/or control the multi-mode device for optimal performance. Command services may also enable MIH users to facilitate optimal handover policies. For example, the network may initiate and control handovers to balance the load of two difference access networks.

The link status varies with time and terminal mobility. Information provided by MICS is dynamic information comprising of link parameters such as signal strength, link speed, etc, where as information provided by MIIS is less dynamic or static in nature and is comprised of parameters such as network operators, higher layer service information, etc. MICS and MIIS Information could be used in combination by the terminal/network to facilitate the handover.

A number of commands have been added to allow the upper layers to configure, control, and get information from the lower layers. A set of command services that are provided by the L2 data link (MAC, functions such as MAC, Radio Resource Management etc. depending upon the L2 access link technology) and PHY should be defined in this specification.

The commands are classified into two categories: MIH Commands and Link Commands.

### MIH Commands

MIH Command includes the commands from upper layer to MIH (e.g. upper layer mobility protocol to MIH, or policy engine to MIH, etc).

MIH Command may be local or remote. Local MIH Commands are issued by Upper Layers to MIH function in the local stack. Remote Command are issued by Upper Layers to MIH function in the peer stack. Figure 7.1 illustrate the differences between local and remote MIH Commands.

Figure 16: Local MIH Commands

Figure 17:  Remote MIH Command

### Link Commands

Link Command includes the commands from MIH Function to lower layers (e.g. MIH Function to MAC, or MIH Function to PHY). These commands mainly control the behavior of lower layer entities. Link Commands may be local or remote. Local Link Commands are issued by MIH Function to lower layers in the local stack. Remote Link Commands are issued by MIH Function to lower layers in the peer stack. Figure 7.2 illustrates the differences between local and remote Link Commands. 802.21 shall use existing media specific link commands for interaction with different access networks. New Link Commands shall be defined as recommendations to media specific technology standards.

Figure 18: Remote Link Command

### Command Service Flow Model

Figures below show thef flow for local and remote commands.

Upper Layers

MIH

Lower Layers

MIH_Command.request

MIH_Command.confirm

Link Command Execution

Decision to request local Commands

Link_Command.request

STA (Client)

Link_Command.confirm

Figure 19: Command Service Flow (Local)

Decision to send Commands to peer STA

Upper Layers

MIH

Upper Layers

Peer (Network)

MIH_Command.request

Remote Command REQUEST Transport

MIH_Command.response

Remote Command RESPONSE Transport

MIH_Command.response

MIH_Command.request

Execution of MIH Command

Peer (Client)

MIH

Figure 20: Command Service Flow (Remote)

Figure 7.3 shows a typical Command Service Flow. The use cases and handover scenarios for the commands defined in this section are provided in Appendix.

Remote commands may be transported over higher layer protocols or L2 management frames. The transport of remote commands is out of the scope of this specification. Some examples for remote command transport are provided in the Appendix.

### Command List

#### MIH Commands

The following set of MIH Commands are defined.

| No | MIH Command | Local, Remote | Media Types | Parameters | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | MIH Poll | L, R | All |  | Poll the status of links |
| 2 | MIH Switch | L, R | All |  | Switch session between links |
| 3 | MIH Configure | L, R | All |  | Configure a link |
| 4 | MIH Scan | L, R | All |  | Scan a link |

#### Link Commands

At this stage no new Link Commands have been defined.

#### Remote MIH Commands

The following set of commands between MIH Function in local stack and MIH Function in remote stack are defined.

| Command Identifier | Command Name | MIHF <> MIHF | Description |
| --- | --- | --- | --- |
| 1 | MIH Handover Initiate | Client <> Network PoA | Initiates handovers and sends a list of suggested networks and suggested PoA. |
| 2 | MIH Handover Prepare | Network (oPoA) <> Network (nPoA) | This command is sent by MIHF on oPoA to MIHF on suggested new network at nPoA. This allows the client to query for resources on nPoA and also allows to prepare the nPoA for handover |
| 3 | MIH Handover Commit | Client <> Network | In this case the client commits to do the handover based on selected choices for network and PoA. |
| 4 | MIH Handover Complete | Network (nPoA) <> Network (oPoA) | This is a notification from nPoA to oPoA that handover has been completed, new PoA has been established and any pending packets may now be forwarded to the new nPoA. |
| 5 | MIH Network Address Information | Network (nPoA) <> Network(oPoA) <> Network (Access Router/ Foreign Agent) | This command is sent by MIHF on oPoA to MIHF on suggested new network at nPoA. nPoA may relay this command to the AR with MIHF. This allows the client to have network address related information prior to the handover to the nPoA. |

## Media Independent Information Service

### Introduction

Media Independent Information Service (MIIS) provides a framework by which a MIH Function both in the mobile node and in the network can discover and obtain homogeneous or heterogeneous network information within a geographical area to facilitate handovers. In the larger scope, the macro objective is to acquire a global view of the heterogeneous networks to facilitate seamless handovers when roaming across these networks.

Media Independent Information Service includes support for various Information Elements (IEs). Information Elements provide information that is essential for a handover module to make intelligent handover decision. Figure 8-1 gives a high level description of scenarios that distinguish between two different types of mobility.

- Horizontal handover: A horizontal handover is performed by switching between different points of attachment of the same access network. For example, switching between different APs within the 802.11 network (e.g. Inter ESS or inter subnet).

- Vertical handover: A vertical handover means a handover from one access network to another access network. For example switching between different PoAs across heterogeneous technologies (e.g. from WLAN to GPRS).

Figure 21:  Horizontal and Vertical Handovers

Depending on the type of mobility support for different types of information elements may be necessary for performing handovers.   For example in case of horizontal handovers across different PoAs of the same access network information available from lower link layers of access network may be sufficient. In such cases information elements like intra-technology neighbor reports and other link layer information required during handovers is directly available from the access network. In such cases the availability of higher layer services offered by the network typically does not change appreciably across different network point of attachments. On the other hand during vertical handovers the terminal may move across different access networks. In such cases there is a need to select appropriate PoA in the new network based on both optimum link layer connectivity as well as availability of appropriate higher layer services to permit service and session continuity for active user applications.

Media Independent Information Service (MIIS) provides the capability for obtaining the necessary information for handovers. This includes information about lower layers such as neighbor maps and other link layer parameters as well as information about available higher layer services such as access to internet connectivity, availability of VPN services, etc. The set of different higher layer services provided by the MIIS may constantly evolve. At the same time the list of access networks that are supported by MIIS may also evolve. As such there is a need for flexibility and extensibility in the way the MIIS provides support for different information elements. Towards this end the MIIS defines a schema. The schema helps a client of MIIS to discover the capabilities of MIIS and also discover the entire set of different access networks and IEs supported by a particular implementation. Schema representation also allows the terminal to query the information in a more flexible and efficient manner. As part of defining this schema the MIIS can also identify a set of basic information elements that can define the core functionality of different implementations of MIIS. Other information elements as they are added can become part of the more extended set of MIIS capabilities.

MIIS provides information about different access networks such as 802 networks, 3GPP networks and 3GPP2 networks. The MIIS also allows this collective information to be accessed from any single network. Thus for example using a 802.11 access network it may be possible to get information about not only all other 802 networks in a particular region but also that of 3GPP and 3GPP2 networks as well. Similarly using a 3GPP2 radio, it may be possible to get access to information about all 802 and 3GPP networks in a given region. This capability allows a terminal device to use its currently active access network and scan for other available access networks in a geographical region. Thus a terminal device is freed from the burden of powering up each of its individual radios and establishing network connectivity for the purpose of accessing heterogeneous network information. MIIS enables this functionality across all available access networks by providing uniform way to retrieve heterogeneous network information in any geographical area.

### Information Service Elements

The information service elements can be classified into three groups:

- General Network Information (GNI): These information elements give a general overview of the network like network ID, location of different PoAs of the network, IP version, operator of the network, and so on.

- Link Layer Information (LLI): These information elements include the information related to link layer layers such as, link layer parameters (channel, frequency, PHY types), data rates, neighbor information, security, QoS, and so forth.

- Higher Layer Information (HLI): These information elements include higher layer services or applications that are supported by the respective network. Some examples are support for Multimedia Message Service (MMS), Mobile IP (MIP), Virtual Private Network (VPN), types of applications supported (e.g. VoIP, e-mail, IPsec VPNs, streaming media, location based ), pricing of access (e.g. “a fee is be required” versus “access to the network is free”),  use of NAT,  roaming partners and so forth.

While all of the above information service elements are necessary and will be available via 802.21 information service framework, it is important to note that link layer information (LLI) will be mostly directly available via media specific technologies. In cases where media specific amendments are not available or possible, link layer information can be enabled via higher layer information.

Following table gives a comprehensive list of information elements (IEs).

| Name of Information Element | Description | Media Types | Comments |
| --- | --- | --- | --- |
| General Network Information (GNI) |  |  |  |
| List of networks available | List all network types that are available given a location or POA information | All | E.g., 802.3, 802.11, 802.16, GSM, GPRS/EDGE networks |
| Location of POA | Geographical Location, Civic address, PoA ID | All | E.g. GML format for LBS or network management purpose |
| Network standards supported | List of all available transmission technologies available | All | Enables MN to decide to make use of other transmission technologies supported by the same provider |
| Network Identifier | Unique ID of the network or network provider | All | E.g. SSID in 802.11 or ITU-T E.212 MCC+MNC |
| Operator | Name of the network provider | All | E.g. Vodafone, Orange. Could be equivalent to Network ID. |
| IP Version | Indicates the version Internet Protocol used | All | IPv4 and/or IPv6 |
| Roaming Partners | List of direct roaming agreements |  | E.g. in form of NAIs or MCC+MNC |
| Cost | Indication of costs for service/network usage | All | E.g, flat rate, hourly, day or weekly rate |
| SLAList | Service level Agreement list | All | RequestingServiceProviderName, NumSLAServiceProvider |

| Link Layer Information (LLI) |  |  |  |
| --- | --- | --- | --- |
| Neighbor Information | Neighboring network information, measurement report | All | Technology specific information |
| Security | Link layer security supported | All | Technology specific, e.g. WEP in 802.11, 802.11i, PKM in 802.16 UEA in 3G, Authentication, EncryptionMode, EncryptionAlgorithm, KeyProvisioning, KeyManagement |
| Quality of Service | Link QoS parameters | All | E.g. BER, SNR, DataRateKbps, MinLatencyMsec, MaxLatencyMsec, MaxJitterMsec |
| AccessRouterInfo |  |  | AccessRouterAddress, IPversion, MobilityProtocolSupport, FASupport |

| Higher layer Service Information (HLSI) |  |  |  |
| --- | --- | --- | --- |
| IMS Access (MMS, SMS, Presence, Instant Message, Push-to-Talk, …) | Indication whether specific service is supported or not | N/A | 802.21 specific methods, e.g., a new flag, query/response |
| ISP Supported | Internet Service Provider that provides the access to internet | N/A | 802.21 specific methods, e.g., a flag, query/response |
| Location based services | List of Local services that are available given a location | N/A | 802.21 specific methods, e.g., a flag, query/response |
| VPN Supported | Network enables VPN services | N/A | 802.21 specific methods, e.g., a flag, query/response |
| MIP Supported | Network enables MIP version and services | N/A | 802.21 specific methods, e.g., a flag, query/response |
| Use of NAT | NAT is used for internet access | N/A | 802.21 specific methods, e.g., a flag, query/response |

Table 8-3:  A Comprehensive list of Information Elements

From the above listed elements a basic set of information elements is extracted and show (Table 8-3) how these elements can be supported via lower layers if they can be incorporated in media specific technologies (e.g. via bits or flags). In such cases, MIHF entity (e.g., a STA or network entity) can extract this basic IE either from media specific broadcast information or by sending a specific request/response primitive. On the other hand, before sending a request/response, the MIHF entity should have the knowledge that the network supports the 802.21 MIH standard (may be obtained by hearing a bit in the management frame). For example, MIHF entity in STA (Figure 8-3) may request the MIHF entity of POA for such information. It is important to note here that before STA gets authenticated to the PoA and ready to exchange data, it first gets the basic IE set.  This IE set may then be used by the STA’s handover function to get more information from the POA in order to determine the right POA to attach.

Figure 22:  STA exchanging IEs from its PoA

| Seq. Nr. | Name of IE | Type | Possible values |
| --- | --- | --- | --- |
| 1 | Network_Operator | String |  |
| 2 | Networks_supported | Enumerated (could also be Flags / Bits) | 802.3, 802.11a, 802.11b, 802.11g, 802.16a, 802.16d, 802.16e, GSM, GPRS, W-CDMA, CDMA2000, … |
| 3 | Cipher_Suites | Flags / Bits | WEP(0), WPA(1), WPA2(2), TKIP(3), AES-CCMP(4), … |
| 4 | Âuthentication_Methods | Flags / Bits | (0), 802.1x(1), UAM(2),… |
| 5 | Link_Cost | String |  |
| 6 | Location_LatLong | Real | (x-value, y-value) |
| 7 | IP_Version | Enumerated | v4, v6, |
| 8 | Data_Rates | Real | (min_rate, max_rate) |
| 9 | HLSI | Flags / Bits | ISP(0), IMS(1), MMS(2), MIP(3), NAT(4), VPN(5), SIP(6), Local_device_support(6), … |
| 10 | Roaming_List_Available | Boolean / Flag / Bit | TURE / FALSE or 1 / 0 |
| 11 | MING_List_Available | Boolean / Flag / Bit | TURE / FALSE or 1 / 0 |
| 12 | Quality_of_Service | String |  |

Table 8-3: Basic Information Element Set

(1) This is the name of the network (PoA) to which the STA is now attached to (not in authenticated state).

(2) If PoA supports different access technologies, this variable is filled with the respective names.

(3) Cipher algorithms which are used to secure the physical link are listed over here. The bit string defined acts like a flag and the support of a specific cipher algorithm is set by the respective bit. In case all the bits are unset, it indicates that the PoA is not supporting any link encryption method.

(4) Authentication methods the PoA is supporting to check the STA credentials. The bit string defined acts like a flag and the support of a specific authentication method is set by the respective bit. In case all the bits are unset, it indicates that the PoA is not supporting any authentication method.

(5) Cost parameter. How the value is set is out of scope.

(6) Geographical location of the PoA (latitude and longitude co-ordinates). This parameter enables to trace the location of the PoA.

(7) IP versions supported by the PoA.

(8) Maximum and Minimum data rates supported by the PoA (in Mbps).

(9) HLSI bit stream gives an overview of the higher layer services provided by that particular PoA.

ISP: Internet Service Provider

IMS: IP Multimedia Subsystem

MMS: Multimedia Message Service

MIP: Mobile IP support, which is having either a home or foreign agent

NAT: Network Address Translation. If this is set, the local IP used between STA and PoA is not seen from outside

VPN: support of Virtual Private Network

SIP: Session Initiation Protocol

Local_device_support: If this flag is set, then STA can ask for the specific list of devices from PoA for example using higher layer protocols like UP&P, SLP, etc.

(10) If this variable is set to TRUE, then STA could ask PoA to issue a list of the roaming partners in the next instance.

(11) If this variable is set to TRUE, then STA could ask PoA to issue a of the network graph in the next instance.

(12) The parameter QoS is specified as a string. Some examples of QoS parameters could be delay, jitter. At this point, it is difficult to specify how this is set.

### Information Service Schema

A schema defines structure of information. A schema is used in the 802.21 information service to define the structure of each information element as well as the relationship among different information elements supported. The 802.21 information service schema needs to be supported by every MIH Function that implements the MIIS to support flexible and efficient information queries. The 802.21 information service defines the various information elements and their structure. The various IEs represent information about lower layers of network stack as well as about higher layer services available in different access networks. A schema is defined by a language and can be represented in multiple ways. Examples include Resource Description Framework (RDF) which is based on XML, ASN.1 which is used in 802 MIBs, Variants or a simple TLV representation of different information elements.

The MIIS schema is classified into two major categories.

Basic schema that is essential for every MIH to support and

Extended schema that is optional and can be vendor specific

#### RDF Schema Representation

This section gives an example of schema using Resource Description Framework (RDF) [8]. RDF uses SPARQL [7] as a query language for querying information.  Both RDF schema [6] and SPARQL are represented in XML. .An RDF schema defines the structure of set of expressions, where the underlying structure of any expression is a collection of triples, each consisting of a subject, a predicate and an object [8]. XML syntax for RDF called RDF/XML is defined in [9].

RDF has the following advantages:

Supports both hierarchical and non-hierarchical information structure.

Allows for flexible data query

Allows for distributed schema definition

Easier way to change the schema definition

As described earlier, the RDF schema definition for MIIS consists of two parts; the basic and the extended schema. The basic schema is not supposed to be updated. An MIH entity is typically pre-provisioned with the basic schema for ease of implementation of schema-based query.  In scenarios where the basic schema is not pre-provisioned methods such as DNS query may be used to access the location (FQDN) of the basic schema.

Unlike the basic schema, the extended schema is expected to be updated periodically, e.g., when a new link-layer technology is introduced. The extended schema can be retrieved from the specified URL via the IEEE 802.21 information service using the schema query capability described in Section 8.5.3 without any pre-provision of such extended schema. The URL of the extended schema can also be obtained via the schema URL query capability described in section 8.5.3. Alternatively a DNS query may be used for finding out the location (FQDN part) of extended schema.  The extended schema is defined as an extension of the basic schema and includes data structure and relationship of media-specific or higher-layer information. In that sense extended schema is the complement of basic schema.

Figure 8-3 below shows an example graphical representation of the 802.21 MIIS basic schema in which ‘Network’ ‘L2”, ‘L3’ ‘ Location’  ‘IPv4’, ‘IPv6’ , ‘Link-Type’ , ‘PoA’,  “Civic Address’, ‘Geo-Coordinates’  are represented as class, while all others are  properties of classes.  The lines indicate either the range or domain of a property or a sub class of a class.  In particular ‘r’ represents the range of a property and ‘d’ represents  domain of a property.  ‘Domain’  defines the class that a particular property belongs to and ‘range’ defines a type of a particular property.

Figure 23: A Graphical Representation of basic schema

#### The MIIS Schema

The basic and extended schema and their textual description in different formats are shown below. Extended schema is optional and contains an exhaustive list of information resources and properties. Therefore extended set has been described in Appendix 11.4

##### Basic Schema representation in RDF/XML

<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf 'http://www.w3.org/1999/02/22-rdf-syntax-ns#'>

<!ENTITY rdfs 'http://www.w3.org/2000/01/rdf-schema#'>

<!ENTITY mihbase 'URL_TO_BE_ASSIGNED'>

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;" xmlns:mihbase="&mihbase;"

xml:base="&mihbase;">

<rdfs:Class rdf:ID="Network">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

Network class has two properties, namely l2 for layer-2 information and l3 for higher-layer information. Any property can be added to this class in an extended schema.

<rdfs:comment/>

</rdfs:Class>

<rdf:Property rdf:ID="l2">

<rdfs:domain rdf:resource="#Network"/>

<rdfs:range rdf:resource="#L2"/>

<rdfs:comment>

This property is of type L2 class.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="l3">

<rdfs:domain rdf:resource="#Network"/>

<rdfs:range rdf:resource="#L3"/>

<rdfs:comment>

This property is of type L3 class.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="L2">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

L2 class has properties that are specific to link-layer. The properties include network-id, operator, location and neighbor-information properties. Any property can be added to this class in an extended schema.

<rdfs:comment/>

</rdfs:Class>

<rdf:Property rdf:ID="operator">

<rdfs:domain rdf:resource="#L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

<rdfs:comment>

This property contains a name of the operator. It could be the same as network-id property.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="network-id">

<rdfs:domain rdf:resource="#L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

<rdfs:comment>

This property contains an identifier of the network. It may contain an SSID.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="link-type">

<rdfs:domain rdf:resource="#L2"/>

<rdfs:range rdf:resource="#Link-Type"/>

<rdfs:comment>

This property is of type Link-Type and carries the type of the link.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="Link-Type">

<rdfs:comment>

An instance of this class is a resource that is used for identifying the type of a link. A distinct instance of this class is to be defined for each link type in an extended schema.

<rdfs:comment/>

<rdfs:Class/>

<rdf:Property rdf:ID="location">

<rdfs:domain rdf:resource="#L2"/>

<rdfs:range rdf:resource="#Location"/>

<rdfs:comment>

This property is of type Location class.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="Location">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

This class has properties that indicate a location in different ways. The properties include poa, civic-address and geo-coordinates properties. Any property can be added to this class in an extended schema.

<rdfs:comment/>

</rdfs:Class>

<rdf:Property rdf:ID="geo-coordinates">

<rdfs:domain rdf:resource="#Location"/>

<rdfs:range rdf:resource="#Geo-Coordinates"/>

<rdfs:comment>

This property is of type Geo-Coordinates class.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="poa">

<rdfs:label>Point of Attachment</rdfs:label>

<rdfs:domain rdf:resource="#Location"/>

<rdfs:range rdf:resource="#PoA"/>

<rdfs:comment>

This property is of type PoA class.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="Geo-Coordinates">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

This class has properties that represent geographic coordinates. The properties include latitude, longitude and altitude attributes.

<rdfs:comment/>

</rdfs:Class>

<rdf:Property rdf:ID="latitude">

<rdfs:domain rdf:resource="#Geo-Coordinates"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

<rdfs:comment>

This property contains the latitude of the location.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="longitude">

<rdfs:domain rdf:resource="#Geo-Coordinates"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

<rdfs:comment>

This property contains the longitude of the location.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="altitude">

<rdfs:domain rdf:resource="#Geo-Coordinates"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

<rdfs:comment>

This property contains the altitude of the location.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="civic-address">

<rdfs:domain rdf:resource="#Location"/>

<rdfs:range rdf:resource="#Civic-Address"/>

<rdfs:comment>

This property is of type Civic-Address class.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="Civic-Address">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

This class has properties that represent a civic address. Properties of this class are to be defined in an extended schema, possibly based on the IETF GEOPRIV work.

<rdfs:comment/>

</rdfs:Class>

<rdf:Property rdf:ID="neighbor-information">

<rdfs:domain rdf:resource="#L2"/>

<rdfs:range rdf:resource="#PoA"/>

<rdfs:comment>

This property is of type PoA class and contains an identifier of a specific neighboring PoA.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="PoA">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

This class has properties that represent a PoA. The properties are poa-type and poa-id.

<rdfs:comment/>

</rdfs:Class>

<rdf:Property rdf:ID="poa-type">

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#Link-Type"/>

<rdfs:comment>

This property is of type Link-Type class.

<rdfs:comment/>

</rdf:Property>

<rdf:Property rdf:ID="poa-id">

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

<rdfs:comment>

This property contains a link-specific identifier of PoA.

<rdfs:comment/>

</rdf:Property>

<rdfs:Class rdf:ID="L3">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

<rdfs:comment>

L3 class has properties that are specific to higher-layer. The properties include IPv4 and IPv6. Any property can be added to this class in an extended schema.

<rdfs:comment/>

</rdfs:Class>

<rdfs:Class rdf:ID="IPv6">

<rdfs:subClassOf rdf:resource="#L3"/>

<rdfs:comment>

IPv6 class has properties that are specific to IPv6. Properties of this class are to be defined in an extended class.

<rdfs:comment/>

</rdfs:Class>

<rdfs:Class rdf:ID="IPv4">

<rdfs:subClassOf rdf:resource="#L3"/>

<rdfs:comment>

IPv4 class has properties that are specific to IPv4. Properties of this class are to be defined in an extended class.

<rdfs:comment/>

</rdfs:Class>

</rdf:RDF>

##### Basic Schema representation in ASN.1

Another example of representation is in the Abstract Syntax Notation One (ASN.1) format which is widely used in the telecommunications. From the ASN.1 schema it possible to either generate a binary format or to have it converted into XML format. The basic set representation in ASN.1 format and its respective XML format is as follows:

| ASN.1 | XML |
| --- | --- |
| BASICSet80221 DEFINITIONS::= BEGIN RealSet::=SEQUENCE { value1 REAL, value2 REAL } MIHbasicIE::= SEQUENCE { networkOperator UTF8String, networksSupported ENUMERATED {e8023, a80211, b80211, g80211, a80216, d80216, e80216, gsm, gprs, wcdma, cdma2000}, chipperSuites BIT STRING {wep(0), wpa(1), wpa2(2), tkip(3), aesccmp(4)}, authMethods BIT STRING {eap(0), x802(1), uam(2)}, linkCost UTF8String, locationLatLong RealSet, ipVersion ENUMERATED {v4, v6}, dateRates RealSet, hlsi BIT STRING{isp(0), ims(1), mms(2), mip(3), nat(4), vpn(5), sip(6), localDeviceSupport(7)}, roamingListAvailable BOOLEAN, mINGListAvailable BOOLEAN, qualityofService UTF8String } END | <MIHbasicIE> <networkOperator> </networkOperator> <networksSupported> </networksSupported> <chipperSuites> </chipperSuites> <authMethods> </authMethods> <linkCost> </linkCost> <locationLatLong> <value1> </value1> <value2> </value2> </locationLatLong> <ipVersion> </ipVersion> <dateRates> <value1> </value1> <value2> </value2> </dateRates> <hlsi> </hlsi> <roamingListAvailable> </roamingListAvailable> <mINGListAvailable> </mINGListAvailable> <qualityofService> </qualityofService> </MIHbasicIE> |

### Information Flow

Figure  8-6-1  describes an information flow based on  L2 transport.  MIIS  within MIHF (MIH Functin)  is communicating with the remote MIHF  that is residing within the access network. MIH POA in this figure is either an AP  or a BS. MIH_information_request from the STA is carried  over L2 frame and delivered to the remote MIH.  Remote MIH  returns the necessary information via MIH_information_response.

Decision to request Information from peer STA

MIH

MAC

MAC

MIH

STA (Client)

AP/BS

MIH_Info.request

Information REQUEST FRAME

MIH_Info.confirm

Neighbour Map Report Availability Confirmation

Information RESPONSE FRAME

MIH_Info.indication

MIH_Info.response

Neighbour Map Report Generation

Figure 24:  L2-based MIIS information flow.

On the other hand, Figure  8-6-2  describes an information flow based on  L3 transport.  MIIS  within MIHF (MIH Function)  is communicating with the remote MIHF  that is residing either within the access network or in the core network. MIH POA in this figure is either an AR or some other entity in the  network. MIH_information_request from the STA is carried  over L3 frame and delivered to the remote MIH.  Remote MIH  returns the necessary information via MIH_information_response.

Decision to request Information from peer STA

MIH

MIH

STA (Client)

e.g, AR

MIH_Info.request

Information REQUEST FRAME

MIH_Info.confirm

Neighbour Map Report Availability Confirmation

Information RESPONSE FRAME

MIH_Info.indication

MIH_Info.response

Neighbour Map Report Generation

IP

IP

Figure 25:  L3-based MIIS information flow.

# MIH SAPs and Primitives

## Introduction

Following are the key SAPs MIH Function uses for interfacing other entities.

- MIH_MGMT_SAP

The MIH Function uses this SAP for interfacing with lower layers of media specific access networks. Primitives defined as part of this SAP are recommended as enhancements to media specific SAPs.

- MIH_SME_SAP

The MIH Function uses this SAP for interfacing with the System Management or Network Management entity.

- MIH_SAP

This is the main SAP which upper layers use for interfacing with MIH Function and accessing the various MIH services. This is a new SAP.

## MIH_MGMT_SAP Primitives

The key primitives defined as part of MIH_MGMT_SAP are described in the table below.

| No | MIH_MGMT_SAP Primitives | Opcode | Service Category | Description |
| --- | --- | --- | --- | --- |
| 1 | Link Event Discover | Request | Event |  |
| 2 | Link Event Register | Request | Event |  |
| 3 | Link Event Deregister | Request | Event |  |
| 4 | Link Event Configure | Request | Event |  |
| 5 | Link Up | Indication | Event |  |
| 6 | Link Going Down | Indication | Event |  |
| 7 | Link Event Rollback | Indication | Event |  |
| 8 | Link Parameters Change | Indication | Event |  |
| 9 | Link SDU Transmit Success | Indication | Event |  |
| 10 | Link SDU Transmit Failure | Indication | Event |  |
| 11 | Link Handoff Imminent | Indication | Event |  |
| 12 | Link Handoff Proceeding | Indication | Event |  |
| 13 | Link Handoff Complete | Indication | Event |  |
| 14 | Link Information Request | Request | Information |  |
| 15 | Link Information Response | Response | Information |  |
| ….. | ….. |  | ….. |  |
| 16+ | Reserved |  | Reserved |  |

### Link_Event_Discover.request

#### Function

This primitive is used by MIH Function or any other upper layer to query and discover the list of supported link layer events.

#### Semantics of service primitive

Link_Event_Discover.request

(

EventSource

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | N/A | The origination point from where list of supported events is requested. This can be used to discover both local and remote events. |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This primitive is generated by an upper layer entity such as MIH Function that is seeking to receive link layer event notifications.

#### Effect on receipt

The recipient responds immediately with Link_Event_Discover.confirm primitive.

### Link_Event_Discover.confirm

#### Function

This primitive returns the result of the query to discover link layer events request

#### Semantics of service primitive

Link_Event_Discover.confirm

(

EventSource,

SupportedEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | N/A | The origination point from where events are requested. |
| SupportedEventList | Set of Event IDs | N/A | List of link layer events supported by the link layer |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This primitive is generated in response to a Link_Event_Discover.request primitive.

#### Effect on receipt

The recipient may examine the returned event list and register for selective link layer indications.

### Link_Event_Register.request

#### Function

This primitive is used by MIH Function (the registrant) to register an interest in a particular or group of event types from a particular event origination point. The response indicates which of the requested event types were successfully registered. Event types not successfully registered will not be delivered to the registrant.

#### Semantics of service primitive

Link_Event_Register.request

(

EventSource,

RequestedEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | N/A | The origination point from where events are requested. |
| RequestedEventList | Set of Event IDs | Set of valid link layer events | List of events that the endpoint would like to receive indications for from the Event source |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This primitive is generated by a registrant such as MIH Function that is seeking to receive event primitives from a point in the protocol stacks terminating either end of the link.

#### Effect on receipt

The recipient responds immediately with Link_Event_Register.confirm primitive.

### Link_Event_Register.confirm

#### Function

This primitive returns the result of the registration request.

#### Semantics of service primitive

Link_Event_Register.confirm

(

EventSource,

ResponseEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | N/A | The origination point from where events are requested. |
| ResponseEventList | Set of Event IDs and their Registration Status | N/A | List of events along with their registration status |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This primitive is generated in response to a Link_Event_Register.request primitive.

#### Effect on receipt

The recipient may examine the returned event list and learn about the registration status of different events.

### Link_Event_Deregister.request

#### Function

This primitive is used by MIH Function (the registrant) to deregister for a set of link layer events.

#### Semantics of service primitive

Link_Event_Deregister.request

(

EventSource,

RequestedEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | N/A | The origination point from where event registration is to be deregistered. |
| RequestedEventList | Set of Event IDs | Set of valid link layer events | List of events for which indications need to be deregistered from the Event source |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This primitive is generated by a registrant such as MIH Function that is seeking to deregister for already registered set of events.

#### Effect on receipt

The recipient responds immediately with Link_Event_Deregister.confirm primitive.

### Link_Event_Deregister.confirm

#### Function

This primitive returns the result of the deregistration request.

#### Semantics of service primitive

Link_Event_Deregister.confirm

(

EventSource,

ResponseEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | N/A | The origination point from where events are to be deregistered. |
| ResponseEventList | Set of Event IDs and their Deregistration Status | N/A | List of events along with their deregistration status |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This primitive is generated in response to a Link_Event_Deregister.request primitive.

#### Effect on receipt

The recipient may examine the returned event list and learn about the deregistration status of different events.

### Link_Up.indication

#### Function

This notification is delivered when a layer 2 connection is established on the specified link interface and when L3MP and other upper layers can send higher layer packets. All layer 2 activities in establishing the link connectivity are expected to be completed at this point of time.

#### Parameters:

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| LinkIdentifer | Media Specific | Unique link identifier that can be used to detect changes in network PoA. (SSID+BSSID) serves as unique network identifier for 802.11 networks |
| MacNewPoA | MAC Address | MAC Address of New PoA (AP/BS) |
| MacOldAccessRouter | MAC Address (Optional) | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address (Optional) | MAC Address of new Access Router (if any) |
| IP_Renewal_Indicator | Flag | Indicates whether the MS shall change IP Address in the new PoA 0 : Change Required 1 : Change Not required |
| Mobility Management Protocol Support Type bitmap | BITMAP | Indicates the type of Mobility Management Protocol supported by the new PoA 0 : Mobile IPv4 with FA (FA-CoA) 1 : Mobile IPv4 without FA (Co-located CoA) 2 : Mobile IPv6 3 : Mobile IPv6 with DHCPv6 4 : SIP |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This notification is generated when a layer 2 connection is established for the specified link interface.

#### Effect on receipt

Higher layers may take different actions on this notification. The handover policy function may start collecting routing metrics for this link and may consider this link as a potential candidate for establishing future connections. When higher layer connections are being established over this link, the Transport layer may adjust transport (TCP) related parameters based on link properties.

### Link_Down.indication

#### Function

This notification is delivered when a layer 2 connection is broken on the specified link and when no more packets can be sent on the specified link

Event Source:	{Local Link}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacoldPoA | MAC Address | MAC Address of old PoA |
| MacOldAccessRouter | MAC Address (Optional) | MAC Address of old Access Router (if any) |
| ReasonCode | REASON_CODE_TYPE | Reason for why the link went down such as disassociated with PoA, failure in transmitting/acknowledging the frames, etc. |

The reason code for Link_Down can be one of the following.

| Reason_Code_Id | Reason Code | Meaning |
| --- | --- | --- |
| 0 | RC_EXPLICIT_DISCONNECT | The link is down because of explicit disconnect procedures initiated either by client or network |
| 2 | RC_PACKET_TIMEOUT | The link is down because no acknowledgements were received for transmitted packets within the specified time limit. |
| 3 | RC_FAIL_NORESOURCE | The link is down because there were no resources to maintain the connection |
| 4 – 127 | Reserved | Reserved |
| 128 - 255 | RC_VENDOR_SPECIFIC | Vendor Specific reason code. |

#### When generated

This notification is generated when layer 2 connectivity is lost. Layer 2 connectivity may be lost explicitly in cases where the terminal may initiate disassociate type procedures. In other cases the terminal may infer loss of link connectivity due to successive timeouts for acknowledgements of retransmitted packets.

#### Effect on receipt

Higher layers may take different actions on this notification. The handover policy may eliminate this link from list of active links for routing connections and may consider handing over any potential active connections to other more suitable links.

### Link_Going_Down.indication

#### Function

This notification is delivered when a Layer 2 connection is expected (predicted) to go down (Link_Down) within a certain time interval. Link_Going_Down event may be the indication to initiate handover procedures. Different applications may have different requirements with respect to the time interval required to anticipate in advance a link going down and make adequate a priori preparation to handover a connection depending on Application Class. Hence upper layers such as a L3 mobility protocol may actually configure the time interval in which a Link_Going_Down event may be delivered. This would allow the flexibility for the time interval to be in msecs, 100s of msecs or even seconds.

Event Type:	Predictive

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MACNewPoA | MAC Address (Optional) | MAC Address of New PoA |
| MacOldAccessRouter | MAC Address (Optional) | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address (Optional) | MAC Address of new Access Router (if any) |
| TimeInterval | Time in msecs | Time Interval in which the link is expected to go down. The link connectivity is expected to be available at least for time specified by TimeInterval |
| ConfidenceLevel | Percentage (0-100) | The confidence level for link to go down within the specified time interval |
| UniqueEventIdentifer | 0 – 65535 | Used to uniquely identify the event. To be used in case of event rollback |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

A Link_Going_Down event implies that a Link_Down is imminent within a certain time interval. If Link_Down is NOT received within specified time interval then actions due to previous Link_Going_Down may be rejected.

A 100% confidence would indicate certainty that the link is definitely going down within the specified time interval. For example a BS that may have decided to shut down for administrative reasons may send down a Link_Going_Down trigger with 100% confidence level. Predictions based on changes in link parameters such as signal strength values would typically have a lower confidence level. Different links would typically use implementation specific methods to predict future loss in link connectivity.

#### Effect on receipt

Higher layers may take different actions on this notification. Upper layers may prepare to initiate handovers based on the confidence level reported as part of the event.

### Link_Event_Rollback.indication

#### Function

Link_Event_Rollback is used in conjunction with Link_Going_Down. In case of Link_Going_Down if the link is no longer expected to go down in the specified time interval, then a Link_Event_Rollback message is sent to the Event destination. The destination should disregard or rollback the changes associated with the event identifier in such cases.

Event Type:	Administrative

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| UniqueEventIdentifer | 0 – 65535 | Used to identify the event which needs to be rolled back |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This notification is generated when a predictive even expected to occur within a certain time interval is no longer expected to occur before the expiry of the indicated time interval.

#### Effect on receipt

Higher layers may reevaluate the link under consideration by retrieving other link quality related parameters. Any handover procedures that may have already been initiated may need to be rescinded.

### Link_Detected.indication

#### Function

Link_Detected indicates that a new type of link has been detected for use.

This may imply that the terminal is in the coverage area and can listen to a beacon, or that the terminal may have received a response to a probe. Link_Detected does not guarantee that the terminal would be able to establish connectivity with the detected link, but just that the terminal can attempt to gain connectivity. Upper layers including MIH Function may evaluate additional properties of the link before attempting to establish a L2 connection with the link.

Event Type:	State Change

Event Source:	{Local Link}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MIH Capability | Flag | 0 : MIH Capability Not Supported 1 : MIH Capability Supported |

#### When generated

This notification is generated when a new link type is detected. The detection can be based on a beacon or response to a probe or even through the Information Service.

#### Effect on receipt

Upper layers include MIH Function may discover additional properties of the link before selecting it for establishing connectivity.

### Link_Parameters_Change.indication

#### Function

Link_Parameters_Change indicates changes in link parameters have crossed specified threshold levels. This may include link layer parameters such as Speed of the link, QoS, BER, etc. The threshold level for each such parameter may have to be configured through a separate command to link layer.

Event Type:	State Change

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MacAccessRouter | MAC Address (Optional) | MAC Address of current Access Router (if any) |
| oldValueOfLinkParameter |  | Specify the old value of link parameter |
| newValueOfLinkParameter |  | Specify the new value of link parameter |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

Examples of link layer parameters that could trigger this event include:

- Link Speed

- Link Bit Error Rate

- Link Frame Loss Rate  before Retransmission

- Link Received Signal Strength

#### When generated

This notification is generated when a link layer parameter crosses a configured threshold.

#### Effect on receipt

Higher layers may take different actions on this notification. If parameters related to link quality cross a certain threshold then that link may need to be evaluated for handing over current connections. The MIH Function may collectively evaluate different parameters and give appropriate indications to higher layers regarding suitability of different links.

### Link_SDU_Transmit_Success.indication

#### Function

During transmission the link layer may fragment higher layer packets. As such a higher layer PDU may be broken into multiple segments by the link layer. These segments may then be individually transmitted by the link layer.

Link_SDU_Transmit_Success indicates that all segments belonging to a higher layer packet (PDU) have been successfully delivered from the ARQ module of link layer in current node to the ARQ module in link layer in peer node. A higher layer intermediate buffer management entity could use this indication to flush higher layer packets stored in any intermediate buffers.

Event Type:	Link Transmission

Event Source:	{Local MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| PacketIdentifier |  | Identifier for higher layer data packet that was delivered successfully. |

LINK_EVENT_SOURCE is defined as follows.

{Local Node, Remote Node}. Local and remote is referred with respect to origination of request

#### When generated

This notification is generated when link ARQ module has received notification from peer entity about successful transmission of all link layer segments corresponding to a higher layer PDU.

#### Effect on receipt

Higher layers may take different actions on this notification. A higher layer intermediate buffer management entity in MIH could use this indication to flush higher layer packets stored in any intermediate buffers.

### Link_SDU_Transmit_Failure.indication

#### Function

During transmission the link layer may fragment higher layer packets. As such a higher layer PDU may be broken into multiple segments by the link layer. These segments may then be individually transmitted by the link layer.

Link_SDU_Transmit_Failure indicates that at least one segment belonging to a higher layer packet (PDU) was not delivered successfully from the ARQ module of link layer in current node to the ARQ module in link layer in peer node.

Event Type:	Link Transmission

Event Source:	{Local MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| PacketIdentifier |  | Identifier for higher layer data packet that was to be delivered. |
| SegmentIdentifier |  | Identifier of specific segment that was not delivered successfully |
| FailureReason |  | Failure reason as to why the specific segment under consideration could not be delivered to the peer entity. |

#### When generated

This notification is generated when link ARQ module has received notification from peer entity about successful transmission of all link layer segments corresponding to a higher layer PDU.

#### Effect on receipt

Higher layers may take different actions on this notification. A higher layer intermediate buffer management entity could use this indication to retransmit higher layer packets stored in any intermediate buffers, especially if there are changes in access network during handovers.

### Link_Handoff_Imminent. Indication

#### Function

Link Handoff Imminent is generated before the handoff occurs. It contains information about the new point of attachment of the mobile. It also contains information about any application specific data that might be useful for the application running on the mobile

Event Type:	L2 Handoff

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MACNewPoA | MAC Address | MAC Address of New AP/BS |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |

#### When generated

Depending on whether it is the Mobile node or the Network, it is generated before the layer 2 handoff begins.

#### Effect on receipt

Application may take necessary actions to cover the handover blackout period. The network can proactively start sending the additional data as soon as it sends the pre-trigger and should expect a request from the mobile specifying how much data it needs to cover the blackout time. This event may be an indication to start buffering packets.

### Link_ Handoff_Proceeding.Indication

#### Function

To improve link layer handoff by buffering copies of layer 3 packets being transmitted via any layer 2 Interface at an intermediate buffer management layer between OSI layer 2 and layer 3 and transferring the group of buffered packets to an intermediate buffer management layer in target OSI L2 interface upon receiving a link layer hard handoff proceeding indication from lower layer. It achieves loss less inter-technology handoff.

Event Type:	L2 Handoff

Event Source:	{Local MAC, Remote MAC }

#### Parameters:

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MACNewPoA | MAC Address | MAC Address of New AP/BS |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |

#### When generated

This indication is generated when layer 2 handoff is detected by MAC layer management entity, PHY layer management entity.

##### Effect on receipt

Transferring, by the intermediate buffering layer in response to the link layer hard handoff indication, the group of buffered packets to a target intermediate buffering layer in a target OSI-based communications interface.

### Link_Handoff_Complete.Indication

#### Function

Improve OSI transport, Session and Application layer responsiveness to link changes. They can better adapt their data flows by resuming flows upon receiving this indication.

Event Type:	L2 Handoff

Event Source:	{Local MAC, Remote MAC}

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | LINK_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| MACNewPoA | MAC Address(Optional) | MAC Address of New AP/BS |
| MacOldAccessRouter | MAC Address (Optional) | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address (Optional) | MAC Address of new Access Router (if any) |

#### When generated

This is generated upon completion of L2 handoff when link is ready for regular traffic.

#### Effect on receipt

Notifying a transport layer, by the target Buffer Management entity in response to the hard handoff complete indication that data transfer may resume. This may be an indication to stop buffering any intermediate packets.

## MIH_SME_SAP Primitives

The key primitives defined as part of MIH_SME_SAP are described in the table below.

| No | MIH_SME_SAP Primitives | Opcode | Service Category | Description |
| --- | --- | --- | --- | --- |
| 1 | Initialize | Request | System Mgmt | Initializes the mIH Function |
| 1 | Reset | Request | System Mgmt | Resets the MIH Function |
| 2 | Install link | Indication | System Mgmt | Informs the MIH Function of a new link that has been added to the terminal device or the network. The interface to new link may have been hot plugged or powered on. |
| 3 | Remove link | Indication | System Mgmt | Informs the MIH Function of an existing link interface that has been removed. The interface to new link may have been removed dynamically or shut down. |
| ….. | ….. | …… | ….. | ….. |
| 4+ | Reserved |  |  |  |

## MIH_SAP Primitives

The key primitives defined as part of MIH_SAP are described in the table below.

| No | MIH_SAP Primitives | Opcode | Service Category | Description |
| --- | --- | --- | --- | --- |
| 1 | MIH Capability Discover | Request | General | Discover list of Events and Cmmands supported by MIH Function. |
| 2 | MIH Event Register | Request | Event |  |
| 3 | MIH Event Deregister | Request | Event |  |
| 4 | MIH Event Configure | Request | Event |  |
| 5 | MIH Link Up | Indication | Event |  |
| 6 | MIH Link Going Down | Indication | Event |  |
| 7 | MIH Link Event Rollback | Indication | Event |  |
| 8 | MIH Link Parameters Change | Indication | Event |  |
| 9 | MIH Link SDU Transmit Success | Indication | Event |  |
| 10 | MIH Link SDU Transmit Failure | Indication | Event |  |
| 11 | MIH Link Handoff Imminent | Indication | Event |  |
| 12 | MIH Link Handoff Proceeding | Indication | Event |  |
| 13 | MIH Link Handoff Complete | Indication | Event |  |
| 14 | MIH Poll | Reqest | Command |  |
| 15 | MIH Switch |  | Command |  |
| 16 | MIH Configure |  | Command |  |
| 17 | MIH Scan |  | Command |  |
| 18 | MIH Handover Initiate |  | Command |  |
| 19 | MIH Handover Prepare |  | Command |  |
| 20 | MIH Handover Commit |  | Command |  |
| 21 | MIH Handover Complete |  | Command |  |
| 22 | MIH Network Address Information |  | Command |  |
| 23 | MIH Information Request | Request | Information |  |
| 24 | MIH Information Response | Response | Information |  |
| ….. | ….. |  | ….. |  |
| 25+ | Reserved |  | Reserved |  |

### MIH Capability Discover

TBD

### MIH Event Register

Please see the description for Link Event Register.

### MIH Event Deregister

Please see the description for Link Event Deregister.

### MIH Event Configure

Please see the description for Link Event Configure.

### MIH Events

Please see the description for corresponding link layer events.

### MIH Poll

A MIH Poll command is issued by upper layer entities to discover and monitor the status of the currently connected and potentially available links. A MIH Poll command may be local or remote. For example, a local polling may help the policy function that resides out of MIH to make optimal handover decisions for different applications when multiple links are available in a terminal. A remote polling initiated from the network side may enable the network to collect the status information on multiple links in a terminal through the currently connected link.

Upper layer entities may poll the lower layers periodically in a specified interval or based on the preferences in a policy engine, which is out of the scope of this specification.

#### MIH_Poll.request

Function

This primitive is issued by uppers to discover the status of the currently connected and potentially available links.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Poll.request (

Source Identifier,

Destination Identifier,

Polling Request Set,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the polling request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the polling request shall be sent. |
| Polling Request Set | Set of polling request each defined in Table 7.1. | Set of polling request each defined in Table 7.1 | Containing a set of interested status information. |

When generated

This primitive is generated when L3 or upper layers request the status information from lower layer links.

Effect of receipt

A local poll command causes the specified MAC to issue a Link Poll command to get the information on the status of the links. A remote polling command causes the peer MAC to issue

#### MIH_Poll.response

Function

This primitive is issued by the MIH entity to report the status of the links in response to the requests of upper layers.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Poll.response (

Source Identifier,

Destination Identifier,

Polling Request Set,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the polling response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the polling response shall be sent. |
| Poll Request Set | Set of polling request each defined in Table 7.1. | Set of polling request each defined in Table 7.1 | Containing a set of interested status information. |

When generated

This primitive is generated when MIH receive the status information and report to L3 or upper layers.

Effect of receipt

Upon receipt of the link status information, the upper layers make decisions and take actions.

| Table 7.1 Poll Request |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Valid Range | Description |
| NETWORK_TYPES | Integer | 0x01 – 802.3 0x02 – 802.11 0x03 – 802.16 0x04 – 3GPP 0x05 – 3GPP2 | The network type of the link that is polled. Upper layers may selectively poll a certain link type. |
| DEVICE_INFO | N/A | N/A | Information on manufacturer, model number, revision number of the software/firmware and serial number in displayable text are returned. |
| OPERATION_MODE | Integer | 0x00 – Normal Mode 0x01 – Power Saving Mode 0x02 – Power Down | Returns the polled link’s current power mode. |
| NETWORK_ID | N/A | N/A | Return the ID of the network that the polled link is currently configured to communicate with. |
| CHANNEL_ID |  |  | The ID of the channel currently in use |
| CHANNEL_QUALITY |  |  | Current Connection Quality |
| LINK_SPEED |  |  | Bits per second |
| BATTERY_LEVEL | Integer | 0-100; battery level in percentage; -1; unknown battery level |  |
| Media Dependent | N/A | N/A | TBD |

### MIH Switch

#### MIH_Switch.request

Function

This primitive is generated by upper layers to switch an active session from one link to another.

After upper layer entities makes handover decision, a MIH Switch command may be issued by upper layer entities to switch the active session from one link to another. MIH Function may execute the MIH_Switch command by using combination of a set of Link Command. For example, the MIH entity may establish new connection by Link Connect command, and disconnect the old link by Link Disconnect command, or turn the old link to sleep mode by Link Sleep command, or even power down it by Link Power Down command.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Switch.request (

Source Identifier,

Destination Identifier,

HandoverMode;

New Link Identifier,

Old Link Identifier,

Old Link Action

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the request shall be sent. |
| HandoverMode | Enumerate | Make-before-Break Break-before-Make | The handover mode decides the sequence of the execution of link command. If it is make-before-break, the Link Connect of the new link is executed before Link Disconnect of old link; if it is break-before-make, Link Connect is after the Link Disconnect |
| New Link Identifier |  |  |  |
| Old Link Identifier |  |  |  |
| Old Link Actions | Enumerate | Link Disconnect; Link Sleep; Link Power Down | Upper layers may choose which action the old link should takes. |

When generated

This primitive is generated when L3 or upper layers makes handover decision and try to disconnect the old link and establish the new link.

Effect of receipt

Upon receipt of the MIH Switch command, MIH checks the HandoverMode field first. If it is make-before-break, MIH tries to execute Link Connect to establish the new link before the old link is gone. If it is break-before-make, MIH tries to disable the old link before the new link is established. Depending on the Old Link Actions, MIH may select the ways to disable the old link: disconnect the old link by calling Link Disconnect; turn the link to sleep mode by calling Link Sleep; or shut down the old link by Link Power Down.

#### MIH_Switch.response

Function

This primitive is issued by the MIH entity to report the result of the MIH Switch command.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Switch.response (

Source Identifier,

Destination Identifier,

ResultCode,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the request shall be sent. |
| ResultCode |  | Success; Failure; Rejected |  |

When generated

This primitive is generated when MIH receive the results of the Link Commands.

Effect of receipt

Upon receipt of the result code, the upper layers make evaluations and take actions.

### MIH Configure

MIH Configure may be issued by upper layer entities to control the behavior of lower layer, for example, to set some feature of the driver of a specific link. MIH Configure may contain the configuration commands for multiple lower layer links. When MIH receives MIH Configure, it issues the Link Configure commands to corresponding links. The configuration parameter should be carried to Link Configure commands.

##### MIH_Configure.request

Function

This primitive is issued by uppers to control the behavior of a set of lower layer links.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure.request (

Source Identifier,

Destination Identifier,

Configure Request Sets,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the request shall be sent. |
| Configuration Request Sets | Set of configuration parameters for corresponding interfaces each defined in Table 7.2. | Set of configuration parameters for corresponding interfaces each defined in Table 7.2. | Containing a set of configuration parameters. |

When generated

This primitive is generated when L3 or upper layers attempt to control the behaviors of lower layer links, for example, set some features in the drivers.

Effect of receipt

A local configure command causes the MIH to issue a Link Configure command to set the lower layers according to the configuration parameters. A remote configure command causes the MIH in the peer MAC entity to issue a Link Configure command to set the lower layers. If multiple links are configured, multiple Link Configure command should be sent to each particular link.

| Table 7.2 Configure Request |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Valid Range | Description |
| NETWORK_TYPES_IN_USE | Integer | 0x01 – 802.3 0x02 – 802.11 0x03 – 802.16 0x04 – 3GPP 0x05 – 3GPP2 | Set the network type that should be used for the lower layer entity, e.g., a driver. |
| OPERATION_MODE | Integer | 0x00 – Normal Mode 0x01 – Power Saving Mode 0x02 – Power Down | Change the device’s power mode |
| DISABLE_TRANSMITTER | N/A | N/A | Enable/disable the transmitter of the interface. |
| NETWORK_ID | N/A | N/A | Change to the specified network of an interface. |
| CURREN_ADDRESS |  |  | Change the current address to the value specified |
| SUSPEND_DRIVER | N/A | N/A | Suspend or resume of the specified interface. |
| Media Dependent | N/A | N/A | TBD |

##### MIH_Configure.response

Function

This primitive is issued by the MIH entity to report the result of the MIH Configure command.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure.response (

Source Identifier,

Destination Identifier,

ResultCode,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the response is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the response shall be sent. |
| ResultCode |  | Success; Failure; Rejected |  |

When generated

This primitive is generated when MIH receive the results of the Link Commands.

Effect of receipt

Upon receipt of the result code, the upper layers make evaluations and take actions.

### MIH Scan

A MIH Scan command is issued by upper layer entities to discover the neighboring POA of the access networks. MIH Scan may contain the scan command for multiple links. The scan command for each link is defined in the related SDO and beyond the scope of this document. If the currently active link is required to scan, it is most likely that the on-going session is interrupted.

#### MIH_Scan.request

Function

This primitive requests a survey of potential POA that the STA may later elect to try to join.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Scan.request (

Source Identifier,

Destination Identifier,

Scan Request Sets,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the polling request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the polling request shall be sent. |
| Scan Request | Defined in related SDOs. | Defined in related SDOs. | Defined in related SDOs. |

When generated

This primitive is generated when L3 or upper layers request a survey of access networks.

Effect of receipt

If multiple links are requested, multiple Link Scan commands are issued to the specified MACs, carrying the media specific Scan Request information. The Link Scan should be defined in related SDOs.

#### MIH_Scan.response

Function

This primitive returns the descriptions of the set of POAs detected by the scan process.

Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Scan.response (

Source Identifier,

Destination Identifier,

Scan Response Sets,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the responseis initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the response shall be sent. |
| Scan Response Set | Defined in related SDOs. | Defined in related SDOs. | Defined in related SDOs. |

When generated

This primitive is generated by the MIH as a result of a MIH_Scan.request.

Effect of receipt

Upper layers take the scan results.

### MIH Handover Initiate

#### MIH_Handover_Initiate.request

#### Function

This primitive is used by MIH Function on a terminal to communicate with peer MIH Function on a network. The primitive is used to communicate intent of handover initiation. The handover can be initiated either by the client or by the network.

#### Semantics of service primitive

MIH_Handover_Initiate.request

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer

SuggestedNewLinkIdentifier,

SuggestedNewPoAIdentifier,

HandoverMode,

CurrentLinkAction,

)

Local or Remote: Both

MIHF (Terminal) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| SuggestedNewLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks |  | This is the identifier of new network to which handover needs to be initiated. |
| SuggestedNewPoAIdentifier | MAC_ADDRESS (Optional) |  | This is the preferred Point of Attachment (AP/BS) on new network |
| HandoverMode | Make-before_Break Break-before-Make |  | The handover mode may influence the manner in which links are prepared for handover |
| CurrentLinkAction | LINK_DISCONNECT LINK_LOW_POWER LINK_POWER_DOWN LINK_NO_ACTION |  | Specifies suggested action on old link once handover procedures have been executed. |

#### When generated

This primitive is generated by an upper layer entity such as a Handover Policy engine, L3 Mobility protocol like MIP, Application or some other entity that may want to cause a change in the current selected link. This command can be sent either to just the local MIH Function or it can be sent to the remote MIH Function as well.

#### Effect on receipt

The local MIH Function responds immediately with a MIH_Handover_Initiate.confirm primitive.

The remote MIH Function responds with MIH_Handover_Initiate.response primitive. The remote MIH Functin may send a MIH_Handover_Prepare message to the MIH Function in the new link under consideration. This message attempts to prepare the new link for impending handover. This may involve querying the new link for available resources and also potentially establishing a path for transfer of any future packets.

#### MIH_Handover_Initiate.response

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that sent out a MIH_Handover_Initiate.request. The primitive is used to communicate the response of handover initiation request. The response can be sent either by the client or the network depending on where the request was initiated.

#### Semantics of service primitive

MIH_Handover_Initiate.response

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer,

HandoverAck,

PreferredLinkIdentifier,

PreferedPoAIdentifier,

AbortReason

)

Local or Remote: Both

MIHF (Terminal) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| HandoverAck | BOOL | 0 or 1 | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a reason code is provided |
| PreferredLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks |  | This is the identifier of new network to which handover needs to be initiated. This may be different than the network that was suggested in the handover request. |
| PreferredPoAIdentifier | MAC_ADDRESS |  | This is the preferred Point of Attachment (AP/BS) on new network. This may be different than what was suggested in handover request. |
| AbortReason |  |  | Lists the reason for aborting/declining the handover request. |

#### When generated

The MIH Function responds with this primitive in response to a MIH_Handover_Initiate.request command from a peer MIH Function entity.

#### Effect on receipt

On receiving the response the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the response. In case the handover is to executed, the peer MIH function may also provide information about preferred network and preferred PoA on the preferred network.

### MIH Handover Prepare

#### MIH_Handover_Prepare.request

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that sent out a MIH_Handover_Initiate.request. The primitive is used to communicate the response of handover initiation request. The response can be sent either by the client or the network depending on where the request was initiated.

#### Semantics of service primitive

MIH_Handover_Prepare.request

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer,

QueryResourceList,

)

Local or Remote: Both

MIHF (old Network) <> MIHF (new Suggested Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| QueryResourceList |  |  | List of resources to be queried at the new suggested network. |

#### When generated

The MIH Function responds with this primitive in response to a MIH_Handover_Initiate.request command from a peer MIH Function entity.

#### Effect on receipt

On receiving the response the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the response. In case the handover is to executed, the peer MIH function may also provide information about preferred network and preferred PoA on the preferred network.

#### MIH_Handover_Prepare.response

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that sent out a MIH_Handover_Initiate.request. The primitive is used to communicate the response of handover initiation request. The response can be sent either by the client or the network depending on where the request was initiated.

#### Semantics of service primitive

MIH_Handover_Prepare.response

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer,

QueryResourceList,

)

Local or Remote: Both

MIHF (old Network) <> MIHF (new Suggested Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| ResourceStatus |  |  | Specifies whether requested resources are available or not at the new PoA. |
| QueryResourceList |  |  | List of resources actually available at he new suggested network and the new networkPoA. |

#### When generated

The MIH Function responds with this primitive in response to a MIH_Handover_Initiate.request command from a peer MIH Function entity.

#### Effect on receipt

On receiving the response the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the response. In case the handover is to executed, the peer MIH function may also provide information about preferred network and preferred PoA on the preferred network.

### MIH_Handover_Complete

#### MIH_Handover_Complete.request

#### Function

This primitive is used by a peer MIH Function on new PoA to communicate with the MIH Function on old PoA. The primitive is used to communicate the status of handover operation. This allows the old PoA to start forwarding any pending or remaining packets

#### Semantics of service primitive

MIH_Handover_Complete.request

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer

)

Local or Remote: Both

MIHF (old Network) <> MIHF (new Suggested Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |

#### When generated

The MIH Function responds with this primitive when handover operations to the new network PoA have been completed.

#### Effect on receipt

On receiving the request the old PoA may forward any pending packets to new PoA.

### MIH_Network_Address_Information

#### MIH_Network_Address_Information.request

#### Function

- This primitive is delivered from MIH Function on old PoA to another MIH Function on new PoA in order to discover mobile terminal’s network address related information before handover. Upon reception of this message, the MIHF on new PoA can either interact with upper layer to achieve enough information or further deliver this message to the MIHF in the network, e.g Access Router with MIHF which may have enough information on this message. This also can trigger the network entity, e.g, Access Router or Foreign Agent to send out Mobility related messages directly to the mobile terminal.

#### Semantics of service primitive

MIH_Network_Address_Information.request

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer

MACMobileTerminal,

HomeAddress,

CoA,

OldAccessRouterAddress,

OldFAAddress

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| MACMobileTerminal | MAC Address |  | MAC Address of Mobile Terminal |
| Home Address | IP Address |  | - Home IP address of Mobile Terminal |
| - CoA | IP Address |  | - Care of Address of Mobile Terminal |
| - Old FA Address | IP Address |  | IP address of old Foreign Agent. This parameter is only included when mobile terminal is using Mobile IPv4, |
| - Old Access Router Address | IP Address |  | IP address of old Access Router This parameter is only included when mobile terminal is using IPv6 |

#### When generated

When the mobile terminal initiates handover which may be informed by Link_Going_Down remote event service, old PoA generates this message to achieve network address related information prior to the handover.

#### Effect on receipt

Upon reception of this primitive, the MIHF on new PoA can either interact with upper layer to achieve enough information or further deliver this message to the MIHF in the network, e.g Access Router with MIHF which may have enough information on this message. When this primitive is delivered to the upper layer of the MIHF in the network, e.g, Access Router with MIHF, this can trigger to send network address related information, e.g, Router Advertisement either directly to the mobile terminal or to MIHF.

#### MIH_Network_Address_Information.response

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that sent out a MIH_Network_Address_Infomration.request. The primitive is used to communicate the response of MIH_Network_Address_Information.request. The response can be sent either by the new PoA or the network entity, e.g, Access Router or Foreign Agent, depending on where the related information is available.

#### Semantics of service primitive

MIH_Network_Address_Information.request

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer

MACMobileTerminal,

HomeAddress,

FAAddress,

AccessRouterAddress

NetworkAddressInformation

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| MACMobileTerminal | MAC Address |  | MAC Address of Mobile Terminal |
| Home Address | IP Address |  | - Home IP address of Mobile Terminal |
| - FA Address | IP Address |  | IP address of Foreign Agent. This parameter is only included when mobile terminal is using Mobile IPv4, |
| - Access Router Address | IP Address |  | IP address of Access Router This parameter is only included when mobile terminal is using IPv6 |
| - Network Address Information |  |  | - In case of Mobile IPv4, Agent Advertisement is encapsulated and in case of Mobile IPv6, Router Advertisement is encapsulated. |

#### When generated

The MIH Function responds with this primitive in response to a MIH_Network_Address_Information.request command from a peer MIH Function entity. The old PoA can also generate this message to deliver the received network address information further to the mobile terminal.

#### Effect on receipt

When the new PoA receives this message from the network entity, new PoA further delivers this primitive to the old PoA. When the mobile terminal receives this primitive, MIHF delivers this to the mobility management protocol or handover policy engine.

### MIH Information

#### MIH_Information.request

#### Function

This primitive is used by a station’s MIH (or an application on behalf of MIH) to request information either from the network or from a remote MIH.  The information query can be related to a specific interface, attributes to the network interface as well as entire network capability.  In other words, the service primitive will have the flexibility to query either a specific data within a network interface or extended schema of a given network. The network interface is not explicitly specified in the primitive, but is rather implicitly selected based on the specific MIH_MGMT SAP that is selected.

#### Semantics of service primitive

MIH_Information.request

(

InfoQueryFilter,

InfoQueryParameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryFilter | Combination of QueryFilterType flags | N/A | A set of query filters which control the type and amount of information that is retrieved. |
| InfoQueryParameters | Combination of QueryParameters, e.g., L2 info, L3 info, etc | N/A | Query filter specific parameters which indicate the type of information the client may be interested in. |

QueryFilterType:

GNI:

- FILTER_INFO_ALL_NETWORKS			Get Info about all networks

- FILTER_INFO_802_NETWORK			Get info about all 802 networks

- FILTER_INFO_GSM_NETWORK			Get info about only GSM network

- FILTER_INFO_CDMA_NETWORK

>

- …..

LLI:

- FILTER_INFO_ALL_ELEMENTS			Get info about all information elements

- FILTER_INFO_ELEMENT_MAC_ADDRESS		Get info about only the MAC Address

- FILTER_INFO_ELEMENT_SECURITY		Get info about only the security

HLSI:

- FILTER_INFO_ALL_SERVICE_PROVISION		Get info about higher layer services

- FILTER_INFO_ELEMENT_ISP			Get info about the ISP

- FILTER_INFO_VPN_SUPPORT			Indicates the support of VPN

- FILTER_INFO_MIP_SUPPORT			Indicates the support of MIP

Examples:

1. If a client is interested in getting information about all the elements of a network, it sends the following request after getting associated to the network:

MIH_Information.request (FILTER_INFO_ALL_ELEMENTS)

Depending on the amount of information available, the response could be big data flow.

2. If a client is interested to know whether VPN is supported by the network of interest, he could send following request:

MIH_Information.request (FILTER_INFO_VPN_SUPPORT)

In this case the response could be quite simple, ‘Yes’ or ‘No’.

3. If a client is interested to know whether EAP support is enabled by a network, the request could be as follows:

MIH_Information.request (FILTER_INFO_ELEMENT_SECURITY,EAP)

In this case too the response could be quite simple, ‘Yes’ or ‘No’.

#### When generated

This primitive is generated by a network L3MP or MIH that is seeking to retrieve information.

#### Effect on receipt

The recipient tries to interpret the query request and retrieve the specified information. Once the information is retrieved the recipient responds with MIH_Information.response primitive.

#### MIH_Information.response

#### Function

This primitive is used by MIH to respond to a previous MIH_Information.request primitive.

#### Semantics of service primitive

MIH_Information.response

(

InfoQueryFilter,

MIH_REPORT,

status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryFilter | Combination of QueryFilterType flags | N/A | A set of query filters which control the type and amount of information that is retrieved. |
| MIH_REPORT | String | N/A | Report consisting of information requested by the L3MP or MIH |
| Status | Success/Failure | N/A | Specifies whether the information was successfully retrieved or not. |

#### When generated

This primitive is generated by MIH that is responding to a previous query retrieve information.

#### Effect on receipt

The L3MP or MIH tries to interpret the MIH_REPORT and take suitable action.

#### Information Service Primitives Supporting RDF schema

Information service primitives as defined above are sufficient for query and response of schema.  However, following new filter types are needed in addition to the filters that are defined earlier.

- FILTER_INFO_DATA is defined for obtaining information using the SPARQL [SPARQL] RDF query language.  This is used, e.g., when detailed query conditions are specified, one result contains multiple different types of information and/or more than one result may be carried.

- FILTER_INFO_SCHEMA_URL is defined for obtaining the URL of the location of the extended schema.

- FILTER_INFO_SCHEMA is defined for obtaining the extended schema that  is stored in a specific URL.

Detailed definition and usage of these filter types are described in the next subsections.

##### General RDF/XML Data Query

A new InfoQueryFilter type “FILTER_INFO_DATA” is defined. When this InfoQueryFilter type is specified, the InfoQueryParameters must be a string which contains a SPARQL (Protocol and RDF Query language) query [2] where the SPARQL query is supposed to contain an appropriate query for obtaining expected RDF/XML data. The MIH_REPORT of the corresponding MIH_information.response must be a string which contains a SPARQL query result [3].

An example query request and response when FILTER_INFO_DATA is specified as InfoQueryFilter to obtain a list of 802.11 point of attachments (i.e., BSSIDs) around a specific location where the location is represented as an 802.11 point of attachment is shown below.

MIH_Information.request (FILTER_INFO_DATA,

"PREFIX mihbase:<URL_TO_BE_ASSIGNED>

SELECT ?poa-id

WHERE (?x1, mihbase:neighbor-information, ?x2)

(?x2, mihbase:poa-type, “802.11”)

(?x2, mihbase :poa-id, ?poa-id)

(?x1, mihbase :location, ?x3)

(?x3, mihbase :poa, ?x4)

(?x4,  mihbase:poa-type, “802.11”)

(?x4,  mihbase:poa-id, “00:11:22:33:44:55”)”, 123)

MIH_Information.response(FILTER_INFO_DATA

"<?xml version="1.0"?>

<sparql xmlns="http://www.w3.org/2001/sw/DataAccess/rf1/result">

<head>

<variable name="poa-id"/>

</head>

<results>

<result>

<poa-id>aa:bb:cc:dd:ee:ff </poa-id>

</result>

<result>

<poa-id>00:11:22:33:44:55</poa-id>

<result>

</result>

<poa-id>01:23:45:67:89:ab</poa-id>

<result>

</results>

</sparql>",

123)

##### RDF schema URL Query

A new InfoQueryFilter type “FILTER_INFO_SCHEMA_URL” is defined. When this InfoQueryFilter type is specified, the InfoQueryParameters must be a null string. The MIH_REPORT of the corresponding MIH_information.response must be a string which contains a URL for the extended schema. It is left to the implementation how to retrieve the extended schema from the obtained URL.

An example query request and response when FILTER_INFO_SCHEMA_URL is specified as InfoQueryFilter is shown below.

MIH_Information.request(FILTER_INFO_SCHEMA_URL, “” , 123)

MIH_Information.response(FILTER_INFO_SCHEMA_URL,

"http://www.networkdiscovery.org/2005/04/rdf-extended-schema/",123)

##### RDF schema Query

A new InfoQueryFilter type “FILTER_INFO_SCHEMA” is defined. When this InfoQueryFilter type is specified, the InfoQueryParameters carries the URL of the schema the querer wants to obtain. The MIH_REPORT of the corresponding MIH_information.response is a string which contains the obtained schema.

An example query request and response when FILTER_INFO_SCHEMA is specified as InfoQueryFilter to obtain the extended schema stored in the specified URL is shown below.

MIH_Information.request(FILTER_INFO_SCHEMA,

"http://www.networkdiscovery.org/2005/04/rdf-extended-schema/", 123)

MIH_Information.response(FILTER_INFO_SCHEMA,

"<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf 'http://www.w3.org/1999/02/22-rdf-syntax-ns#'>

<!ENTITY rdfs 'http://www.w3.org/2000/01/rdf-schema#'>

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;">

…

</rdf:RDF>",123)

# Media Independent Handover Protocol

## Introduction

The MIH Function provides asynchronous and synchronous services through well defined SAPs for lower layers and upper layers. The services provided include the Event Service (ES), Command Service (CS), and Information Service (IS). Detailed description about MIH services are found in Section 5.3. MIH SAPs include the MIH upper layer SAP, which is used by the users of MIH to gain access to various MIH Function services, and MIH lower layer SAPs, which are used by MIH Function to gain access and control of a variety of media dependent lower layer resources.

The Media Independent Handover protocol defines frame formats for exchanging messages between peer MIH Function entities. These messages are based on the primitives which are part of Media Independent Event service, Media Independent Command service and Media Independent Information service. IEEE802.21 supports Media Independent Handover Function in mobile terminal, and network. The MIHF Protocol allows peer MIH Function entities to interact with each other.

## MIH Protocol Transport

The table below shows the various transport options for different media types.

| No | Media Type | Preferred Transport | L2 Transport | L3 Transport |
| --- | --- | --- | --- | --- |
| 1 | Ethernet | L2 | Data Frames | IP based |
| 2 | 802.11 | L2 | Data Frames, Management Frames | IP based |
| 3 | 802.16 | L2 | Data Frames, Management Frames | IP based |
| 4 | 3GPP | L3 | Requires protocol stack changes | IP based |
| 5 | 3GPP2 | L3 | Requires protocol stack changes | IP based |

### L2 Data Frames Packet Format

Preamble

SFD

Destination

Source

Len/Type

LLC

Data

CRC

Len/Type

MIH Message Header

MIH Message Payload

MIH Ethernet Type

Figure 26: L2 Data Frame Format for transporting MIH Packet

### L2 Management Frame Packet Format

For 802.11 use Management frames with a new Action Category (MIH).

### L3 Frame Packet Format

One can use a standard L3 protocol (UDP/TCP) with a well defined MIH port. A new specific L3 protocol can be defined as well. The discovery of MIH Function entities at L3 is outside of scope.

## Description

The Media Independent Handover Protocol provides the following services:

- MIH capability discovery: MIHF in mobile terminal or MIHF in the network discovers which entity supports MIHF. Thereafter the peer MIH Functions negotiate/discover an optimum transport for communication. The MIH Function entities also discover list of supported events and commands. The MIH Function can also query the information schema for list of supported information elements.

- MIH remote registration: Remote MIHF in different entities can register with each other to receive Media Independent Handover Messages including remote MIES. No registration is required for command services.

- MIH message exchange: MIHF can exchange MIH messages using MIH payload and MIH protocol over a suitable transport. As part of message exchange the peer MIH Function entities can use the MIES, MICS and MIIS for effective handovers.

The standard describes the MIH packet format, message formats, and the procedures for MIH message exchange to facilitate handover in a media independent manner. However, handover policy and handover decision making is outside the scope of the standard. Following figure shows possible MIH message delivery and its relation with other protocol layers.

Figure 27: MIHF interaction for Inter-MIHF MIH message exchange

### MIH General Packet Format

The figure below describes MIH Packet format and essential details of MIH messages. MIH messages can be transported over L2 using data frames or media specific management frames. MIH messages can also be sent over L3 using a suitable L3 transport protocol.

Protocol Version

MIH Service ID

MIH Opcode

Transaction ID

Message Length

Reserved

octet 1

octet 2

octet 3

octet 4

MIH Message Identifier

MIH Message Data

Figure 28:  MIH General Packet Format

Fragment No

Fr

MIH Message Header

MIH Message Payload

MIH Function Destination Identifier

MIH Function Source Identifier

MIH Function Identifier Length

| Name of the Field | Size | Optional/ Mandatory | Description |
| --- | --- | --- | --- |
| Protocol Version | 1 octet | Mandatory | Version of the MIH protocol. Default: 0x01 |
| MIH Service ID | 1 octet | Mandatory | MIH Service Identifier 1:Event Service 2:Command Service 3: Information Service |
| MIH Opcode | 1 octet | Mandatory | Operation to be performed 1: Request 2: Response 3: Indication |
| Transaction ID | 1 octet | Mandatory | Transaction Identifier used to match requests and responses wherever applicable. |
| Fr | 1 bit | Mandatory | Fragmentation flag. 1: Specifies current packet has more than 1 fragment 0: Current packet is not fragmented |
| Fragment No | 7 bit | Mandatory | Fragment Number of this packet |
| Message Length | 2 octets | Mandatory | Total length of whole message in bytes (header + payload) |
| MIH Function Identifier Length (MIHFL) | 4 octets | Mandatory | Length of each of Source and Destination MIH Identifier fields |
| Source MIH Function Identifier | MIHFL octets | Mandatory | This is the Source MIHF identifier. This can be the L2 hardware address of the interface at the source node. This can also be IP based for L3 transports. This can also be something generated. |
| Destination MIH Function Identifier | MIHFL octets | Mandatory | This is the Destination MIHF identifier. This can be the L2 hardware address of the interface at the destination node. This can also be IP based for L3 transports. This can also be something generated during MIHF discovery and connectivity phase. |
| MIH Message ID | 4 octet | Mandatory | Actual MIH Message identifier for the service identified above. e.g. Link_Going_Down Event Indication for Event Service |
| MIH Message Data | Variable | Mandatory | MIH Service specific data of variable length |

### MIH Function Messages

MIH Function messages are used to discover MIH Function capability in the peer MIH Function entities, register with peer MIHF entity and provide MIES, MICS and MIIS services to MIHF clients.

| No | MIH Message Identifier | MIH Opcode | Category |
| --- | --- | --- | --- |
| 1 | MIH Capability Discover | Request, Response | Capability Discovery |
| 2 | MIH Event Register | Request, Response | Registration |
| 3 | MIH Event Deregister | Request, Response | Registration |
| 4 | MIH Link Event Configure | Request, Response | Event Configuration |
| 5 | MIH Link Up | Indication | Event Service |
| 6 | MIH Link Going Down | Indication | Event Service |
| 7 | MIH Link Event Rollback | Indication | Event Service |
| 8 | MIH Link Parameters Change | Indication | Event Service |
| 9 | MIH SDU Transmit Success | Indication | Event Service |
| 10 | MIH SDU Transmit Failure | Indication | Event Service |
| 11 | MIH Command Discover | Request, Response | Command Service |
| 12 | MIH Network Address Information | Request, Response | Command Service |
| 13 | MIH Handover Initiate | Request, Response | Command Service |
| 14 | MIH Handover Prepare | Request, Response | Command Service |
| 15 | MIH Handover Complete | Request, Response | Command Service |
| 16 | MIH Information Request | Request | Information Service |
| 17 | MIH Information Response | Response | Information Service |
| ….. | ….. |  | ….. |
| 20+ | Reserved |  | Reserved |

## MIH Function Messages

### MIH Capability Discovery Messages

#### MIH_Capability_Discover.request

This message doesn’t contain MIH message data, and only MIH header and MIH Message Identifier is transmitted. If a requesting MIHF entity doesn’t know the MIHF capable entity’s addresses, requesting MIHF entity fills its source addresses and broadcast this message. If a requesting MIHF entity knows the address of entity but doesn’t know whether the entity is MIHF capable, this message is delivered to the entity in a unicast way.

#### MIH_Capability_Discover.response

This message may contain SupportedEventList, SupportedCommandList, in the MIH message data. Only MIHF capable entity can receive MIH_Capability_Discover.request. Upon reception of MIH_Capability_Discover.request, MIH_Capability_Discover.response is transmitted. Destination addresses are copied from the MIH_Capability_Discover.request and source addresses are filled with its addresses. An entity with MIHF may also broadcast unsolicited MIH_Capability_Discover.response message to advertise its MIH capability.

| Name | Length | Value |
| --- | --- | --- |
| SupportedEventList | 4 | For each Bit location, a value of ‘1’ indicates the link event is supported. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |
| SupportedCommandList | 4 | For each Bit location, a value of ‘1’ indicates the link command is supported. Bit #0: Link Power Up Bit #1: Link Power Down Bit #2: Link Configure Bit #3: Link Connect Bit #4: Link Disconnect Bit #5: Link Sleep Bit #6: Link Scan Bit #7: Link Poll Bit #8~31: Reserved |

### MIH Remote Registration Messages

#### MIH_Event_Register.request

This message is used by remote MIH Function (the registrant) to register an interest in a particular or group of event types from a particular event origination point.

| Name | Length | Value |
| --- | --- | --- |
| RequestedEventList | 4 | For each Bit location, a value of ‘1’ indicates the link event is requested to register to receive indication from the Event source. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |

#### MIH_Event_Register.confirm

The response indicates which of the event types were successfully registered.

| Name | Length | Value |
| --- | --- | --- |
| ResponseEventList | 4 | For each Bit location, a value of ‘1’ indicates the requested link event is registered correctly. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |

#### MIH_Event_Deregister.request

This message is sent by remote MIHF (the registrant) to deregister for a set of link layer events.

| Name | Length | Value |
| --- | --- | --- |
| RequestedEventList | 4 | For each Bit location, a value of ‘1’ indicates the link event requested to de-register from the event source. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |

#### MIH_Event_Deregister.confirm

The response indicates which of the event types were successfully deregistered.

| Name | Length | Value |
| --- | --- | --- |
| ResponseEventList | 4 | For each Bit location, a value of ‘1’ indicates the requested link event is de-registered correctly. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |

### MIH Event Service Packet Format

Packets for MIH Event service have MIH Service ID set to 1.

#### MIH_Link_Up.indication

This notification is delivered from MIHF in the PoA to MIHF in the network when a layer 2 connection is successfully established with mobile terminal.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address |
| MacNewPoA | Variable | MAC Address of New PoA (AP) |
| MacOldAccessRouter | Variable | MAC Address of old Access Router (if any) |
| MacNewAccessRoute | Variable | MAC Address of new Access Router |

#### MIH_Link_Down.indication

This notification is delivered from MIHF in the PoA to MIHF in the network when a layer 2 connection is disconnected due to certain reason with mobile terminal.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address |
| MacNewPoA | Variable | MAC Address of New PoA (AP) |
| MacOldAccessRouter | Variable | MAC Address of old Access Router (if any) |
| ReasonCode | 1 | Reason for why the link went down 0: RC_EXPLICIT_DISCONNECT ( 1: RC_PACKET_TIMEOUT 2: RC_FAIL_NORESOURCE 3: 127: Reserved 128~255: RC_VENDOR_SPECIFIC |

Note: Meaning of Reason code presented above is as following

- RC_EXPLICIT_DISCONNECT: The link is down because of explicit disconnect procedures initiated either by client or network

- RC_PACKET_TIMEOUT: The link is down because no acknowledgements were received for transmitted packets within the specified time limit.

- RC_FAIL_NORESOURCE: The link is down because there were no resources to maintain the connection

- RC_VENDOR_SPECIFIC: Vendor Specific reason code.

#### MIH_Link_Going_Down.indication

This message is transmitted to the remote MIHF when a layer 2 connectivity is expected (predicted) to go down (Link_Down) within a certain time interval.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address |
| MacNewPoA | Variable | MAC Address of New PoA (AP) |
| MacOldAccessRouter | Variable | MAC Address of old Access Router |
| MacNewAccessRouter | Variable | MAC Address of new Access Router |
| TimeInterval | 2 | Time Interval in which the link is expected to go down. The link connectivity is expected to be available at least for time specified by TimeInterval . Unit: 1ms |
| Confidence Level | 1 | The confidence level for link to go down within the specified time interval Expressed in percentage (0~100) |
| UniqueEventIdentifer | 2 | To be used in case of event rollback |

#### MIH_Link_Event_Rollback.indication

This message is used in conjunction with Link_Going_Down. In case of Link_Going_Down if the link is no longer expected to go down in the specified time interval, the a Link_Event_Rollback message is sent to the remote Event destination.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address |
| UniqueEventIdentifer | 2 | To be used in case of event rollback |

#### MIH_Link_Parameters_Change.indication

This message indicates changes in link parameters have crossed configured threshold levels.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address |
| MacAccessRouter | Variable | MAC Address of current Access Router (if any) |
| oldValueOfLinkParameter | Variable | Specify the old value of link parameter |
| newValueOfLinkParameter | Variable | Specify the new value of link parameter |

### MIH Command Service Packet Format

Packets for MIH Command service have MIH Service ID set to 2.

#### MIH_Network_Address_Information.request

The ‘MIH_Network_Address_Information.request’ is delivered from one MIHF in the network to another MIHF in the network in order to discover mobile terminal’s network address related information before handover. Upon reception of this message, the receiving remote MIHF can either interact with upper layer to achieve enough information or further deliver this message to the MIHF which may have enough information on this message.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address of Mobile Terminal |
| Home Address | 4 or 6 | Home IP address of Mobile Terminal |
| CoA | 4 or 6 | Care of Address of Mobile Terminal |
| Old FA Address / Old Access Router Address | 4 or 6 | In case of Mobile IPv4, this parameter represents “IP address of old Foreign Agent, and in case of Mobile IPv6, this parameter represents IP address of old Access Router. |

#### MIH_Network_Address_Information.response

This is sent as a response to MIH_Network_Address_Information.response.

| Name | Length | Value |
| --- | --- | --- |
| MacMobileTerminal | Variable | MAC Address of Mobile Terminal |
| Home Address | 4 or 6 | Home IP address of Mobile Terminal |
| FA Address / Access Router Address | 4 or 6 | In case of Mobile IPv4, this parameter represents “IP address of Foreign Agent, and in case of Mobile IPv6, this parameter represents IP address of Access Router. |
| Network Address Information | Variable | In case of Mobile IPv4, Agent Advertisement is encapsulated and in case of Mobile IPv6, Router Advertisement is encapsulated. |

### MIH Information Service Packet Format

Packets for MIH Information service have MIH Service ID set to 3. The packet structure for MIIS is as follows.

Protocol Version

MIH Service ID=3

MIH Opcode

Transaction ID

Message Length

Reserved

octet 1

octet 2

octet 3

octet 4

MIH Message Identifier

MIH Message Data

Figure 29: MIIS Packet Format

Fragment No

Fr

MIH Message Header

MIH Message Payload

MIH Function Destination Identifier

MIH Function Source Identifier

MIH Function Identifier Length

Language Code

Character Encoding

Language Code: This defines the language used for query coding (e.g. ASN.1, UML, XML).

Charter Encoding: Examples of character encoding are as follows.

ASCII		 pure ASCII

UTF8		 UTF-8

UTF16LE		 UTF-16 little endian

UTF16BE		 UTF-16 big endian

UCS4LE		 UCS-4 little endian

UCS4BE		 UCS-4 big endian

UCS4_2143	 UCS-4 unusual ordering

UCS4_3412	 UCS-4 unusual ordering

UCS2		 UCS-2

8859_1		 ISO-8859-1 ISO Latin 1

8859_2		 ISO-8859-2 ISO Latin 2

8859_3		 ISO-8859-3

8859_4		 ISO-8859-4

8859_5		 ISO-8859-5

8859_6		 ISO-8859-6

8859_7		 ISO-8859-7

8859_8		 ISO-8859-8

8859_9		 ISO-8859-9

2022_JP		 ISO-2022-JP

## Protocol Flow

### MIH Capability Discovery

The network entities and the terminals with MIHF shall discover which entity supports MIHF in order to utilize MIHF for media independent handover. MIH capability discovery can be done through media specific broadcast messages or through on demand MIH capability discovery messages.

#### MIH Capability Discovery through media specific broadcast messages

The network may be able to indicate to the mobile terminal if it is MIH capable by broadcasting MIH capability over the medium, e.g. beacon in 802.11 and DL-MAP in 802.16.

#### On Demand MIH Capability Discovery

MIHF in mobile terminal or in the network can discover which entity in the network or which mobile terminal supports MIHF by using MIH capability discovery procedure. MIH capability discovery procedure consists of capability discovery handshake and capability advertisement. MIH capability discovery can be achieved by exchanging MIH messages, MIH_Capability_Discover.request and MIH_Capability_Discover.response. An MIHF entity may also advertise its MIH capability to the neighbors by broadcasting MIH_Capability_Discover.response message. Following figure shows the on demand MIH capability discovery procedure.

Figure 30: On demand MIH capability discovery procedure

# Appendix

## Handover Procedures

TBD

### MIP based Handover Sequence Diagrams

TBD

### SIP based Handover Sequence Diagrams

TBD

### MIH Service Selection based Handover Sequence Diagrams

TBD

### Network Initiated Handover Sequence Diagrams

TBD

## Media Independent Neighbor Graph (MING): An Example

### Media Independent Neighbor Graph

Neighbor graph is a network graph that connects a set of vertices (network PoAs) and a set of edges (a path between two PoAs) around a station’s (STA) location. It is a way of representing neighborhood information.  In case of homogeneous networks, network PoAs are of the same kind while for heterogeneous networks they are different.  Media independent neighbor graph (MING) consists of homogeneous and heterogeneous network elements.  In that sense media independent neighbor graph is a superset of all media specific neighbor graphs. Stations can obtain media specific neighbor graph information directly from the access networks. However, it is limited by the media the stations are currently attached to.  802.21 media independent neighbor information on the other hand can help obtain a global view of the network neighbor graph consisting of multiple media types.

Thus MING contains a set of different types of neighbors relative to a given PoA. This set can be kept by the MIIS functional entity and can eventually help a STA to identify quickly the potential candidate PoAs for handover. MING may also serve other purposes. For example, if a multi-mode STA knows (via MING) that some modes may not be  supported by the neighboring access networks, the STA may like not to power up (for power  management) the respective radio interfaces and then scan for neighbor information.  A graphical representation of MING is shown in figure 32. The graph contains a set of media independent information elements.

Figure 31: An Example Representation of Media Independent Neighbor Graph (MING)

A MING may be manually configured, e.g., manually listing the neighbors for a given AP, or obtained via the management interface, or other access networks. Rather than incurring such management overhead, the PoA can learn its neighbors of other media type dynamically through the course of report/response message exchanges. However, how this information is reported and the corresponding responses are outside the scope of this document. A neighbor information element shall only contain entries of neighboring PoAs that are legitimate neighbors relative to a given PoA satisfying the query. A MING should also prevent the addition of bogus neighbors. The exact form of the implementation of MING is vendor dependent and outside the scope of this standard, however, a dynamic mechanism to construct and maintain MING is described in Appendix 11.2.  A MING query may also be performed to obtain the neighbor information. In such cases, neighbor information IE is used in the neighbor information query. The format of the neighbor information map is described in Appendix.

Media Independent Neighbor Graph (MING) is a way to represent the MIIS neighborhood information. MING contains the pertinent information on a set of different types’ neighbors that are candidates to which STAs can handover. This MING is compiled by the MIIS entity in the network side and can include information from measurement reports sent by STAs who successfully performed heterogeneous handovers, as well as the information via the management interface.

MING can be defined as a set of MING vertices and a set of MING edges representing a binary relation between MING vertices.

- MING vertex: A PoA with a list of information elements related with the PoA.

- MING edges: The neighboring relationship between two PoAs. Two PoAs i and j are neighbors if there exists a path of motion between i and j such that it is possible for a mobile STA to perform a handover successfully. For example, if a mobile STA can successfully handover from 802.11 AP1 to a cellular BS2, AP1 and BS2 are neighbors and (AP1, BS2) is a MING Edge.

If a STA is anticipating handover, it might request information service from MIIS entity at network side. Whenever receiving such a request, MIIS entity at network side should reply with a partial set of MING. The partial set shall only contain entries of neighboring PoAs with legitimate relationship of the PoA the STA now makes a connection to. If supported, unsolicited advertisement can be used for distributing the partial set as well. The following Figure 11-1 describes the partial set of MING, which can be obtained when a STA requests neighboring information while connecting to PoA #2.

Figure 32: Partial Set of MING

### Management of Neighboring Network Information

Two PoAs directly connected in MING represents the neighboring relationship. That is to say, a STA can handover between the two PoAs if MING contains a MING edge between the two PoAs. In most cases, this neighboring network information is manually managed by an administrator. Such manual administration is effective if the topology of network is almost fixed and hardly changes. However, some network systems have inherent feature that their topology is a little dynamic. In such network system, the quality of MING can decrease by topology change of existing PoAs. When a PoA is taken away, all the edges to the PoA should be evicted due to their inactivity. If a directional antenna is equipped with a PoA, the change of direction of the antenna may lead the eviction. So, this section describes a dynamic method to construct or manage MING.

Each MING vertex or edge might be managed based on a given timer. Whenever getting a report about a PoA’s existence or a neighboring relation from STAs, MIIS entity at network side should add a MING vertex (or edge) or refresh the existing one. If no STA reports the same MING vertex (or edge) during a given time, the corresponding one is deleted. The information elements of the deleted MING vertex can be stored in a temporal storage to re-use them when the corresponding MING vertex is again added.

A 802.21-compatible STA is recommended to send a report message to MIIS entity at network side. This can occur after any successful handover. This implies that MING vertex (or edge) is created only if roaming agreement is satisfied, when STA handovers between two PoAs within a managed operator’s network. If supported, the report message can be required by the network. Whatever mechanisms, such a report message itself should be verified.

The following section defines the primitives for supporting the dynamic management of neighboring network information

### MING_POA Report

MAC

MIH

MIH

MAC Orig.

MAC New

MAC

MIH

Mobile Terminal

Original Network

Information Server

Successful Handover

New Network

Old Communication

Link Up

MING_POA_REPORT.request

MING_POA_Report.indication

MING_POA_Report.response

MING_POA_REPORT.confirm

REQUEST FRAME

RESPONSE FRAME

### MING_POA_Report.request

#### Function

This primitive is used by STA’s MIH to report the discovered PoAs to MIIS entity at network side. This primitive requests that a MING_POA_REPORT be sent to the MIIS with which the STA is associated.

#### Semantics of service primitive

MING_POA_Report.request

(

Current PoA,

MING_POA_REPORT,

Neighbor Status,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Current PoA |  | N/A | Current PoA the sending STA makes a connection with |
| MING_POA_REPORT |  | N/A | Discovered PoAs and the related ANI, LLI, and HLSI found. |
| Neighbor Status | Yes / Don’t know yet | N/A | For the PoA known after successful handover, the Neighbor Status is “Yes” For the PoA discovered anyhow at any time (except handover), the Neighbor Status is “Don’t know yet” |

[Note: MING_POA_REPORT is not yet clear.]

#### When generated

This primitive is generated by STA’s MIH to report the discovered PoAs to MIIS entity at the network side.

#### Effect on receipt

On receipt of this primitive, the management entity constructs a MING POA Report Request message. The STA then attempts to transmit this to the MIIS entity with which it is associated at the network side.

### MING_POA_Report.confirm

#### Function

This primitive reports the result of a MING_POA_REPORT.

#### Semantics of service primitive

MING_POA_Report.confirm

(

ResultCode,

MING Vertex Set,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PAMAMETERS, REFUSED, TIMEOUT, TRANSMISSION_FALIURE, UNSPECIFIED_FALILURE | Indicates the result of the corresponding MING_POA_Report.request |
| MING Vertex Set |  | N/A | A set of MING Vertexes being reported. Only present if ResultCode = SUCCESS |

#### When generated

This primitive is generated by STA’s management entity as a result of an MING_POA_Report.request and indicates the result of the request.

#### Effect on receipt

On receipt of this primitive, the ResultCode may be evaluated and the reported data may be used.

### MING_POA_Report.indication

#### Function

This primitive indicates that an MING_POA_Report was received from a STA. It is valid only at an MIH capable AP.

#### Semantics of service primitive

MING_POA_Report.indication

(

PeerSTAAddress,

Neighbor Status,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the STA from which a MING_POA_Report was received. |
| Neighbor Status | Yes / Don’t know yet | N/A | For the PoA known after successful handover, the Neighbor Status is “Yes” For the PoA discovered anyhow at any time (except handover), the Neighbor Status is “Don’t know yet” |

#### When generated

This primitive is generated when a valid PoA report is received at the network side.

#### Effect on receipt

On receipt of this primitive, MIIS entity at network side tries to update its MING. If Neighbor Status is “Yes”, it should also update the neighboring relationship between the Current PoA and the reported PoA.

### MING_POA_Report.response

#### Function

This primitive is generated in response to an MING_POA_Report.indication reporting the result of an MING POA REPORT to the peer-STA.

#### Semantics of service primitive

MING_POA_Report.response

(

PeerSTAAddress,

ResultCode,

MING Vertex Set,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the STA from which a MING_POA_Report was received. |
| ResultCode | Enumeration | SUCCESS, INVALID_PAMAMETERS, REFUSED, TIMEOUT, TRANSMISSION_FALIURE, UNSPECIFIED_FALILURE | Indicates the result of the corresponding MING_POA_Report.indication |
| MING Vertex Set |  | N/A | A set of MING Vertexes being reported. Only present if ResultCode = SUCCESS |

#### When generated

This primitive is generated by the network in response to an MING_POA_Report.indication. If the returned ResultCode is SUCCESS, the primitive contains the vertexes to be sent to the STA.

#### Effect on receipt

On receipt of this primitive, the MIH constructs a MING POA Report Response message. The STA then attempts to transmit this to the STA indicated by the PeerSTAAddress parameter.

### Media Independent Neighbor Graph (MING) Query

Information service primitives as defined in Section 8.5  are sufficient for query and response of neighbor information. However the following new filter type is needed in addition to the filters that are defined earlier.

- FILTER_INFO_NEIGHBOR_INFO

InfoQueryParameters while using the above filter includes 802.21neighbor information IE containing a PoA. The format of such neighbor information IE is discussed in section 11.5.1.  On the other hand, MIH_REPORT in query response contains the list of neighbor information IEs along around the POA.

More flexible query with detailed search conditions as well as query for obtaining higher layer information is better performed using the RDF/XML based query as described below.

### Neighbor Information IE

A neighbor information IE is a variable-length IE that carries the identification of a particular POA along with the information that is associated with the particular point of attachment.

| Length (1 byte) | Zero, One or More Neighbor Information Entry (Variable) |
| --- | --- |

- Length: Length specifies the length of the fields after the length field.

- Neighbor Information Entry : Neighbor information entry contains all the information relevant to a particular neighbor and is defined below

| Link Type (1 byte) | POA ID Length (1 byte) | POA ID (Variable) | Link Information length (1 byte) | Link Information (Variable) |
| --- | --- | --- | --- | --- |

- Link Type : 1  --  802.3 , 2 --- 802.11, 3- 802.16 ,4—GSM , 5—GPRS ,6—CDMA2000,  7--- WCDMA

- POA ID length: POA ID length is the length of the POA ID field

- POA ID:  POA ID is the identification of the point of attachment of a specific link type.  For example, POA ID can be MAC address   of the AP, BS or Ethernet switch.

- Link Information length: Link information length specifies the length of the link information

- Link Information: Link information contains the information related to that particular link

## Schema Representation

### Extended Schema Representation in RDF/XML: An Example

Following example describes an extended schema definition represented in RDF/XML format.

<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf 'http://www.w3.org/1999/02/22-rdf-syntax-ns#'>

<!ENTITY rdfs 'http://www.w3.org/2000/01/rdf-schema#'>

<!ENTITY mihbase 'URL_TO_BE_ASSIGNED/#'>

<!ENTITY mihextended 'URL_TO_BE_ASSIGNED/#'>

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;" xmlns:mihbase="&mihbase;" xmlns:mihextended="&mihextended;"

xml:base="&mihextended;">

<mihbase:Link-Type rdf:ID="ieee802.3"/>

<mihbase:Link-Type rdf:ID="ieee802.11"/>

<mihbase:Link-Type rdf:ID="ieee802.16"/>

<mihbase:Link-Type rdf:ID="gsm"/>

<mihbase:Link-Type rdf:ID="gprs"/>

<mihbase:Link-Type rdf:ID="cdma2000"/>

<mihbase:Link-Type rdf:ID="w-cdma"/>

<rdf:Property rdf:ID="nsp-code">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="nsp-tariff">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdfs:Class rdf:ID="IEEE802.11">

<rdfs:subClassOf rdf:resource="&mihbase;L2"/>

</rdfs:Class>

<rdf:Property rdf:ID="l2-standard">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#L2-Standard"/>

</rdf:Property>

<rdfs:Class rdf:ID="L2-Standard"/>

<mihextended:L2-Standard rdf:ID="ieee802.11a"/>

<mihextended:L2-Standard rdf:ID="ieee802.11b"/>

<mihextended:L2-Standard rdf:ID="ieee802.11g"/>

<mihextended:L2-Standard rdf:ID="ieee802.16e"/>

<rdf:Property rdf:ID="ssid">

<rdfs:domain rdf:resource="#IEEE802.11"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="channel">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="phy">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#Phy"/>

</rdf:Property>

<rdfs:Class rdf:ID="Phy"/>

<mihextended:Phy rdf:ID="cck"/>

<mihextended:Phy rdf:ID="dsss"/>

<mihextended:Phy rdf:ID="dfdm"/>

<rdf:Property rdf:ID="l2-data-rates">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="l2-chipher-suites">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#L2-Cipher-Suites"/>

</rdf:Property>

<rdfs:Class rdf:ID="L2-Cipher-Suites">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="WEP">

<rdfs:subClassOf rdf:resource="#L2-Cipher-Suites"/>

</rdfs:Class>

<rdfs:Class rdf:ID="TKIP">

<rdfs:subClassOf rdf:resource="#L2-Cipher-Suites"/>

</rdfs:Class>

<rdfs:Class rdf:ID="AES-CCMP">

<rdfs:subClassOf rdf:resource="#L2-Cipher-Suites"/>

</rdfs:Class>

<rdf:Property rdf:ID="authenticated-key-management-suites">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#Authenticated-Key-Management-Suites"/>

</rdf:Property>

<rdfs:Class rdf:ID="Authenticated-Key-Management-Suites">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="Psk">

<rdfs:subClassOf rdf:resource="#Authenticated-Key-Management-Suites"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.1x">

<rdfs:subClassOf rdf:resource="#Authenticated-Key-Management-Suites"/>

</rdfs:Class>

<rdf:Property rdf:ID="l2-key-management-protocol">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#L2-Key-Management-Protocol"/>

</rdf:Property>

<rdfs:Class rdf:ID="L2-Key-Management-Protocol">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.11i-4Way-Hand-Shake">

<rdfs:subClassOf rdf:resource="#L2-Key-Management-Protocol"/>

</rdfs:Class>

<rdf:Property rdf:ID="l2-qos">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#L2-QoS"/>

</rdf:Property>

<rdfs:Class rdf:ID="L2-QoS">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.11e">

<rdfs:subClassOf rdf:resource="#L2-QOS"/>

</rdfs:Class>

<rdf:Property rdf:ID="cost">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="ieee802.11-roaming-list">

<rdfs:domain rdf:resource="#IEEE802.11"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="l2-mobility">

<rdfs:domain rdf:resource="&mihbase;L2"/>

<rdfs:range rdf:resource="#L2-Mobility"/>

</rdf:Property>

<rdfs:Class rdf:ID="L2-Mobility">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.11r">

<rdfs:subClassOf rdf:resource="#L2-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.11u">

<rdfs:subClassOf rdf:resource="#L2-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.21">

<rdfs:subClassOf rdf:resource="#L2-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="L2-Pre-Auth">

<rdfs:subClassOf rdf:resource="#L2-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IEEE802.16">

<rdfs:subClassOf rdf:resource="&mihbase;L2"/>

</rdfs:Class>

<rdf:Property rdf:ID="bsid">

<rdfs:domain rdf:resource="#IEEE802.16"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdfs:Class rdf:ID="GSM">

<rdfs:subClassOf rdf:resource="&mihbase;L2"/>

</rdfs:Class>

<rdf:Property rdf:ID="cid">

<rdfs:domain rdf:resource="#GSM"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdfs:Class rdf:ID="GPRS">

<rdfs:subClassOf rdf:resource="&mihbase;L2"/>

</rdfs:Class>

<rdfs:Class rdf:ID="CDMA2000">

<rdfs:subClassOf rdf:resource="&mihbase;L2"/>

</rdfs:Class>

<rdfs:Class rdf:ID="W-CDMA">

<rdfs:subClassOf rdf:resource="&mihbase;L2"/>

</rdfs:Class>

<rdf:Property rdf:ID="router-address">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="dhcp-server-address">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="domain-name">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="subnet">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="sip-server-address">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="l3-key-management-protocol">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="#L3-Key-Management-Protocol"/>

</rdf:Property>

<rdfs:Class rdf:ID="L3-Key-Management-Protocol">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IKEv1">

<rdfs:subClassOf rdf:resource="#L3-Key-Management-Protocol"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IKEv2">

<rdfs:subClassOf rdf:resource="#L3-Key-Management-Protocol"/>

</rdfs:Class>

<rdf:Property rdf:ID="l3-authentication">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="#L3-Authentication"/>

</rdf:Property>

<rdfs:Class rdf:ID="L3-Authentication">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="PANA">

<rdfs:subClassOf rdf:resource="#L3-Authentication"/>

</rdfs:Class>

<rdf:Property rdf:ID="paa-address">

<rdfs:domain rdf:resource="#PANA"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="ep-address">

<rdfs:domain rdf:resource="#PANA"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdfs:Class rdf:ID="UAM">

<rdfs:subClassOf rdf:resource="#L3-Authentication"/>

</rdfs:Class>

<rdf:Property rdf:ID="l3-packet-chiphering">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="#L3-Packet-Ciphering"/>

</rdf:Property>

<rdfs:Class rdf:ID="L3-Packet-Ciphering">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="IPsec">

<rdfs:subClassOf rdf:resource="#L3-Packet-Ciphering"/>

</rdfs:Class>

<rdf:Property rdf:ID="vpn-gateway-address">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="isp-code">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="isp-name">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="isp-tariff">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="l3-mobility">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="#L3-Mobility"/>

</rdf:Property>

<rdfs:Class rdf:ID="L3-Mobility">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

<rdfs:Class rdf:ID="MIP">

<rdfs:subClassOf rdf:resource="#L3-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="MIPv4">

<rdfs:subClassOf rdf:resource="#MIP"/>

</rdfs:Class>

<rdfs:Class rdf:ID="MIPv6">

<rdfs:subClassOf rdf:resource="#MIP"/>

</rdfs:Class>

<rdf:Property rdf:ID="home-agent-address">

<rdfs:domain rdf:resource="#MIP"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdf:Property rdf:ID="foreign-agent-address">

<rdfs:domain rdf:resource="#MIP"/>

<rdfs:range rdf:resource="&rdfs;Literal"/>

</rdf:Property>

<rdfs:Class rdf:ID="CT">

<rdfs:subClassOf rdf:resource="#L3-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="CARD">

<rdfs:subClassOf rdf:resource="#L3-Mobility"/>

</rdfs:Class>

<rdfs:Class rdf:ID="L3-Pre-Authentication">

<rdfs:subClassOf rdf:resource="#L3-Mobility"/>

</rdfs:Class>

<rdf:Property rdf:ID="l3-qos">

<rdfs:domain rdf:resource="&mihbase;L3"/>

<rdfs:range rdf:resource="#L3-QoS"/>

</rdf:Property>

<rdfs:Class rdf:ID="L3-QoS">

<rdfs:subClassOf rdf:resource="&rdfs;Resource"/>

</rdfs:Class>

</rdf:RDF>

### ASN.1 Representation of Extended Information Elements (An Example)

MIH-INFORMATION-SERVICES-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, NOTIFICATION-TYPE,

Integer32, Counter32, IpAddress, enterprises

FROM SNMPv2-SMI

TEXTUAL-CONVENTION, RowStatus, DisplayString, TruthValue,

FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP, NOTIFICATION-GROUP

FROM SNMPv2-CONF

;

-- For information only. Neeed to acquire the official MIB identifier:

mih OBJECT IDENTIFIER ::= { iso(1) member-body(2) us(840) ieee802dot21(10999) }

is OBJECT IDENTIFIER ::= { mih 2 }

mihIsMIB MODULE-IDENTITY

LAST-UPDATED "200504271400Z"  -- 27 April 2005 14:00:00 EDT

ORGANIZATION "Lucent Technologies"

CONTACT-INFO "

Peretz Feder

15H-228 67 Whippany Rd

Whippany, NJ  07981-0903  U S

Tel:	+1 973 386 6976

Email: pfeder@lucent.com

-- Additional contacts can be added here

DESCRIPTION "

This MIB module contains manged objects for the MIH Information Services"

REVISION "200504271400Z"  -- 27 April 2005 14:00:00 EDT

DESCRIPTION "

Initial version."

::= { is 2 }

BSAttributes ::= SEQUENCE

BEGIN

{

BSID_LEN			INTEGER,

BSID				INTEGER,

ServiceProviderName		IA5String,

State				IA5String OPTIONAL,

Country				IA5String OPTIONAL,

AccessTechnology ::= CHOICE

{

802.3, 802.11a, 802.11b, 802.11g, 802.11n, 802.16d, 802.16e, GSM, GPRS, EDGE, UMTS, HSDPA, 3G1X, EV-DO-Rel0, EV-DO-RevA, EV-DV

},

NumSectors			INTEGER OPTIONAL,

ChannelBandwidthMHz		REAL,

StartChannelMHz		REAL,

DLPeakDataRateKbps		REAL,

ULPeakDataRateKbps		REAL,

SecuritySuite ::= SEQUENCE

{

Authentication		ENUMERATED {None, EAP-TLS, EAP-TTLS, EAP-SIM, EAP-AKA, 802.1x },

EncryptionMode		ENUMERATED {None, WEP, AES},

EncryptionAlgorithm	ENUMERATED {None}

KeyProvisioning		ENUMERATED {None, PSK},

KeyManagement		ENUMERATED {None, TKIP, WPA-PSK, AES}

},

Currency			IA5String DEFAULT “US$”,

ClassCost ::= SEQUENCE

{

CoS			IA5String,

Cost			REAL,

QoS ::= SEQUENCE

{

DataRateKbps		REAL,

MinLatencyMsec	INTEGER,

MaxLatencyMsec	INTEGER,

MaxJitterMsec		INTEGER

}

},

ClassofService			BITString {RealTime, DelaySensitive, DelayInSensitive, BestEffort},

AccessRouterInfo ::= SEQUENCE

{

AccessRouterAddress		IA5String,

IPversion			ENUMERTAED {IPv4, IPv6},

MobilityProtocolSupport		ENUMERATED {None, MIPv4, MIPv6, SIP},

FASupport			BOOLEAN DEFAULT FALSE OPTIONAL

}

CartesianCoordinateLocation ::= SEQUENCE (this could supply access tech info as well per location

{

Latitude		[0]	REAL OPTIONAL,

Longitude	[1]	REAL OPTIONAL,

Altitude		[2]	REAL OPTIONAL

},

}

END

--The RequestingBSID in the following structure is the BS with respect to which

--the NeighorList is generated. The number of neighbors the information element

--returns is dependent on the requesting BSID.

--NeighborList ::= SEQUENCE

--{

--	RequestingBSID	INTEGER,

--	NumNeighbors		INTEGER,

--	BS	[0]		ANY DEFINED BY BSAttributes,

--	BS	[1]		ANY DEFINED BY BSAttributes,

--	 :	 :		     :

--	 :	 :		     :

--	BS	[N]		ANY DEFINED BY BSAttributes

--}

NeighborList 	OBJECT-TYPE

SYNTAX      NeighborList

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"."

::= { mihIsMIB 12 }

--Max of 10 statically allocated Neighbors

--simplistic  model

NeighborList ::=

SEQUENCE {

RequestingBSID	INTEGER,

NumNeighbors		INTEGER,

BS1				OCTET STRING,

BS2				OCTET STRING,

BS3				OCTET STRING,

BS4				OCTET STRING,

BS5				OCTET STRING,

BS6				OCTET STRING,

BS7				OCTET STRING,

BS8				OCTET STRING,

BS9				OCTET STRING,

BS10				OCTET STRING

}

RequestingBSID	OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 1 }

NumNeighbors		OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 2 }

BS1	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 3 }

BS2	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 4 }

BS3	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 5 }

BS4	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 6 }

BS5	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	7 }

BS6	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 8 }

BS7	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 9 }

BS8	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 10 }

BS9	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 11 }

BS10	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { NeighborList 12 }

--Similarly, RequestingServiceProviderName is the following structure is the

--Service Provider with respect to which the SLAList is generated. The number

--of service provider entries that the information element returns is dependent

--on the specific requesting service provider.

--SLAList ::=

--{

--	RequestingServiceProviderName	IA5String,

--	NumSLAServiceProvider		INTEGER,

--	SP	[0]				IA5String,

--	SP	[1]				IA5String,

--	 :	 :				  :

--	 :	 :				  :

--	SP	[N]				IA5String

-}

SLAList 	OBJECT-TYPE

SYNTAX      SLAList

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"."

::= { mihIsMIB 13 }

--Max of 10 statically allocated SLAs

--simplistic  model

SLAList 	::=

SEQUENCE {

RequestingServiceProviderName	DisplayString,

NumSLAServiceProvider		INTEGER,

SP1				DisplayString,

SP2				DisplayString,

SP3				DisplayString,

SP4				DisplayString,

SP5				DisplayString,

SP6				DisplayString,

SP7				DisplayString,

SP8				DisplayString,

SP9				DisplayString,

SP10				DisplayString

}

RequestingServiceProviderName	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	1 }

NumSLAServiceProvider		OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	2 }

SP1	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	3 }

SP2	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	4 }

SP3	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	5 }

SP4	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	6 }

SP5	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	7 }

SP6	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	8 }

SP7	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	9 }

SP8	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	10 }

SP9	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	11 }

SP10	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { SLAList 	12 }

--This provides the access technology list in order of the preference as provisioned

--by the service provider in case an inter-technology handover is desired.

--This preference list view is Service Provider specific and would change with the

--requesting service provider name. This information is provided to the terminal as

--part of selecting the preferred access technology as provisioned by the service provider.

--This information is not unique to a particular user but rather the service provider

--global preference attributes.

--AccessPreferenceList ::=

--{

--	RequestingServiceProviderName	IA5String,

--	NumAccessTechnologies		INTEGER,

--	AccessTechnology	[0]		IA5String,

--	AccessTechnology	[1]		IA5String,

--	  :			 :		  :

--	  :			 :		  :

--	AccessTechnology	[N]		IA5String

--}

AccessPreferenceList OBJECT-TYPE

SYNTAX      AccessPreferenceList

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"."

::= { mihIsMIB 14 }

--Max of 10 statically allocated access technologies

--simplistic  model

AccessPreferenceList ::=

SEQUENCE {

RequestingServiceProviderName	DisplayString,

NumAccessTechnologies		INTEGER,

AccessTechnology1		DisplayString,

AccessTechnology2		DisplayString,

AccessTechnology3		DisplayString,

AccessTechnology4		DisplayString,

AccessTechnology5		DisplayString,

AccessTechnology6		DisplayString,

AccessTechnology7		DisplayString,

AccessTechnology8		DisplayString,

AccessTechnology9		DisplayString,

AccessTechnology10		DisplayString

}

RequestingServiceProviderName	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 1 }

NumAccessTechnologies		OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 2 }

AccessTechnology1		OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 3 }

AccessTechnology2 	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 4 }

AccessTechnology3	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 5 }

AccessTechnology4	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 6 }

AccessTechnology5	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 7 }

AccessTechnology6	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 8 }

AccessTechnology7	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 9 }

AccessTechnology8	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 10 }

AccessTechnology9	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 11 }

AccessTechnology10	OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

". "

::= { AccessPreferenceList 12 }

--PreferenceListPolicyOverride ::=

--{

--	UserSpecificConfigurationOverride	BOOLEAN DEFAULT TRUE,

--	ServiceProviderOverride		BOOLEAN DEFAULT FALSE,

--	UserOverride				BOOLEAN DEFAULT FALSE

--}

--The PreferenceListPolicyOverride is to specify how user based policies

--could be used. The UserSpecificConfigurationOverride being TRUE implies that the

--policies that are downloaded or set by a user specific configuration would override

--any policies that are provided by the Information Services.

PreferenceListPolicyOverride OBJECT-TYPE

SYNTAX      PreferenceListPolicyOverride

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PreferenceListPolicyOverride is to specify how user based policies

could be used. The UserSpecificConfigurationOverride being TRUE implies that the

policies that are downloaded or set by a user specific configuration would override

any policies that are provided by the Information Services."

::= { mihIsMIB 15 }

PreferenceListPolicyOverride ::=

SEQUENCE {

UserSpecificConfigurationOverride	TruthValue,

ServiceProviderOverride			TruthValue,

UserOverride				TruthValue

}

UserSpecificConfigurationOverride	OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Default Value is TRUE."

::= { PreferenceListPolicyOverride 1 }

ServiceProviderOverride		OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Default Value is FALSE"

::= { PreferenceListPolicyOverride 2 }

UserOverride			OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Default Value is FALSE"

::= { PreferenceListPolicyOverride 3 }

-- Object(s) associated with 802.3 Interaction

-- Object(s) associated with 802.11 Interaction

AccessPointID ::= SEQUENCE

{

BSSID		IA5String,

ESSID		IA5String

}

-- Object(s) associated with 802.16 Interaction

-- Object(s) associated with 3GPP Interaction

-- Object(s) associated with 3GPP2 Interaction

-- Object(s) associated with IP Interaction

END

## Media specific Amendments

### 802.3 MAC/PHY Amendments

A new ethertype value must be defined for proper accommodation of MIH PDUs in IEEE 802.3 data frames.

### 802.11 MAC/PHY Amendments

The following 802.11 specific amendments are recommended.

- Broadcast based MIH capability discovery

- The indication of MIH capability in the Capability Information field of the 802.11 beacon frame body. The 802.11 access point periodically transmits a beacon frame to announce its presence and relay information (such as timestamp, SSID, and other parameters regarding the access point) to 802.11 network interfaces (NIC's) that are within range. The 802.11 radio NICs continually scan all 802.11 radio channels and listen to beacons as the basis for choosing which access point is best to associate with. This proposal recommends using the Capability Information field in the beacon frame body to advertise MIH capability. The access point can set the MIH-capability bit to indicate that the 802.11 network is MIH-capable and therefore able to facilitate MIH handover for multi-modal subscriber devices.

- MIH_MGMT_SAP based amendments

The MIH_MGMT_SAP identifies a set of primitives that need to be supported in the management plane. Appropriate amendments need to be planned for MLME based SAPs. These primitives specify additional capabilities for Event Service, Command Service and Information service.

- Information Service based amendments

Many media specific essential information elements have been identified. These need to be supported in the access network specific standard. Information elements specific to 802.11 access network need to be supported in the base specification as well.

- Transport based amendments

New Action frames need to be defined that can transfer MIH Services related messages between the station and the AP.

### 802.16 MAC/PHY Amendments

#### MIH Support and Primitives in 802.16

##### Overview of management model for MIH

The IEEE802.21 specification shall define generic SAPs and other primitives that provide generic link layer intelligence. IEEE 802.16 thereafter needs to provide corresponding media specific SAPs and primitives which can satisfy these generic abstractions as specified by IEEE 802.21 specification.

##### 802.16 MIH Management Interfaces

Management entity in MS or BS interfaces with Network Control and Management System (NCMS). NCMS needs to be extended to have functions like SME in 802.11.

NCMS is a layer-independent entity. MIH Function layer in MS or BS can interface with management plane either directly with MIH_MGMT_SAP or MIH_ME_SAP via NCMS. When MIH Function layer communicates through NCMS, NCMS receives primitives through MIH_ME_SAP and further delivers them to management plane through M_SAP or C_SAP.

- MIH_MGMT_SAP: This defines the interface between the MIH Function layer and the management plane of 802.16. This SAP can be used for sending MIH messages between peer MIH entities. Messages based on management frames can be sent in the unauthenticated state as well. The MIH_MGMT_SAP defines primitives for MIES, MICS and MIIS.

- MIH_ME_SAP: This SAP defines the interface between MIH Function layer and the NCMS. This SAP can be the same as MIH_MGMT_SAP as well.

##### Extension of Network Control and Management System (NCMS)

NCMS is required to be extended to support MIH event services, command services, and information services. When NCMS in terminal or BS interfaces with MIH Function layer, NCMS may provide means for MIH Function layer to interface with management plane in MS or BS.

In case of NCMS in BS, further extension may be required for information services in a network agnostic manner.

Figure 34. Illustration of the Network Control and Management System

##### 802.16 Primitives for MIH

[TBD]

#### MIH Capability Delivery in 802.16

##### Overview of MIH Capability Delivery

There are two ways for MS to obtain MIH Capability of IEEE 802.16 BSs. One is the method of Pre-acquisition before handover, and the other is the method of Post-acquisition after handover.

##### Pre-MIH Capability Acquisition

MS may obtain the information whether there is an available other type of PoAs with or without MIH Capability by receiving MOB_NBR-ADV message from the Serving BS before performing handover from 802.16 system to other interface networks.

In MOB_NBR-ADV message, MIH enabled 802.16 BS may provide MSs with MIH Capability information of other interface PoAs (e.g, WLAN AP, or 3GPP/3GPP2 BS). MIH INFO bitmap indicates availability and MIH capability of the WLAN AP and Cellular System BS which is located near the serving 802.16 BS. For each bit location, a value of  “1” indicates the correspondent is supported.

| Syntax | Size | Notes |
| --- | --- | --- |
| MOB_NBR-ADV_Message_Format() { |  |  |
| Management Message Type = 53 | 8 bits |  |
| Skip-Optional-Fields bitmap | 8 bits | Bit [0]: if set to ‘1’, omit Operator ID field Bit [1]: if set to ‘1’, omit NBR BS ID field Bit [2]: if set to ‘1’, omit HO process optimization field Bit [3]: if set to ‘1’, omit QoS related fields Bit [4]: if set to ‘1’, omit MIH INFO bitmap Bit [5]-[7]: reserved |
| …… | …… | …… |
| If (Skip-Optional-Fields-[4]=0) { |  |  |
| MIH INFO bitmap | 16bits | [0] Available WLAN AP [1] Available WLAN AP MIH Enabled [2] Available WLAN AP MIH Capability unknown [3] Available 3GPP BS [4] Available 3GPP BS MIH Enabled [5] Available 3GPP BS MIH Capability unknown [6] Available 3GPP2 BS [7] Available 3GPP2 BS MIH Enabled [8] Available 3GPP2 BS MIH Capability unknown [9]-[15] reserved |
| } |  |  |
| } |  |  |

Table XX. MOB-NBR-ADV management message.

##### Post-MIH Capability Acquisition

MS may obtain MIH Capability information for target BS by receiving DL-MAP message during network entry procedure after handover from other interface networks to 802.16 system.

BS could inform MS of its MIH Capability information through DL-MAP message, and MS obtains MIH Capability for serving BS after receiving DL-MAP message.

Table XX shows MIH_Capability_IE that could be included in DL-MAP message

| Syntax | Size | Notes |
| --- | --- | --- |
| MIH_Capability_IE() { |  |  |
| Extended DIUC | 4bits | MIH_Capability_IE = XX |
| Length | 4bits | Length = 0x01 |
| MIH Capability | 1bit | 0 : MIH Not Supported 1 : MIH Supported |
| } |  |  |

Table XX . DL-MAP message

#### MIH Message Delivery in 802.16

.

IEEE 802.16 shall only provide ways for MIHF to deliver MIH messages, and shall not process MIH related messages.

##### MIH message delivery through Data Plane

MIH message delivery through data plane requires CS extension to support new service flow for MIH.

Information Service Message Delivery

Remote Event/Command Service Message Delivery

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].28 | 1 | 0: No CS 1: Packet, IPv4 2: Packet, IPv6 3: Packet, 802.3/Ethernet 4: Packet, 802.1Q VLAN 5: Packet, IPv4 over 802.3/Ethernet 6: Packet, IPv6 over 802.3/Ethernet 7: Packet, IPv4 over 802.1Q VLAN 8: Packet, IPv6 over 802.1Q VLAN 9: ATM 10: Packet, IPv4 with Header Compression (ROHC) 11: Packet, IPv4 with Header Compression (ECRTP) 12: Packet, IPv6 with Header Compression (ROHC) 13: Packet, IPv6 with Header Compression (ECRTP) 14: Packet, IPv4 over 802.3/Ethernet with Header Compression (ROHC) 15: Packet, IPv4 over 802.3/Ethernet with Header Compression (ECRTP) 16: Packet, IPv6 over 802.3/Ethernet with Header Compression (ROHC) 17: Packet, IPv6 over 802.3/Ethernet with Header Compression (ECRTP) 18: Packet, IPv4 over 802.1Q VLAN with Header Compression (ROHC) 19: Packet, IPv4 over 802.1Q VLAN with Header Compression (ECRTP) 20: Packet, IPv6 over 802.1Q VLAN with Header Compression (ROHC) 21: Packet, IPv6 over 802.1Q VLAN with Header Compression (ECRTP) 22: MIH (Media Independent Handover) 23~255: reserved | DSx-REQ |

### Cellular specific Amendments

#### Events in MIH_RRC_SAP for 3GPP

As per the current 3GPP standard, PHY and MAC SAPs would be used to communicate information to the RRC as shown in the Figure x below.  The RRC will communicate PHY and MAC information to the MIH using a MIH-RRC SAP.  The list of event s and triggers carried over MIH RRC SAP are as follows.

- 3GPP-Link-Up: This trigger specifies that the link is up. This trigger should be sent from the LAC to the MIH after the RRC determines that both the MAC and PHY are up.

- 3GPP-Link-Down: This trigger specifies that the link is down. This trigger should be sent from the LAC to the MIH after the RRC determined that either the MAC or the PHY is down.

- 3GPP-Link-Going-Down: This trigger specifies that the link is going down in the “near future”. This trigger should be based on the fact that either the MAC or the PHY is going down.

- 3GPP-Link-Signal-Strength: This trigger indicates the signal strength at the physical layer and it is expected that the RRC receive this information from the PHY as part of the PHY SAP.

- 3GPP-Link-SNR: This trigger indicates the signal-to-noise ratio at the physical layer.  Again, it is expected that the RRC receive this information from the PHY as part of the PHY SAP.

#### Events in MIH_LAC_SAP for 3GPP2

- 3GPP2-Link-Up: This trigger specifies that the link is up. This trigger should be sent from the LAC to the MIH after the LAC determines that both the MAC and Phy are up.

- 3GPP2-Link-Down: This trigger specifies that the link is down. This trigger should be sent from the LAC to the MIH after the LAC determined that either the MAC or the Phy is down.

- 3GPP2-Link-Going-Down: This trigger specifies that the link is going down in the “near future”. This trigger should be based on the fact that either the MAC or the Phy is going down.

- 3GPP2-Link-Signal-Strength: This trigger indicates the signal strength at the physical layer and it is expected that the LAC receive this information from the Phy as part of the Phy SAP.

- 3GPP2-Link-SNR: This trigger indicates the signal-to-noise ratio at the physical layer.  Again, it is expected that the LAC receive this information from the Phy as part of the Phy SAP.

#### Events in MIH_PPP_SAP for 3GPP2

PPP triggers are sent over the 3GPP-PPP SAP during both the Link Control Phase and the Network Control Phase of PPP link establishment. These are triggers that can be produced within the current PPP state machine implementation as shown illustrated below and thus does not require changes to the normal PPP state machine implementation.

There are two phases to the establishment of a PPP session in 3GPP2 namely the Network Control Phase (NCP) and the Link Control Phase (LCP). The Network Control Phase is a component within the PPP state machine and takes place when the Link Control Protocol has opened the PPP link. The Link Control Phase continues until the Link is terminated. We propose that PPP implementations provide the capability for these triggers to be generated and communicated to the MIH using the 3GPP2-PPP-SAP.

Figure 33: PPP (LCP, IPCP) state machine applicable to 3GPP and 3GPP2.

The proposed triggers are summarized below with appropriate descriptions. We first consider the triggers generated during the LCP and then the NCP.

#### LCP Triggers

As shown in the above state machine sequence for PPP link negotiation, maintenance and termination, when a PPP link is to be established, the availability of the physical layer is checked and if it is available it is deemed that PPP negotiation could be initiated. This is indicated by the Established state. From this state, the end-points exchange LCP parameters to open the LCP link. If this parameter exchange fails due to some reason, a 3GPP2-LCP-Configure-Failure event is triggered and the PPP link establishment fails. If the parameter exchange is successful, the end-points move into the LCP_Authenticate state. During this transition a

3GPP2-LCP-Link-Open trigger is sent which indicates that the link is open but authentication (that is CHAP/PAP) is yet to be performed. Once the authentication is successful, the state machine moves into the LCP_Opened state. The successful authentication triggers a 3GPP2-LCP-Link-Up event. If the authentication is unsuccessful, a 3GPP2-LCP-Auth-Failure event is triggered and the link is terminated by moving the state machine to the LCP_Terminate state. When the PPP link is to be closed, it could be initiated both by the local end-point or the remote-end point. In either case, when appropriate messages are received, the state machine moves to the LCP_Terminate state. Depending on who initiated the closing of the PPP link, appropriate triggers 3GPP2-LCP-Closing trigger is initiated.

In addition, three triggers that are not dependent on state transitions but could happen any time when the state machine is in either the Established, LCP_Authenticate and LCP_Opened states are:

3GPP2-LCP-Carrier-Failure,, which indicates the failure of a lower layer link and that the PPP link will be terminated.

3GPP2-LCP-Link-Quality-Failure, which indicates that the link quality is below a configured threshold (this does not necessarily mean that PPP link should be terminated), and

3GPP2-LCP-Timeout,  which indicates that the PPP link will be terminated because of some time-out (possibly because an expected message does not arrive before the expiration of a timer).

To summarize the LCP triggers:

3GPP2-LCP-Configure-Failure: Link parameter configuration failed.

3GPP2-LCP-Link-Open: Link is up but authentication is not complete (pre-authentication trigger).

3GPP2-LCP-Link-Up: Link is up and authentication has been completed (post-authentication trigger).

3GPP2-LCP-Auth-Failure: Link authentication failed.

3GPP2-LCP-Closing: Link is being closed (normal closing)

3GPP2-LCP-Carrier-Failure: Link failed because of lower layer link failure (abnormal closing).

3GPP2-LCP-Link-Quality-Failure: Link quality below configured threshold.

3GPP2-LCP-Timeout: Link terminated because of timeout (abnormal closing).

#### NCP Triggers

Within the LCP_Opened state, during NCP, the sub-protocol of interest is the IPCP. The (sub) state machine for IPCP is shown within the LCP_Opened state in the above figure. From the IPCP_Open state, if the IPCP parameter configuration is successful, the state machine moves to the IPCP_Opened state and the trigger 3GPP2-IPCP-Link-Open  is generated. If the parameter configuration is unsuccessful, the state machine moves to the IPCP_Close state and the trigger 3GPP2-IPCP-Config-Failure is generated. If the IPCP link is closed normally, the state machine moves from the IPCP_Opened state to the IPCP_Closed state and the trigger 3GPP2-IPCP-Link-Closed is generated. In addition, when IPCP parameters are being exchanged, if there is a time-out event, a 3GPP2-IPCP-Timeout trigger is generated and the state machine moves to the IPCP_Closed state.

To summarize the IPCP triggers:

3GPP2-IPCP-Link-Open: Link is open.

3GPP2-IPCP-Link-Closed: Link is closed normally.

3GPP2-IPCP-Config-Failure: IPCP parameter configuration failure and link is closed abnormally.

3GPP2-IPCP-Timeout: Link is closed abnormally because of timeout.