| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Suggested Amendments to Draft 802.21 Specification |  |
| Date Submitted | 2005-07-11 |  |
| Source(s) | Ronny Kim, Jin Lee LG Electronics, Inc. 533,Hogye-1dong,Dongan-gu, Anyang-shi,Kyongki-do,Korea | Voice: +82-31-450-1879 Fax: +82-31-450-7912 [mailto : {ronnykim, jins978}@lge.com] |
| Re: | IEEE 802.21 Session #09 in San Francisco |  |
| Abstract | This document contains proposed text for fixing a number of places in the 802.21 initial draft, 21-05-0271-00-0000-One_Proposal_Draft_Text. Summary of the text proposal: - Missing primitives and messages - MIH_Handover_Commit is defined but corresponding primitives and MIH messages are missing - MIH_Handover_Initiate.request/response primitives are defined but MIH messages are missing. - Change the figure 27 to make more generic. - Insert “Type” column in MIH Function messages for standard TLV format. |  |
| Purpose | This document proposes text changes on the draft 802.21 specification. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

## 7.4 MIH_SAP Primitives

Insert the subsection, 7.4.12 as indicated below:

### MIH_Handover_Commit

#### MIH_Handover_Commit.request

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function on a network. The primitive is used to communicate performing the handover based on selected choices for network and PoA.

#### Semantics of service primitive

MIH_Handover_Commit.request

(

CommandSource,

CommandDestination,

MACNewPoA

CurrentLinkAction

)

Local or Remote: Both

MIHF (Terminal) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| MACNewPoA | MAC Address |  | MAC Address of New Point of Attachment. |
| CurrentLinkAction | Bitmap |  | Specifies suggested action on old link once handover procedure have been executed. 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4~7: Reserved |

#### When generated

This primitive is generated when successful MIH_Handover_Initiate.response is received to allow a terminal begin handover and the mobile terminal decides to perform actual handover based on selected choices for network and PoA.

#### Effect on receipt

The recipient may identify that a terminal starts handovers to the selected network.

#### MIH_Handover_Commit.response

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that sent out a MIH_Handover_Commit.request. This primitive is used to communicate the response of handover commit request.

#### Semantics of service primitive

MIH_Handover_Commit.response

(

CommandSource,

CommandDestination,

CurrentLinkAction,

)

Local or Remote: Both

MIHF (old Network) <> MIHF (new Suggested Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkAction | Bitmap |  | Specifies suggested action on old link once handover procedure have been executed. 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4~7: Reserved |

#### When generated

This primitive is generated in response to a MIH_Handover_Commit.request primitive.

#### Effect on receipt

The recipient may begin to set up layer 2 connection.

## 8.3 Description

Replace the figure 27 with the figure below:

## 8.4 MIH Function Messages

Insert “type column” in all tables in the 8.4 section following the example below; editor shall insert type column in all tables in the 8.4 section and assign subsequent values in ascending order:

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| SupportedEventList | 1 | 4 | For each Bit location, a value of ‘1’ indicates the link event is supported. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |
| SupportedCommandList | 2 | 4 | For each Bit location, a value of ‘1’ indicates the link command is supported. Bit #0: Link Power Up Bit #1: Link Power Down Bit #2: Link Configure Bit #3: Link Connect Bit #4: Link Disconnect Bit #5: Link Sleep Bit #6: Link Scan Bit #7: Link Poll Bit #8~31: Reserved |

### MIH Command Service Packet Format

Insert the subsection, 8.4.4.1, 8.4.4.2, 8.4.4.3, 8.4.4.4 as indicated below:

#### MIH_Handover_Initiate.request

This message is delivered by MIH Function on a terminal to communicate with peer MIH Function on a network. The primitive is used to communicate intent of handover initiation. The handover can be initiated either by the client or by the network.

| Name | Length | Value |
| --- | --- | --- |
| SuggestedNewLinkIdentifier | Variable | The identifier of new network to which handover needs to be initiated |
| SuggestedNewPoAIdentifier | Variable | MAC address of the preferred Point of Attachment (AP/BS) on new network |
| HandoverMode | 1 | Handover mode may influence the manner in which links are prepared for handover 0: Make-Before-Break 1: Break-Before-Make 2~7: Reserved |
| CurrentLinkAction | 1 | Specifies suggested action on old link once handover procedures have been executed. 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_NO_ACTION 3~7: Reserved |

#### MIH_Handover_Initiate.response

This message is delivered by MIH Function on a terminal to communicate with peer MIH Function on a network. The primitive is used to communicate intent of handover initiation. The handover can be initiated either by the client or by the network.

| Name | Length | Value |
| --- | --- | --- |
| HandoverAck | 1 | The result to the handover request 0: Initiate Handover 1: Abort Handover |
| PreferredLinkIdentifier | Variable | The identifier of new network to which handover needs to be initiated |
| PreferredPoAIdentifier | Variable | MAC address of the preferred Point of Attachment (AP/BS) on new network |
| AbortReason | 1 | Only included when HandoverAck is 1: Abort Handover |

#### MIH_Handover_Commit.request

This message is delivered by MIH Function on a terminal to commit to do the handover based on selected choices for network and PoA.

| Name | Length | Value |
| --- | --- | --- |
| MACNewPoA | Variable | MAC Address of New Point of Attachment. |
| CurrentLinkAction | 1 | Specifies suggested action on old link once handover procedure have been executed. Combination of selection is possible. 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4~7: Reserved |

#### MIH_Handover_Commit.response

This message is delivered as a response to MIH_Handover_Commit.request.

| Name | Length | Value |
| --- | --- | --- |
| CurrentLinkAction | 1 | Specifies suggested action on old link once handover procedure have been executed. Combination of selection is possible. 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4~7: Reserved |