| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Requirements and Suggested Amendments for IEEE 802.16 |  |
| Date Submitted | August, 2005 |  |
| Source(s) | Vivek Gupta (Editor) |  |
| Re: | 21-05-0335-01-0000-Requirements_Amendments_802_16 |  |
| Abstract | This contribution has initial set of requirements and suggested amendments for 802.16 access technology |  |
| Purpose | Requirements and Suggested Amendments for 802.16 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Key Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Ronny Kim Jin Lee | LG Electronics Inc. | 533, Hogye1-dong, Dongan-Gu, Anyang-shi, Kyoungki-do, Korea, 431-749 | +82-31-450-2945 | +82-31-450-7912 | ronnykim@lge.com jins978@lge.com |

# 1   Introduction

# The purpose of this document is to identify requirements to be satisfied by IEEE 802.16 specification for supporting MIH Services as specified by the IEEE 802.21 specification. This document also includes suggestions for possible amendments to the IEEE 802.16g specification for satisfying the identified requirements.

# Scope of IEEE 802.16g Specification

## The IEEE 802.16g specification (current revision IEEE 802.16g-05/008, August 2005 http://ieee802.org/16/netman/docs/80216g-05_008.pdf specifies amendments to 802.16 Air Interface for Fixed and Mobile Broadband Wireless Access Systems. These amendments are for Management Plane Procedures and Services.

The 802.16g specification includes primitives which are exposed to upper layers in a consistent manner for use by control and management plane protocols. The network that manages and controls the 802.16 device is abstracted as a Network Control and Management System (NCMS) that interfaces with the base stations. The NCMS is made up of several different functional entities that may be centrally located or distributed across the network. The exact functionality of these entities is outside the scope of the 802.16g specification. Any necessary inter-BS coordination in is handled through the NCMS. NCMS protocols are not included in the 802.16g specification.

The 802.16g specification only describes the management and control interactions between MAC/PHY/CS layers of the 802.16 devices and the NCMS. The specification also includes the definition of various Information Elements (IEs) and other protocol primitives that are exposed using Service Access Points (SAPs). This includes CS, MAC and PHY layer context information used by NCMS protocols to manage and control the air interface.

# 2   Requirements

## This section identifies specific 802.21 requirements that need to be met by the 802.16g specification.

General Requirements

## The NCMS shall support MIH Function services for MS and BS.

## The 802.16 Reference Model shall support MIH Function services. Both the 802.16 SS and BS shall support these MIH Function services.

SAP Requirements

## The 802.16 specification shall support link layer events as specified in the 802.21 specification. For each link layer event this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.16 specification. The link layer events are identified in Table-1 in IEEE 802.21 specification.

## The 802.16 specification shall support link layer commands as specified in the 802.21 specification. For each link layer command this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.16 specification.

## The 802.16 specification shall support a L2 transport for transferring remote events and remote command messages over the air interface between MIH Function on SS/MS and MIH Function on PoA (BS). The Remote event and remote command messages shall be transferable over either a basic connection or a primary connection.

Information Element Requirements

## The 802.16 specification shall support Information Elements identified by the 802.21 specification.

Transport Requirements

## The 802.16 specification shall support a new ethertype for MIH.

## The 802.16 specification shall support interactions between MIH Function on BS and MIH Function on some other PoA (such as AP in 802.11 network). These shall be supported as interactions between 802.16 BS and NCMS.

## The 802.16 specification shall support a L2 transport for querying/setting information elements (IEs) over the air interface between MIH Function on SS/MS and MIH Function on PoA (BS). The IEs shall be transferable over either a basic connection or a primary connection.

## The 802.16 specification shall support MIH capable BS’s MIH Capability Delivery through broadcast message. This shall be accomplished by including capability information recommended by 802.21 specification in both DL-MAP message and Neighbor Advertisement (MOB_NBR-ADV) MAC management message.

## The 802.16 specification shall support advertisement of heterogeneous network information as specified by 802.21 specification. The advertisement shall be achieved by including heterogeneous network information in the Neighbor Advertisement (MOB_NBR-ADV) MAC management message.

# Suggested Amendments

## The NCMS shall include support for MIH Function Services as shown in Fig-1 below. The MIH Function Services shall provide support for MIH Event service, MIH Command service and MIH Information service.

- Fig 1: Update to NCMS Entities

## The 802.16 Reference Diagram shall be updated as per Figure 2. The C_SAP and M_SAP SAPs shall be amended to add support for 802.21 primitives.

- Fig 2: MIH Reference Diagram for 802.16

## The following set of events supported in the 802.21 specification shall be supported. (Table-1)

| Event Identifier | Event Type | Event Name | Description | Comments | 802.16 Primitive |
| --- | --- | --- | --- | --- | --- |
| 1 | State Change | Link Up | L2 connection has been established | Successful registration response message is received | M_Registration.confirmation |
| 2 | State Change | Link Down | L2 connection has been broken | MS can not demodulate the downlink and exceeds number of RNG-REQ retries with the serving BS. This can be check by counting ranging retries (M_Ranging.confirmation). | M_Ranging.confirmation |
| 3 | Predictive | Link Going Down | L2 connection loss is imminent | Either when MS receives BS initiated MOB_BSHO-REQ[1] message to request handover or when link quality delivered through M_Scanning.confirmation is bad below certain threshold. | M_Scanning.confirmation or M_ScanReport.confirmation. |
| 4 | State Change | Link Detected | A new link has been detected | Successful scanning of new BS. | M_Scanning.confirmation |

## The following set of link commands shall be specified in the 802.16 specification:

<No link level commands have currently been identified in 802.21 draft>

## The 802.16 specification shall support Information Elements identified by the 802.21 specification. The following set of basic information elements shall be supported:

- General set of 802.21 IEs: (These are independent of access technology)

| Seq. No. | Name of IE | Type | Possible values | Comments |
| --- | --- | --- | --- | --- |
| 1 | Network_Operator | String |  |  |
| 2 | Networks_supported | Bitmap | 802.3, 802.11a, 802.11b, 802.11g, 802.16a, 802.16d, 802.16e, GSM, GPRS, W-CDMA, CDMA2000, … |  |
| 3 | Location_LatLong | Real | (x-value, y-value) |  |
| 4 | Roaming_List_Available | Boolean | Specifies if list of roaming partner agreements is available |  |
| 5 | Media_Independent_Neighbor_List_Available | Boolean | Specifies if neighbor reports comprising of list of Points of Attachments (PoA) for different networks is available. |  |

- Basic set of 802.16 specific IEs

| Seq. Nr. | Name of IE | Type | Possible values | Comments |
| --- | --- | --- | --- | --- |
| 1 | Cipher_Suites | Bitmap | None, |  |
| 2 | Âuthentication_Methods | Bitmap | None, |  |
| 3 | Link_Cost | String | Cost of using access link and other services |  |
| 4 | IP_Version | Bitmap | v4, v6 | IP version supported by network |
| 5 | Data_Rates | Real | (min_rate, max_rate) |  |
| 6 | Higher Layer Services Supported | Bitmap | ISP(0), IMS(1), MMS(2), MIP(3), NAT(4), VPN(5), SIP(6), Local_device_support(6), … | Are these really useful? |
| 7 | Quality_of_Service | Bitmap |  | Do we need this? |
| 8 | Neighbor Maps |  |  | A new MIH neighbor report format shall be defined and recommended by 802.21 |

## A new MIH ethertype shall be defined. The MIH payload shall be encapsulated in 802.3 frames and sent using CS_SAP over the air interface (data plane).

MIH message delivery through data plane requires CS extension to support new service flow for MIH.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].28 | 1 | 0: No CS 1: Packet, IPv4 2: Packet, IPv6 3: Packet, 802.3/Ethernet 4: Packet, 802.1Q VLAN 5: Packet, IPv4 over 802.3/Ethernet 6: Packet, IPv6 over 802.3/Ethernet 7: Packet, IPv4 over 802.1Q VLAN 8: Packet, IPv6 over 802.1Q VLAN 9: ATM 10: Packet, IPv4 with Header Compression (ROHC) 11: Packet, IPv4 with Header Compression (ECRTP) 12: Packet, IPv6 with Header Compression (ROHC) 13: Packet, IPv6 with Header Compression (ECRTP) 14: Packet, IPv4 over 802.3/Ethernet with Header Compression (ROHC) 15: Packet, IPv4 over 802.3/Ethernet with Header Compression (ECRTP) 16: Packet, IPv6 over 802.3/Ethernet with Header Compression (ROHC) 17: Packet, IPv6 over 802.3/Ethernet with Header Compression (ECRTP) 18: Packet, IPv4 over 802.1Q VLAN with Header Compression (ROHC) 19: Packet, IPv4 over 802.1Q VLAN with Header Compression (ECRTP) 20: Packet, IPv6 over 802.1Q VLAN with Header Compression (ROHC) 21: Packet, IPv6 over 802.1Q VLAN with Header Compression (ECRTP) 22: MIH (Media Independent Handover) 23~255: reserved | DSx-REQ |

## The 802.16 specification shall support interactions between MIH Function on SS and MIH Function on some other PoA (such as AP in 802.11 network). These shall be supported as interactions between BS and NCMS. Following is a list of handover commands and interactions between MIH Function entities supported by 802.21 specification.

| Id | Command Name | MIHF <> MIHF | Description | Comments |
| --- | --- | --- | --- | --- |
| 1 | MIH Handover Initiate | Client <> Network PoA | Initiates handovers and sends a list of suggested networks and suggested PoA. | Covered by HO Request |
| 2 | MIH Handover Prepare | Network (oPoA) <> Network (nPoA) | This command is sent by MIHF on oPoA to MIHF on suggested new network at nPoA. This allows the client to query for resources on nPoA and also allows to prepare the nPoA for handover | New primitive needs to be defined |
| 3 | MIH Handover Commit | Client <> Network | In this case the client commits to do the handover based on selected choices for network and PoA. | Covered by HO Confirmation |
| 4 | MIH Handover Complete | Network (nPoA) <> Network (oPoA) | This is a notification from nPoA to oPoA that handover has been completed, new PoA has been established and any pending packets may now be forwarded to the new nPoA. | New primitive needs to be defined |
| 5 | MIH Network Address Information | Network (nPoA) <> Network(oPoA) <> Network (Access Router/ Foreign Agent) | This command is sent by MIHF on oPoA to MIHF on suggested new network at nPoA. nPoA may relay this command to the AR with MIHF. This allows the client to have network address related information prior to the handover to the nPoA. | New primitive needs to be defined |

## MIH Capability delivery through broadcast message shall be supported to advertise BS’s MIH capability.

- Downlink MAP (DL-MAP)

- Neighbor Advertisement (MOB_NBR-ADV)

### MIH Capability broadcast through DL-MAP

MS may obtain MIH Capability information for target BS by receiving DL-MAP message from the target BS.

BS could inform MS of its MIH Capability information through DL-MAP message, and MS obtains MIH Capability for the BS after receiving DL-MAP message.

Table XX shows MIH_Capability_IE that could be included in DL-MAP message

| Syntax | Size | Notes |
| --- | --- | --- |
| MIH_Capability_IE() { |  |  |
| Extended DIUC | 4bits | MIH_Capability_IE = XX |
| Length | 4bits | Length = 0x01 |
| MIH Capability | 1bit | 0 : MIH Not Supported 1 : MIH Supported |
| } |  |  |

Table XX . DL-MAP message

### MIH Capability broadcast through MOB_NBR-ADV

MS may obtain MIH Capability information for target BS by receiving MOB_NBR-ADV broadcast message during scanning the target BS.

BS could inform MS of its MIH Capability information through MOB_NBR-ADV message, and MS obtains MIH Capability of the BS after receiving MOB_NBR-ADV message.

| Syntax | Size | Notes |
| --- | --- | --- |
| MOB_NBR-ADV_Message_Format() { |  |  |
| Management Message Type = 53 | 8 bits |  |
| Skip-Optional-Fields bitmap | 8 bits | Bit [0]: if set to ‘1’, omit Operator ID field Bit [1]: if set to ‘1’, omit NBR BS ID field Bit [2]: if set to ‘1’, omit HO process optimization field Bit [3]: if set to ‘1’, omit QoS related fields Bit [4]: if set to ‘1’, omit Current BS’s MIH Capability INFO Bit [5]-[7]: reserved |
| …… | …… | …… |
| If (Skip-Optional-Fields-[4]=0) { |  |  |
| Current BS’s MIH Capability INFO | 1 bit | [0] : MIH not Supported [1] : MIH Supported |
| } |  |  |
| } |  |  |

Table XX. MOB-NBR-ADV management message.

## Neighbor Advertisement shall support heterogeneous network information to cover inter-technology handovers.

MS may obtain the information whether there is an available other type of PoAs with or without MIH Capability by receiving MOB_NBR-ADV message from the Serving BS before performing handover from 802.16 system to other interface networks.

In MOB_NBR-ADV message, MIH enabled 802.16 BS may provide MSs with MIH Capability information of other interface PoAs (e.g, WLAN AP, or 3GPP/3GPP2 BS). MIH INFO bitmap indicates availability and MIH capability of the WLAN AP and Cellular System BS which is located near the serving 802.16 BS. For each bit location, a value of  “1” indicates the correspondent is supported.

| Syntax | Size | Notes |
| --- | --- | --- |
| MOB_NBR-ADV_Message_Format() { |  |  |
| Management Message Type = 53 | 8 bits |  |
| Skip-Optional-Fields bitmap | 8 bits | Bit [0]: if set to ‘1’, omit Operator ID field Bit [1]: if set to ‘1’, omit NBR BS ID field Bit [2]: if set to ‘1’, omit HO process optimization field Bit [3]: if set to ‘1’, omit QoS related fields Bit [4]: if set to ‘1’, omit Current BS MIH Capability INFO Bit [5] : if set to ‘1’, omit MIH INFO bitmap Bit [6]-[7]: reserved |
| …… | …… | …… |
| If (Skip-Optional-Fields-[5]=0) { |  |  |
| MIH INFO bitmap | 16bits | [0] Available WLAN AP [1] Available WLAN AP MIH Enabled [2] Available WLAN AP MIH Capability unknown [3] Available 3GPP BS [4] Available 3GPP BS MIH Enabled [5] Available 3GPP BS MIH Capability unknown [6] Available 3GPP2 BS [7] Available 3GPP2 BS MIH Enabled [8] Available 3GPP2 BS MIH Capability unknown [9]-[15] reserved |
| } |  |  |
| } |  |  |

Table XX. MOB-NBR-ADV management message.

## The Mobility parameters specified in section 14.5.9.1 shall be modified to cover inter-technology handovers.

- Handover Context for Connections

- Neighbor BS List Management

## The following handover primitives defined in 802.16g specification in section 14.5.9.7.1, interactions between BS and NCMS, shall be amended to support inter-technology handovers as per 802.21 specification.

HO Request

HO Indication

HO Response

HO Confirmation

HO Start

HO Cancel

HO Directive

The parameters defined in the Handover primitives need to be changed/updated as follows.

| No | Current Parameter | Changes or Equivalent New Parameter | Comments |
| --- | --- | --- | --- |
| 1 | HO Type | Add Inter-Technology Handover (MIH) as an option | Specifies type of handovers |
| 2 | Serving BS ID | Use (Network Id + PoA ID) for MIH based inter-technology handovers | Specifies current Point of Attachment |
| 3 | Candidate Target BS List | Use (Network Id + Candidate Target PoA list) for MIH based inter-technology handovers | Specifies candidate list of target PoA on available networks |
| 4 | Recommended Target BS List | Use (Network Id + Recommended Target PoA list) for MIH based inter-technology handovers | Specifies recommended candidate list of target PoA on available networks |
| 5 | HO Quality Information |  |  |
| 6 | MSS Access Information |  |  |
| 7 | Newly Allocated Information |  |  |

The primitives in section 14.5.9.7.1 can be updated as follows:

14.5.9.7.1.1 HO Request

- Serving BS ID (Network + PoA ID)

- MSS ID

- HO Type: Add MIH (inter technology Handover) option

- Mode

- Candidate target BS list

- Service Flow Information

- (Network Id + Candidate target PoAs) list

14.5.9.7.1.2 HO indication

- Serving BS ID (Network + PoA ID)

- MSS ID

- HO Type: Add MIH (inter technology Handover) option

- Mode

- Service Flow Information

- HO Quality Information

14.5.9.7.1.3 HO response

- MSS ID

- HO Type: Add MIH (inter technology Handover) option

- Mode

- Recommended Target BS List

- (Network ID + Recommended Target PoAs) List

14.5.9.7.1.4 HO confirmation

- Target BS ID

- (Target Network ID + target PoA ID)

- MSS ID

- Result Flag

- HO Type: Add MIH (inter technology Handover) option

- Mode

- MSS Access Information

- Newly Allocated Information

- HO Quality Information

14.5.9.7.1.5 HO start

- MSS ID

- HO Type: Add MIH (inter technology Handover) option

- Mode

- Target BS ID

- (Target Network ID + Target PoA ID)

14.5.9.7.1.6 HO cancel

- MSS ID

- HO Type: Add MIH (inter technology Handover) option

- Mode

14.5.9.7.1.3 HO directive

- MSS ID

- HO Type: Add MIH (inter technology Handover) option

- Mode

- Recommended Target BS List

- (Network ID + Recommended Target PoAs) List

## The following additional primitives shall be defined:

The IEEE 802.16 MAC shall support the following primitives which are delivered through C_SAP (Control Service Access Point) or M_SAP (Management Service Access Point) interfacing with NCMS (Network Control and Management System).

  - M_Ranging.request/indication/response/confirmation

  - M_Registration.request/indication/response/confirmation

  - M_Neighbor.indication

  - M_ScanScheduling.request/indication/response/confirmation

  - M_Scanning.request/confirmation

  - M_MACHandover.request/indication/response/confirmation

  - M_HOIND.request/confirmation

  - M_Management.request/indication/response/confirmation

The use of these primitives to provide peer communication is shown in Figure 3. The use of primitive can be divided into two categories. The first category is with interaction with the peer entity, and the second category is primitive exchange within local stack.

The initial request for service from a higher layer through NCMS is provided by the “Request” primitive. The request triggers to generate appropriate MAC management message and the MAC management message is sent across the air interface to the peer MAC. Upon reception of the MAC management message over the air interface, corresponding “Indication” primitive is generated to inform NCMS of the request; When the response for the request is made from the higher layer, the response is delivered through the NCMS by the “Response” primitive. The response triggers to generate appropriate response MAC management message and this message is transmitted over the air interface to the originating side. Upon reception of the response MAC management message over the air interface, corresponding “Confirmation” primitive is generated and delivered to higher layer via NCMS.

Primitives exchange for the unidirectional MAC management messages, which don’t require response messages, such as MOB_HO-IND, MOB_TRF-IND, and for the local management of the MAC state machine, is shown in Fig. 3, (2). The initial request for service from a higher layer through NCMS is provided by the “Request” primitive. The request triggers either to generate appropriate unidirectional MAC management message or MAC state change depending on the primitive. “Confirmation” primitive conveying the result of the request is delivered to the higher layer through NCMS.

- Fig 3: The use of primitives to generate MAC management messages

[Note] Recommended primitives’ functional description, semantics, time of generation and effect of receipt are written below. It is recommended to evaluate and modify the suggested text if necessary for inclusion in the 802.16g specification.

### M_Ranging.request/indication/response/confirmation:

Upper layers can control ranging procedure with these primitives. Upper layers shall commence 802.16 link setup procedure by sending M_Ranging.request primitive through NCMS.

Note) M_Ranging.request primitive with ranging type: “initial” is an 802.16 Link Switch Link Command (link setup) which corresponds to Link Switch MIH command.

- Fig 4: The use of Ranging Primitives

#### M_Ranging.request

##### Function

This primitive requests ranging. Upper layer management entities shall request ranging by sending this primitive to the MAC layer through NCMS.

##### Semantics

Semantics

M_Raning.request

(

Source ,

Destination ,

Ranging Type

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Ranging Type | Enumeration | Initial, Handoff, Location Update, Periodic | This identifies the ranging type |

##### When generated

This primitive is generated by the upper layer management entities to initiate ranging procedure for initial network entry, network re-entry after handover, periodic ranging, network re-entry from Idle mode, and location update of Idle Mode mobile terminals.

##### Effect of receipt

MAC layer shall generate RNG-REQ MAC management message including corresponding TLVs depending on the Ranging type and RNG-REQ message shall be sent to the BS over air interface.

#### M_Ranging.indication

##### Function

This primitive notifies the upper layer management entity in BS that the mobile terminal requests ranging with RNG-REQ.

##### Semantics

M_Ranging.indication

(

Source ,

Destination ,

MS Address ,

CDMA code ,

MAC Version ,

Required Downlink Burst Profile ,

Serving BS ID ,

Target BS ID ,

HO Indication ,

Location Update Request ,

Paging Controller ID

)

| Name | Type | Valid | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS Address | MAC Address | Any valid individual MAC Address | MAC Address of MS that requests ranging |
| CDMA code |  |  | CDMA code received for ranging |
| MAC Version | Enumeration | IEEE Std 802.16-2001, IEEE Std 802.16-2004, IEEE Std 802.16e | MAC version supported by MS |
| Required Downlink Burst profile |  |  | DIUC value of Downlink Burst Profile |
| Serving BS ID |  |  | Serving BS ID during ranging |
| Target BS ID |  |  | Target BS ID during ranging |
| HO Indication |  |  | This parameter indicates the MS is currently attempting to HO or Network Re-entry from Idle Mode to the BS. |
| Location Update Request |  |  | This parameter indicates MS action of Idle Mode Location Update Process |
| Paging Controller ID |  |  | This is a logical network identifier for the serving BS or other network entity retaining MS service and operational information and/or administering paging activity for the MS while in Idle Mode. |

##### When generated

This primitive is generated by MAC layer when MAC layer receives RNG-REQ message over the air interface.

##### Effect of receipt

Upon receipt ranging indication, M_Ranging.response is generated

#### M_Ranging.response

##### Function

This primitive returns the result of ranging request.

##### Semantics

M_Ranging.response

(

Source ,

Destination ,

MS Address ,

Result Code ,

Management CIDs ,

Resource Retain Flag ,

HO Process Optimization ,

Location Update Response ,

Paging information ,

Paging Controller ID ,

Next Periodic Ranging

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS Address | MAC Address | Any valid individual MAC Address | MAC Address of MS that requests ranging |
| Result Code | Enumeration |  | Result of ranging request |
| Management CID | Enumeration | Basic CID Primary Management CID | Management CID of MT if ranging succeeded |
| Resource Retain Flag |  |  | MT information retained |
| HO Process Optimization |  |  | Network re-entry process optimization after handover |
| Location Update Response | Enumeration | Success Failure | Location Update result in idle mode |
| Paging information |  |  | Changed paging information if location update succeeded |
| Paging Controller ID |  |  | Idle mode management entity (Paging controller ID) |
| Next Periodic Ranging |  |  | Frame offset of next ranging during sleep mode |

##### When generated

This primitive is generated when decided to notify the ranging result after receiving M_Ranging.indication

##### Effect of receipt

MAC layer sends RNG-RSP message

#### M_Ranging.confirmation

##### Function

This primitive notifies the result of ranging from M_Ranging.response to upper layer entity

##### Semantics

M_Raning.confirmation

(

Source ,

Destination ,

MS Address ,

ResultCode ,

ManagementCIDs ,

Resource Retain Flag ,

HO Process Optimization ,

Location Update Response ,

Paging Information ,

Paging Controller ID ,

Next Periodic Ranging

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS Address | MAC Address | Any valid individual MAC Address | MAC Address of MS that requests ranging |
| Result Code | Enumeration |  | Result of ranging request |
| Management CID | Enumeration | Basic CID Primary Management CID | Management CID of MT if ranging succeeded |
| Resource Retain Flag |  |  | MT information retained |
| HO Process Optimization |  |  | Network re-entry process optimization after handover |
| Location Update Response | Enumeration | Success Failure | Location Update result in idle mode |
| Paging information |  |  | Changed paging information if location update succeeded |
| Paging Controller ID |  |  | Idle mode management entity (Paging controller ID) |
| Next Periodic Ranging |  |  | Frame offset of next ranging during sleep mode |

##### When generated

This primitive is generated when MAC layer receives RNG-RSP message.

##### Effect of receipt

The upper layer entity receives the result of ranging

### M_Registration.request/indication/response/confirmation

Upper layers can control registration procedure with these primitives. Upper layers are notified of link setup by M_Registration.confirmation.

Note) M_Registration.confirmation primitive is an 802.16 Link_Up Link Event.

- Fig 4: The use of Registration Primitives

#### M_Registration.request

##### Function

This primitive is initiated by the upper layer entity to request registration.

##### Semantics

M_Registration.request

(

Source ,

Destination ,

IP management mode ,

IP Version ,

Method of Allocating IP Address ,

Previous IP Address

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| IP management mode | Enumeration | Unmanaged Mode Managed Mode |  |
| IP Version | Enumeration | Version 4 Version 6 | IP Version |
| Method of Allocation IP Address | Enumeration | DHCP Mobile IPv4 DHCPv6 Mobile IPv6 IPv6 Stateless address auto configuration | IP Address configuration method |
| Previous IP Address | IP Address |  | Previously assigned IP Address of MS on the secondary management connection. |

##### When generated

This primitive is generated when upper layer entity requests registration

##### Effect of receipt

REG-REQ message including necessary TLV parameter is sent

#### M_Registration.indication

##### Function

This primitive notifies that upper layer entity requests registration

##### Semantics

M_Registration.indication

(

Source ,

Destination ,

IP management mode ,

IP Version ,

Method of Allocating IP Address ,

Previous IP Address

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| IP management mode | Enumeration | Unmanaged Mode Managed Mode |  |
| IP Version | Enumeration | Version 4 Version 6 | IP Version |
| Method of Allocation IP Address | Enumeration | DHCP Mobile IPv4 DHCPv6 Mobile IPv6 IPv6 Stateless address auto configuration | IP Address configuration method |
| Previous IP Address | IP Address |  | Previously assigned IP Address of MS on the secondary management connection. |

##### When generated

This primitive is generated when MAC layer receives REG-REQ message .

##### Effect of receipt

M_Registraion.response is generated.

#### M_Registration.response

##### Function

This primitive returns the result of registration request.

##### Semantics

M_Regisration.response

(

Source ,

Destination ,

IP management mode ,

IP Version ,

Method of Allocating IP Address ,

Skip IP Address Acquition

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| IP management mode | Enumeration | Unmanaged Mode Managed Mode |  |
| IP Version | Enumeration | Version 4 Version 6 | IP Version |
| Method of Allocation IP Address | Enumeration | DHCP Mobile IPv4 DHCPv6 Mobile IPv6 IPv6 Stateless address auto configuration | IP Address configuration method |
| Skip IP Address Acquisiton | Enumeration | No IP address change Re-acquire IP address | This indicates to an MS whether it should reqcquire its IP address on the secondary management connection and related context or reuse its prior context |

##### When generated

This primitive is generated to notify the result of registration after M_Registration.indication is received

##### Effect of receipt

MAC layer sends REG-RSP message

#### M_Registration.confirmation

##### Function

This primitive notifies the registration result from M_Registration.response to upper layer entity

##### Semantics

M_Registration.confirmation

(

Source ,

Destination ,

IP management mode ,

IP Version ,

Method of Allocating IP Address ,

Skip Address Acquisition

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| IP management mode | Enumeration | Unmanaged Mode Managed Mode |  |
| IP Version | Enumeration | Version 4 Version 6 | IP Version |
| Method of Allocation IP Address | Enumeration | DHCP Mobile IPv4 DHCPv6 Mobile IPv6 IPv6 Stateless address auto configuration | IP Address configuration method |
| Skip Address Acquisition | Enumeration | No IP address change Re-acquire IP address | This indicates to an MS whether it should reqcquire its IP address on the secondary management connection and related context or reuse its prior context |

##### When generated

This primitive is generated when REG-RSP is received

##### Effect of receipt

Registration result is notified to the upper layer entity

### M_Neighbor.indication

When 802.16 MAC receives neighbor advertisement (MOB_NBR-ADV), this primitive is used to deliver the information to upper layers.

Note) When 802.21’s recommendation is accepted by 802.16 working group and heterogeneous network information can be delivered through MOB_NBR-ADV, this primitive shall include heterogeneous network information.

- Fig 4: The use of Neighbor Advertisement Indication Primitives

##### Function

This primitive is generated by MAC layer to notify the upper layer entity of reception of neighbor advertisement (MOB_NBR-ADV) from BS.

##### Semantics

M_Neighbor.indication

{

Source,

Destination,

Operator ID,

N_Neighbors,

Neighbor BS-ID,

HO Process Optimization,

Current BS’s MIH Capability INFO

MIH INFO Bitmap

}

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Operator ID |  |  | Unique ID assigned to the operator |
| N_Neighbors |  |  | The count of the unique combination of Neighbor BSID, Preamble Index and DCD. |
| Neighbor BS-ID |  |  | Base station ID |
| HO Process Optimization | Enumeration | Bit #0: Omit SBC-REQ/RSP management messages during re-entry processing Bit #1: Omit PKM Authentication phase except TEK phase during current re-entry processing Bit #2: Omit PKM TEK creation phase during reentry processing Bit #3: Omit REG-REQ/RSP management during current re-entry processing Bit #4: Omit Network Address Acquisition management messages during current reentry processing Bit #5: Omit Time of Day Acquisition management messages during current reentry processing Bit #6: Omit TFTP management messages during current re-entry processing Bit #7: Full service and operational state transfer or sharing between serving BS and target BS (ARQ, timers, counters, MAC state machines, etc…) | Network re-entry process optimization after handover |
| Current BS’s MIH Capability MIH INFO | Enumeration | MIH Not Supported MIH Supported | This indicates whether current BS delivering neighbor advertisement supports MIH or not. |
| MIH INFO bitmap | Enumeration | Available WLAN AP, Available WLAN AP MIH Enabled, Available WLAN AP MIH Capability unknown, Available 3GPP BS, Available 3GPP BS MIH Enabled, Available 3GPP BS MIH Capability unknown, Available 3GPP2 BS, Available 3GPP2 BS MIH Enabled, Available 3GPP2 BS MIH Capability unknown, | This indicates existence of different network point of attachments and their MIH capability. |

##### When generated

This primitive is generated for the MAC layer to notify the upper layer entity of MOB_NBR-ADV contents received from the BS.

##### Effect of receipt

Upper layer entity acquires information of BSes.

### M_ScanScheduling.request/indication/response/confirmation

Upper layers can schedule scanning period with BS. During scanning period BS may buffer downlink traffic to the mobile terminal.

Note) In 802.21 working group, usefulness of Scan Scheduling shall be investigated.

- Fig 5: The use of Scan Scheduling Primitives

#### M_ScanScheduling.request

##### Function

This primitive requests the MAC layer to send MOB_SCN-REQ message

##### Semantics

M_ScanScheduling.request

(

Source ,

Destination ,

Scan duration ,

BSID

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Scan duration |  |  | Scan duration time |
| BSID |  |  | Peer BS ID for SCN-REQ |

##### When generated

This primitives is generated when the upper layer entity indicates to send MOB-SCN-REQ

##### Effect of receipt

MAC layer sends MOB-SCN-REQ to BS

#### M_ScanScheduling.indication

##### Function

This primitive carries the information related MOB-SCN-REQ message to the upper layer entity

##### Semantics

M_ScanScheduling.indication

(

Source ,

Destination ,

MS MAC Address ,

Scan duration

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS MAC Address |  |  | MAC Address of scan request MS |
| Scan duration |  |  | Scan duration time |

##### When generated

This primitive is generated after BS receives MOB_SCN-REQ from MS

##### Effect of receipt

The upper layer decides whether allowing scan request or not

#### M_ScanScheduling.response

##### Function

This primitive transmits the result of scan request to MAC layer

##### Semantics

M_ScanScheduling.response

(

Source ,

Destination ,

MS MAC Address ,

Scan duration ,

Start frame

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS MAC Address | MAC Address |  | MAC Address of scan request MS |
| Scan duration |  |  | Scan duration time |
| Start frame |  |  | Scan start frame |

##### When generated

This primitive is generated when the upper layer entity decide to carry the result of scan request

##### Effect of receipt

BS sends MOB_SCN-RSP message to carry the information from the received M_ScanScheduling.response

#### M_ScanScheduling.confirmation

##### Function

This primitive is to transmit the information related to MOB_SCN-REQ message to the upper layer entity

##### Semantics

M_ScanScheduling.confirmation

(

Source

Destination ,

Scan duration ,

Start frame

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Scan duration |  |  | Scan duration time |
| Start frame |  |  | Scan start frame |

##### When generated

This primitive is generated when MS sends scan information from BS to the upper layer entity

##### Effect of receipt

The upper layer entity indicates scanning to MS with the information from M_ScanScheduling.confirmation

### M_Scanning.request/confirmation

Upper layers can command autonomous scanning with these primitives.

Note) These primitives are Scan Link Command for MIH Scan command in 802.21.

- Fig 6: The use of Scanning Primitives

#### M_Scanning.request

##### Function

This primitive is for upper layer entity to request scanning to MS

##### Semantics

M_Scanning.request

(

Source ,

Destination ,

Scan duration

Link Quality Threshold

Link Status Report Period

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Scan duration |  |  | Scan duration time |
| Link Quality Threshold |  |  | Signal Quality threshold. Scanning report shall be made when link quality goes worse than this threshold. |
| Link Status Report Period |  |  | Time period that the scanning report shall be made. |

##### When generated

This primitive is generated when the upper layer entity requests scanning to MAC layer

##### Effect of receipt

MAC layer starts to scan

#### M_Scanning.confirmation

##### Function

This primitive is for MAC layer to notify the upper layer entity of scan result.

##### Semantics

M_Scanning.confirmation

(

Source ,

Destination ,

ResultCode ,

ResultCode

BS ID ,

CINR ,

RSSI

MIH Capability

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| ResultCode | Enumeration | Available BS No Available BS | Scan Result |
| BS ID |  |  | Scanned BS ID |
| CINR |  |  | CINR of Available BS |
| RSSI |  |  | RSSI of Available BS |
| MIH Capability | Enumeration | MIH not supported MIH supported | MIH Capability of available BS |

##### When generated

This primitive responds to M_Scanning.request to notify the upper layer entity of scan result

##### Effect of receipt

The upper layer entity receives the channel status of available BSes as a scanning result.

### Scan Report primitives

Usage scenario is shown in Figure 6. Only primitives delivered by NCMS are shown. Delivery of the primitives shall be based on the pre-registration procedure between upper layer management entities and NCMS.

Scan report can be made remotely to the BS or locally to the upper layer entity depending on the report target value in M_ScanReport.request.

Figure 6. Use of Scan Report Primitives

#### M_ScanReport.request

##### Function

This primitive is for the MAC layer to report scan result locally or remotely.

##### Semantics

M_ScanReport.request

(

Source ,

Destination

Report Target

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Report Target | Enumeration | Local Remote | This indicates the object to which report shall be made. |

##### When generated

This primitive is generated when the upper layer entity requests to send MOB_SCAN-REPORT message to the BS or to report the scan result to the upper layer entity.

##### Effect of receipt

MAC layer sends MOB_SCAN-REPORT to BS in case of remote report. In case of local report, upper layer entity transmits scan report with M_ScanReport.confirmation.

#### M_ScanReport.confirmation

##### Function

This primitive notifies the upper layer entity of the result of M_ScanReport.request.  In case of remote report, this primitive carries the remote message transmission result. In case of local report, this primitive carries scanning report to the upper layer entity.

##### Semantics

M_ScanReport.confirmation

(

Source ,

Destination ,

ResultCode

BS ID

RSSI

CINR

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Result Code | Enumeration | Success Fail No Available BS | The result of scan report message transmission . When there is no available BS to scan, ‘No Available BS’ result code shall be included. |
| BS ID |  |  | Scanned BS ID |
| RSSI |  |  | CINR of Available BS |
| CINR |  |  | RSSI of Available BS |

##### When generated

When this primitive is generated as a response to M_ScanReport.request with remote report target, this primitive is generated after MAC layer sends scan report to BS. When this primitive is generated as a response to M_ScanReport.request with local report target, this primitive is generated after scanning the BSes.

##### Effect of receipt

An upper layer entity receives the result of remote scan report message (MOB_SCAN-REPORT) or scanning result.

### M_MACHandover.request/indication/response/confirmation

Upper layers can control handover procedure by using these primitives.

Note) These primitives can be used as link commands for MIH_Handover_initiate.request/response. For remote command service, 802.16 MAC management messages shall be used. (MOB_MSHO-REQ/MOB_BSHO-REQ/MOB_BSHO-RSP) Currently only parameters relevant to 802.16 handover are included. Parameters for Median Independent Handover shall be identified and added.

- Fig 7: The use of Handover Primitives

#### M_MACHandover.request

##### Function

This primitive requests MAC layer to send handover request message.

##### Semantics

M_MACHandover.request

(

Source ,

Destination ,

N_Recommended ,

Neighbor BS ID ,

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| N_Recommended |  |  | The Number of target BS |
| Neighbor BS ID |  |  | Available neighbor BS ID |

##### When generated

This primitive is generated when the upper layer requests MAC layer to send MOB_MSHO-REQ message

##### Effect of receipt

MAC  layer sends MOB_MSHO-REQ to BS

#### M_MACHandover.indication

##### Function

This primitive is for MAC layer of BS to deliver handover request received from the MS to the upper layer entity.

##### Semantics

M_MACHandover.indication

(

Source ,

Destination ,

MS MAC Address ,

N_Recommended ,

Neighbor BS ID ,

BS CINR Mean

BS RSSI Mean

Relative delay

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS MAC Address |  |  | MS MAC Address |
| N_Recommended |  |  | The Number of target BS |
| Neighbor BS ID |  |  | Available neighbor BS ID |
| BS CINR Mean |  |  | This indicates the CINR in dB measured at the MS on the downlink signal of a particular BS. |
| BS RSSI Mean |  |  | This indicates the Received Signal Strength measured by the MS from the particular BS |
| Relative delay |  |  | This indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. |

##### When generated

This primitive is generated when the MAC layer receives MOB_MSHO-REQ message

##### Effect of receipt

The upper layer selects recommended target BSes by signaling with other BSes through backbone messages.

#### M_MACHandover.response

##### Function

The upper layer entity transfers the result of handover request to MAC layer

##### Semantics

M_MACHandover.response

(

Source ,

Destination ,

N_Recommended ,

Neighbor BS ID ,

HO Process Optimization ,

HO ID

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| N_Recommended |  |  | The Number of target BS |
| Neighbor BS ID |  |  | Available neighbor BS ID |
| HO Process Optimization |  |  | Network re-entry process optimization after handover |
| HO ID |  |  | ID assigned for use in initial ranging to the target BS once this BS is selected as the target BS. |

##### When generated

This primitive response to M_MACHandover.request

##### Effect of receipt

MAC layer of BS sends MOB_BSHO-RSP to MS

#### M_MACHandover.confirmation

##### Function

MAC layer transmits the result of handover request to the upper layer entity

##### Semantics

M_MACHandover.confirmation

(

Source ,

Destination ,

N_Recommended ,

Neighbor BS ID ,

HO Process Optimization ,

HO ID

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| N_Recommended |  |  | The Number of target BS |
| Neighbor BS ID |  |  | Available neighbor BS ID |
| HO Process Optimization |  |  | Network re-entry process optimization after handover |
| HO ID |  |  | ID assigned for use in initial ranging to the target BS once this BS is selected as the target BS. |

##### When generated

This primitive is generated after MAC layer of MS receives MOB_BSHO-RSP message

##### Effect of receipt

The upper layer entity decides to do handover to target BS

### M_HOIND.request/confirmation

An MS transmits a MOB_HO-IND message for final indication that is about performing a HO.

Note) This primitive is a Link Command of MIH_Handover_Commit.request/response MIH Command in the .21 draft.  For remote service, 802.16 MAC management message shall be used. (MOB_HO-IND)

- Fig 8: The use of Handover Indication Primitives

#### M_HOIND.request

##### Function

This primitive transfers the information about target BS to MAC layer

##### Semantics

M_HOIND.request

(

Source ,

Destination ,

HO_IND_type

Target_BS_ID

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| HO_IND_type | Enumeration | Handover Handover cancel Handover reject |  |
| Target_BS_ID |  |  | Target BS ID for Handover |

##### When generated

This primitive is generated when the upper layer entity requests MAC layer to start handover

##### Effect of receipt

MS starts handover when HO_IND_type indicates ‘handover’

#### M_HOIND.confirmation

##### Function

This primitive informs the upper layer entity whether MOB_HO-IND message transmission is carried out successfully or not

##### Semantics

M_HOIND.confirmation

(

Source ,

Destination ,

Result Code

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Result code | Enumeration | Success Fail | The result of MOB_HO-IND message transmission |

##### When generated

This primitive responds to M_HOIND.request

##### Effect of receipt

The upper layer recognizes that MOB_HO-IND message transmission has been carried out successfully

#### M_HOIND.indication

##### Function

This primitive is for MAC layer of BS to deliver handover indication (MOB-HO-IND) received from the MS to the upper layer entity.

##### Semantics

M_HOIND.indication

(

Source ,

Destination ,

HO_IND_type

Target_BS_ID

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| HO_IND_type | Enumeration | Handover Handover cancel Handover reject |  |

##### When generated

This primitive is generated when the MAC layer of BS receives MOB_HO-IND message

##### Effect of receipt

The upper layer is notified of MS’s handover decision.

### M_Management.request/indication/response/confirmation

These primitives are used to manage the status of mobile terminal. Upper layer can change the status of mobile terminal into power on/down/hold/de-register, etc.

Note) These can be mapped to MIH Configure MIH Command Service. For remote service, 802.16 MAC management messages can be used. (RES-CMD/ DREG-CMD)

- Mobile registration deregistration from the BS and power off

- Mobile node status management (reset / Power Down / Power On)

- Remote Reset by BS

- Remote control by BS

- Remote control by BS

#### M_Management.request

##### Function

Upper layer entities in BS or MS can request MS’s status change with this primitive.

##### Semantics

M_Management.request

(

Source ,

Destination ,

MS Address ,

Action Code

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS Address | MAC Address | Any valid individual MAC Address | This parameter is included when BS makes request. |
| Action Code | Enumeration | Power on Power off Reset Deregistration Hold Normal | Type of management |

##### When generated

When the upper layer entity of MS generates this primitive, this primitive is used to change its MAC status, such as power on, power off, reset, etc with or without interaction with BS. When the upper layer entity of BS generates this primitive, this primitives is used to change the status, such as hold, reset, normal, etc. of the specific MSes, identified by MS addresses.

##### Effect of receipt

In case of local request in MS, MAC layer changes its status into the requested status and send M_Management.confirmation with the status change result. If MS needs to interact with BS for its status change, MS’s MAC layer transmits MAC management message with corresponding action code.

In case of remote request from BS, BS’s MAC layer transmits MAC management messages to specific MS in order to change MS’s status.

#### M_Management.indication

##### Function

This primitive delivers received status change MAC management message to the upper layer entity.

##### Semantics

M_Managment.indicaiton

(

Source ,

Destination ,

MS Address ,

Action Code

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS Address | MAC Address | Any valid individual MAC Address | MAC Address of MS |
| Action Code | Enumeration | Power on Power off Reset Deregistration Hold Normal | Type of management |

##### When generated

This primitive is generated by MAC layer upon reception of status change MAC management messages, such as, DREG-REQ, DREG-CMD, RES-CMD, etc.

##### Effect of receipt

The upper layer entity is able to control the status of MS.

#### M_Management.response

##### Function

As a response to the MS’s request, upper layer entity in the BS can command the MS’s status change.

##### Semantics

M_Management.response

(

Source ,

Destination ,

MS Address ,

Action Code

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| MS Address | MAC Address | Any valid individual MAC Address | MAC Address of MS |
| Action Code | Enumeration | Power on Power off Reset Deregistration Hold Normal | Type of management |

##### When generated

This primitive is generated, when the upper layer entity receives M_Management.indication with Deregistration Action Code and upper layer entity decides to deregister the specific MS.

##### Effect of receipt

Upon reception of this primitive, MAC layer of BS transmits DREG-CMD to the specific MS.

#### M_Management.confirmation

##### Function

This primitive transmits the result of status changes.

##### Semantics

M_Management.confirmation

(

Source ,

Destination ,

Result

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source | EVENT_SOURCE | N/A | The origination point from where this primitive is initiated |
| Destination | EVENT_DESTINATION | N/A | This specifies the destination where this primitive finally arrives |
| Result | Enumeration | Success Fail | The result of status changes |

##### When generated

This primitive is generated when the status of MS is changed.

##### Effect of receipt

The upper layer can correctly update the status change of MS. Depending on the result, the upper layer entity can perform correct action, e.g, MS’s status update, re-issuing status change command.

[