| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Service (IS) Reference Model, Use Case Scenario and higher Layer requirements for 802.21 Information Service (IS) |  |
| Date Submitted | September, 2005 |  |
| Source(s) | Subir Das, Yoshihiro Ohba, Farooq Bari ….. |  |
| Re: | 21-05-0348-00-0000-Higher_layer_Requirements_Information_Service |  |
| Abstract | This contribution has IS reference model, Use case scenario and initial set of higher layer requirements for 802.21 Information Service |  |
| Purpose | To generate higher layer requirements for Information Service |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# 1   Introduction

# The purpose of this document is to identify the requirements to support 802.21 Information Service (IS) via higher layers (L3 and above in terms of OSI layering model). These requirements are derived from the high level IS reference model and use case scenarios.  At first we discuss the model and use cases and then specify the initial set of requirements.

# Information Service (IS)  Reference Model

## Figure 2.1 describes both single hop and multi-hop IS models.  UE refers to a user entity that contains 802.21 Information Service function. NISP refers to a Network Information Service Provider that provides necessary information to the UE via query and response mechanisms.  NISP has two logical components: i) 802.21 IS Server function and ii) Information Database. UE communicates with the NISP via interface ‘Ia’ in a single-hop model. In multi-hop model a proxy is introduced that helps routing the information query and response to the appropriate NISP and UE respectively [Note: In such a scenario, proxy does not have information database]. UE communicates with the proxy NISP via interface “Ia” while proxy communicates with the NISP via interface “Ia’ ”.   Interface “Ix” represents the communication between an Information Server and Information Database.  However, interface “Ix” is out of scope in current 802.21 PAR.   Regarding interfaces “Ia” and “Ia’”, the communication between UE and NISP over L3 and higher layers is our focus here.

# Use Cases

## Case I: Information Service co-located with Information Database

### Client-Server model

Figure 3.1.1 :  Use Case Scenario I (Client-Server Model)

Figure 3.1.1 presents a client-server use case model whereby Information Service (Server) is co-located with the Information Database.  In this scenario UE communicates with the individual NISP via interface “Ia”.

### Client-Proxy-Server model

Figure 3.1.2: Use Case Scenario II (Client-Proxy-Server Model)

Figure 3.1.2 presents a Client-Proxy-Server use case model whereby Information Service (Server) is co-located with the Information Database.  In this scenario, UE communicates with an NISP via interface “Ia” and if the NISP does not have the required information, it contacts another NISP via interface “Ia’” and the former NISP acts as a proxy to the latter NISP. [Note: Interface “Ia’” however does not support the Server-to-Server communication]

## Case 2: Information Service with separate Information Database

### Client-Server Model

Figure 3.2.1:  Use Case Scenario III (Client-Server Model)

Figure 3.2.1 presents a Client-Server  use case  model whereby Information Service (Server)  and  Information Database are two separate entities.  In this scenario UE communicates with the individual NISP via interface “Ia” as earlier and interface “Ix” is out of scope in current 802.21 PAR.

### Client-Proxy-Server Model

Figure 3.2.2: Use Case Scenario IV (Client-Proxy-Server Model)

Figure 3.2.2 presents a Client-Proxy-Server use case  model whereby Information Service (Server) and Information Database are two separate entities.  In this scenario, UE communicates with an NISP via interface “Ia” and if the NISP does not have the required information, it contacts the other NISP via interface “Ia’” and the former NISP acts as a proxy to the latter NISP. [Note: Interface “Ia’” however does not support the Server-to-Server communication].  Interface “Ix” is out of scope in current 802.21 PAR.

# Requirements

## This section identifies higher layer requirements for 802.21 Information Service. Although some of the requirements may be categorized as general IS requirements, however, it seems appropriate to list them here.

## The 802.21 IS Reference Model shall support both single-hop and multi-hop model for information exchange between a User Element (UE) and a Network IS Provider (NISP).

## The 802.21 IS specification shall define the interface “Ia” between UE and Information Server.

## The 802.21 IS specification shall define the interface “Ia` “between Information Proxy and Server in multi-hop model.

## The 802.21 IS specification shall define appropriate SAPs for higher layers, primitives and information elements to support the 802.21 MIH IS functionality. Both UE and Information Server shall support the MIH Information Service functionality.

## The 802.21 IS specification shall support multiple ways of discovering the Information Server. The specification shall neither define a new mechanism nor mandate a specific mechanism for this purpose.

## The 802.21 IS specification shall define a basic set of   Information Elements (IEs) (both media independent and dependent) that are mandatory for supporting IS service.

## The 802.21 IS specification shall define an extended set of Information Elements (IEs) that are optional for supporting IS service

## The 802.21 IS specification shall define the relationship among the Information Elements (IEs)

## The 802.21 IS specification shall support multiple options in representing the IEs and the corresponding query/response format.

## The 802.21 IS specification shall specify appropriate transport for carrying IS query and response between a UE and an Information Server.

## The 802.21 IS specification shall specify appropriate transport to proxy IS query and response between a UE and an Information Server via intermediate NISPs.

## The 802.21 IS specification shall specify appropriate security requirements that are needed to be established between a UE and an Information Server before exchanging any information between them.

## The 802.21 IS specification shall allow information exchange between a UE and NISP both secure and non-secure way

## The 802.21 IS specification shall support fragmentation of IS query and response frame where needed.

## The 802.21 IS specification shall support the use cases that are defined in this document

## ..

# Information Elements

### The following basic set of IEs (Table-1 and Table II ) shall be supported by 802.21 IS specification

### Media Independent Information Elements

| Seq.No. | Name of IE | Type | Possible values |
| --- | --- | --- | --- |
| 1 | Point of Attachment (POA) | String |  |
| 2 | Neighbor_Network_Information | String | 802.3, 802.11a, 802.11b, 802.11g, 802.16a, 802.16d, 802.16e, GSM, GPRS, W-CDMA, CDMA2000, … |
| 3 | Network_Operator | String | Name of the network Operator |
| 4 | QoS_supported | Boolean | Information about QoS support |
| 5 | Location | Real | Geo-coordinates (as per RFC 3825) |
| 6 | IP_Version | String | Operator A has v4, Operator B has v6 |
| 7 | Security_Features_Supported | String | NAT, VPN, IPsec, TLS, SMIME, |
| 8 | Roaming_Partner_List | String | List of roaming partners available |
| 9 | Network_Connection_Cost | String | Free, Not free, Tarriff ($$/hr) |

### Media Dependent Information Elements

| Seq.No. | Name of IE | Type | Possible values |
| --- | --- | --- | --- |
| 1 | Âuthentication_Methods | Bitmap | 802.1x, UAM, SIM |
| 2 | Cipher_Suites | Bitmap | WEP, WPA, PSK, AES-CCMP |
| 3 | Data_Rates | Real | (Min_rate, Max_rate) |
| 4 | Quality_of_Service | Boolean | 802.11e |
| 5 | Neighbor_Maps | String | 802.11AP, 802.16BS, Cellular BS, |