| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Requirements and Suggested Amendments for IEEE 802.11 |  |
| Date Submitted | August, 2005 |  |
| Source(s) | Vivek Gupta (Editor) |  |
| Re: | 21-05-0340-00-0000-Requirements_Amendments_802_11 |  |
| Abstract | This contribution has initial set of requirements and suggested amendments for 802.11 access technology |  |
| Purpose | Requirements and Suggested Amendments for 802.11 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# 1   Introduction

# The purpose of this document is to identify requirements to be satisfied by IEEE 802.11 specification for supporting MIH Services as specified by the IEEE 802.21 specification. This document also includes suggestions for possible amendments to the IEEE 802.11 specification for satisfying the identified requirements.

## 2   Requirements

## This section identifies specific 802.21 requirements that need to be met by the 802.11 specification.

## The 802.11 Reference Model shall support Media Independent Handover (MIH) Function services. The 802.11 specification shall define appropriate SAPs, primitives and information elements to support IEEE 802.21 MIH functionalities. Both the IEEE 802.11 STA and AP shall support the MIH functionality.

## The IEEE 802.11 specification shall provide a mechanism (for APs and STAs) to indicate Media Independent Handover capability. The 802.11 specification shall support MIH Capability Discovery through a broadcast message as well as through an explicit query and response mechanism. The broadcast discovery shall be supported by including capability information in the 802.11 beacon.

## The 802.11 specification shall support link layer events as specified in the 802.21 specification. For each link layer event this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.11 specification. The following set of events supported in the 802.21 specification (Table-1) shall be supported.

| Event Identifier | Event Type | Event Name | Description | Comments |
| --- | --- | --- | --- | --- |
| 1 | State Change | Link Up | L2 connection has been established |  |
| 2 | State Change | Link Down | L2 connection has been broken |  |
| 3 | Predictive | Link Going Down | L2 connection loss is imminent |  |
| 4 | State Change | Link Detected | A new link has been detected |  |
| 5 | State Change | Link Parameters Change | Link parameters has crossed specified threshold |  |
| 6 | Administrative | Link Event Rollback | Link event needs to be rolled back since the predictive event did not occur as predicted |  |
| 7 | Link Transmission | Link SDU Transmit Success | Success in transmitting SDU |  |
| 8 | Link Transmission | Link SDU Transmit Failure | Failure in SDU transmission |  |
| 9 | Link Synchronous | Link Handoff Imminent |  |  |
| 10 | Link Synchronous | Link Handoff Proceeding |  |  |
| 11 | Link Synchronous | Link Handoff Complete |  |  |
| ….. | …… | …… |  |  |

## The 802.11 specification shall support link layer commands as specified in the 802.21 specification. For each link layer command this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.11 specification.

## The 802.11 specification shall support a L2 transport for transferring remote events and remote command messages over the air interface between MIH Function on STA and MIH Function on AP. The remote event and remote command messages shall be transferable over either a data frame or a management frame.

## The 802.11 specification shall support a new ether type for supporting MIH functionality over the data plane.

## The 802.11 specification shall support interactions between MIH Function on AP and MIH Function on some other PoA (such as BS in 802.16 network). These shall be supported as interactions between 802.11 AP and the 802.11 DS. Following is a list of handover commands and interactions between MIH Function entities supported by 802.21 specification.

| Command Identifier | Command Name | MIHF <> MIHF | Description |
| --- | --- | --- | --- |
| 1 | MIH Handover Initiate | Client <> Network PoA | Initiates handovers and sends a list of suggested networks and suggested PoA. |
| 2 | MIH Handover Prepare | Network (oPoA) <> Network (nPoA) | This command is sent by MIHF on oPoA to MIHF on suggested new network at nPoA. This allows the client to query for resources on nPoA and also allows to prepare the nPoA for handover |
| 3 | MIH Handover Commit | Client <> Network | In this case the client commits to do the handover based on selected choices for network and PoA. |
| 4 | MIH Handover Complete | Network (nPoA) <> Network (oPoA) | This is a notification from nPoA to oPoA that handover has been completed, new PoA has been established and any pending packets may now be forwarded to the new nPoA. |
| 5 | MIH Network Address Information | Network (nPoA) <> Network(oPoA) <> Network (Access Router/ Foreign Agent) | This command is sent by MIHF on oPoA to MIHF on suggested new network at nPoA. nPoA may relay this command to the AR with MIHF. This allows the client to have network address related information prior to the handover to the nPoA. |

## The 802.11 specification shall support Information Elements identified by the 802.21 specification. The following set of basic information elements shall be supported

| Seq. Nr. | Name of IE | Type | Possible values |
| --- | --- | --- | --- |
| 1 | Network_Operator | String |  |
| 2 | Networks_supported | Enumerated | 802.3, 802.11a, 802.11b, 802.11g, 802.16a, 802.16d, 802.16e, GSM, GPRS, W-CDMA, CDMA2000, … |
| 3 | Cipher_Suites | Enumerated | None, WEP, WPA-1, WPA-2, TKIP, AES-CCMP, … |
| 4 | Âuthentication_Methods | Enumerated | (None), WEP, Psk, 802.1x,…. |
| 5 | Link_Cost | String | Cost of using services at the link |
| 6 | Location_LatLong | Real | (x-value, y-value) |
| 7 | IP_Version | Enumerated | v4, v6, |
| 8 | Data_Rates | Real | (min_rate, max_rate) |
| 9 | Higher Layer Services Supported | Enumerated | ISP(0), IMS(1), MMS(2), MIP(3), NAT(4), VPN(5), SIP(6), Local_device_support(6), … |
| 10 | Roaming_List_Available | Boolean | Specifies if list of roaming partner agreements is available |
| 11 | Media_Independent_Neighbor_List_Available | Boolean | Specifies if neighbor reports comprising of list of Points of Attachments (PoA) for different networks is available. |
| 12 | Quality_of_Service | Enumerated | 802.11e |

## The 802.11 specification shall support a L2 transport for querying/setting information elements (IEs) over the air interface between MIH Function on STA and MIH Function on AP. The IEs shall be transferable over either a data frame or a management frame. The transport of IEs shall also be supported in state 1: unauthenticated and unassociated (class 1 frames).

# Suggested Amendments

## The 802.11 Reference Diagram shall be updated as per Figure 1.

- Fig 1: MIH Reference Diagram for 802.11