| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER |  |
| Date Submitted | September, 2005 |  |
| Source(s) | Harmonized Section 5 Contribution |  |
| Re: | 21-05-0361-00-0000-Section5_Changes |  |
| Abstract | Updates for Section 5 of current MIH Draft |  |
| Purpose | This is an initial contribution to capture changes and other suggested architectural amendments for Section 5 in the current 802.21 Draft document. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

List of Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Vivek Gupta | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503 712 1754 | 503 264 4230 | vivek.g.gupta@intel.com |
| Ulises Olvera Juan | Interdigital | Two Huntington Quadrangle Third Floor, Melville, NY 11747, USA | 514 904 6282 |  | ulises.olvera- |
| Ronny Kim Jin Lee | LG Electronics Inc. | 533, Hogye1-dong, Dongan-Gu, Anyang-shi, Kyoungki-do, Korea, 431-749 | +82-31-450-2945 | +82-31-450-7912 | ronnykim@lge.com jins978@lge.com |
| Yogesh Bhatt Qiaobing Xie | Motorola | 600 North U.S. Highway 45, Libertyville, IL 60048 U.S.A | 847-523-0126 | 847 523 5660 | yogeshbhatt@motorola.com |
| Stefano Faccin Srinivas Sreemanthula | Nokia | NH3:400, 6000 Connection Drive, Irving, Texas 75039 | 9728944994 | 972 894 4589 | stefano.faccin@nokia.com |
| Xiaoyu Liu | Samsung | Mt.14-1,Nongseo-Ri, Giheung-Eup, Yongin-Si,Gyeonggi-Do,Korea 449-712 | +82-31-280-9615 | +82-31-280-9569 | xiaoyu.liu@samsung.com |
| Wolfgang Gröting Stefan Berg Kalyan Koora | Siemens AG | Frankenstr. 2, 46395 Bocholt, Germany | +49 2871 91 2142 | +49 2871 91 3387 | wolfgang.groeting@siemens.com stefan.berg@siemens.com kalyan.koora@siemens.com |
| Yoshiro Ohba | Toshiba America Research Inc. | P.O. Box 429, Piscataway, NJ 08854-4151 | 732-699-5305 | 732-336-3578 | yohba@tari.toshiba.com |
| Subir Das | Telcordia Technologies Inc | One Telcordia Drive, RRC-1B229, Piscataway, NJ 08854-4157 | 732 699 2483 | 732 336 7026 | subir@research.telcordia.com |

1 Overview of Specification	4

2 References	4

3 Definitions	4

4 Acronyms and Abbreviations	4

5 General Architecture	5

5.1 Introduction	5

5.2 General Design Principles	5

5.3 Media Independent Handover Reference Architecture	5

5.3.1 Logical Network Reference Model	5

5.3.2 MIH Interfaces between different entities	5

5.3.3 Relation between MIH Functions	7

5.4 MIH Function Services	8

5.4.1 Media Independent Event Service	8

5.4.2 Media Independent Command Service	8

5.4.3 Media Independent Information Service	8

5.5 MIH Reference Models for Access Networks	8

5.5.1 MIH Reference Model for 802.3	8

5.5.2 MIH Reference Model for 802.11	8

5.5.3 MIH Reference Model for 802.16	10

5.5.4 MIH Reference Model for 3GPP	10

5.5.5 MIH Reference Model for 3GPP2	10

5.6 Service Access Points (SAPs)	11

5.6.1 Media Specific SAPs	12

5.6.2 Media Independent SAPs	13

5.6.3 Network Management SAPs	13

5.7 Media Independence	13

# Overview of Specification

# References

# Definitions

# Acronyms and Abbreviations

# General Architecture

## Introduction

## General Design Principles

## Media Independent Handover Reference Architecture

### Logical Network Reference Model

### MIH Interfaces between different entities

MIH

MN

(802.3, 802.11,

802.16, 3GPP, 3GPP2)

MIH

Current PoA

MIH

Candidate PoA(s)

MIH

Ia

Ia’

Ib

Ic

Ic’

Id

MIH Capable

Network Entity

(e.g., MIH IS Function,

MIH Controller)

Information Database

Ix

Figure A: Communication Model and various Interfaces

The interfaces in the above reference diagram are addressed by the 802.21 specification as follows.

- Ia – specifies the interface between the MIH Function on the mobile node and the MIH Function on the current PoA in the access network. Communication between the two MIH Functions may occur at L2 or L3 in this case. The definition of Ia interface is within the scope of the current specification.

- Ia’ – specifies the interface between the MIH Function on the mobile node and the MIH Function on a candidate PoA on some other access network to which the current connection needs to be handed over. Communication between the two MIH Functions may occur at L2 or L3 in this case as well. The Ia’ interface is likely to be the same as the Ia interface. The definition of Ia’ interface is within the scope of the current specification.

- Ib – specifies the interface between the MIH Function on the current PoA in the acces network currently under use and the MIH Function on a candidate PoA on some other access network to which the current connection needs to be handed over. Communication between the two MIH Functions is likely to occur at L3 in this case. The Ib interface is outside the scope of the current specification.

- Ic – specifies the interface between the MIH Function on the current PoA in the acces network currently under use and the MIH Function on some other network entity. This network entity could act as MIH Information Service Function or as a MIH controller which can exchange commands and events with the mobile node. Communication between the two MIH Functions may occur at L2 or L3 depending on the location of the MIH based network entity and the specific deployment scenarios. The Ic interface is within the scope of the current specification.

- Ic’ – is same as the Ic interface except that it specifies the interface between the MIH Function on the candidate PoA on some other acces network and the MIH Function on some other network entity. The Ic’ interface is defined within the scope of the current specification.

- Id – specifies the interface between the MIH Function on the mobile node and the MIH Function on some other network entity. Communication between the two MIH Functions is likely to occur at L3 in this case. The Id interface is defined within the scope of the current specification.

- Ix – specifies the interface between the MIH Function on a network entity and the Information repository. The Ix interface is outside the scope of the current specification.

The table below summarizes the scope of different interfaces within the current specification.

| No | Interface | Comments | Transport | Scope |
| --- | --- | --- | --- | --- |
| 1 | Ia, Ia’ | Single instance per access network | Usually L2, can be L3 | In scope of 802.21 |
| 2 | Ib | Single instance between any two access networks | Usually L3 | Out of scope of 802.21 |
| 3 | Id | Can support multiple instances over single access network at L3 | L3 | In scope of 802.21. |
| 4 | Ic, Ic’ | Independent of access network | L2, L3 | In scope of 802.21. |
| 5 | Ix | Independent of access network | L2, L3 | Out of scope of 802.21. |

### Relation between MIH Functions

MIH

MN

MIH

Case 1: Loose interworking scenario

PoA AN-1

MIH

PoA AN-2

Ia

Ia

AN-2

MIH

MN

MIH

MIH

MN

MIH

PoA AN-1

MIH

PoA AN-2

Case 2: Tight interworking scenario (same domain or connected domains)

Case 3: Combined loose and tight interworking scenarios

MIH

Remote

MIH

Remote

MIH

Remote

MIH

Remote

MIH

Remote

MIH

Remote

MIH

Remote

MIH

Remote

AN-1

Ib

Ib

Ic

Ib

Ic

Ia

Ia

AN-1

AN-2

AN-1

AN-2

Ia

Ia

Id

Id

Ib

Ic

Ic

PoA

(AN-1, AN-2)

Figure B: Interworking scenarios and MIH Relationships

## MIH Function Services

### Media Independent Event Service

### Media Independent Command Service

### Media Independent Information Service

## MIH Reference Models for Access Networks

### MIH Reference Model for 802.3

Figure 4: MIH Reference Model for 802.3

The MIH reference model for 802.3 is illustrated above. The IEEE 802.21 standard supports the Media Independent Event service, Media Independent Command service and Media Independent Information service. In case of wired Ethernet networks the payload of MIH services is supported over the data plane by using existing primitives defined by the LSAP SAP.

### MIH Reference Model for 802.11

Figure 5: MIH Reference Model for 802.11

The above figure shows the MIH functions for 802.11 stations and network PoA (APs).

The LSAP SAP defines the MIH interface to the data plane and can encapsulate MIH payload in data packets. However since 802.11 does not currently support Class 1 data frames, traffic can be sent over the data plane only after the station has associated with the AP.

The MIH_LINK_SAP SAP specifies the interface between MIH and the management plane (MLME) and allows MIH payload to be encapsulated in management frames (such as action frames). Thus primitives specified by MIH_LINK_SAP can be used to transfer packets before a station has associated with an AP, whereas the LSAP can be used to transfer packets after association has been established with an AP.

The MIH_SAP specifies the interface of MIH Function with other higher layer entities such as Transport, Handover policy, L3 Mobility protocol, etc.

The MIH Function interfaces with the OS or the system using the MIH_SME_SAP.

### MIH Reference Model for 802.16

The figure below shows the MIH function for 802.16 based system. The M_SAP and C_SAP service access points are common between the MIH Function and NCMS. These SAPs specify the interaction between MIH Function and Management Entity and also help with transporting MIH payload across to peer MIH Function entities. The MIH_SAP shows the interface of MIH Function with other higher layer entities such as Transport, Handover policy, L3 Mobility protocol, etc.

Figure 6: MIH Reference Model for 802.16

### MIH Reference Model for 3GPP

### MIH Reference Model for 3GPP2

Figure 9: MIH Reference Model for Mobile Stations with Multiple Protocol Stacks

## Service Access Points (SAPs)

The MIH Function exchanges messages with other layers and functional planes using Service Access Points (SAPs). Each SAP consists of a set of service primitives that specify the information to be exchanged and the format of the information exchanges.

The specification of the MIH Function includes the definition of SAPs that are media-independent (i.e., insensitive to the interface types that the mobile terminal can support) and recommendations to define or extend other SAPs that are media-dependent. Media-independent SAPs allow the MIH Function to provide services to the upper layers of the mobility-management protocol stack, the network management plane, and the data bearer plane. The MIH_SAP and associated primitives provide the interface from MIH_Function to the upper layers of the mobility-management stack. Upper layers need to register with MIH Function as MIHF users to receive MIHF-generated events and also for link layer events that originate at layers below the MIH Function but can be passed on to upper layers through MIH Function. Upper layers can directly send commands to MIH Function. Similarly MIH Function entities can also send commands to other remote (peer) MIH Function entities. Primitives corresponding to all these services described above are within the scope of MIH_SAP.

Media-dependent SAPs allow the MIH Function to use services from the lower layers of the mobility-management protocol stack and their management planes. All inputs (including the events) from the lower layers of the mobility-management stack into the MIH Function are provided through existing media-specific SAPs such as MAC SAPs, PHY SAPs, and LLC SAPs. Link Commands generated by the MIH Function to control the PHY and MAC layers during the handover are part of the media specific MAC/PHY SAPs and are already defined elsewhere.

Figure below shows the key MIH related SAPs for different networks. The key MIH related SAPs are as follows:

Figure 10: Relationship between different MIH SAPs

### Media Specific SAPs

#### MIH_LINK_SAP

This defines the interface between the MIH Function and the management plane of 802.11 network. This SAP can be used for sending MIH messages between MIH Function and local link layer entities as well as between peer MIH Function entities. Messages based on management frames can be sent in the unauthenticated state as well. The MIH_LINK_SAP defines primitives for MIES, MICS and MIIS.

#### M_SAP

This defines the interface between the MIH Function and the management plane of 802.16 network.

#### C_SAP

This defines the interface between the MIH Function and the control plane of 802.16 network.

#### MIH_RRC_SAP

This defines the interface between the MIH Function and RRC.

#### MIH_PPP_SAP

This defines the interface between the MIH Function and PPP.

#### MIH_LAC_SAP

This defines the interface between the MIH Function and LAC.

### Media Independent SAPs

#### MIH_SAP

This SAP defines the interface between MIH Function and a L3 mobility protocol, or a handover function which might reside at higher layer or a higher layer transport as well. Definition of this SAP is required to define the scope and functionality of MIH Function.

### Network Management SAPs

#### MIH_SME_SAP

This SAP defines the interface between MIH Function and the Network Management Entity This SAP defines the MIH interface with the management system and is used for MIH configuration and operation.

## Media Independence