| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER |
| Date Submitted | September, 2005 |
| Source(s) | Qiaobing Xie, Peretz Feder, Andrea Francini, Subir Das, Vivek Gupta |
| Re: | 21-05-0365-02-0000-communication_model_text |
| Abstract | Draft Text for MEDIA INDEPENDENT HANDOVER Specification |
| Purpose | Proposal for 802.21 |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

List of Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Peretz Feder, Andrea Francini | Lucent Technologies | 67 Whippany Rd , Whippany, NJ 07981, USA | 919 932 4369 |  | pfeder@lucent.com |
| Qiaobing Xie | Motorola | 1501 W. Shure Drive, Arlington Heights, Il 60004 | 847-632-3028 |  | qxie1@motorola.com |
| Subir Das | Telcordia Technologies Inc | One Telcordia Drive, RRC-1B229, Piscataway, NJ 08854-4157 | 732 699 2483 |  | subir@research.telcordia.com |

### 5.3.0 Communication Model

MIH functions may communicate with each other for various purposes, such as in the case of remote MIIS, MIES, and MICS. MIH communication happens when MIH related information is passed from one MIHF to another.

Figure X shows the MIH communication model. The model intends to show MIH functions in different distinctive roles and the communications relationships amongst them.

It is important to note that each of the communication relationships shown in the diagram below does not imply a particular interface or transport mechanism. Rather, a communication relationship only intends to show that MIH related information passing is possible between the two distinctive MIH functions. Moreover, each communication relationship shown in the diagram may encompass different type of 802.21 interfaces, different transport mechanisms used (e.g., L2, L3), and/or different MIH related content being passed (e.g., IS, CS, ES, etc.).

Figure X: Communication Model

Depending on its position in the system, four distinctive roles of MIH function are identified in the communication model:

- MIHF on a UE.

- MIHF on a serving PoA (which is the PoA that the UE is currently attached to).

- MIHF on a candidate PoA (which is a PoA that the UE is aware of but not currently attached to, and can potential become the target PoA when a handover occurs).

- MIHF on a network entity that is not a PoA.

The following six communication reference points are identified in the communication model:

- Communication reference point R1

Reference Point R1 refers to MIH procedures between the MIH on a UE and the MIH on the serving PoA. R1 may encompass communications interfaces over both L2 and L3 and above, and content passed over R1 can be related to MIIS, MIES, or MICS.

- Communication reference point R2

Reference Point R2 refers to MIH procedures between the MIH on the serving PoA and the MIH on an 802.21 NE that is not a PoA. R2 may encompass communications interfaces over L3 and above, and MIH content passed over R2 may be related to MIIS, MIES, or MICS.

- Communication reference point R3

Reference Point R3 refers to MIH procedures between the MIH on a UE and the MIH on a candidate PoA. R3 may encompass communications interfaces over L2, and content passed over R3 can be related to MIIS, MIES, or MICS.

- Communication reference point R4

Reference Point R4 refers to MIH procedures between the MIH on a condidate PoA and the MIH on an 802.21 NE that is not a PoA. R4 may encompass communications interfaces over L3 and above, and MIH content passed over R4 may be related to MIIS, MIES, or MICS.

- Communication reference point R5

Reference Point R5 refers to MIH procedures between the MIH on the serving PoA and the MIH on a candidate PoA. R5 may encompass a direct PoA to PoA communication over L3 and above. In this release of the specifications R5 is out of scope since direct communication or services between PoAs serving disparate technologies can't be realistically achieved at present.

- Communication reference point R6

Reference Point R6 shows MIH procedures between the MIH on a UE and the MIH on an 802.21 NE that is not a PoA. R6 may encompass communications interfaces over L3 and above and MIH content passed over R6 may be related to MIIS, MIES, or MICS.