| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | Proposed modification of the MIH Capability Discovery |  |  |
| Date Submitted | September 13, 2005 |  |  |
| Source(s) | Eunah Kim, Junghoon Jee, Jong-Hwa Yi ETRI 161 Gajeong-Dong, Yuseong-Gu Daejeon, 305-700, KOREA | Voice: +82-42-860-4820 Fax: +82-42-861-5404 {eakim, jhjee, jhyiee}@etri.re.kr |  |
| Re: | IEEE 802.21 Session #NN in Garden Grove, California |  |  |
| Abstract | This contribution proposes amendments related to the MIH Capability Discovery. First, we define a new value, “Capability Discovery”, in MIH Service ID. Second, we suggest the MIH_Capability_Discover.request to contain the MIH message data. Third, we specify the transport way for discovering the MIH capability on each different media types. Finally, we define new SupportedTransportProtocol in MIH message data. |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- MIH Service ID of MIH General Packet Format in section 8.3.1, page 94

MIH service ID which is the first identifier field of MIH general packet format, does not include any value for MIH capability discovery messages, thus resulting in the problem of encoding and decoding of MIH capability discovery messages.

Therefore, we propose to add a new MIH Service ID, Capability Discovery with value ‘4’ in Table 13.

| Name of the Field | Size | Optional/Mandatory | Description |
| --- | --- | --- | --- |
| MIH Service ID | 1 octet | Mandatory | MIH Service Identifier 1 : Event Service 2 : Command Service 3 : Information Service 4 : Capability Discovery |

- MIH Capability Discovery Messages in section 8.4.1, page 96-97

MIH_Capability_Disover.request message does not contain the MIH message data, thus requiring two capability discovery hand-shakes or capability advertisements between the MIH function of mobile node and the MIH function in the network for them to recognize each other’s MIH capabilities. By containing the MIH message data in the MIH_Capability_Discover.request message, both MIH functions in the terminal and network can discover each other’s capabilities by only one capability discovery procedure.

Therefore, we suggest replacing the first sentence of the paragraph of section 8.4.1.1 with the new sentence as follows.

8.4.1.1 MIH_Capability_Discover.request

This message doesn’t contain MIH message data, and only MIH header and MIH Message Identifier is transmitted. This message may contain SupportedEventList and SupportedCommandList in the MIH message data. If a requesting MIHF entity doesn’t know the MIHF capable entity’s addresses, requesting MIHF entity fills its source addresses and broadcast this message. If a requesting MIHF entity knows the address of entity but doesn’t know whether the entity is MIHF capable, this message is delivered to the entity in a unicast way.

- MIH Protocol Transport in section 8.2, page 92

There exists a difference between which transport to use in delivering MIH capability discovery messages and which transport to use in delivering actual MIH service.

Without regulation about transport way for MIH capability discovery, peer MIH function entities undergo difficulties in finding out whether each other’s peer supports MIH function or not. Therefore, we propose to specify the transport way for discovering the MIH capability according to different media types by adding a new column, “Transport for MIH Capability Discovery”, in Table 11.

Table 11 – Transport Options

| No | Media Type | Transport for MIH Capability Discovery | Preferred Transport | L2 Transport | L3 Transport |
| --- | --- | --- | --- | --- | --- |
| 1 | Ethernet | L2 (Data Plane) | L2 | Data Frames | IP based |
| 2 | 802.11 | L2 (Management Plane) | L2 | Data Frames, Management Frames | IP based |
| 3 | 802.16 | L2 (Management Plane) | L2 | Data Frames, Management Frames | IP based |
| 4 | 3GPP | L3 (will be defined in IETF?) | L3 | Requires protocol stack changes | IP based |
| 5 | 3GPP2 | L3 (will be defined in IETF?) | L3 | Requires protocol stack changes | IP based |

- MIH Capability Discovery Messages in section 8.4.1, page 96-97

Peer MIH function entities can use different transport way for each MIH service. This different transport way should be identified before the actual MIH service exchange. Therefore, we propose this information can be conveyed during the MIH capability discovery handshake by adding the SupportedTransportProtocol in the MIH message data of MIH_Capability_Discover.request and MIH_Capability_Discover.response messages, which specifies what transport way to use for each MIH service.

| Name | Length | Value |
| --- | --- | --- |
| SupportedTransportList | 4 | Octet 1 specifies the transport protocol for the event service. For each Bit location, a value of ‘1’ indicates the transport protocol is supported Bit #0 : L2, Data Plane Bit #1 : L2, Management Plane Bit #2 : L3, IP Protocol Bit #3 : L3, TCP Protocol Bit #4 : L3, UDP Protocol Bit #7 – Bit #5 : Reserved Octet 2 specifies the transport protocol for the command service. For each Bit location, a value of ‘1’ indicates the transport protocol is supported Bit #0 : L2, Data Plane Bit #1 : L2, Management Plane Bit #2 : L3, IP Protocol Bit #3 : L3, TCP Protocol Bit #4 : L3, UDP Protocol Bit #7 – Bit #5 : Reserved Octet 3 specify the transport protocol for the Information service. For each Bit location, a value of ‘1’ indicates the transport protocol is supported Bit #0 : L2, Data Plane Bit #1 : L2, Management Plane Bit #2 : L3, IP Protocol Bit #3 : L3, TCP Protocol Bit #4 : L3, UDP Protocol Bit #7 – Bit #5 : Reserved Octet 4 is reserved |
| SupportedEventList | 4 | For each Bit location, a value of ‘1’ indicates the link event is supported. Bit #0: Link Up Bit #1: Link Down Bit #2: Link Going Down Bit #3: Link Detected Bit #4: Link Parameters Change Bit #5: Link Event Rollback Bit #6: Link SDU Transmit Success Bit #7: Link SDU Transmit Failure Bit #8~31: Reserved |
| SupportedCommandList | 4 | For each Bit location, a value of ‘1’ indicates the link command is supported. Bit #0: Link Power Up Bit #1: Link Power Down Bit #2: Link Configure Bit #3: Link Connect Bit #4: Link Disconnect Bit #5: Link Sleep Bit #6: Link Scan Bit #7: Link Poll Bit #8~31: Reserved |

- Motion

802.21 adopts proposed texts related to MIH capability discovery in this contribution.