| Project | IEEE 802.21 <http://www.ieee802.org/21/> |
| --- | --- |
| Title | MIH Information Elements for terminals. |
| Date Submitted | September 14, 2005 |
| Source(s) | Mathieu PERESSE, Eric NJEDJOU (France Telecom) |
| Re: | IEEE 802.21 Session #10 in Garden Grove, CA |
| Abstract |  |
| Purpose | To Introduce an Information Element set for terminals. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# 1 – Introduction

Although not explicitly stated in the draft, the MIH Information Service only provides a set of Information Elements for Point of Attachments (Access Networks). In order for a Network Operator to control handover, it has to gather information from PoAs and UEs. Therefore there is a need for a new set of IEs that specify information to be sent by the terminal to the network. For example a Mobility Management Entity (MME), located in the network, makes use of these Information Elements to select the optimum link for a given terminal.

Information Elements for UEs are transported via the MIH Information Service (static IEs) or via the MIH Command and Event Services (dynamic IEs).

# 2 - Problem

A MME (preferably in the network) can also use Mobile Terminals information, such as their Radio Parameters, the User Preferred Networks, or the QoS needs.

Information Elements for UEs (to be sent to the network) are not defined in the current draft.

# 3 - Solution

For coherence, a new Information Element set is needed for terminals, in addition to the already defined Information Set (for PoAs).

To meet what has already been described in the 802.21 draft, the IEs that contain static information are to be carried through the Information Service and the IEs that contains dynamic information are to be carried through the Command and Event Services.

Another requirement is that the reference model for IS should explicitly state that all the interfaces between entities shall be bidirectional (Ia, Ia'…), to stress the fact that not only an UE can gather IEs from PoAs / Information Servers in the Network, but there could be some Entity (MME for example) in the network that could be interested in gathering info from UEs (change in user preferences…).

# 4 - Presentation of the Information Element set for the terminal.

The Information Elements can be carried through Information Service, Event Service or Command Service. Examples of utilization will be given in a simple scenario in the next section.

| Static/Quasi-static IEs (carried through IS) |  |  |  |
| --- | --- | --- | --- |
| Name of Information Element | Primitive Used to carry the IE | Tech Specific | Comments |
| List of supported technologies | MIH_Information.response or .indication | NO | E.g. 802.3, 802.11a/b/g, 802.16, GSM, GPRS/EDGE networks, UMTS |
| Supported technologies options | MIH_Information.response or .indication | NO | 802.11i, 802.11e, 802.16e, EDGE Class 10… |
| Cost preferences | MIH_Information.response or .indication | NO | Quality/Cost ratio |
| IP Version | MIH_Information.response or .indication | YES | Version 4 or Version 6 |
| Authentication Methods | MIH_Information.response or .indication | YES | EAP-TLS, EAP-SIM,SIM auth.(GSM) |
| Hardware ID | MIH_Information.response or .indication | YES | HW MAC Address for 802 technologies / IMEI for cellular technology. |

| Dynamic IEs (carried through ES/CS) |  |  |  |
| --- | --- | --- | --- |
| Name of Information Element | Description | Tech Specific | Comments |
| Current Interface Type | MIH_Measurement_Report (see the New_ECI contribution, for info) | YES | 802.11a, GPRS |
| Current Network ID | MIH_Measurement_Report | YES | SSID, PLMN |
| Current PoA ID | MIH_Measurement_Report | YES | BSSID, Cell ID |
| Measured Bandwidth | MIH_Measurement_Report | YES | Kbps |
| BER | MIH_Measurement_Report | YES | % |
| Current Data Rate | MIH_Measurement_Report | YES | Kbps |
| Current Signal Level | MIH_Measurement_Report | YES | dBm |
| Current Radio Link Quality | MIH_Measurement_Report | YES | dB |
| Quality of Service | MIH_Service_Request.request/response (see the New_ECI contribution, for info) | NO | MinLatencyMsec, MaxLatencyMsec, MaxJitterMsec |
| Neighbor Information | MIH_Information.xxx ?? | NO | List(Network ID, PoA ID, technology, BER, Signal Level) |

5 – A simple example scenario

Case 1: The UE communicates directly with the MME over L3.

Case 2: The UE communicates with PoA over L2 that relays messages to the MME over L3.

Scenario: The UE periodically sends its Measurements Reports to MME (either via a PoA or directly through L3). If the signal is below a certain level, the MME will find UE's neighbors (based on the location ID or on the AP BSSID and on a neighbor map) and sends a Neighbor Report to the UE. The UE then performs measurements on these neighbors and sends them to the MME. If the UE's signal (11k RCPI) is below a second threshold, the MME then applies an algorithm to select the best PoA to handover to and sends the decision to the UE.

Case 1:

UE

PoA

MME

UE Measurement Reports over L3 (1)

Neighbor Report

UE's Neighbors Measurement Reports over L3 (2)

Load Report over L3

- A measurement report example:

| Current Interface Type | 802.11g |
| --- | --- |
| Current Network ID | Orange (SSID) |
| Current PoA ID | AB:CD:EF:12:35:67 |
| BER | 0,1% |
| Current Data Rate | 5500 Kbps |
| Current Signal Level | - 100 dBm |
| Current Radio Link Quality | 40 dB |

- A neighbour report example:

| Current Interface Type | 802.11g | UMTS |
| --- | --- | --- |
| Current Network ID | Orange (SSID) | Orange (PLMN) |
| Current PoA ID | AB:CD:EF:12:35:67 | Cell ID |
| BER | 0,1% | 0,1% |
| Current Data Rate | 5500 Kbps | 384 Kbps |
| Current Signal Level | - 80 dBm | - 65 dBm |
| Current Radio Link Quality | 40 dB | 38 dB |

Case 2: (example with 802.11)

UE

PoA

MME

Measurement Reports over L2 (11k)

Consolidated Measurement Reports over L3 (21)

11k Neighbor Report

21 Neighbor Report

11k Beacon Report (with Measurements)

21 Neighbor Report (with measurements)

Handover Command (new .11 primitive)