| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | Amendment of the MIH Protocol Format |  |  |
| Date Submitted | November 5, 2005 |  |  |
| Source(s) | Junghoon Jee, Eunah Kim ETRI 161 Gajeong-Dong, Yuseong-Gu Daejeon, 305-700, KOREA | Voice: +82-42-860-4820 Fax: +82-42-861-5404 {jhjee, eakim}@etri.re.kr |  |
| Re: | IEEE 802.21 Session #NN in Hyatt Regency, Vancouver |  |  |
| Abstract | This contribution proposes an amendment for protocol formats of Media Independent Handover which is defined in IEEE P802.21/D00.03. |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

Amendment of MIH General Packet Format in section 8.3.1, page 98

- Problem Definition

First problem in the current MIH general packet format is redundant definition of identifiers, MIH service identifier and MIH message identifier. Currently, MIH message identifier field defines separate identifiers for all MIH function messages, which are related with MIH capability discovery, MIH remote event registration and MIH services. Only with this MIH message identifier, the receiving peer MIH function entity can decode the MIH packets and then perform the expected behavior from the sending MIH peer. Therefore, the MIH service identifier does not act any active role in the MIH packet processing.

Second problem in the current MIH general packet format is the inclusion of L2 or L3 address like MIH function destination and source identifier. Generally, L2 and L3 address are defined in the L2 or L3 protocol itself and this address information is revealed to the functional entity which utilizes the specific transport protocol. Thus, MIH packet format needs not to include MIH function destination and source identifier field in itself.

- Proposition

We propose to delete the MIH Service identifier, MIH Function identifier length, Source MIH Function identifier and Destination MIH Function identifier fields in the previous MIH packet format with modification of message length field to denote only the payload length in bytes.

| Name of the Field | Size | Optional/ Mandatory | Description |
| --- | --- | --- | --- |
| Protocol Version | 1 octet | Mandatory | Version of the MIH protocol. Default: 0x01 |
| MIH Message ID | 4 octet | Mandatory | Actual MIH Message identifier for the service identified above. MIH Message identifier for MIH services, MIH capability discovery and MIH event registration. e.g. Link_Going_Down Event Indication for Event Service |
| MIH Service ID | 1 octet | Mandatory | MIH Service Identifier 1:Event Service 2:Command Service 3: Information Service |
| MIH Opcode | 1 octet | Mandatory | Operation to be performed 1: Request 2: Response 3: Indication |
| Transaction ID | 1 octet | Mandatory | Transaction Identifier used to match requests and responses wherever applicable. |
| Fr | 1 bit | Mandatory | Fragmentation flag. 1: Specifies current packet has more than 1 fragment 0: Current packet is not fragmented |
| Fragment No | 7 bit | Mandatory | Fragment Number of this packet |
| Message Length | 2 octets | Mandatory | Total length of whole message in bytes (header + payload) Total length of payload in bytes |
| MIH Function Identifier Length (MIHFL) | 4 octets | Mandatory | Length of each of Source and Destination MIH Identifier fields |
| Source MIH Function Identifier | MIHFL octets | Mandatory | This is the Source MIHF identifier. This can be the L2 hardware address of the interface at the source node. This can also be IP based for L3 transports. This can also be something generated. |
| Destination MIH Function Identifier | MIHFL octets | Mandatory | This is the Destination MIHF identifier. This can be the L2 hardware address of the interface at the destination node. This can also be IP based for L3 transports. This can also be something generated during MIHF discovery and connectivity phase. |