| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements |  |
| Date Submitted | January, 2006 |  |
| Source(s) | Vivek Gupta, Ajoy Singh, Yoshi Ohba, Kalyan Koora |  |
| Re: | 21-06-0401-03-0000-Information_Elements |  |
| Abstract | TLV Representation of Information Elements |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Information Element Representation and Length

| No | Information Element | Representation | Length (octets) |
| --- | --- | --- | --- |
| 1 | Link Type | For example, RADIUS NAS-Port-Type attribute value which is a unique identifier defined in http://www.iana.org/assignments/radius-types, e.g., 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - 1X-EV Etc. | 4 |
| 2 | Operator Identifier | A combination of RADIUS Operator-Namespace and Operator-Name attributes defied in draft-ietf-geopriv-radius-lo-04.txt. Operator Name space: {GSM, CDMA, REALM,..} Operator Name: | Variable |
| 3 | PoA Location | Two types of location information are supported in draft-ietf-geopriv-radious-lo-04.txt, i.e., geospatial location information and civic location information. Geospatial location information consists of: - Latitude Resolution - Latitude - Longitude Resolution - Longitude - Altitude Type {Floor, Meters) - Altitude Resolution - Altitude - Map Datum (Co-ordinate reference system) Civic location information consists of: - Country Code - Civic address elements Method of obtaining location information - GPS - Manual | Variable |
| 4 | PoA Address | Link-layer address of PoA (Usually the MAC Address of the PoA) | Variable |
| 5 | Data Rate | Kbps | 4 (32-bit unsigned integer) |
| 6 | Channel Range | (Low, High range) MHz | 4,4 32-bit unsigned integer |
| 7 | PoA subnet info | (IPv4/IPv6) Each PoA may be assigned to multiple subnets | Variable |
| 8 | PoA Capabilities | Bitmap of different PoA capabilities | 4 |
| 9 | PHY Type | Classification of the Physical layer | Variable |
| 10 | MAC Type | Classification of the MAC layer | Variable |
| 11 | Cost | Should include the following: Value: (Actual cost denomination) Unit: (Country/Currency code) - enumerated Time Duration: Duration for which cost is applicable - enumerated Service Info: (whether unrestricted or restricted) | Variable |
| 12 | Security | Cipher Suites and Authentication Methods | Variable |
| 13 | QoS | QoS classes and Traffic Specifications | Variable |

2.1 TLV Representation for 802.21 Information Elements

| Type (4 octets) | Length (Variable octets) | Value (Variable octets) |
| --- | --- | --- |
| Represents the Type or ID of the Information Element | Length of the Value field of IE | The value of the IE |

Interpretation of the length field:

Case 1: If the number of octets occupied by the value field is less than 128, the size of the length field is always 1 octet and the MSB of the octet is set to the value ‘0’.

Case 2: If the number of octets occupied by the value field is greater than 127, then the length field is always greater than 1 octet. The MSB of the first octet of the length field is set to the value ‘1’ and the remaining 7 bits of the first octet indicate the number of octets that are appended further. The number represented by the 2nd octet of the length field indicates the total size of the value field.

2.2  Definition of Information Element Name Space

The 802.21 Information Elements can be assigned identifiers IDs as follows.

Each Information Element ID can be a 32 bit value.

| Range | Description | Comments |
| --- | --- | --- |
| 0x0000000 | Reserved |  |
| 0x00000001 - 0x1FFFFFFF | Reserved for 802.21 | Core 802.21 specific IEs |
| 0x2 ¦¦ (16 bit OUI) ¦¦ (14 bit ID) | Vendor specific IE | IE defined on a per vendor basis |
| 0x30 ¦¦ (8 bit Working Group identifier) ¦¦ (16 bit WG specific ID of IEs) | Reserved for different Working Groups. | Can be reserved for other different SDOs and 802 WGs such as 802.11, 802.16 etc., if they want to define anything specific for heterogeneous handovers |
| 0x31 ¦¦ (24 bit ID) | Reserved for IETF. | Can be reserved for IETF protocols and other higher layer IEs. |
| 0x32 ¦¦ ( 24 bits) | Reserved for playpen area. | Can be used in development and testing. Should not be used in released products. Avoids collision during development. |
| 0x33000000 – 0xFFFFFFFF | Reserved | For future use |

3. TLV Représentations

3.1 General Information

List of Access Networks (Link Types)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x1000001 | List of Neighboring Access Networks | Variable | List of different link types. 4 octects for each link |

Number of Operators (for each Access Network)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000002 | Number of operators for each link type | 2 octets | Number of different operators for each link type |

List of Operators (for each Access Network)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000003 | List of Network Operators for a specific link | Variable | List of different operators for each link type. The value field contains zero or more Operator Identifier TLVs. |

Operator Identifier

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000004 | An identifier of an operator | Variable | A combination of operator namespace and operator name is encoded in this field. The encoding format is defined in Table 1. |

Table 1: Value field format of Operator Identifier TLV

| Syntax | Length (octets) | Note |
| --- | --- | --- |
| L_OperatorNamespace | 1 | Length of OperatorNamespace in octets |
| L_OperatorName | 1 | Length of OperatorName in octets |
| OperatorNamespace | Variable | A value of RADIUS Operator-Namespace attribute defined in draft-ietf-geopriv-radius-lo-04.txt: GSM : GSM Association TADIG WG CDMA : IMSI Oversight Council REALM: IANA or delegate |
| OperatorName | Variable | A value of RADIUS Operator-Name attribute defined in draft-ietf-geopriv-radius-lo-04.txt. The value is a non-NULL terminated string whose Length MUST NOT exceeds 253 bytes. The value uniquely identifies the operator name within the scope of the operator namespace. |

3.2 Access Network specific information

Number of PoAs (for Access network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000101 | Number of PoAs | 2 octets | Number of PoAs for a specific link type + Operator combination |

Roaming Partners (for Access Network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000102 | Roaming partners | Variable | List of different operators. The value field contains zero or more Operator Identifier TLVs. |

Cost (for Access Network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000103 | Cost | Variable | Cost of accessing the network. |

An example could be:

Currency/Unit : enum values could be taken from standard values like in http://www.jhall.demon.co.uk/currency/by_country.html

Duration: {YEAR, MONTH, WEEK, DAY, HOUR, MINUTE, SECOND}

Service-Info: {Unrestricted, Restricted }

Security of Network (for Access Network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000104 | Security of Network | Variable | Authentication Methods and Cipher suites used. The length, format and semantics of this field are specific to each link type and defined by each media-specific WG or SDO. In many cases, this field contains values of media-specific MIB objects used for representing security characteristics of the media. |

QoS in Network (for Access Network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000105 | QoS of Network | Variable | QoS classes and Traffic Specifications. The length, format and semantics of this field are specific to each link type and defined by each media-specific WG or SDO. In many cases, this field contains values of media-specific MIB objects used for representing QoS characteristics of the media. |

3.3 PoA specific information

PoA Location (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000201 | PoA Location | Variable | The Value field of RADIUS Location-Information attribute is encoded in this field. The encoding format is defined in draft-ietf-geopriv-radius-lo-04.txt. |

Table 2: Value field format of PoA Location TLV

| Syntax | Length (octets) | Note |
| --- | --- | --- |
| LocationFormat | 1 | Format of location information 0: Civic location information 1: Geospatial location information |
| Method | 1 | Way location information was derived or discovered: 0: GPS 1: Assisted GPS 2: Manual 3: Provided by DHCP 4: Triangulation 5: Cell 6: IEEE 802.11 WLAN Access Point |
| L_LocationInformation | 1 | Length of LocationInformation in octets |
| LocationInformation | Variable | Civic location or Geospatial location information attributes |

Table 3: Value field format of PoA LocationInformation (Civic Address)

| Syntax | Length (octets) | Note |
| --- | --- | --- |
| Country Code | 2 | Two-letter ISO 3166 country code in capital ASCII letters |
| L_CivicAddressElements | 1 | Length of CivicAddressElements in octets |
| CivicAddressElements | Variable | Civic address elements |

Table 4: Value field format of PoA LocationInformation (Geospatial location)

| Syntax | Length (bits) | Note |
| --- | --- | --- |
| LatitudeResolution | 6 | Latitude Resolution |
| Latitude | 34 | Latitude |
| LongitudeResolution | 6 | Longitude Resolution |
| Longitude | 34 | Longitude |
| AltitudeType | 4 | Following codes are defined: 1: Meters 2: Floors |
| AltitudeResolution | 6 | Altitude Resolution |
| Altitude | 30 | Altitude |
| Datum | 8 | 1: WGS 2: NAD 83 (with associated vertical datum for North American vertical datum for 1998) 3: NAD 83 (with associated vertical datum for Mean Lower Low Water (MLLW)) |

PoA Data Rate (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000202 | PoA Data Rate | 4 octets | Data rate |

PoA MAC Type (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000203 | MAC Type | Variable | The length, format and semantics of this field are specific to each link type and defined by each media-specific WG or SDO. In many cases, this field contains values of media-specific MIB objects used for representing a MAC type of the media. An example for 802.16 is WmanIfMacVersion (of type INTEGER) of WMAN-IF-MIB. Another example for 802.11 is dot11StationConfigEntry (of type Dot11StationConfigEntry) of IEEE802dot11-MIB. |

PoA Channel Range (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000204 | Channel range | 8 octets | Channel Range (Low, High) |

PoA Subnet Information (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000205 | PoA subnet Info | Variable | This field contains a list of subnets to which the link of the PoA belongs. The encoding format is defined in Table 2. |

Table 5: Value field format of PoA Subnet Information TLV

| Syntax | Size (octets) | Note |
| --- | --- | --- |
| N_Subnet | 1 | The number of Subnets. |
| For (i=0; i<N_Subnet; i++) { | - | - |
| Address | Variable | An IP address of the PoA encoded as Address base type of Diameter protocol (RFC3588). AddressType is encoded in the first 2-octet, which must be either 1 (IPv4) or 2 (IPv6). |
| L_Prefix | 1 | The bit length of the prefix of the subnet to which Address belongs. L_Prefix<=32 for IPv4 subnet and L_Prefix<=128 for IPv6 subnet |
| } | - | - |

PoA Capabilities (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000206 | PoA Capabilities | 4 octets | PoA Capability bitmap Bit 0: Security Y/N Bit 1: QoS Y/N Bit 2: Internet Access Y/N Bit 3: IP Version 4 Y/N Bit 4: IP Version 6 Y/N Bit 5: Emergency Services Y/N Bit 6-31 : Reserved |

PoA PHY Type (for each PoA of Access Network + Operator combination )

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000207 | PHY Type | Variable | The length, format and semantics of this field are specific to each link type and defined by each media-specific WG or SDO. In many cases, this field contains values of media-specific MIB objects used for representing a PHY type of the media. An example for 802.16 is an OID suffix (4-octet integer) of an object defined as a sub-class of wmanIfBsPhy in WMAN-IF-MIB. Another example for 802.11 is the value of dot11PHYType instance (of type INTEGER) in IEEE802dot11-MIB. |

PoA Address

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| 0x10000208 | Link-layer address of PoA | Variable | The link-layer address of the PoA encoded as Address base type of Diameter protocol (RFC3588). AddressType is encoded in the first 2-octet. |

4. Information Request and Response

The client sends an Information Request to the MIH PoS. The MIH PoS sends an Information response to the client.

Information Request (From Client to Information Server)

| Type | Length | Additional Parameters |
| --- | --- | --- |
| Type value of IE | Length of Additional parameters | Any additional parameters for request |

Information Response (from Information Server to Client)

| Type | Length | Value |
| --- | --- | --- |
| Type value of IE | Length of Value | The value of corresponding IEs |

4.1 General Information

4.1.1 General Network Information Report (for all link types)

Request

| Type | Length | Additional Parameters |
| --- | --- | --- |
| 0x100000FF | 0 |  |

Response

| Type | Length |  |  |  |
| --- | --- | --- | --- | --- |
| 0x100000FF | Variable | Number of Links (1) |  |  |
|  |  | Link Type_1 (4) | Number of Operators for Link_Type_1 (1) | List of Operators TLV for Link_Type_1 (variable) |
|  |  | Link Type_2 (4) | Number of Operators for Link_Type_2 (1) | List of Operators TLV for Link_Type_2 (variable) |
|  |  | Link Type_3 (4) | Number of Operators for Link_Type_3 (1) | List of Operators TLV for Link_Type_3 (variable) |
|  |  | ……. | ……. | ……. |

4.1.2 List of Networks (Link Types)

Request

| Type | Length | Additional Parameters |
| --- | --- | --- |
| 0x10000001 | 0 |  |

Response

| Type | Length | Value |
| --- | --- | --- |
| 0x10000001 | Variable | List of different link types. Four (4) octets for each link |

4.1.3 Number of Operators (for each link type)

Request

| Type | Length | Additional Parameters |
| --- | --- | --- |
| 0x10000002 | 4 | Link type |

Response

| Type | Length | Value |
| --- | --- | --- |
| 0x10000002 | 2 | Number of Operators |

4.1.4 List of Operators (for each link type)

Request

| Type | Length | Additional Parameters |
| --- | --- | --- |
| 0x10000003 | 4 | Link type (4) |

Response

| Type | Length | Value |
| --- | --- | --- |
| 0x10000003 | Variable | List of Operators TLV |

4.2 Access Network specific Information

4.2.1 Access Network Information Report (for each link type and Operator)

Request

| Type | Length | Additional Parameters |  |
| --- | --- | --- | --- |
| 0x100001FE | Variable | Link Type (4) | Operator Identifier TLV (Variable) |

Response

| Type | Length |  | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x100001FE |  | Variable | Link Type_1 (4) | Operator Identifier TLV (Variable) | Number of PoAs (1) | List of Roaming Partners TLVs (variable) | Cost TLV (variable) | QoS TLV (variable) | Security of Network TLV (variable) |

4.2.2 Access Network Information Report (for all link types)

Request

| Type | Length | Additional Parameters |
| --- | --- | --- |
| 0x100001FF | 0 |  |

Response

| Type | Length | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x100001FF | Variable | Number of Links (1) |  |  |  |  |  |  |
|  |  | Link Type_1 (4) | Operator Identifier TLV (variable) | Number of PoAs (1) | List of Roaming Partners TLV (variable) | Cost TLV (variable) | QoS TLV (variable) | Security of Network TLV (variable) |
|  |  | Link Type_1 (4) | Operator Identifier TLV (variable) | Number of PoAs (1) | List of Roaming Partners TLV (variable) | Cost TLV (variable) | QoS TLV (variable) | Security of Network TLV (variable) |
|  |  | ………. |  |  |  |  |  |  |
|  |  | ……. |  |  |  |  |  |  |

4.3 PoA specific information

4.3.1 PoA Information Report (for each Link type and Operator)

Request

| Type | Length | Additional Parameters |  |
| --- | --- | --- | --- |
| 0x100002FF | 5variable | Link Type (4) | Operator Identifier TLV (variable) |

Response

| Type | Length |  | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x100002FF |  | Variable | Link Type_1 (4) | Operator Identifier TLV (variable ) | Number of PoAs (1) |  |  |  |  |  |
|  |  |  | PoA Address TLV (variable) | PoA Location TLV (variable) | Data Rate (4) | PoA PHY Type TLV (variable) | PoA MAC Type TLV (variable ) | Channel Range (4) | PoA Subnet information TLV (variable ) | Capability Bitmap (4) |
|  |  |  | PoA Address TLV (variable) | PoA Location TLV (variable ) | Data Rate (4) | PoA PHY Type TLV (variable ) | PoA MAC Type TLV (variable ) | Channel Range (4) | PoA Subnet information TLV (variable ) | Capability Bitmap (4) |
|  |  |  | …… |  |  |  |  |  |  |  |