| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Revised Basic Schema |  |
| Date Submitted | January 9th, 2006 |  |
| Source(s) | Yoshihiro Ohba, Kenichi Taniuchi, Subir Das |  |
| Re: | 21-06-0460-00-0000-Basic_Schema |  |
| Abstract | This document contains revised definition of Basic Schema. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

## This document contains revised definition of Basic Schema.

# Graphical Representation of Basic Schema

# Basic Schema Definition

<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf "http://www.w3.org/1999/02/22-rdf-syntax-ns#">

<!ENTITY rdfs "http://www.w3.org/2000/01/rdf-schema#">

<!ENTITY mihbase "URL_TO_BE_ASSIGNED">

<!ENTITY owl "http://www.w3.org/2002/07/owl#">

<!ENTITY xsd "http://www.w3.org/2001/XMLSchema#">

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;" xmlns:mihbase="&mihbase;"

xml:base="&mihbase;" xmlns:owl="&owl;" xmlns:xsd="&xsd;">

<owl:Ontology rdf:about="">

<rdfs:comment></rdfs:comment>

<rdfs:label>Basic Schema for IEEE 802.21 Information Service</rdfs:label>

</owl:Ontology>

<owl:Class rdf:ID="PoA">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#data-rate"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#phy-type"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#phy-type"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#mac-type"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#mac-type"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#channel-range"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#channel-range"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#qos"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#security"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#neighboring-poa"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#poa-location"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#poa-location"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator-identifier"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator-identifier"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#poa-capabilities"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#poa-capabilities"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#roaming-partner"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#poa-subnet-info"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#poa-address"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#link-type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

A distinct instance of this class must be assigned for each point of attachment.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="data-rate">

<rdfs:label>Data Rate</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

The data-rate property contains a maximum data rate of the link layer of the PoA in unit of Kbps.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="phy-type">

<rdfs:label>PHY Type</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;string"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="mac-type">

<rdfs:label>MAC Type</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;string"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="Channel-Range">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#low-channel-range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#high-channel-range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="channel-range">

<rdfs:label>Channel Range</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#Channel-Range"/>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="low-channel-range">

<rdfs:label>Low Channel Range</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Channel-Range"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="high-channel-range">

<rdfs:label>High Channel Range</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Channel-Range"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="security">

<rdfs:label>Security</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;boolean"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="uam">

<rdfs:label>UAM</rdfs:label>

<rdfs:subPropertyOf rdf:resource="#security"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="pana">

<rdfs:label>PANA</rdfs:label>

<rdfs:subPropertyOf rdf:resource="#security"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="qos">

<rdfs:label>QoS</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;boolean"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="diffserv">

<rdfs:label>Diffserv</rdfs:label>

<rdfs:subPropertyOf rdf:resource="#qos"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="neighboring-poa">

<rdfs:label>Neighboring PoAs</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#PoA"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="poa-location">

<rdfs:label>PoA Location</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#PoA-Location"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="operator-identifier">

<rdfs:label>Operator Identifier</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#Operator-Identifier"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="roaming-partner">

<rdfs:label>Roaming Partner</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#Operator-Identifier"/>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="cost">

<rdfs:label>Cost</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;string"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="poa-capabilities">

<rdfs:label>PoA Capabilities</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

PoA Capability bitmap

Bit 0:Security

Bit 1:QoS

Bit 2:Internet Access

Bit 3:IP Version 4

Bit 4:IP Version 6

Bit 5:Emergency Services

Bit 6:-31:Reserved

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="poa-subnet-info">

<rdfs:label>PoA subnet info</rdfs:label>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#Subnet-Information"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="Subnet-Information">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#subnet-address"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#prefix-length"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="subnet-address">

<rdfs:label>Subnet Address</rdfs:label>

<rdfs:domain rdf:resource="#Subnet-Information"/>

<rdfs:range rdf:resource="#Address"/>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="prefix-length">

<rdfs:label>Prefix Length</rdfs:label>

<rdfs:domain rdf:resource="#Subnet-Information"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="PoA-Location">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#geospatial-location-information"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#geospatial-location-information"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#civic-location-information"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#civic-location-information"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#method"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#method"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

This class has properties that indicate a location of PoA.

The properties include geospatial-location-information, civic-location-information and method. Any property can be added to this class

in an extended schema.

</rdfs:comment>

</owl:Class>

<owl:ObjectProperty rdf:ID="geospatial-location-information">

<rdfs:label>Geospatial Locaiton Information</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty" />

<rdfs:domain rdf:resource="#PoA-Location"/>

<rdfs:range rdf:resource="#Geospatial-Location-Information"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="Geospatial-Location-Information">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#latitude-resolution"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#latitude"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#longtitude-resolution"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#longitude"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#altitude-type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#altitude-resolution"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#altitude"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#datum"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

This class has properties that represent geographic coordinate.

The format is based on the Location Configuration Information (LCI) defined in RFC 3825.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="latitude-resolution">

<rdfs:label>Latitude Resolution</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:integer.minInclusive rdf:datatype="&xsd;integer">0</xsd:integer.minInclusive>

<xsd:integer.maxInclusive rdf:datatype="&xsd;integer">34</xsd:integer.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;integer"/>

</rdfs:range>

<rdfs:comment>

This property contains the latitude resolution.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="latitude">

<rdfs:label>Latitude</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:hexBinary.minInclusive rdf:datatype="&xsd;hexBinary">0</xsd:hexBinary.minInclusive>

<xsd:hexBinary.maxInclusive rdf:datatype="&xsd;hexBinary">3FFFFFFFF</xsd:hexBinary.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;hexBinary"/>

</rdfs:range>

<rdfs:comment>

This property contains the integer part of latitude of the location.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="longitude-resolution">

<rdfs:label>Longitude Resolution</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:integer.minInclusive rdf:datatype="&xsd;integer">0</xsd:integer.minInclusive>

<xsd:integer.maxInclusive rdf:datatype="&xsd;integer">34</xsd:integer.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;integer"/>

</rdfs:range>

<rdfs:comment>

This property contains the longitude resolution.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="longitude">

<rdfs:label>Longitide</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:hexBinary.minInclusive rdf:datatype="&xsd;hexBinary">0</xsd:hexBinary.minInclusive>

<xsd:hexBinary.maxInclusive rdf:datatype="&xsd;hexBinary">3FFFFFFFF</xsd:hexBinary.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;hexBinary"/>

</rdfs:range>

<rdfs:comment>

This property contains the longitude of the location.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude-type">

<rdfs:label>Altitude Type</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:integer.minInclusive rdf:datatype="&xsd;integer">0</xsd:integer.minInclusive>

<xsd:integer.maxInclusive rdf:datatype="&xsd;integer">2</xsd:integer.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;integer"/>

</rdfs:range>

<rdfs:comment>

This property contains the altitude type.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude-resolution">

<rdfs:label>Altitude Resolution</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:integer.minInclusive rdf:datatype="&xsd;integer">0</xsd:integer.minInclusive>

<xsd:integer.maxInclusive rdf:datatype="&xsd;integer">30</xsd:integer.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;integer"/>

</rdfs:range>

<rdfs:comment>

This property contains the altitude resolution.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude">

<rdfs:label>Altitude</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:hexBinary.minInclusive rdf:datatype="&xsd;hexBinary">0</xsd:hexBinary.minInclusive>

<xsd:hexBinary.maxInclusive rdf:datatype="&xsd;hexBinary">3FFFFFFF</xsd:hexBinary.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;hexBinary"/>

</rdfs:range>

<rdfs:comment>

This property contains the altitude of the location.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="datum">

<rdfs:label>Datum</rdfs:label>

<rdfs:domain rdf:resource="#Geospatial-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:integer.minInclusive rdf:datatype="&xsd;integer">1</xsd:integer.minInclusive>

<xsd:integer.maxInclusive rdf:datatype="&xsd;integer">2</xsd:integer.maxInclusive>

<rdfs:subClassOf rdf:resource="&xsd;integer"/>

</rdfs:range>

<rdfs:comment>

This property contains the datum type.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="civic-location-information">

<rdfs:label>Civic Location Information</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty" />

<rdfs:domain rdf:resource="#PoA-Location"/>

<rdfs:range rdf:resource="#Civic-Location-Information"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="Civic-Location-Information">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#country-code"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#country-address-elements"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

This class has properties that represent civic address.

The format is defined in draft-ietf-geopriv-radius-lo-04.txt.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="country-code">

<rdfs:label>Country Code</rdfs:label>

<rdfs:domain rdf:resource="#Civic-Location-Information"/>

<rdfs:range rdf:parseType="Resource">

<xsd:string.length rdf:datatype="&xsd;integer">2</xsd:string.length>

<rdfs:subClassOf rdf:resource="&xsd;string"/>

</rdfs:range>

<rdfs:comment>

This property contains the two-letter ISO 3166 country code in capital ASCII letters.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="country-address-elements">

<rdfs:label>Country Address Elements</rdfs:label>

<rdfs:domain rdf:resource="#Civic-Location-Information"/>

<rdfs:range rdf:resource="#Country-Address-Elements"/>

<rdfs:comment>

This property contains the location information element.The format of the civic address elements is described in Section 3.3 of draft-ietf-geopriv-dhcp-civil-08.txt with a TLV pair (whereby the Type and Length fields are one octet long).

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="Country-Address-Elements">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#country-address-element"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="country-address-element">

<rdfs:label>Country Address Element</rdfs:label>

<rdfs:domain rdf:resource="#Country-Address-Elements"/>

<rdfs:range rdf:resource="#Country-Address-Element"/>

<rdfs:comment>

This property contains the country address elements.The format of the civic address elements is described in Section 3.4 of draft-ietf-geopriv-dhcp-civil-08.txt with a TLV pair (whereby the Type and Length fields are one octet long).

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="Country-Address-Element">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#catype"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cavalue"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="catype">

<rdfs:label>CAtype</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Country-Address-Element"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

A one-octet descriptor of the data civic address value.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cavalue">

<rdfs:label>CAvalue</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Country-Address-Element"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

The civic address value.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="method">

<rdfs:label>Method</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty" />

<rdfs:domain rdf:resource="#PoA-Location"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="link-type">

<rdfs:label>Link Type</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

This property contains a RADIUS NAS-Port-Type attribute value defined in

http://www.iana.org/assignments/radius-types.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="poa-address">

<rdfs:label>PoA Address</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#PoA"/>

<rdfs:range rdf:resource="#Address"/>

<rdfs:comment>

This property contains a link-specific identifier of PoA.

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="Address">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#address-type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#address"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="address-type">

<rdfs:label>Address Type</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Address"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="address">

<rdfs:label>Address</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Address"/>

<rdfs:range rdf:resource="&xsd;hexBinary"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="Operator-Identifier">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator-namespace"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">0</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator-namespace"/>

<owl:maxCardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:maxCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator-name"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

This class has properties that represent an operator.

The properties are operator-namespace and operator-name.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="operator-namespace">

<rdfs:label>Operator Namespace</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Operator-Identifier"/>

<rdfs:range rdf:parseType="Resource">

<xsd:string.minLength rdf:datatype="&xsd;integer">1</xsd:string.minLength>

<xsd:string.maxLength rdf:datatype="&xsd;integer">253</xsd:string.maxLength>

<rdfs:subClassOf rdf:resource="&xsd;string"/>

</rdfs:range>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="operator-name">

<rdfs:label>Operator Name</rdfs:label>

<rdf:type rdf:resource="&owl;FunctionalProperty"/>

<rdfs:domain rdf:resource="#Operator-Identifier"/>

<rdfs:range rdf:parseType="Resource">

<xsd:string.minLength rdf:datatype="&xsd;integer">1</xsd:string.minLength>

<xsd:string.maxLength rdf:datatype="&xsd;integer">253</xsd:string.maxLength>

<rdfs:subClassOf rdf:resource="&xsd;string"/>

</rdfs:range>

<rdfs:comment>

This property contains the name of the operator.

</rdfs:comment>

</owl:DatatypeProperty>

</rdf:RDF>