| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | MIH Link Handover Complete Event Definition |
| Date Submitted | January 9th, 2006 |
| Source(s) | Mathieu PERESSE, Eric NJEDJOU, Qiaobing XIE |
| Ref: | 21-06-0465-00-0000-MIH_Link_Handover_Complete |
| Abstract | This contribution proposes the definition for MIH Link Handover Complete primitive |
| Purpose | This contribution proposes the definition for MIH Link Handover Complete primitive |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

### 7.4.x MIH Link Handover Complete

MIH Link Handover Complete is issued by the MIH Function to report the successful completion of link handover to its upper layers. The event notified may be either an intra-technology or inter-technology link handover. The link handover can be a result of either the execution of an MIH Switch command previously issued by an MIH user or an autonomous L2 handover/switching event indicated by a Link Handover Complete indication from a link layer.

### 7.4.x.1 MIH_Link_Handover_Complete.indication

#### 7.4.x.1.1 Function

This primitive is issued by the MIH Function to report the completion of a link handover to upper layers. This event may notify MIH users of a successful intra-technology or inter-technology handover.

This MIH event can be local or remote.

Event Type:	MIH event

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | MIH_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC Address | MAC Address of Mobile Terminal |
| HO_Type |  | Indicate whether this is an intra-technology or inter-technology HO. |
| NewLinkIdentifier | Media dependent | Identifier of the new active link (e.g. MAC Address) |
| OldLinkIdentifier | Media dependent | Identifier of the old active link (e.g. MAC Address) |
| MACNewPoA | MAC Address | Identifier of the new serving PoA. |
| MacOldAccessRouter | MAC Address | MAC Address of old Access Router (if any) |
| MacNewAccessRouter | MAC Address | MAC Address of new Access Router (if any) |

MIH_EVENT_SOURCE is defined as follows:

{Local Node, Remote Node} Local is referred with respect to origination of event.

#### 7.4.x.1.2 When generated

This notification is generated by the MIH Function when a link layer inter- or intra-technology handover is completed. In the case of an intra-technology handover, the event could be triggered by the reception of a Link_Handover_Complete.indication from a link. In the case of an inter-technology handover, the event could be triggered by the successful execution of an MIH Switch primitive issued earlier by an MIH user.

#### 7.4.x.1.3 Effect on receipt

Upper layer entities may take different actions on this notification. Typically, an MIH user may make use of this notification to configure other layers (IP, Mobile IP) for various upper layers handovers (e.g. L3 Handovers) that are needed. Transport layers (e.g. TCP) may also make use of this primitive to fine tune their flow control & flow congestion mechanisms.