| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | MIH Link Parameter Report primitive |
| Date Submitted | January 9th, 2006 |
| Source(s) | Mathieu PERESSE, Eric NJEDJOU, Qiaobing XIE |
| Ref: | 21-06-0466-00-0000-MIH_Link_Parameter_Report |
| Abstract | This contribution proposes the definition and enhancements of the current MIH Link Parameter Change primitive. It also proposes to change the name of the primitive to MIH Link Parameter Report. |
| Purpose | The definition and enhancements of the MIH Link Parameter Change primitive. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

### 7.4.x MIH Link Parameter Report

Upper layer entities may want to monitor certain link parameters to assess the status of the currently connected and potentially available links. MIH Link Parameter Report is an MIH event that indicates the current value as well as any changes in certain link layer parameters to upper layer entities. This MIH Event can be local or remote.

MIH gets current link layer parameters by receiving Link Parameter Change indication from a link, or by some other means such as using Radio Resource Management facilities provided by the link layers (e.g. 802.11k, 802.16g, cellular radio measurements)

MIH Link Parameter Report event can be triggered upon changes in link parameters (e.g., when a given parameter crosses a specified threshold), or at a regular interval pre-determined by a user configurable timer.  The upper layers can use MIH Configure primitive to set the thresholds as well as the report timer.

### 7.4.x.1 MIH_Link_Parameter_Report.indication

#### 7.4.x.1.1 Function

MIH_Link_Parameters_Report is issued by the MIH Function to upper layers to indicate various values of link parameters. The list of available link parameters is TBD.

The event can be local or remote.

Event Type:	MIH event

#### Parameters

| Name | Type | Description |
| --- | --- | --- |
| EventSource | MIH_EVENT_SOURCE | The origination point from where the event is generated. |
| MacMobileTerminal | MAC ADDRESS | Mac Address of Mobile Terminal |
| LinkIdentifer | TBD | Unique link identifier that can be used by the upper layer to identify from which link this parameter report is about. |
| LinkParameterList | TBD | A list of triples of {link parameter type, oldValueofLinkParameter, newValueOfLinkParameter) |

MIH_EVENT_SOURCE is defined as follows:

{Local Node, Remote Node}. Local and remote is referred with respect to origination of event.

#### 7.4.x.1.2 When generated

This notification is generated either at a predefined regular interval determined by a user configurable timer or when a specified parameter of the currently active interface crosses a configured threshold.

#### 7.4.x.1.3 Effect on receipt

Upper layer entities may take different actions on this notification. For example, a Mobility Management Entity locally or remotely located from the event origination may take this event into account in its decision algorithm.