| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Commentary attachments for event and command updates |  |
| Date Submitted | January 16th, 2005 |  |
| Source(s) | Srinivas Sreemanthula and Vivek Gupta |  |
| Ref: | 21-06-0477-00-0000-Events-Commands-Updates |  |
| Abstract | This contribution provides clarifications to events and commands defined in commentary. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Remedy #1 - MIH Event Directions

| Event Id | Link Event Type | Link Event Name | Local (L), Remote (R) | Remote direction (Client->Network, Network->Client or Network<->Network) | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | State Change | MIH_Event_Up | L, R | Client->Network Network<->Network | Link is initialized and available for use |
| 2 | State Change | MIH_Event_Down | L, R | Client->Network Network<->Network | Link is down and not available |
| 3 | State Change | MIH_Event_Detected | L, R | Client->Network Network<->Network | A new link is detected |
| 4 | State Change | MIH_Event_Parameters_Change | L, R | All | Link parameters have changed |
| 5 | Predictive | MIH_Event_Going_Down | L, R | All | Link conditions are degrading |
| 6 | Administrative | MIH_Event_Roll_back | L, R | All | To roll back a previous event |
| 7 | Link Transmission | MIH_Event_SDU_Transmit_Success | L | n/a | Link transmit success of a higher layer PDU fragments |
| 8 | Link Transmission | MIH_Event_SDU_Transmit_Failure | L | n/a | Link transmit failure of a higher layer PDU fragments |
| 9 | Link Synchronous | MIH_Event_Handover_Imminent | L, R | All |  |
| 10 | Link Synchronous | MIH_Event_Handover_Proceeding | L, R | All |  |
| 11 | Link Synchronous | MIH_Event_Handover_Complete | L, R | All |  |

# Remedy #2 - Command updates

## 7.4.10.1.4

The local MIH Function responds immediately with a MIH_Handover_Initiate.confirm primitive. The remote MIH Function responds with MIH_Handover_Initiate.response primitive. The remote MIH Function may send a MIH_Handover_Prepare message to the MIH Function in the new link under consideration. This message attempts to prepare the new link for impending handover. This may involve querying the new link for available resources and also potentially establishing a path for transfer of any future packets.

### Suggested text

. The remote MIH Function responds with MIH_Handover_Initiate.response primitive. If the recipient of the command is a network MIH function, it may perform a MIH_Handover_Prepare message exchange with the MIH Function in the target link under consideration before sending the response message.

## 7.4.10.2.4

On receiving the response the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the response. In case the handover is to executed, the peer MIH function may also provide information about preferred network and preferred PoA on the preferred network.

### Suggested text

On receiving the response the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the response. If the recipient of the command is a client MIH function and in case the handover is to be executed, it may also provide information about selected network and selected PoA on the selected network with a MIH Handover Commit message.

## 7.4.12.1.1

This primitive is used by a peer MIH Function to communicate with the MIH Function on network. The primitive is used to communicate commitment to perform handover based on selected choices for candidate network and PoA.

### Suggested text

This primitive is used by a peer MIH Function to communicate with the remote MIH Function. The primitive is used to communicate commitment to perform handover based on selected choices for candidate network and PoA.

## 7.4.12.2.4

The receipient may initiate handover process and begin setting up of new layer 2 connection.

### Suggested text

The client recipient may initiate handover process and begin setting up of new layer 2 connection. The network recipient may determine that the handover procedure is in progress to the intended network.

## 7.4.13.1.1

This primitive is used by a peer MIH Function on new PoA to communicate with the MIH Function on old PoA. The primitive is used to communicate the status of handover operation. This allows the old PoA to start forwarding any pending or remaining packets.

### Suggested text

This primitive is used by a peer MIH Function to communicate with the remote MIH Function. This is used to communicate the completion of handover operation.

## 7.4.13.1.4

On receiving the request the old PoA may forward any pending packets to new PoA.

### Suggested text

Upon receipt, the receiving MIH Function in the old link can start forwarding any pending or remaining packets and also release any resources in use by the old link. Upon receipt, the receiving MIH Function on the new link must forward the message to the MIH Function in the old link.

## 7.4.13.2 MIH_Handover_Complete.response

## 7.4.13.2.1 Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that has sent out the MIH_Handover_Complete.request. This primitive is used to send a response to the handover complete request.

7.4.13.1.2 Semantics of service primitive

MIH_Handover_Complete.response

(

CommandSource,

CommandDestination,

CurrentLinkIdentifier

)

Local or Remote: Both

MIHF (old Network) <> MIHF (new Network)

MIHF (network) <> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYPE | n/a |  |
| CommandDestination | MIH_LOCAL MIH_REMOTE | n/a |  |
| CurrentLinkIdentifier | MAC Address | n/a |  |

## 7.4.13.2.3 When generated

The MIH Function responds with this primitive after processing the handover complete request.

## 7.4.13.2.4 Effect on receipt

Upon receipt, the client MIH Function can determine that the handover complete request was processed successfully.  Upon receipt, the MIH Function in the new network may forward this message to the client.