| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Capability Discovery Updates |  |
| Date Submitted | January 16th, 2005 |  |
| Source(s) | Srinivas Sreemanthula |  |
| Ref: | 21-06-0478-00-0000-capability-discovery-update |  |
| Abstract | This contribution provides clarifications to events and commands defined in commentary. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Remedy #1

# 7.4.1.1 MIH_Capability_Discover.request

7.4.1.1.1 Function

This primitive is used by MIH Function or upper layer entity to discover MIH capability of different links.

7.4.1.1.2 Semantics of service primitive

MIH_Capability_Discover.request (

SourceIdentifier,

DestinationIdentifier,

SupportedEventList,

SupportedCommandList

)

Local or Remote: Both

MIHF (network) <> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier |  |
| Destination Identifier | Identifier | Any valid individual or group identifier |  |
| SupportedEventList | List | Enumerated Event Codes | Supported event codes |
| SupportedCommandList | List | Enumerated Command Codes | Supported command codes |

## 7.4.1.1.3 When generated

This primitive is generated by the upper layers that need to determine MIH capability information of the link layers.

## 7.4.1.1.4 Effect on receipt

MIH Function capable entities may respond with MIH_Capability_Discover.response primitive.

# Remedy #2

## 8.4.1.2

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| SupportedEventList | TBD | Variable (padded) | Array of event codes formatted as message-id or enhanced-message-id (length is defined in MIH header) Code-1; Code-2; Code-3; …. |
| SupportedCommandList | TBD | Variable (padded) | Array of command codes formatted as message-id or enhanced-message-id (length is defined in MIH header) Code-1; Code-2; Code-3; …. |
| SupportedTransportList | TBD | 4 | Octet 1 specifies the transport option for the event service. Octet 2 specifies the transport option for the command service. Octet 3specifies the transport option for the information service. For each bit location a value of ‘1’ indicates that the transport option is supported. Bit #0: L2 Bit #1: L3 or higher layer protocol Bit #2~7: Reserved |

## 8.4.2.1 & 8.4.2.3

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| RequestedEventList | TBD | Variable (padded) | Array of event codes formatted as message-id or enhanced-message-id (length is defined in MIH header) Code-1; Code-2; Code-3; …. |

## 8.4.2.2 & 8.4.2.4

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| RequestedCommandList | TBD | Variable (padded) | Array of commanf codes formatted as message-id or enhanced-message-id (length is defined in MIH header) Code-1; Code-2; Code-3; …. |