| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | M_SAP & C_SAP Update based on the accepted Requirement and Amendment document of 802.16 |  |
| Date Submitted | January, 2006 |  |
| Source(s) | Ronny Kim, Jin Lee LG Electronics, Inc. 533,Hogye-1dong,Dongan-gu, Anyang-shi,Kyongki-do,Korea Vivek Gupta Intel Corporation JF3-206, 2111 NE 25th Avenue, Hillsboro, OR 97124, USA | Voice: +82-31-450-1879 Fax: +82-31-450-7912 [mailto : {ronnykim, jins978}@lge.com] Voice: 503 712 1754 Fax: 503 264 4230 [mailto:vivek.g.gupta@intel.com] |
| Re: | IEEE 802.21 Session #12 in Hawaii |  |
| Abstract | This contribution is to reflect the primitives accepted by 802.16g into 802.21 draft. |  |
| Purpose | Update 802.21 draft |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# 7.2.1.2 Management SAP (M_SAP )

# M_SAP is defined by IEEE 802.16g and it may include, but is not limited to primitives related to:

# System configuration

# Monitoring Statistics

# Notifications/Triggers

# Multi-mode interface management

# [Note] Text is from IEEE 802.16g baseline document (IEEE802.16g/008r2) and Multi-mode interface management is for 802.21 multimode interfaces. IEEE802.21 shall provide clear definition of it.

# 7.2.1.3 C_SAP

# C_SAP is defined by IEEE 802.16g and it may include, but is not limited to primitives related to:

# Handovers (e.g. notification of HO request from MS, etc.)

# Idle mode mobility management (e.g. Mobile entering idle mode)

# Subscriber and session management (e.g. Mobile requesting session setup)

# Radio resource management, etc.

# AAA server signaling (e.g. EAP payloads)

# Media Independent Function Services

# [Note] Text is from IEEE 802.16g baseline document (IEEE802.16g/008r2) and IEEE802.21 shall provide clear definition of Media Independent Function Services..

## The key primitives defined as part of C_SAP are described in the table below.

| No | Primitives | Description |
| --- | --- | --- |
| 1 | M_Ranging | Upper layers can control ranging procedure with these primitives. Upper layers shall commence 802.16 link setup procedure by sending M_Ranging.request primitive through NCMS. Note) M_Ranging.request primitive with ranging type: “initial” is an 802.16 Link Switch Link Command (link setup) which corresponds to Link Switch MIH command. |
| 2 | M_Registration | Upper layers can control registration procedure with these primitives. Upper layers are notified of link setup by M_Registration.confirmation. Note) M_Registration.confirmation primitive is an 802.16 Link_Up Link Event. |
| 3 | M_Neighbor | When 802.16 MAC receives neighbor advertisement (MOB_NBR-ADV), this primitive is used to deliver the information to upper layers. |
| 4 | M_ScanScheduling | Upper layers can schedule scanning period with BS. During scanning period BS may buffer downlink traffic to the mobile terminal. |
| 5 | M_Scanning | Upper layers can command autonomous scanning with these primitives. Note) These primitives are Scan Link Command for MIH Scan command in 802.21. |
| 6 | M_ScanReport | Delivery of the primitives shall be based on the pre-registration procedure between upper layer management entities and NCMS. Scan report can be made remotely to the BS or locally to the upper layer entity depending on the report target value in M_ScanReport.request. |
| 7 | M_MACHandover | Upper layers can control handover procedure by using these primitives. Note) These primitives can be used as link commands for MIH_Handover_initiate.request/response. For remote command service, 802.16 MAC management messages shall be used. (MOB_MSHO-REQ/MOB_BSHO-REQ/MOB_BSHO-RSP) Currently only parameters relevant to 802.16 handover are included. Parameters for Media Independent Handover shall be identified and added. |
| 8 | M_HOIND.request/confirmation | An MS transmits a MOB_HO-IND message for final indication that is about performing a HO. Note) This primitive is a Link Command of MIH_Handover_Commit.request/response MIH Command in the .21 draft. For remote service, 802.16 MAC management message shall be used. (MOB_HO-IND). Currently only parameters relevant to 802.16 handover are included. Parameters for Media Independent Handover shall be identified and added. |
| 9 | M_Management | These primitives are used to manage the status of mobile terminal. Upper layer can change the status of mobile terminal into power on/down/hold/de-register, etc. Note) These can be mapped to MIH Configure MIH Command Service. For remote service, 802.16 MAC management messages can be used. (RES-CMD/ DREG-CMD) |