| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Service Category Allocation to MIH Primitives and Link Primitives |  |
| Date Submitted | January 9, 2006 |  |
| Source(s) | Eunah Kim, Junghoon Jee, Hongseok Jeon ETRI 161 Gajeong-Dong, Yuseong-Gu Daejeon, 305-700, KOREA | Voice: +82-42-860-4820 Fax: +82-42-861-5404 {eakim, jhjee, jeonhs}@etri.re.kr |
| Re: | IEEE 802.21 Session #NN in Hilton Waikoloa Village, Hawaii |  |
| Abstract | This contribution proposes a proper allocation of service category to MIH primitives and their corresponding Link primitives. |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Proposed Changes

Table 5-Link Commands (page 44)

| No | Link Command | Local(L), Remote(R) | Remote Direction | Comments |
| --- | --- | --- | --- | --- |
| 1 | Link Capability Discovery | L |  | Discover the 802.21 capabilities supported by the link layer |
| 2 | Link Event Register | L |  | Register to receive link event notifications |
| 3 | Link Event Deregister | L |  | Deregister from receiving link event notifications |
| 41 | Link Configure Thresholds | L |  | Configure the thresholds for various link triggers such as Link Going Down |

Table 7-MIH_LINK_SAP Primitives (Page 53)

| No | Primitives | Service Category | Description |
| --- | --- | --- | --- |
| 1 | Link EventCapability Discover | Command General | Discover link capabilities |
| 2 | Link Event Register | Command Event | Register for event notifications |
| 3 | Link Event Deregister | Command Event | Deregister for event notification |
| 4 | Link Configure Thresholds | Command | Configure link thresholds for Link Parameter Change notifications |
| … | … | … | … |

Table 8-MIH_SAP Primitives (Page 54)

| No | Primitives | Service Category | Description |
| --- | --- | --- | --- |
| 1 | MIH Capability Discover | Capability DiscoveryGeneral | Discover list of Events and Commands supported by MIH Function |
| 2 | MIH Event Register | Event | Register for MIH event notifications |
| 3 | MIH Event Deregister | Event | Deregister for MIH event notifications |
| 4 | MIH Event Configure Link Configure Threshold | EventCommand | Configure thresholds for MIH events |
| … | … | … | … |

Table 13-MIH Function Messages

| No | MIH Message Identifier | MIH Opcode | Service Category |
| --- | --- | --- | --- |
| 1 | MIH Capability Discover | Request, Response | Capability DiscoveryGeneral |
| 2 | MIH Event Register | Request, Response | RegistrationEvent |
| 3 | MIH Event Deregister | Request, Response | RegistrationEvent |
| 4 | MIH Link Event Configure Link Configure Threshold | Request, Response | Event ConfigurationCommand |
| … | … | … | … |