| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Amendment of MIH Capability Discover in Section 7.4.1 |  |
| Date Submitted | January 9, 2006 |  |
| Source(s) | Eunah Kim, Hongseok Jeon, Junghoon Jee ETRI 161 Gajeong-Dong, Yuseong-Gu Daejeon, 305-700, KOREA | Voice: +82-42-860-4820 Fax: +82-42-861-5404 {eakim, jeonhs, jhjee}@etri.re.kr |
| Re: | IEEE 802.21 Session #NN in Hilton Waikoloa Village, Hawaii |  |
| Abstract | This contribution proposes several amendments regarding the MIH Capability Discover request/response primitives. |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

7.4.1 MIH Capability Discover

7.4.1.1 MIH_Capability_Discover.request

7.4.1.1.1 Function

This primitive is used by MIH Function or upper layer entity to discover MIH capability of different links.

This primitive is generated by upper layers to MIH layer to discover MIH capability of different links.

7.4.1.1.2 Semantics of service primitive

MIH_Capability_Discover.request (

SourceIdentifier,

DestinationIdentifier,

SupportedEventList,

SupportedCommandList,

SupportedTransportList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the polling request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the polling request shall be sent. |
| SupportedEventList | Bitmap |  | List of supported events. Refer to 8.4.1.2 |
| SupportedCommandList | Bitmap |  | List of supported commands. Refer to 8.4.1.2 |
| SupportedTransportList | Bitmap |  | List of supported transport for each service. Refer to 8.4.1.2 |

7.4.1.1.3 When generated

This primitive is generated by the upper layers that need to determine MIH capability information of the link layers.

7.4.1.1.4 Effect on receipt

MIH Function capable entities may respond with MIH_Capability_Discover.response primitive.

7.4.1.2 MIH_Capability_Discover.response

7.4.1.2.1 Function

This primitive is used by MIH Function to convey supported MIH capabilities about Event Service, Command Service and Information Service.

7.4.1.2.2 Semantics of service primitive

MIH_Capability_Discover.response (

SourceIdentifier,

DestinationIdentifier,

MIH Capability,

SupportedEventList,

SupportedCommandList,

SupportedTransportList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the polling requestresponse is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of local or peer MIH entity to which the polling requestresponse shall be sent. |
| MIH Capability | Flag |  | 0: MIH functionality not supported 1: MIH functionality is supported |
| SupportedEventList | Flag Bitmap | Set of supported events | List of supported events. Refer to 8.4.1.2 |
| SupportedCommandList | Flag Bitmap | Set of supported commands | List of supported commands. Refer to 8.4.1.2 |
| SupportedTransportList | Bitmap |  | List of supported transport for each service. Refer to 8.4.1.2 |

7.4.1.2.3 When generated

This primitive is generated when MIH_Capability_Discover.request primitive is received by MIH Function.

7.4.1.2.4 Effect on receipt

Upon reception of this primitive the receiving entity becomes aware of supported MIH capabilities.