| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | 21-06-0492-01-0000-GenRefModel.doc |  |
| Date Submitted | January, 2006 |  |
| Source(s) | Andrea Francini, Peretz Feder, Shyam Parekh | Lucent Technologies francini@lucent.com, pfeder@lucent.com, sparekh@lucent.com |
| Re: | P802-21-D00-04.pdf |  |
| Abstract | Revision proposal for Sections 5.5, 5.5.1, 5.5.2, and 5.5.3 (General MIH Reference Model and IEEE 802 MIH Reference Models). |  |
| Purpose | Replace text in current draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

## MIH Reference Models for Access Networks

The messaging exchanges between peer MIH Function instances, in particular the type of transport that they use, are sensitive to several factors, such as the nature of the network nodes that contain the peer MIH Function instances (whether or not one of the two is a Mobile Node or a PoA), the nature of the access network (whether IEEE 802 or 3G cellular), and the availability of MIH capabilities at the PoA.

Figure 4 presents a summary of the types of relationship that may exist between the MIH Function and other functional components in the same network node, and between the MIH Function and a peer MIH Function instance residing in a different network node.

Figure 4—Types of MIH Function relationship

The General MIH Reference Model enables a simple representation of the broad variety of MIH Function relationships shown in Figure 4. In the Model, a mobility-management protocol stack (or plane) is logically identified within each network node that includes an MIH Function instance. The abstraction provided by the mobility-management plane makes it easy to isolate and represent the MIH relationships with all pre-existing functional entities within the same network node. Such relationships are both internal (with functional entities that share with the MIH function the logical inclusion in the mobility-management plane) and external (with functional entities that belong to other planes, such as the transport plane and the network/element management plane).

Figure 5 illustrates the placement of the MIH Function within the mobility-management plane and its interaction with the transport and network/element management planes.

Figure 5—General MIH Reference Model

All exchanges between the MIH Function and other functional entities occur through service primitives, grouped in Service Access Points (SAPs). Each SAP identifies an interface with a distinct functional entity within the same network node.

The technology-agnostic General MIH Reference Model includes the following four SAPs:

- MIH_SAP: Media-independent interface with the upper layers of the mobility-management protocol stack.

- MIH_LINK_SAP: Generic name for the media-dependent interface with the lower layers of the mobility-management protocol stacks.

- MIH_NET_SAP: Generic name for the media-dependent interface with the transport plane, which supports the exchange of MIH information and messages with remote MIH Function instances. In IEEE 802 networks the MIH_NET_SAP typically maps onto a technology-specific link-layer transport SAP and a common higher-layer transport interface. In 3G cellular networks the MIH_NET_SAP typically maps onto the common higher-layer transport interface.

- MIH_SME_SAP: Media-independent interface with the management system that controls the network element.

The General MIH Reference Model works as a template for the definition of the technology-specific reference models. In those models, the media-independent SAPs (MIH_SAP and MIH_SME_SAP) always maintain the same names and sets of primitives. Conversely, the media-dependent SAPs (MIH_LINK_SAP and MIH_NET_SAP) assume media-specific names and sets of primitives, often reusing names and primitives that already exist in the respective pre-existing lower-layer SAPs. The MIH_LINK_SAP and MIH_NET_SAP never appear in the technology-specific reference models with their generic names.

The inclusion in the General MIH Reference Model of media-independent SAPs enables the specification of a single, homogeneous set of service primitives across all link-layer technologies. Then, in the technology-specific reference models, the general service primitives map onto functionally identical service primitives, which are found in the pre-existing technology-specific SAPs and/or newly defined in recommended extensions of those SAPs.

### MIH Reference Model for 802.3

The logical placement of the MIH Function in the 802.3 protocol stack is shown in Figure 6.

Figure 6—Logical Placement of the MIH Function in the IEEE 802.3 protocol stack

Figure 7 shows the instantiation of the General MIH Reference Model when the access link complies with the IEEE 802.3 standard.

Figure 7—MIH Reference Model for IEEE 802.3

In the 802.3 reference model, the Logical Link Control (LLC) layer is the immediate MIH counterpart in the lower layers of the mobility-management plane.

The pre-existing LLC SAP (LSAP) instantiates the MIH_LINK_SAP of the General MIH Reference Model: the MIH Function uses the LSAP to exchange information with the control portion of LLC for mobility-management purposes. The LSAP also instantiates the link-layer portion of the generic MIH_NET_SAP for the transport of MIH messages over L2. The MIH Function interface that instantiates the higher-layer portion of the MIH_NET_SAP is TBD.

[Note 1: The TBD on the higher-layer portion of the MIH-NET_SAP must be resolved. Is it L3 or L4? If L4, is it TCP or UDP? The resolution should be common to all access technologies, since the higher-layer portion of the MIH_NET_SAP is media-independent.]

[Note 2: Is there a Management Plane in the 802.3 case? If not, the Management Plane box and the MIH_SME_SAP label should be removed from Figure 7.]

### MIH Reference Model for 802.11

The logical placement of the MIH Function in the protocol stack of 802.11 stations and access points is shown in Figure 8.

Similarly to 802.3, the LLC SAP (LSAP) defines the interface of the MIH Function with the 802.11 data plane and can encapsulate MIH messages in data frames. However, since 802.11 does not currently support Class 1 data frames, MIH messages can be transported over the 802.11 data plane only after the Mobile Node has associated with the 802.11 access point. Before the association between Mobile Node and access point takes place, the L2 transport of MIH messages can rely on 802.11 management frames from the 802.11 management plane (MLME). The MIH_MLME_SAP defines the interface between the MIH Function and the MLME.

Figure 8—Logical Placement of the MIH Function in the IEEE 802.11 Protocol Stack

Figure 9 shows the instantiation of the General MIH Reference Model when the access link complies with the IEEE 802.11 standard.

Figure 9— MIH Reference Model for IEEE 802.11

Since the MAC Layer Management Entity (MLME) is the immediate MIH counterpart in the lower layers of the 802.11 mobility-management plane, the MIH_MLME_SAP instantiates the MIH_LINK_SAP in the MIH Reference Model for 802.11.

The MIH_MLME_SAP and the pre-existing LLC SAP (LSAP) instantiate the link-layer portion of the generic MIH_NET_SAP for the transport of MIH messages over L2, respectively before and after the association of the Mobile Node with the 802.11 access point takes place. The MIH Function interface that instantiates the higher-layer portion of the MIH_NET_SAP is TBD [See Note 1].

### MIH Reference Model for 802.16

The logical placement of the MIH Function in the 802.16 protocol stack is shown in Figure 10.

The MIH Function and the Network Control and Management System (NCMS) share the C_SAP and M_SAP for access to the mobility-management services of the Mobility Control Entity and Management Entity in the 802.16 Management Plane.

[Note 3: Appropriate mechanisms for the direct encapsulation of MIH messages into 802.16 data frames are still to be identified. The Service-Specific Convergence Sublayer instances currently available in the 802.16 standard only enable the encapsulation of IP packets and Ethernet frames. The only option available for L2 transport would be to first encapsulate the MIH messages into Ethernet frames with an Ethertype value that identifies MIH, and then mandate the adoption of Ethernet CS for 802.16 connections that carry MIH messages. This approach limits both the efficiency of the L2 transport of MIH messages (since it imposes the addition of full Ethernet overhead – at least 18 bytes – to the MIH frame) and the availability of L2 transport capabilities for MIH (since Ethernet CS is not ubiquitous). A solution that enables better efficiency and easier accessibility of L2 transport capabilities could become available with the possible standardization of the Generic Packet Convergence Sublayer (GPCS) recently proposed within 802.16g. With GPCS a more efficient LLC/SNAP encapsulation (8 bytes overhead) could create the needed room for the MIH Ethertype in 802.16 frames.]

Figure 10—Logical Placement of the MIH Function in the IEEE 802.16 Protocol Stack

Figure 11—MIH Reference Model for IEEE 802.16

Figure 11 shows the instantiation of the General MIH Reference Model when the access link complies with the IEEE 802.16 standard.

Since the Mobility Control Entity (CE) and the Management Entity (ME) are the immediate MIH counterparts in the lower layers of the 802.16 mobility-management plane, the C_SAP and M_SAP instantiate the MIH_LINK_SAP in the MIH Reference Model for 802.16.

The MIH Function interfaces that instantiate the MIH_NET_SAP for 802.16 are TBD [See Note 3].