| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER Functions and Services Specification |  |
| Date Submitted | January 9th, 2006 |  |
| Source(s) | Ulises Olvera |  |
| Re: | IEEE 802.21 Session #12 in Waikoloa, Hawaii |  |
| Abstract | Clarifications and enhancement to the current 802.21 baseline draft (D00.04), including clarification to 802.21 and 3GPP and 3GPP2 inter-action |  |
| Purpose | Enhancement to existing 802.21 baseline draft D00.04,November 2005 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1	Overview	2

2	References	2

3	Definitions	2

4	Acronyms	2

Clarifications and enhancement to 3GPP2 inter-action within the 802.21 baseline draft	3

5	Scope of InterDigital 802.21 Contribution	3

5.1	Proposed Amendments	3

5.1.1	Section 5.5.5 “MIH Reference Model for 3GPP2”	3

5.1.2	Section 5.5.5 “MIH Reference Model for 3GPP2”	3

5.1.2.1	Sections 5.5.5, Lines 48 to 65	4

5.1.3	Sections 5.6.1.5 and 5.6.1.6  “MIH-PPP-SAP” and “MIH-LAC-SAP”	4

5.1.4	Section 5.8.3 “3GPP Amendments”	5

5.1.5	MIH-3GLINK-SAP Primitives	5

5.1.5.1	New description for “MIH-3GLINK-SAP Primitives”	5

# Overview

This contribution describes amendments to the current IEEE 802.21 draft specified in reference 1 with regards to integration of 3GPP2 based systems. The amendments are founded on the assumption that existing 3GPP2 primitives can be used in order to achieve 3GPP2-IEEE 802.21 integration.

# References

- IEEE P802.21/D00.04 Draft IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services

- IS-2000.4-D “Signaling Link Access Control (LAC) Standard for cdma2000 Spread Spectrum Systems - Release D”

# Definitions

# Acronyms

3G				Third Generation

3GPP2				3G Partnership Project 2

AS				Access Stratum

CS				Convergence Sub-Layer

IEEE				Institute of Electrical and Electronics Engineers

IP				Internet Protocol

L1                			Physical Layer (PHY)

L2			         	Medium Access Control (MAC) and Link Layer Control (LLC)

LAN				Local Area Network

LLC            			Logical Link Control

LTE	          			Long Term Evolution

MAC				Medium Access Control

MAP				Mobility Anchor Point

MIH	          			Media Independent Handover

NAS	          			Non-Access Stratum

SAE	          			System Architecture Evolution

SAP             			Service Access Point

SDO             			Standards Development Organization

Clarifications and enhancement to 3GPP2 inter-action within the 802.21 baseline draft

# Scope of InterDigital 802.21 Contribution

Define 802.21 Interaction with 3GPP2 technology including its integration within an overall solution that includes radio access technologies such as 802.11, 802.16, 3GPP2 and 3GPP.

# Proposed Amendments

# Section 5.5.5 “MIH Reference Model for Mobile Stations with Multiple Protocol Stacks”

Replace figure 10 with the following figure:

Figure 10 MIH Reference Model for Mobile Station with Multiple Protocol Stacks

# Section 5.5.5 “MIH Reference Model for 3GPP2”

We propose to replace the current Figure 9 with the following figure:

Figure 9 MIH Reference Model for 3GPP2 Mobile Stations

# Sections 5.5.5, Page 28, Lines 48 to 65

Replace the existing text with the following:

The figure above illustrates the interaction between IEEE 802.21 Service and 3GPP2 based systems IEEE 802.21 services are accessed through the MIH-3GLINK-SAP. Note however that no new primitives or protocols need to be defined within 3GPP2 specification. Instead, a mapping between IEEE 802.21 Link Layer services and 3GPP2 primitives as defined in references 2 is established. Primitive information available from a layer such as LAC can be directly used and mapped to 802.21 service primitives in order to generate an event. This mapping is defined in Table 33a. For example MIH-3GLINK-SAP can directly use or map to L3_SAP in order to obtain LAC related elements in 3GPP2. There will be no direct communication between the PHY and MAC layers with the MIH layer.

For the case where MIH function is placed in the 3GPP2 network, its PoS may be remote. The architecture placement of any MIH function shall be left out for the 3GPP2 SDO.

# Sections 5.6.1.5 and 5.6.1.6  “MIH-PPP-SAP” and “MIH-LAC-SAP”

Remove these SAPs.

# Section 5.8.2 and 5.8.3 “3GPP Amendments/3GPP2 Amendments”

The amendments suggested in this section emphasize reuse of 3GPP and 3GPP2 functionality. 3GPP system already broadcast information regarding other Radio Access Technologies. It is recommended extending this concept so that 802 RATs be also considered along with other existing RATs such as 3GPP2, GERAN and UTRAN.

3GPP and 3GPP2 services primitives already provide access to the underlying layers that support GSM, UMTS and LTE/SAE and CDMA2000 connections including session management, mobility management and radio resource management.

As described in table 33a, a direct mapping can be established between these primitives and primitives defined in this standard, in order to access 3GPPand 3GPP2 services provides through existing 3GPP and 3GPP2 SAPs. This does not require addition of extra primitives although extensions might be useful. Therefore no new 3GPP or 3GPP 2 primitives are foreseen.

The following recommendations are suggested:

      - Include 802.xx Radio Access Technology within the Inter-RAT cell info list information element broadcast by 3GPP systems currently defined in [5.]

      - Extend existing primitives to reflect awareness of 802.xx RATs.

# MIH-3GLINK-SAP Primitives

Insert this section after section 7.4.15.3.3 “schema Query:

# New description for “MIH-3GLINK-SAP Primitives”

Insert the following text and table:

3GPP and 3GPP2 services primitives for GSM, UMTS, LAC, PPP and LTE/SAE can be use to access IEEE 802.21 services. This can is done by establishing a concrete relationship between the 3GPP/3GPP2 primitives and IEEE 802.21 primitives at both Link Layer and MIH level.

The following table provides this mapping. Note that a 3GPP primitive group can be mapped to more than one IEEE 802.21 primitives. E.g., “SMREG-PDP-MODIFY” can be mapped to a Link Up Primitive and it could also indicate the successful completion of a Handover, thereby mapping to a Handover Complete MIH-primitive. Likewise “L2 Condition Notification” as defined in reference [2] can be mapped to a Link Down Indication.

| 3GPP Layer | 3GPP Service Primitive Descriptions | 3GPP Primitive | 802.21 Service Primitives |
| --- | --- | --- | --- |
| RR | Page received by RR layer | GMRR-PAGE | Link Parameter Change |
|  | Successful reception of data supporting specific QoS | GRR-DATA | Link Parameter Change |
| LLC | Peer LLC layer is established | LL-ESTABLISH | Link Up/Link Parameter Change |
|  | Peer LLC layer is released | LL-RELEASE | Link Gong Down/Link Parameter Change |
|  | LLC Layer unrecoverable error | LL-STATUS | Link Down |
| GMM | Station is attached | GMMREG-ATACH | Link Parameter Change |
|  | Station is detached | GMMREG-DETACH | Link Parameter Change |
| SM | Data session active | SMSM-ACTIVE | Link Up/Link Parameter Change/ Handover Initiate/Handover Prepare |
|  | Data session is deactivated | SMSM-DEACTIVEATE | Link Down/Link Parameter Change/Handover Switch |
|  | Data session modified | SMSM-MODIFY | Link Parameter Change |
|  | Data session terminated due to unrecoverable error | SMSM-STATUS | Link Down |
|  | PDP Context is active | SMREG-PDP-ACTIVATE | Link Up/Handover Complete/Parameter Change/ Handover Complete |
|  | PDP Context is modified | SMREG-PDP-MODIFY | Link Up/Parameter Change/ Handover Complete |
|  | PDP Context is deactivated | SMREG-PDP-DEACTIVATE | Link Going Down/Link Down |
| GMM/SM (NAS) | Station is attached | GMMREG-ATACH | Parameter Change |
|  | Station is detached | GMMREG-DETACH | Parameter Change |
|  | PDP Context is active | SMREG-PDP-ACTIVATE | Link Up/Handover Complete/Parameter Change/Handover Initiate/Handover Prepare |
|  | PDP Context is modified | SMREG-PDP-MODIFY | Parameter Change/ Link Up |
|  | PDP Context is deactivated | SMREG-PDP-DEACTIVATE | Link Down/Link Parameter Change |
|  | Radio Access Bearer is activated for data transfer | RABMSM-ACTIVATE | Link Up/Parameter Change/ Handover Complete |
|  | Radio Access Bearer is deactivated for data transfer | RABMSM-DEACTIVATE | Link Down/Link Parameter Change |
|  | Radio Access Bearer is modified for data transfer | RABMSM-MODIFY | Parameter Change/ Link Up/ Handover Complete |
|  | Radio Access Bearer data transfer error | RABMSM-STATUS | Link Down |
| RRC (AS) | Radio Access Bearer has been activated | RABMAS-RAB-ESTABLISH | Link Up/ Handover Initiate/Handover Complete |
|  | Radio Access Bearer has been released | RABMAS-RAB-RELEASE | Link Down |
|  | AS failure Indication | RABMAS-STATUS | Link Down |
|  | Information regarding geographical area. | Information Broadcast | Parameter Change/Information Request/Information Response |
|  | Notification of paging for particular user or terminal | Paging Request | Parameter Change |
|  | Notification for all users | Notification Broadcast | Parameter Change |
|  | Notification information for a specific or for many user | Notification Indication | Parameter Change |
|  | UE initiated connection establishment | Connection Establishment | Link Up |
|  | Network initiated connection release | IF Initiated Connection Release | Link Down |
|  | Network initiated Radio Access Bearer Establishment | IF Side Initiated Radio Access Bearer Establishment | Link Up/Link Detected |
|  | Network initiated Radio Access Bearer Release | IF Side Initiated Radio Access Bearer Release | Link Down |
|  | Indication that the connection might be aborted unless streamlining is done | Streamlining Require Indication | Link Going Down |
|  | Location information provided by the network for a specific UE | UE location information | Parameter Change |
|  | Connection loss indications | Connection loss indication | Link Down |
| E-UTRAN/E-CORE* | The location of the UE is now know by the network | LTE-detached | Parameter Change |
|  | The UE is known to the network but not transport channel is established | LTE-idle | Parameter Change/Link Down |
|  | Radio resources have been established and the UE is able to perform uplink and downlink transport of PDU | LTE-Active | Link Up/Link Handover Complete |
| 3GPP2 LAC | L3 is notified of a relevant event detected at Layer 2. E.g., an abnormal condition | L2.Condition.Notification | Link-Up/Link Down/Link Going Down |
|  | Reception of the specified SDU was acknowledge at L2 | L2.Data.Confirm | Handover Complete |
|  | L2 executes a control command as directed by L3 | L2.Suppervision.Request | Handover Initiate |

Table 33a

# Section 5.3.2“Logical Network Reference Model”

Remove lines 16 to 26 from page 19 and ad the following text after lines 48 in page 20:

The network reference model shown in Figure 2 includes an MIH-capable mobile node (or UE, for User Equipment) that supports multiple Media Access technology (MA) options. MIH capabilities can be retrieved either through an explicit query and response mechanism or they might be broadcast natively by the relevant access technology.

On the network side, the location of the MIH-capable nodes is not deterministic and it might vary even within the same Media Access Technology. The MIH functionality may reside in the network point of attachment (PoA) such as an AP or RBS, but alternatively even deeper inside the access or core portions of the technology specific network. Event service triggers can originate at both side of an active link. However, the UE is typically the first node to react to link layer changes that might alter the condition of the underlying access technology.

The overall mobility management architecture might include Mobile IP infrastructure (clien, FA, HA, H-AAA), SIP mobility infrastructure (client, SIP servers) or reflect other mobility schemes. Note however that the procedures required for performing Media Independent Handovers, including determination of the need for handing over a particular call or session, the selection of the system to which the MS should handover and determination of when to perform the handover are all outside the scope of this specification.