| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER Functions and Services Specification |  |
| Date Submitted | January 9th, 2006 |  |
| Source(s) | Ulises Olvera |  |
| Re: | IEEE 802.21 Session #12 in Waikoloa, Hawaii |  |
| Abstract | Enhancement to the current 802.21 baseline draft (D00.04), including flow diagrams exemplifying the use of MIH commands for executing handover procedures. |  |
| Purpose | Enhancement to existing 802.21 baseline draft D00.04,November 2005 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1	Overview	2

2	References	2

3	Definitions	2

4	Acronyms	2

Flow Diagrams exemplifying MIH Commands and procedures	2

5	Scope of InterDigital Contribution	2

5.1	Proposed Amendments	2

5.1.1	Section Anex A “Handover Procedures”	2

# Overview

This contribution describes flow diagrams that exemplified the use of IEEE 802.21 MIH Handover commands and procedures

# References

- IEEE P802.21/D00.04 Draft IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services

# Definitions

# Acronyms

Flow Diagrams exemplifying MIH Commands and procedures

# Scope of InterDigital Contribution

Provide an example of a flow diagram that exemplifies the use of command primitives and their interaction across remote MIH peers

# Proposed Amendments

# Section Anex A “Handover Procedures”

Insert the following flow diagram and accompanying text

:

Figure X Handover from 802.11 to 3GPP2

The following steps describe the flow depicted in figure. XX

          - The UE is associated to an 802.11 AP and there is currently a session established through the corresponding Home Agent

          - An IEEE 802.21 Link Parameter change event is triggered due to E.g., undesirable radio conditions. The MIH function notifies the MIH user through a corresponding MIH Link Parameter Change message. The MIH might also notifies the local MIH User. This is illustrated with the dotted arrow.

          - The MIH Function notifies the MIH Peer at the network (which previously register for the event service) indicating that a Link Parameter Change has been triggered. The MIH user can optionally contact the MIH Information Server through an MIH Capability Discovery Request to query about the available network for the user that generated the event. Note that this information could have been previously obtained of it could be readily available within the network entity where the MIH user resides.

          - The MIH user might use a Handover Prepare message in order to request resource from alternate networks.

          - The MIH user selects a candidate based on the information received in through the relevant MIH event indication. How this is done is out of scope of the 802.21 specification

          - The MIH user generates a Handover Initiate Request toward its local MIH Function. In turn, the local MIH Function contacts its peer at the STA/UE to transfer the MIH Handover Initiate message. E.g., over a layer 3 transport using the MIH Protocol. The remote MIH Function at the UE relay the MIH Handover Initiate Request, using an MIH Handover.Indication primitive.

          - The MIH User issues an MIH Switch.Request towars its local MIH Function to initiate the establishment of a new radio connection according to the information received in the MIH Handover Initiate Message.

          - The MIH Function issues an Link Up request (Note to theEditor, please note that this command needs to be added to the relevant Link Technology if not already available).

          - The 3GPP2 network interface establishes a new radio connection and a new PPP session.

          - The cellular interface responds indicating the radio link is up.

          - An MIH Handover Initiate.Response message is sent to the requesting MIH User indicating the successful establishment of the new radio connection. Note that the MIH User might have direct access to the relevant cellular technology. This is however out of the scope of this specification. Note that the establishment of the PPP session might trigger a Link Up event at the network side.

          - Mobil IP registration might be now trigger.

          - An MIH Handover Commit message is sent to MIH User indicating the successful establishment of the new radio connection.

          - The MIH User can now tear down the connection using an MIH complete request. side

          - A Link Tear Down is use to release resources.

          - A Link Tear Down response is issue indicating success release of resources.

          - An MIH Handover Complete response is issue notifying the user of successful release of resources.

          - An MIH Initiate Response can now be issue toward the peer.

          - The session is now transferred toward the new link layer

          - The UE is disassociated from the previous PoA