| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Network selection |  |
| DCN | 21-06-0500-00-0000 |  |
| Date Submitted | Jan, 06, 2006 |  |
| Source(s) | Qu Bingyun, Zhuang Hongcheng |  |
| Re: | IEEE 802.21 Session #12 in Hawaii |  |
| Abstract | This document updates MIH_Handover_Initiate message to extend MIH function. Firstly, it supports negotiation between network and terminal about which side could do MIH decision. Secondly, it enables multiple network selection. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

#### Introduction

MIH is defined as a helper and facilitator function which helps to make handover decision. Upper layers make handover decisions and link selection based on inputs and context from MIH.

To meet different requirements, such decisions and selection could be made in terminal or network. But there is no way to negotiate who could make such a decision during a specific MIH handover process, terminal or network? As a solution, terminal and network could put up its own control point preferred at handover initial stage.

To originate handover, terminal could send a MIH_Handover_Initiate.request message including suggested networks that terminal wants to access to. In some cases, there may exist several networks that terminal could handover to and terminal hasn’t enough information to select. If so, MIH_Handover_Initiate.request message should include multiple links instead of single link. MIHF at network may collect information of these links, or others satisfying terminal’s requirements, and send this information to terminal for terminal to select one of networks.

In general, a terminal will select the most preferred networks, but in some situation, for example, the terminal energy is not enough to support the most preferred networks when beginning to handover, it will select the less preferred networks, avoiding the corruption of services.

# Amendment

7.4.10.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.request (

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer

SuggestedNewLinkList,

SuggestedNewPoAList,

SuggestedControlPoint,

HandoverMode,

CurrentLinkAction,

QueryResourceList

)

Local or Remote: Both

MIHF (Terminal) <> MIHF (Network)

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYP E | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| SuggestedNewLinkList | NetworkIdentifier. Can be different 802 and Cellular networks |  | This is the identifier List of new networks to which handover needs to be initiated. |
| SuggestedNewPoAList | MAC_ADDRESS (Optional) |  | This is the Suggested Point of Attachment List (AP/BS) on new networks |
| SuggestedControlPoint | BOOL |  | Suggested control point at terminal or network. 1: terminal; 0: network. |
| HandoverMode | Make-before_Break Break-before-Make |  | The handover mode may influence the manner in which links are prepared for handover |
| CurrentLinkAction | LINK_DISCONNECT LINK_LOW_POWER LINK_POWER_DOW N LINK_ NO_ACTION |  | Specifies suggested action on old link once handover procedures have been executed. |
| QueryResourceList | BOOL |  | Flag to query the available list of resources on the new link |

If SuggestedControlPoint is 1, it suggests that the terminal want to select the networks.

7.4.10.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.response (

CommandSource,

CommandDestination,

CurrentLinkIdentifier,

TransportLayer,

HandoverAck

PreferredLinkIdentifierList,

PreferedPoAIdentifierList,

PreferedControlPoint,

AbortReason

)

Local or Remote: Both

MIHF (Terminal) <> MIHF (Network)

| Name | Type | Valid Range | Value |
| --- | --- | --- | --- |
| CommandSource | UPPER_LAYER_TYP E | N/A | The origination point from where the command is initiated. This is usually some form of upper layer such as a policy engine, or a L3 Mobility protocol, transport, application etc. |
| CommandDestination | MIH_LOCAL, MIH_REMOTE |  | This specifies the command destination which can be either local or remote MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. Can be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| TransportLayer | TRANSPORT_L2, TRANSPORT_L3 |  | This identifies the transport layer L2 or L3 over which the command needs to be sent to remote MIHF entity. This is valid only for remote commands. |
| HandoverAck | BOOL | 0 or 1 | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a reason code is provided |
| PreferredLinkList | NetworkIdentifier. Can be different 802 and Cellular networks |  | This is the identifier List of new networks to which handover needs to be initiated. This may be different than the network that was suggested in the handover request. The front is most suitable, backward is less. |
| PreferredNewPoAList | MAC_ADDRESS |  | This is the preferred Point of Attachment (AP/BS) List on new networks. The front is most suitable, backward is less. |
| PreferredControlPoint | BOOL |  | Suggested control point at terminal or network. 1: terminal controlled; 0: network controlled |
| AbortReason |  |  | Lists the reason for aborting/declining the handover request. |
| Available Resource List |  |  | List of available resources on new link |

If PreferredControlPoint is 1, it suggests that the terminal can select the most suitable network according its situation and current information such as power. Otherwise, the terminal can only handover to the preferred network. At this time, PreferredLinkList and PreferredNewPoAList has only one element.

# Editorial comment

- Section 7.4.12.2.2, MIH_Handover_Commit message, MIHF (old Network) <> MIHF (new Suggested Network) should be MIHF (Terminal) <> MIHF (Network).

- In 7.4.10.2.1 Function,7.4.11.1.1 Function,7.4.11.2.1 Function, the description is the same. For MIH_Handover_Prepare message, such description may inappropriate.

- In section 5.2, the sequence number ‘a)’ should follow ‘b)’ instead of ‘e)’.

- Section 8.4 MIH Function Messages miss the description about MIH Handover Prepare message, MIH SDU Transmit Success message, MIH SDU Transmit Failure message.