| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Higher Layer Requirements for Event and Command Services |  |
| Date Submitted | January 16th, 2005 |  |
| Source(s) | ES/CS Adhoc Group Edited: Srinivas Sreemanthula |  |
| Ref: | 21-06-0503-01-0000-ES-CS-HL-Reqs.doc |  |
| Abstract | This contribution provides clarifications to events and commands defined in commentary. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

MIH Event and Command Services are used for inter-technology handovers during network selection and handover control processes. The ES/CS require a higher layer transport to enable network controlled usage scenarios and also station centric scenarios to a certain extent. This document covers material related to the ES/CS usage scenarios, additional protocol functionality and transport requirements for higher layer transport of ES/CS messages. This work is the output of the ad-hoc teleconference meetings and email exchanges.

This is a working document and should therefore, be considered only as a draft.

# Usage Scenarios

The usage scenarios can be broadly split as i) direct communications between involved MIH entities and ii) proxy communications that involve one or more proxy MIH nodes in between two MIH end nodes. The higher layer transport for MIH ES/CS should ensure that the two usage models are enabled. The transport can be defined a connection to carry between two MIH functions, including the proxy node. As such, there are no specific transport layer implications for the proxy node functionality. In order to have end-to-end MIH communications, there may be more than one transport leg involved.

Figure 1 Direct Communications

Figure 2 Proxy Communications

# Functional Requirements

As part of ES/CS higher layer requirements, there were discussions related to new MIH functionality related to event/command services.

## MIH Registration

MIH Registration is a MIH level message exchange that allows creating MIH pairing between two MIH entities. The MIH registration request is sent by a client MIH function to a peer MIH function in the network to setup a session and possibly, assign an identifier. This identifier can be used in future MIH communication between the two nodes while the registration session is active. Upon successful registration, the MIH peers can exchange MIH messages for MIH ES/CS services. MIH peer shall not receive or provide any MIH ES/CS services without through the registration process.

The following figure shows a reference MIH ES/CS communication state diagram. In the MIH-idle state, MIH node performs a MIH peer discovery to determine the end point address. After MIH discovery, MIH node will perform capability discovery to determine what services are supported at the other end. MIH node can perform the MIH registration to create the pairing and transition to MIH-Active state where it can exchange MIH ES/CS protocol messages. In the MIH-Active state, the MIH peers can request for specific event registrations or send MIH commands to request specific actions.

### MIH_Register.Request

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function in the network to perform registration.

#### Semantics of service primitive

MIH_Register.Request

(

Source Identifier,

Destination Identifier,

TransactionId,

RegistrationCredentials,

More …

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) <> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Source of this message |
| Destination Identifier | Identifier | Any valid individual or group identifier | Destination of this message |
| Transaction Id | Identifier |  | Transaction Identifier copied from received message |
| Registration Credentials | Credentials |  |  |
| More ..? (e.g. TTL) |  |  |  |

#### When generated

The MIH Function generates this message when it needs to perform a MIH registration to a particular destination MIH function.

#### Effect on receipt

Upon receipt, the MIH Function can verify the credentials and perform additional security measures (not defined in 802.21) to complete the registration.

### MIH_Register.Response

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function in the network to indicate the result of a registration request.

#### Semantics of service primitive

MIH_Register.Response

(

Source Identifier,

Destination Identifier,

TransactionId,

Result Code,
Session Id,

More …

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) <> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Source of this message |
| Destination Identifier | Identifier | Any valid individual or group identifier | Destination of this message |
| Transaction Id | Identifier |  | Transaction Identifier copied from received message |
| Result code | BOOL | SUCCESS, FAILURE |  |
| Session Id | Identifier |  | A session identifier assigned to the registration period |
| More ..? (e.g. TTL) |  |  |  |

#### When generated

The MIH Function generates this message when after completion of registration request processing.

#### Effect on receipt

Upon receipt, the MIH Function can determine that the result of registration request.

### Multiple MIH Registration

Do we need this?

Multiple MIH registration can be useful when the different transports are used for different MIH services e.g. L2 for IS and L3 for ES/CS. There is a need for a little more discussion on these usage scenarios. Do we need registration for IS?

## MIH ES Message Reliability

MIH event messages need reliability for remote communication on an end-to-end basis to ensure the receipt of data to the intended destination. Reliability can achieved either at the MIH protocol level or at the transport level. Reliability at the MIH protocol level will ensure efficiency and reduce the latency in the delivery of MIH ES messages.

A generic MIH level ACK packet can be used to acknowledge the successful receipt of various ES/CS messages at the destination end point. The source shall receive the ACK to determine the reliable delivery to the destination. If the message or the MIH ACK is lost, the source shall timeout (value to be TBD) and retransmit the same MIH ES message. The back-off mechanism and the number of retransmissions a MIH source can retry for successful delivery is outside the scope of the document.

### MIH ACK Message

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that has sent out an MIH event to acknowledge the receipt of the message. This primitive generates an ACK response to the message.

#### Semantics of service primitive

MIH_ACK

(

Source Identifier,

Message Code,

Transaction Id

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) <> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Source of this message |
| Destination Identifier | Identifier | Any valid individual or group identifier | Destination of this message |
| Message Code | EnhancedMessageId | TBD | Message code |
| Transaction Id | Identifier |  | Transaction Identifier copied from received message |

#### When generated

The MIH Function responds with this primitive upon receipt of any remote event.

#### Effect on receipt

Upon receipt, the client MIH Function can determine that the event was received successfully. And discard the MIH message from the buffer.

# Transport Layer Requirements

- MIH ES/CS transport shall support both IPv4 and IPv6 based networks

- MIH ES/CS transport shall preserve the message boundaries i.e. the MIH messages are delivered as a single complete unit

- MIH ES/CS transport shall provide means for NAT traversal

- MIH ES/CS transport shall provide means for firewall traversal

- MIH ES/CS transport shall provide security as defined by IETF standards

- MIH ES/CS transport shall provide efficient, optimized and timely delivery of ES/CS messages