| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | MIH General Relay Message |
| Date Submitted | February 10th, 2006 |
| Source(s) | Qu Bingyun |
| Ref: | 21-06-0531-00-0000-MIH_General_Relay_Message.doc |
| Abstract | This contribution proposes the definition of MIH General Relay Message primitive. |
| Purpose |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

### 7.3.x General Relay Message

MN and candidate PoA may want to exchange message when MN decided to start handover. Because there does not exist connect between MN and candidate PoA , MIH General Relay Message could be used which take old PoA as anchor point to relay message between MN and candidate PoA.

Old PoA has constructed connection with MN and candidate PoA. So it is more reliable to exchange messages through old PoA than exchange messages directly between MN and candidate PoA at that time.

MN and candidate PoA could do service connection, service configuration and negotiation, querying status of links through General Relay Message.

### 7.3.x. Link_General_Relay_Message.indication

#### 7.3.x.1 Function

#### This primitive is used by MIH Function between serving network a terminal. This primitive could also be used by MIH Function between serving network and candidate network.

7.3.x.2 Semantics of service primitive

The parameters of the primitive are as follows:

Link_General_Relay_Message.indication {

SourceIdentifier

DestinationIdentifier,

RelayMessageList

}

Local or Remote: Both

MIHF (old Network) <> MIHF (new Suggested Network), MIHF (old Network) <> MIHF (Terminal)

| Name | Type | Description |
| --- | --- | --- |
| SourceIdentifier | Identifier | The origination point from where the event is generated. |
| DestinationIdentifier | Identifier | The destination identifier |
| RelayMessageList |  | Message exchange between MN and candidate network |

#### 7.3.x.3 When generated

#### This primitive is generated when MN or candidate network want to exchange messages each other through serving network.

#### 7.3.x.4 Effect on receipt

#### Receiver would take different action according to content of RelayMessageList receiving.

Usage scenario

As an example, there illustrates a handover flow from 802.16 to 3GPP2 cdma20001x.

| 1 | The MN is associated to an 802.16 BS. |
| --- | --- |
| 2 | MN decided to handover to cdma2001x network. MN sends General Relay Message which includes Origination Message described in 3GPP2 standards. |
| 3 | 802.16 networks relay this message to 3GPP2 network. |
| 4 | 3GPP2 network sends General Relay Message which includes ExtendedChannelAssignMessage described in 3GPP2 standards. |
| 5 | 802.16 networks relay this message to MN. |
| 6 | MN notifies 802.16 networks it will handover to 3GPP2 network. |
| 7 | MN switches to 3GPP2 network to acquire forward null service frame for setup traffic connection. |

MN and 3GPP2 network exchange messages for preparing service connection Through 802.16 network. So that MN could avoid exchanging messages with 3GPP2 network directly by random access process, which is less reliable than relaying message by 802.16 networks.