Comments:

The document discussed and accepted (21-05-0418-00-0000-communication_model_text) in November meeting had some definitions of different entities. It would be good to add those definitions for better understanding and readability.

Current Text:

Table Y: Summary of Communication Reference Points

| Reference Point | Comments | Transport Choices | Definition of Communication Relationship |
| --- | --- | --- | --- |
| R1 | Between the MIH on a UE and an MIH PoS on the Network Entity of the serving PoA. | Usually L2 (L3 possible) | In scope of 802.21 |
| R2 | Between the MIH on a UE and an MIH PoS on the Network Entity of the candidate PoA. | Usually L2 (L3 possible) | In scope of 802.21 |
| R3 | Between the MIH on a UE and an MIH PoS on a non-PoA network entity. | Usually L3 (L2 possible) | In scope of 802.21 |
| R4 | Between an MIH PoS and a non-PoS MIH Function instance in distinct Network Entities. | Usually L3 | In scope of 802.21 |
| R5 | Between two MIH PoS instances in distinct Network Entities. | Usually L3 | In scope of 802.21 |

Proposed Text :

Table Y: Summary of Communication Reference Points

| Reference Point | Comments | Transport Choices | Definition of Communication Relationship |
| --- | --- | --- | --- |
| R1 | Between the MIH on a UE and an MIH PoS on the Network Entity of the serving PoA. | L2 or L3 | In scope of 802.21 |
| R2 | Between the MIH on a UE and an MIH PoS on the Network Entity of the candidate PoA. | L2 or L3 | In scope of 802.21 |
| R3 | Between the MIH on a UE and an MIH PoS on a non-PoA network entity. | L2 or L3 | In scope of 802.21 |
| R4 | Between an MIH PoS and a non-PoS MIH Function instance in distinct Network Entities. | L2 or L3 | In scope of 802.21 |
| R5 | Between two MIH PoS instances in distinct Network Entities. | L2 or L3 | In scope of 802.21 |