IEEE P802.21/D01.00
Draft IEEE Standard for Local and Metropolitan Area Networks:
Media Independent Handover Services

| Normative Text Proposal for QoS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-June-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Nada Golmie | National Institute of Standards and Technology | 100 Bureau Dr, Stop 8920 Gaithersburg, MD | +1301-975-4190 | Nada.golmie@nist.gov |
| Reijo Salminen | Seesta | Kumitehtaankatu 5, FIN-04260 KERAVA, FINLAND | +358-9-23163636 | Reijo.Salminen@seesta.com |
| Ulises Olvera | InterDigital Communications | 100, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6258 | Ulises.Olvera-Hernandez@InterDigital.com |

- Abstract

This document contains a normative text proposal in support of a Media Independent Handover Quality of Service model.

Notice: This document has been prepared to assist IEEE 802.21. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

Motivation and Objectives	3

Proposed MIH QoS Model and normative text additions to the draft D01.	3

References	3

# Motivation and Objectives

This contribution proposes to include a QOS framework within the IEEE 802.21 specifications. Key features of this framework include a set of performance measurements that:

  - are derived from media and technology specific parameters.

  - scale well across different media types

  - can be provided as a service to the MIH user (e.g., a mobility management application)

In addition, this  MIH QOS framework includes a set of primitives in order to: (for cases below, see references [1]-[4] first)

  - communicate MIH user (or application) QOS requirements to the MIH. The purpose of the  provision of this information is two fold: 1)  MIHF uses these requirements to determine the acceptable QoS levels supporting a specific application on multiple layers and  2) MIH might use this information to set QoS thresholds across multiple layers

  - extract network specific measurements . Note that some metrics and measurements might  already exist in other technologies, however the primitives to make these measurements available to other layers and/or devices may not necessarily be there.

  - set trigger thresholds for these measurements . In order to track some measurements, the MIHF is either notified when these parameters exceed certain thresholds, or it could inquire about the parameter value every time it is needed.

  - report QoS events upon threshold crossing. Note that no new events are required.

  - report real-time measurements to the MIH user.  In this context, real-time refers to network measurements that vary over time.

QoS Requirements

MIH services are geared towards facilitating handovers among heterogeneous network technologies in order to provide seamless mobility and ubiquitous connectivity (“always best connected”). The idea being that the MIH services are expected to  improve the network performance during a handover (let us call this type 1 QOS) in addition to improving the overall network performance end-to-end (type 2 QOS). Observe that type 1 relates to the seamless mobility while type 2 refers to ubiquitous connectivity and “best connected”

Providing seamless mobility has specific performance implications with respect to the application requirements, this includes:

  - Minimize handover latency so as to minimize the end-to-end delay and meet the application delay requirements

  - Minimize the packet loss incurred during a handover so as to minimize the end-to-end packet loss and meet the application packet loss requirements.

Providing seamless mobility has specific performance implications with respect to the network performance perceived by the application and how well it meets the application quality of service requirements. For example,

  - Minimizing the handover latency can result in reducing the end-to-end delay and therefore is able to better meet the application delay requirements.

  - Minimizing the packet loss incurred during a handover leads to reducing the end-to-end packet loss and therefore is able to better meet the application packet loss requirements.

# Proposed MIH QoS Model and normative text additions to the draft D01.

We propose to replace section 5.1.3 on page 13 with the following text:

MIH Quality of Service Model

Note that MIHF supports seamless mobility as an enabler, however  MIHF alone cannot guarantee seamless mobility ,The MIH services defined by the IEEE 802.21, including event, command, and information service, need to consider the resulting network performance objectives and how well they meet the application quality of service requirements. Note however that this does not necessarily refer to requirements placed on the particular service but rather the QOS achieved as a result of the provision of a particular MIHF service.The performance implications to consider from the MIH perspective include both the transient network performance achieved during a handover as well as the continuous monitoring of current network conditions. Depending on the QOS requirements of the end-2-end application, ,seamless mobility implies minimizing the latency,  and potential loss incurred during a handover so as to minimize the end-to-end delay and loss perceived by the application. Seamless mobility also implies the timely assessment of network conditions, such as the monitoring of the signal strength, packet loss, on the link, on both current and target networks in order to optimize the handover decision and its execution

The IEEE 802.21 specification provides Quality of Service Model (QOSM) to quantitatively characterize seamless mobility It allows us to determine the specific QoS that can satisfy the requirements of an application so that a user of such application is not disrupted by the change in link layer technology or access network, see  also ref [3]) and the services provided by the MIH function. This model consists of a set of QoS performance metrics and primitives to handle the exchange and configuration of media specific QoS parameters and application requirements.  Key features include performance measurements that scale well across different media types and can be provided as a service to the user of MIH function. Reference [1] specifies in detailed the required QoS parameters outlined in this document. Reference [2] provides a view of how QoS can be measured at ingress and egress.

The MIH QOSM consists of the following performance metrics:

- Packet Transfer Delay (PTD): upper bound on the mean delay.

- Packet Delay Variation (PDV): upper bound on the 1-10-3 quantile on the PTD minus the minimum IPTD.

- Packet Loss Ratio (PLR): upper bound on the packet loss probability.

- Packet Error Ratio (PER): upper bound on the number  of errored packets. An error in a packet could be characterized as  E.g.,a bit flipped see ref [1]) per total packets sent.

- Throughput (bits/s): the number of bits (only good packets are considered) successfully received divided by the time it took to transmit them over the medium.. Note that this considerations need to be aligned/liaisoned with the ongoing work at ITU-T SG 12 for definition of parameters related to throughput in ITU-T Rec. Y.1540)

The MIH QOSM is a key component in supporting the application quality of service requirements end-to-end  Along a network path where multiple access network technologies are concatenated together, the end-to-end network performance is obtained by concatenating the performance achieved on each network segment. Since at the IP layer, the network performance is agnostic to the underlying layer 2 technologies, it is envisioned that existing IP layer (and higher layer) performance measures and transportation mechanisms, such as the QOS signaling defined by the IETF Next Steps in Signaling (NSIS) group are used to assess the end-to-end network performance.. (see ref [1] and ref [3])

QoS Parameter Discovery

We propose to add the following MIH/Link commands for discovering  QoS related link and network specific parameters.

Add the following command to Table 4, section 6.2.6.1, on page 41

- MIH Parameter Discover: Discover higher layers specific QoS parameters as follows:

| No | MIH Command | (L) ocal, (R) emote | Remote Direction | Comments |
| --- | --- | --- | --- | --- |
| 1 | MIH Get Status | L, R | Network -> Client | Get the status of links |
| 2 | MIH Switch | L, R | Network -> Client | Switch the links as specified |
| 3 | MIH Configure | L, R | Network -> Client | Configure a link |
| 4 | MIH Configure Thresholds | L,R | Network-> Client | Configures thresholds for link events |
| 5 | MIH Scan | L, R | Network -> Client | Scan a link |
| 6 | MIH Handover Ini tiate | L, R | Client -> Network Network -> Client | Network or client may initiate handover and send a list of suggested networks and associ ated Points of Attachment |
| 7 | MIH Handover Pre pare | L, R | Network -> Net work | This command is sent by current MIHF entity to target MIHF entity to allow for resource query and handover preparation. |
| 8 | MIH Handover Com mit | L, R | Client -> Network, Network -> Client | In this case the client or network commits to do the handover and sends the choice of selected network and associated PoA. |
| 9 | MIH Handover Com plete | L, R | Client -> Network, Network -> Net work | Notification from new serving MIHF to previ ous serving MIHF indicating handover com pletion, and any pending packets may now be forwarded to the new MIHF. |
| 10 | MIH Network Address Information | L, R | Network -> Net work | Sent from current serving MIHF entity to tar get MIHF entity to obtain reconfigured net work address on target network for the client. |
| 11 | MIH Parameter Discovery | L, R | Client->Network | Discover Higher Layer specific QoS parameters |

Add the following command to Table 5, section 6.2.5.2, page 41

- Link Parameter Discover: Discover link layer specific QoS parameters as follows:

| No | Link Command | Comments |
| --- | --- | --- |
| 1 | Link Configure Thresholds | Configure the thresholds for various link layer events such as Link Going Down |
| 2 | Link QoS Parameter Discovery | Discover link layer specific QoS parameters (Why discover?…this should already be avialable from the different links..The information is indeed available, however MIHF needs to requested as is not delivery automatically to it) |

Add the following MIH primitive for discovering network specific QoS parameters as follows:

Table 14-MIH_SAP Primitives

| No | Primitives | Service Category | Description |
| --- | --- | --- | --- |
| 1 | MIH Capability Discover | System Management | Discover list of Events and Commands supported by MIH Function. |
| 2 | MIH Event Register | Event | Register for MIH event notifications |
| 3 | MIH Event DeRegister | Event | Deregister for MIH event notifications |
| 4 | MIH Link Up | Event | L2 connection has been established |
| 5 | MIH Link Down | Event | L2 connectivity is lost |
| 6 | MIH Link Going Down | Event | L2 connectivity is predicted to go down |
| 7 | MIH Link Event Rollback | Event | Predicted event has not occurred and hence event indication must be rolled back |
| 8 | MIH Link Parameters Report | Event | Link parameters have crossed specified threshold |
| 9 | MIH Link SDU Transmit Status | Event | Indicate transmission status of all PDU segments |
| 10 | MIH Link Handover Imminent | Event | L2 handover is imminent |
| 11 | MIH Link Handover Complete | Event | L2 handover has been completed |
| 12 | MIH Get Status | Command | Get the status of link |
| 13 | MIH Switch | Command | Switch session between specified links |
| 14 | MIH Configure | Command | Configure link parameters and parameter thresholds |
| 15 | MIH Configure Thresholds | Command | Configure thresholds for Link events |
| 16 | MIH Scan | Command | Scan the network |
| 17 | MIH Handover Initiate | Command | Initiate handover |
| 18 | MIH Handover Prepare | Command | Prepare for handover and query available resources |
| 19 | MIH Handover Commit | Command | Mobile node has committed to handover |
| 20 | MIH Handover Complete | Command | Handover has been completed |
| 21 | MIH Network Address Information | Command | Obtain network address on new link |
| 22 | MIH Get Information | Information | Request to get information from repository |
| 23 | MIH QoS Parameter Discovery | Command | Discover higher layer sepecific QoS parameters |

Add the following link layer primitive for discovering link layer specific QoS parameters as follows:

| No | Primitives | Service Category | Description |
| --- | --- | --- | --- |
| 1 | Link Event Discover | System Management | Discover link capabilities |
| 2 | Link Event Register | Event | Register for event notifications |
| 3 | Link Event Deregister | Event | Deregister for event notifications |
| 4 | Link Configure Thresholds | Command | Configure link thresholds for Link events |
| 5 | Link Up | Event | L2 connectivity is established |
| 6 | Link Down | Event | L2 connectivity is lost |
| 7 | Link Going Down | Event | L2 connectivity loss is imminent |
| 8 | Link Event Rollback | Event | Predicted event has not occurred and hence event indication must be rolled back. |
| 9 | Link Parameters Change | Event | Link parameters have crossed specified thresholds |
| 10 | Link SDU Transmit Status | Event | Indicate transmission status of all PDU segments |
| 11 | Link Handover Imminent | Event | L2 handover is imminent |
| 12 | Link Handover Complete | Event | L2 handover has been completed |
| 13 | Link Get Information | Information | Request for IEs |
| 14 | Link QoS Parameter Discovery | Command | Discover link specific QoS parameters (Again why do you really need this? See above) |

This primitive can be derived from existing link layer information. Existing link layer technologies already provide some form of QoS information albeit not in an uniforme fashion.

7.3.20 Link_QoS_Parameter_Discovery.request

7.3.20.1 Function

This primitive is used by the MIH function to request the set of QoS parameters supported by a specific link layer technology.

7.3.20.2 Semantics of Service Primitive

Link_QoS_Parameter_Discovery.request		(

)

7.3.20.3 When generated

This primitive is generated by the MIH function when attempting to retrieve the set of QoS parameters supported by a specific link layer technology.

7.3.20.4 Effect on receipt

The specific link layer entity returns the set of QoS parameters supported through a Link_QoS_Parameters_Discovery.response primitive.

7.3.21 Link_QoS_Parameter_Discovery.confirm

7.3.21.1 Function

This primitive is used by a link layer entity to provide the set of QoS parameters that it supports.

7.3.21.2 Semantics of Service Primitive

Link_QoS_Parameter_Discovery.request		(

SupportedQoSParameterList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedQoSParameterList | Set of QoS Parameter IDs | Set of valid QoS parameters | List of QoS Parameters supported by the link layer |

7.3.21.3 When generated

This primitive is generated by a link layer entity when responding to a request for supported QoS parameters.

7.3.21.4 Effect on receipt

The MIH function might use this information to provide a higher layer function with a list of QoS parameters supported by the link layers providing services to MIH.

7.4.16 MIH_QoS_Parameter_Discovery.request

7.4.16.1 Function

This primitive is used by the MIH user to request the set of QoS parameters supported by a specific link layer technology

7.4.16.2 Semantics of Service Primitive

MIH_QoS_Parameter_Discovery.request		LinkLayerTechnology (3GPP, 3GPP2, 802.11, 802.16)

)

7.4.16.3 When generated

This primitive is generated by the MIH user when attempting to retrieve the set of QoS parameters supported by a specific link layer technology.

7.4.16.4 Effect on receipt

The specific network entity returns the set of QoS parameters supported through a MIH_QoS_Parameters_Discovery.response primitive. These parameters are the parameter defined in section “QoS Parameters Requirements

‘

7.4.17 MIH_QoS_Parameter_Discovery.response

7.4.17.1 Function

This primitive is used by a network entity to provide the set of QoS parameters that it supports.

7.4.17.2 Semantics of Service Primitive

MIH_QoS_Parameter_Discovery.request		(

SupportedQoSParameterList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SupportedQoSParameterList | Set of QoS Parameter IDs (what are these IDs) | Set of valid QoS parameters (what are these parameters) | List of QoS Parameters supported by the Network |

7.4.17.3 When generated

This primitive is generated by a network entity when responding to a request for supported QoS parameters.

7.4.17.4 Effect on receipt

The MIH function might use this information to provide a list of QoS parameters available by different networks.

QoS Parameter Requirements

We propose the add the following primitives to provide an MIH User (e.g., a mobility management application) with a standardized interface that allows a consistent and a generic mechanism for defining QoS requirements and retrieving media specific QOS measurements.

The primitives are defined as follows:

X.1.1 MIH_Required_QoS.request

X.1.1.1 Function

This primitive is used by the MIH user to set QoS parameter requirementsThe MIHF function can set thresholds within the relevant Link Layer technology that will trigger events when crossed. The MIHF user sets these requirements based on the particular application supported by the link layer technology. By setting these requirements the application guarantees that the supporting link layer technology complies with its requirements. See also ref [1] and ref [3] and INFO_SPEC in ref [4].

X.1.1.2 Semantics of Service Primitive

MIH_Required_QoS.request		(

QoSParameterType1

.

.

.

QoSParameterTypen

)

| Type | Type | Length | Description |
| --- | --- | --- | --- |
| PacketTransferDelay | Identifier | 8 | Upper bound on the mean delay measured in ms |
| PacketDelayVariation | Identifier | 8 | Difference between the upper and lower quantile of the delay variation measured in ms. (E,g.Difference between measurements taken at 99.9 percentile and the 0.1 percentile) |
| PacketLossRatio | Identifier | 8 | Upper bound on the packet loss probablility |
| PakcetErrorRatio | Identifier | 8 | Upper bound on the number of packets in error per total packets sent |
| Throughput | Identifier | 8 | Number of bits successfully received divided by the time it took to transmit them over the medium, measured in bits per second |

| QoS Parameter Type | Identifier |
| --- | --- |
| PacketTransferDelay | 1 |
| PacketDelayVariation | 2 |
| PacketLossRatio | 3 |
| PakcetErrorRatio | 4 |
| Throughput | 5 |

X.1.1.3 When generated

This primitive is generated by the MIH User when setting the required values for one or many QoS parameters

X.1.1.4 Effect on receipt

The MIH Function shall set the relevant parameter requirements as specified by the QoSParameterType.

X.1.2 MIH_Required_QoS.response

X.1.2.1 Function

This primitive is used by the MIH Function to provide the result of a command to set the QoS parameter requirements

X.1.2.2 Semantics of Service Primitive

MIH_Required_QoS.response		(

QoSParameterSettingResultList

)

| Type | Length | Description |
| --- | --- | --- |
| QoSParamterSettingResultList | Variable | For each request, a value “1” indicates that the request was granted and a value “0” indicates that the request was rejected |

X.1.2.3 When generated

This primitive is generated by the MIH Function when responding to a request to set QoS parameter requirements.

X.1.2.4 Effect on receipt

The MIH User might use this information to determine subsequent operation on the current link

X.1.3 MIH_Required_QoS.indication

X.1.3.1 Function

This primitive is used by the MIH function to provide an indication  to an MIH User that requirements on specific QoS parameters can no longer be met. Note that the application itself (i.e.,MIHF user) determines specific requirements that ought to be met. See also ref [1] and ref [3] and INFO_SPEC in ref [4].)

X.1.3.2 Semantics of Service Primitive

MIH_Required _QoS.indication		(

QoSParameterType1

.

.

.

QoSParameterTypen

)

X.1.3.3 When generated

This primitive is generated by the MIH function when requirements on specific QoS parameters can no longer be met as determined by the application. See also ref [1] and ref [3] and INFO_SPEC in ref [4].

X.1.3.4 Effect on receipt

The MIH User might use this information to determine whether or not a handover operation shall be triggered.

QoS Modifications to existing draft text

In section 7.3.7. we propose to replace the exiting table with the following table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| QoSParameterList | List | N/A | A list of QoS parameters supported by the link layer for which a threshold setting operation is required . |

In section 7.3.8.2 we propose to replace the existing table with the following table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| QoSParameterListStatus | List | N/A | A list of QoS parameter for which thresholds have been set |
| Status | Enumerated | Success Error | Status of operation |

In section 7.3.14.2 modify the existing table according to the following:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileNode | MAC Address | N/A | MAC Address of Mobile Node |
| MacAccessRouter | MAC Address (Optional) | N/A | MAC Address of current Access Router (if any) |
| Num LinkParameters | INTEGER | 0-65535 | Number of parameters in LinkParamterList |
| QoSParameterList | List | N/A | A list of triplets of {QoSParameter, oldValueofQoSParameter, newValueOfQoSParameter) |

# References

- ITU-T Rec. Y.1540 (12/2002), Internet protocol aspects – Quality of Service and network performance: Internet protocol data communication service – IP packet transfer and availability performance parameters

- 3GPP TR 23.882 1.2.0, 3rd Generation Partnership Project, Technical Specification Group Services and System Aspects; 3GPP System Architecture Evolution: Report on Technical Options and Conclusions (Release 7)

- Next Steps In Signaling (NSIS), http://www.ietf.org/html.charters/nsis-charter.html

- NSLP for Quality-of-Service Signaling, http://www.ietf.org/internet-drafts/draft-ietf-nsis-qos-nslp-10.txt