IEEE P802.21/D01.00
Draft IEEE Standard for Local and Metropolitan Area Networks:
Media Independent Handover Services

| Normative Text Proposal for QoS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-July-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Nada Golmie | National Institute of Standards and Technology | 100 Bureau Dr, Stop 8920 Gaithersburg, MD | +1301-975-4190 | Nada.golmie@nist.gov |
| Reijo Salminen | Seesta | Kumitehtaankatu 5, FIN-04260 KERAVA, FINLAND | +358-9-23163636 | Reijo.Salminen@seesta.com |
| Ulises Olvera | InterDigital Communications | 100, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6258 | Ulises.Olvera-Hernandez@InterDigital.com |

- Abstract

This document contains a normative text proposal in support of a Media Independent Handover Quality of Service model.

Notice: This document has been prepared to assist IEEE 802.21. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed MIH QoS Model and normative text additions to the draft D01.

We propose to replace section 5.1.3 on page 13 with the following text:

MIH Quality of Service Model

The IEEE 802.21 specifications provide support for application QOS requirements end-to-end. Considering a network path where multiple access and core network technologies are concatenated together, the end-to-end network performance is obtained by concatenating the performance achieved on each network segment. The MIH services defined by the IEEE 802.21, including event, command, and information service, need to consider the resulting network performance objectives and how well they meet the application quality of service requirements. QOS in the context of 802.21 does not refer to requirements placed on any particular MIHF service, but rather the QOS achieved as a result of the provision of a particular MIHF service. The performance implications to consider from the MIH perspective include both the transient network performance achieved during a handover as well as the continuous monitoring of current network conditions.

The 802.21 specifications provide mechanisms to support seamless mobility as an enabler, however the MIHF alone cannot guarantee seamless mobility. Depending on the QOS requirements of the end-to-end application, seamless mobility implies minimizing the latency, and potential loss incurred during a handover so as to minimize the end-to-end delay and loss perceived by the application. Seamless mobility also implies the timely assessment of network conditions, such as the monitoring of the signal strength, packet loss, on the link, on both current and target networks in order to optimize the handover decision and its execution.

The following set of generic QOS performance parameters are exchanged at the MIH level:

- Throughput: representing the link throughput in kbit/s.

- Delay: representing the frame delay in ms.

- Jitter: representing a frame jitter in ms

- Frame loss rate: representing the frame loss probability.

- Frame error rate: representing the probability of receiving a frame with errors.

We propose to modify section 7.4.4.1 on page 84 with the following text:

7.4.4.1 MIH_Link_Parameters_Report.indication

7.4.4.1.1 Function

This primitive is generated by the MIH Function to inform Upper layers that parameters have crossed thresholds or that the timer for periodic update has expired.

7.4.4.1.2 Semantics of primitives

The primitive parameters are as follows:

MIH_Link_Parameters_Report.indication (

LinkIdentifier

LinkParameterReportList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| LinkIdentifier | TBD | N/A | ID of the link for which the parameters changed. |
| LinkParameterReportList | List | N/A | A list of following set of parameters: LinkParameterType OldValue NewValue |
| LinkParameterType | INTEGER | 0-255 | Parameter type as defined in table 17. |
| OldValue | Threshold values dependent on parameter type. | N/A | Old parameter value. |
| NewValue | Threshold values dependent on parameter type. | N/A | New parameter value. |

Table 17: LinkParameterType values

| Value | Name | Value Size (octets) | Valid Range | Description |
| --- | --- | --- | --- | --- |
| 0 | Throughput | 4 | 0-(232-1) | Link throughput in Kbit/s |
| 1 | Delay | 2 | 0-65535 | Frame delay in ms |
| 2 | Jitter | 2 | 0-65535 | Frame jitter in ms |
| 3 | Frame loss rate | 1 | 0-100 | Percentage of the number of frame lost over the number of frames successfully received. |
| 4 | Frame error rate | 1 | 0-100 | Percentage of the number of frame received with errors over the number of frames successfully received. |
| 5~255 | Reserved | N/A | N/A | Reserved for future use. |

7.4.4.1.3 When generated

This primitive is generated when the MIH Function detected that some link parameters have crossed thresholds or that it is time to send a periodic update.

7.4.4.1.4 Effect on receipt

Upper layers entities inspect the new values and may take actions such as preparing for handover.

We propose to replace section 7.4.7 on page 90 with the following text:

7.4.8 MIH_Configure_Threshold

7.4.8.1 MIH_Configure_Threshold.request

7.4.8.1.1 Function

This primitive is generated by an upper layer to configure thresholds for MIH_Link_Parameter_Report.indication.

7.4.8.1.2 Semantics of primitives

The primitive parameters are as follow:

MIH_Configure_Threshold.request (

SourceIdentifier

LinkIdentifier

LinkParameterList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be left empty if the command is local. |
| LinkIdentifier | TBD | N/A | ID of the link for which the thresholds are being configured. |
| LinkParameterList | List | N/A | A list of the following set of parameters: LinkParameterType InitiateActionThreshold RollbackActionThreshold ExecuteActionThreshold UpdateFrequency |
| LinkParameterType | INTEGER | 0-255 | Parameter type as defined in table 17. |
| InitiateActionThreshold | Threshold values dependent on the parameter type. | N/A | Threshold value which may cause Upper layers to start “setup” type activities in response to actual parameter values crossing this thresholds. |
| RollbackActionThreshold | Threshold values dependent on parameter type. | N/A | Threshold value which may cause Upper layers to cancel or rollback the above setup type operation if the actual parameter values retreat to this threshold. |
| ExecuteActionThreshold | Threshold values dependent on parameter type. | N/A | Threshold value which may cause Upper layers to execute taking appropriate action if the actual parameter values cross this threshold. |
| UpdateFrequency | INTEGER | 0-65535 | Interval time (in ms) at which the MIH must generate an MIH_Link_Parameters_Report to provide updated values to Upper layers. |

7.4.8.1.3 When generated

This primitive is generated by an upper layer entity that wishes to be notified of lower layers changes.

7.4.8.1.4 Effect on receipt

The MIH entity receiving this command will generate a series of link commands in order to monitor the status of the target lower layers and inform upper layers of the changes.

7.4.8.2 MIH_Configure_Threshold.response

7.4.8.2.1 Function

This primitive is generated by MIH function in response to MIH_Configure_Threshold.request primitive. It specifies the status of the configuration operation.

7.4.8.2.2 Semantics of primitives

The primitive parameters are as follow:

MIH_Configure_Threshold.confirm (

SourceIdentifier

LinkIdentifier

LinkParameterStatusList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be left empty if the command is local. |
| LinkIdentifier | TBD | N/A | ID of the link for which the threshold have been set. |
| LinkParameterStatusList | List | N/A | A list of following set of parameters: LinkParameterType Status |
| LinkParameterType | INTEGER | 0-255 | Parameter type as define in table 17 |
| Status | BOOLEAN | True/False | Status of operation: True: success False: failure |

7.4.8.2.3 When generated

This primitive is generated in response to MIH_Configure_Threshold.request primitive

7.4.8.2.4 Effect on receipt

The Upper layers receiving the confirmation can prepare to receive notification of MIH_Link_Parameters_Report for successful configuration.

Annex A-5: QoS

Table 1: An example of a QoS Parameter Mapping Table

Figure 1: Event Flow Diagram: Initial Stage

Figure 2: Event Flow Diagram: Link Selection

Figure 3: Event Flow Diagram: Handover