| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | 21-06-0620-00-0000-New_section_5_5.doc |  |
| Date Submitted | May, 2006 |  |
| Source(s) | Ajay Rajkumar, Peretz Feder, Ulises Olvera-Hernandez | Lucent Technologies dhuo@lucent.com, pfeder@lucent.com ulises.olvera-hernandez@interdigital.com |
| Re: | P802-21-D01.00.pdf |  |
| Abstract | New section 5.5 |  |
| Purpose | Replace text in current draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

5.5 MIH Reference Model for Access Networks

The MIH Function provides asynchronous and synchronous services through well-defined Service Access

Points for upper layers. The following sections describe the reference model for various access networks

with MIH functionality.

Figure 4 – General MIH Reference Model and SAPs

Figure 4 illustrates the position of MIH Function in different protocol stacks and the interaction of MIH

Function with other different elements of the system. All exchanges between the MIH function and other

functional entities occur through service primitives, grouped in Service Access Points (SAPs). The media

agnostic General MIH Reference Model includes the following SAPs:

a) MIH_SAP: Media-independent interface of MIH Function with the upper layers of the protocol

stack.

b) MIH_LINK_SAP: Generic SAP which represents the abstract media-dependent interface of MIH

Function with the lower layers of the protocol stacks.

c) MIH_NMS_SAP: Media-independent interface of MIH Function with the Network Management

System.

d) MIH_NET_SAP: Generic name for the media-dependent interface with the transport facilities available on the local node, supporting the exchange of MIH information and messages with remote MIH Function instances. In an IEEE 802 network, the MIH_NET_SAP typically maps onto (i) the link-layer transport SAP that is specific of that network and (ii) the interface to higher-layer transport facilities that is common to all link-layer technologies. In a 3G cellular network, the MIH_NET_SAP only maps onto the common interface to higher-layer transport facilities. Such interface to higher-layer transport facilities consists of the basic socket APIs (bind(), connect(), sendmsg() and others) that are described in IETF RFC 3493.

The General MIH SAP Reference Model works as a template for the definition of the technology-specific reference models. In those models, the media-independent SAPs (MIH_SAP and MIH_NMS_SAP) always maintain the same names and sets of primitives. Conversely, the media-dependent SAPs (MIH_LINK_SAP and MIH_NET_SAP) assume media-specific names and sets of primitives, often reusing names and primitives that already exist in the respective pre-existing lower-layer SAPs. The MIH_LINK_SAP and MIH_NET_SAP never appear in the technology-specific reference models with their generic names.

The inclusion of media-independent SAPs in the General MIH SAP Reference Model enables the specification of a single, homogeneous set of service primitives across all link-layer technologies. Then, in the technology-specific reference models, the general service primitives map onto functionally identical service primitives, which are found in the pre-existing technology-specific SAPs and/or newly defined in recommended extensions of those SAPs.

The following table provides the specific mapping of the SAPs depicted in the general MIH Reference Model to the specific technologies. The following sections describe this mapping.

| General MIH Ref. Model | 802.3 | 802.11 | 802.16 |
| --- | --- | --- | --- |
| MIH_NMS_SAP | Not Applicable | MIH_NMS_SAP | MIH_NMS_SAP |
| MIH_SAP | MIH_SAP | MIH_SAP | MIH_SAP |
| MIH_Link_SAP | LSAP | MLME_SAP | C-SAP M-SAP |
| MIH_NET_SAP for layer 2 MIH_NET_SAP for layer 3 | LSAP Socket APIs | LSAP/MLME_SAP Socket APIs | Eth_CS_SAP/GPCS_SAP/C-SAP Socket APIs |