| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Event Register and DeRegister Commands |  |
| DCN | 21-06-0622-00-0000 |  |
| Date Submitted | April 29, 2006 |  |
| Source(s) | Ajay Rajkumar |  |
| Re: | IEEE 802.21 Session #14 in Jacksonville, Florida |  |
| Abstract | Describe MIH Event Register and DeRegister Call flow and description |  |
| Purpose | Binding Technical Comment in response to Letter Ballot #1 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

    - MIH_Event_Register.request

#### Function

This primitive is used by MIH User (the registrant) to register an interest in one or more event types from a particular MIH Function. The response indicates which of the requested event types were successfully registered. Event types not successfully registered will not be delivered to the registrant.

#### Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Register.request			 	(

RequestedEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| RequestedEvent List | Set of Event IDs | Set of valid link events | List of link layer events that the endpoint would like to receive indications for, from the Event Source |

#### When generated

This primitive is generated by a registrant such as MIH Function that is seeking to receive event primitives from a point in the protocol stacks terminating either end of the link.

#### Effect on receipt

The recipient responds immediately with MIH_Event_Register.confirm primitive.

### MIH_Event_Register.confirm

#### Function

This primitive returns the result of the registration request.

#### Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Register.confirm			 	(

ResponseEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResponseEvent List | Set of Event IDs and their registration status | Set of valid link events | List of link layer events along with their registration status |

#### When generated

This primitive is generated in response to a MIH_Event_Register.request primitive.

#### Effect on receipt

The recipient may examine the returned event list and learn about the registration status of different events.

### MIH_Event_Deregister.request

#### Function

This primitive is used by MIH Function (the registrant) to deregister for a set of link layer events.

#### Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Deregister.request			 	(

RequestedEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| RequestedEvent List | Set of Event IDs | Set of valid link events | List of link layer events for which indications need to be deregistered from the Event Source |

#### When generated

This primitive is generated by a registrant such as MIH Function that is seeking to deregister for already registered set of events.

#### Effect on receipt

The recipient responds immediately with MIH_Event_Deregister.confirm primitive.

### MIH_Event_Deregister.confirm

#### Function

This primitive returns the result of the deregistration request.

#### Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Deregister.confirm			 	(

ResponseEventList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResponseEvent List | Set of Event IDs and their Deregistration status | Set of valid link events | List of link layer events along with their deregistration status |

#### When generated

This primitive is generated in response to a MIH_Event_Deregister.request primitive.

#### Effect on receipt

The recipient may examine the returned event list and learn about the deregistration status of different events.

Rationale: There are two ways to look at Link Event Registration.

One would be that Link Event Registration is initiated when an MIH Function receives an MIH Event Registration. On the other hand, an MIH User could directly send an explicit Link Event Registration Command. Since there is a clear separation between MIH Events and Link Events, the first option is a better.

### Call Flows to show the impact of MIH Event Registration and DeRegistration on the Link Event Registration and DeRegistration

MIH User                     MIHF                    Link Interface

MIH_Event_Register.request

Link_Event_Register.request

Link_Event_Register.confirm

MIH_Event_Register.confirm

A similar call flow Event DeRegistration is as follows:

MIH User                       MIHF                        Link Interface

MIH_Event_DeRegister.request

Link_Event_DeRegister.request

Link_Event_DeRegister.confirm

MIH_Event_DeRegister.confirm