| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | 21-06-0625-01-0000-New_section_5_5_2.doc |  |
| Date Submitted | May, 2006 |  |
| Source(s) | Andrea Francini, Ajay Rajkumar, Peretz Feder, Ulises Olvera-Hernandez | Lucent Technologies Ajayrajkumar/pfeder/francini@lucent.com ulises.olvera-hernandez@interdigital.com |
| Re: | P802-21-D01.00.pdf |  |
| Abstract | New section 5.5.2 |  |
| Purpose | Replace text in current draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

5.5.2 MIH Reference Model for 802.11

Figure 7 –  MIH Reference Model for 802.11

The logical placement of the MIH Function in the 802.11 protocol stack for stations and access points is shown in Figure 7.

Similarly to 802.3, the LLC SAP (LSAP) defines the interface of the MIH Function with the 802.11 data plane and can encapsulate MIH messages in data frames. However, since 802.11 does not currently support Class 1 data frames, MIH messages can be transported over the 802.11 data plane only after the Mobile Node has associated with the 802.11 access point. Before the association between Mobile Node and access point takes place, the L2 transport of MIH messages can rely on 802.11 management frames from the 802.11 management plane (MLME). The MLME_SAP defines the interface between the MIH Function and the MLME.

Since the MAC Layer Management Entity (MLME) is the immediate MIH counterpart in the lower layers of the 802.11 mobility-management, the MLME_SAP instantiates the MIH_LINK_SAP in the MIH Reference Model for 802.11.

The MLME_SAP and the pre-existing LLC SAP (LSAP) instantiate the link-layer portion of the generic MIH_NET_SAP for the transport of MIH messages over L2, respectively before and after the association of the Mobile Node with the 802.11 access point takes place. The MIH Function interface that instantiates the higher-layer portion of the MIH_NET_SAP are the basic socket APIs (bind(), connect(), sendmsg(), others) functions described in RFC-3493.