| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Higher Layer Requirements for Event and Command Services |  |
| Date Submitted |  |  |
| Source(s) | Srinivas Sreemanthula …. |  |
| Ref: | 21-06-0644-00-0000-mih-registration.doc |  |
| Abstract | This contribution provides clarifications to events and commands defined in commentary. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

## MIH Registration (Insert in section 8.4.1.1)

MIH Registration enables two MIH entities to create an MIH pairing. The MIH pairing allows network MIH entities to be aware of a UE’s presence in a certain network so that MIH services can be exchanged. Specifically, it allows MIH network entities to register for events as seen by the multi-access UE and from a UE point of view, the pairing provides authorization for network entities to send commands to enhance the overall user experience utilizing the inter-technology access resources.

The MIH registration request is sent by a client MIH function to a peer MIH function in the network to setup a session with an identifier. This identifier can be used in future MIH communication between the two nodes while the registration session is active. Upon successful registration, the MIH peers can exchange MIH messages for MIH ES/CS services. An MIH network entity may not be to provide any MIH services to the UE without the registration process.  The registration procedure is defined to be originated by UE, but the network MIH entities may not have to perform any MIH registration with the UE. This facility would enable MIH function in the UE to make MIH event registrations with more than one MIH entity in the network so that MIH events may be generated towards the UE. However, UE can reject commands if no MIH pairing is available with a certain MIH network entity.

The following figure shows a reference communication state diagram for MIH event and command services. Initially, MIH node performs a MIH peer discovery to determine the end point address. After MIH discovery, MIH node will perform capability discovery to determine what services are supported at the other end followed by the MIH registration to create the pairing. Upon successful registration, it can exchange MIH protocol messages with the peer node. In the MIH-Registered state, the MIH peers can request for specific event registrations or send MIH commands to request for specific actions.

### MIH_Register.Request (Optional insertion into draft in section 7)

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function in the network to perform registration.

#### Semantics of service primitive

MIH_Register.Request

(

Source Identifier (MIH-ID),

Destination Identifier (MIH-ID),

Session ID,

RequestCode

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) <- MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier (e.g. NAI or FQDN) | Source of this message. Optional for re-registration. |
| Destination Identifier | Identifier | Any valid individual or group identifier (e.g. NAI or FQDN) | Destination of this message |
| Session Id | Identifier |  | Session Identifier |
| Request Code | Enumerated type | 0 - Registration 1 - Re-Registration | Re-registration scope to be defined. |

#### When generated

The MIH Function generates this message when it needs to perform a MIH registration to a particular destination MIH function.

#### Effect on receipt

Upon receipt, the MIH Function can verify the credentials and perform additional security measures (not defined in 802.21) to complete the registration.

### MIH_Register.Response (Optional insertion into draft in section 7)

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function in the network to indicate the result of a registration request.

#### Semantics of service primitive

MIH_Register.Response

(

Source Identifier ,

Destination Identifier,

Session Id,

ValidityTime,

Result Code,
Failure Code

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) -> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Source of this message |
| Destination Identifier | Identifier | Any valid individual or group identifier | Destination of this message |
| Session Id | Identifier |  | A session identifier assigned to the registration period |
| ValidityTime | Integer |  | Time in secs |
| Result code | BOOL | SUCCESS, FAILURE |  |
| Failure code | Enumerated type | Integer | If Result code indicates failure |

#### When generated

The MIH Function generates this message when after completion of registration request processing.

#### Effect on receipt

Upon receipt, the MIH Function can determine that the result of registration request.

### MIH_DeRegister.Request

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function in the network to perform a de-registration.

#### Semantics of service primitive

MIH_DeRegister.Request

(

Source Identifier (MIH-ID),

Destination Identifier (MIH-ID),

Session ID

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) <- MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier (e.g. NAI or FQDN) | Source of this message. Optional. |
| Destination Identifier | Identifier | Any valid individual or group identifier (e.g. NAI or FQDN) | Destination of this message |
| Session Id | Identifier |  | Session Identifier |

#### When generated

The MIH Function generates this message when it needs to perform a MIH de-registration to a particular destination MIH function by providing a valid session-id obtained from previous registration exchange.

#### Effect on receipt

Upon receipt, the MIH Function can perform de-registration and delete the session context.

### MIH_DeRegister.Response

#### Function

This primitive is used by a peer MIH Function to communicate with the MIH Function in the network to indicate the result of a deregistration request.

#### Semantics of service primitive

MIH_DeRegister.Response

(

Source Identifier ,

Destination Identifier,

SessionId,

Result Code,

Failure Code

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) -> MIHF (client)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Source of this message |
| Destination Identifier | Identifier | Any valid individual or group identifier | Destination of this message |
| Session Id | Identifier |  | Session Identifier |
| Result code | BOOL | SUCCESS, FAILURE |  |
| Failure code | Enumerated type | Integer | If Result code indicates failure |

#### When generated

The MIH Function generates this message when after completion of deregistration request processing.

#### Effect on receipt

Upon receipt, the MIH Function can determine that the result of deregistration request.

### MIH_Registration Request (Create 8.5.1.3)

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Source Identifier | xxx | variable | MIHF-ID (e.g. NAI or FQDN) |
| Destination Identifier | xxx | variable | MIHF-ID (e.g. NAI or FQDN) |
| Request Code | xxx | BOOL | 0 - Registration 1 - Reregistration |
| Session ID | xxx | 4 | String |

### MIH_Registration Response (Create 8.5.1.4)

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Source Identifier | xxx | variable | MIHF-ID (e.g. NAI or FQDN) |
| Destination Identifier | xxx | variable | MIHF-ID (e.g. NAI or FQDN) |
| Session ID | xxx | 4 | String |
| Validity Period | Integer | 4 | Time Registration is valid |
| Reg Result Code | BOOL | 1 | 1 - Success; 0 - Failure |
| Reg Failure Code | Enumerated value | Enum | TBD |

### MIH_DeRegistration Request (Insert into 8.5.1.5)

| Name | Type* | Length | Value |
| --- | --- | --- | --- |
| Source Identifier | Xxx | variable | MIHF-ID (e.g. NAI or FQDN) |
| Destination Identifier | Xxx | Variable | MIHF-ID (e.g. NAI or FQDN) |
| Session ID | Xxx | Variable | String |

### MIH_DeRegistration Response (Insert into 8.5.1.6)

| Name | Type* | Length | Value |
| --- | --- | --- | --- |
| Source Identifier | Xxx | variable | MIHF-ID (e.g. NAI or FQDN) |
| Destination Identifier | Xxx | Variable | MIHF-ID (e.g. NAI or FQDN) |
| Reg Result Code | Xxx | BOOL | 1 - Success; 0 - Failure |
| DeReg Failure Code | Xxx | Enum | TBD |
| Session ID | Xxx | Variable | String |

* Need to define the type codes for new elements.