| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements TLV Representation (Comment Assignment #3) |  |
| Date Submitted | June, 2006 |  |
| Source(s) | Qiaobing Xie |  |
| Re: | 21-06-0xxx-00-0000-IE_TLV_representation |  |
| Abstract | Representation of Information Elements |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Replace current 6.3.4.1, 6.3.4.2, and 6.3.4.3 with the following new 6.3.4.1:

6.3.4.1. TLV IE Representation

For a given geophysical location, the top level IE is the List of Neighboring Access Networks containing at least 1 Network IE

List of neighboring networks IE

| Type=LIST_OF_NETWORKS | Length= Variable |
| --- | --- |
| NETWORK IE #1 |  |
| NETWORK IE #2 (optional) |  |
| … |  |
| NETWORK IE #k (optional) |  |
| VENDER_SPECIFIC IE (optional) |  |

A Network IE consists of all IEs depicting the network information and a Network IE contains at least 1 PoA IE:

Network IE

| Type=NETWORK | Length= Variable |
| --- | --- |
| NETWORK TYPE IE (e.g., 802.11, 802.16, CDMA2000, UMTS, 1X-EV) |  |
| OPERATOR_IDENTIFICATION IE |  |
| ROAMING_PARTNERS IE (optional) |  |
| COST IE (optional) |  |
| NETWORK_SECURITY IE (optional) |  |
| QOS IE (optional) |  |
| POA IE #1 |  |
| POA IE #2 (optional) |  |
| … |  |
| POA IE #k (optional) |  |
| VENDER_SPECIFIC_NETWORK IE (optional) |  |

A PoA IE consists of all IEs depicting the PoA:

PoA IE

| Type=POA | Length= Variable |
| --- | --- |
| ADDRESS IE (MAC address of PoA) |  |
| LOCATION IE |  |
| DATA_RATE IE |  |
| MAC_TYPE IE |  |
| CHANNEL_RANGE IE |  |
| SUBNET_INFORMATION IE |  |
| CABABILITIES IE |  |
| VENDER_SPECIFIC_POA IE (optional) |  |

Replace current 6.3.5, 6.3.6, and 6.3.7 with the following new 6.3.5:

6.3.5 Information Retrieval

MIIS Query: MIIS client sends Information_query(QUERY_TLV) to MIIS server to initiate a query, where QUERY_TLV has the following format:

| Type=MIIS_QUERY | Length= Variable |
| --- | --- |
| LIST_OF_IE_TYPES_TO_RETURN |  |
| QUERY_LOCATION IE (optional) |  |
| LIST_OF_INCLUSION_CONDITIONS (optional) |  |
| LIST_OF_EXCLUSION_CONDITIONS (optional) |  |

Note, the inclusion/exclusion conditions employs typical OR, AND, ==, !=, >, <, etc. operators and possibly wildcards.

MIIS Response: MIIS server sends Information_response(RESPONSE_TLV) in response to a query, where RESPONSE_TLV contains one or more returned IEs:

| Type=MIIS_RESPONSE | Length= Variable |
| --- | --- |
| RETURNED IE #1 |  |
| RETURNED IE #2 |  |
| … |  |
| RETURNED IE #k |  |

6.3.6 Information Query and Response Examples

We use the following examples to illustrate how neighboring network information is represented and retrieved. The values of the IEs used in the examples are fictitious and are for illustrative purposes only.

At a given location, we assume that there are three neighboring networks – a UMTS network owned by “red-mobile”, a 802.16 network also owned by “red-mobile”, and another 802.16 network owned by “blue-mobile” – with the following attributes, respectively:

1. a UMST network:

- Network Type: “umts”

- Operator_ID: “red-mobile”

- Roaming_Partners: “red-partner1, red-partner2”

- Cost: 5

- Network_Security: “strong-cipher”

- QoS: “class1/class2/class3”

- PoA #1:

  - MAC_address: mac_addr1

  - Location: coord_1

  - Data_rate: 50

  - MAC_type: n/a

  - Channel_range: 8

  - Subnet_info: “IPv4-192.35.0.0”

  - Capabilities: 0x4A

- PoA #2:

  - MAC address: mac_addr2

  - Location: coord_2

  - Data_rate: 50

  - MAC_type: unknown

  - Channel_range: 6

  - Subnet_info: “IPv4-192.31.0.0”

  - Capabilities: 0x4C

2. a 802.16 (WiMAX) network:

- Network Type: “802.16”

- Operator_ID: “red-mobile”

- Roaming_Partners: “none”

- Cost: 3

- Network_Security: “strong-cipher”

- QoS: “class1”

- PoA #1:

  - MAC address: mac_addr3

  - Location: coord_3

  - Data_rate: 80

  - MAC_type: 802.16

  - Channel_range: 7

  - Subnet_info: “IPv4-192.201.0.0”

  - Capabilities: 0xB7

3. a 802.16 (WiMAX) network:

- Network Type: “802.16”

- Operator_ID: “blue-mobile”

- Roaming_Partners: “blue-partner1”

- Cost: 3

- Network_Security: “open”

- QoS: “class1/class2”

- PoA #1:

  - MAC address: mac_addr4

  - Location: coord_4

  - Data_rate: 95

  - MAC_type: 802.16

  - Channel_range: 7

  - Subnet_info: “IPv6-2002:522d:94d1::/48”

  - Capabilities: 0xB1

Example Query 1: IS client asking for all neighboring network information

a) The client sends Information_query(QUERY_TLV) to information server, with QUERY_TLV set to:

| Type=MIIS_QUERY | Length= Variable |
| --- | --- |
| LIST_OF_IE_TYPES_TO_RETURN= LIST_OF_NETWORKS |  |

b) The information server sends back Information_response(RESPONSE_TLV), with RESPONSE_TLV containing information of all three networks:

| Type=MIIS_RESPONSE |  |  |  | Length= Variable |  |
| --- | --- | --- | --- | --- | --- |
| Type=LIST_OF_NETWORKS |  |  |  | Length= Variable |  |
|  | Type=NETWORK |  |  | Length= Variable |  |
|  | NETWORK_TYPE IE = “umts” |  |  |  |  |
|  | OPERATOR_IDENTIFICATION IE = “red-mobile” |  |  |  |  |
|  | ROAMING_PARTNERS IE = “red-partner1, red-partner2” |  |  |  |  |
|  | COST IE = 5 |  |  |  |  |
|  | NETWORK_SECURITY IE = “strong-cipher” |  |  |  |  |
|  | QOS IE = “class1/class2/class3” |  |  |  |  |
|  |  | Type=POA | Length= Variable |  |  |
|  |  | ADDRESS IE = mac_addr1 |  |  |  |
|  |  | LOCATION IE = coord_1 |  |  |  |
|  |  | DATA_RATE IE = 50 |  |  |  |
|  |  | MAC_TYPE IE = NULL |  |  |  |
|  |  | CHANNEL_RANGE IE = 8 |  |  |  |
|  |  | SUBNET_INFORMATION IE = “IPv4-192.35.0.0” |  |  |  |
|  |  | CABABILITIES IE = 0x4A |  |  |  |
|  |  | Type=POA | Length= Variable |  |  |
|  |  | ADDRESS IE = mac_addr2 |  |  |  |
|  |  | LOCATION IE = coord_2 |  |  |  |
|  |  | DATA_RATE IE = 50 |  |  |  |
|  |  | MAC_TYPE IE = NULL |  |  |  |
|  |  | CHANNEL_RANGE IE = 6 |  |  |  |
|  |  | SUBNET_INFORMATION IE = “IPv4-192.31.0.0” |  |  |  |
|  |  | CABABILITIES IE = 0x4C |  |  |  |
|  | Type=NETWORK |  | Length= Variable |  |  |
|  | NETWORK_TYPE IE = “802.16” |  |  |  |  |
|  | OPERATOR_IDENTIFICATION IE = “red-mobile” |  |  |  |  |
|  | ROAMING_PARTNERS IE = “none” |  |  |  |  |
|  | COST IE = 3 |  |  |  |  |
|  | NETWORK_SECURITY IE = “strong-cipher” |  |  |  |  |
|  | QOS IE = “class1” |  |  |  |  |
|  |  | Type=POA |  |  | Length= Variable |
|  |  | ADDRESS IE = mac_addr3 |  |  |  |
|  |  | LOCATION IE = coord_3 |  |  |  |
|  |  | DATA_RATE IE = 80 |  |  |  |
|  |  | MAC_TYPE IE = 802.16 |  |  |  |
|  |  | CHANNEL_RANGE IE = 7 |  |  |  |
|  |  | SUBNET_INFORMATION IE = “IPv4-192.201.0.0” |  |  |  |
|  |  | CABABILITIES IE = 0xB7 |  |  |  |
|  | Type=NETWORK |  | Length= Variable |  |  |
|  | NETWORK_TYPE IE = “802.16” |  |  |  |  |
|  | OPERATOR_IDENTIFICATION IE = “blue-mobile” |  |  |  |  |
|  | ROAMING_PARTNERS IE = “blue-partner1” |  |  |  |  |
|  | COST IE = 3 |  |  |  |  |
|  | NETWORK_SECURITY IE = “open” |  |  |  |  |
|  | QOS IE = “class1/class2” |  |  |  |  |
|  |  | Type=POA |  |  | Length= Variable |
|  |  | ADDRESS IE = mac_addr4 |  |  |  |
|  |  | LOCATION IE = coord_4 |  |  |  |
|  |  | DATA_RATE IE = 95 |  |  |  |
|  |  | MAC_TYPE IE = 802.16 |  |  |  |
|  |  | CHANNEL_RANGE IE = 7 |  |  |  |
|  |  | SUBNET_INFORMATION IE = “IPv6-2002:522d:94d1::/48” |  |  |  |
|  |  | CABABILITIES IE = 0xB1 |  |  |  |

Example Query 2: IS client asking for only UMTS neighboring network information

a) The client sends Information_query(QUERY_TLV) to information server, with QUERY_TLV set to:

| Type=MIIS_QUERY | Length= Variable |
| --- | --- |
| LIST_OF_IE_TYPES_TO_RETURN= LIST_OF_NETWORKS |  |
| LIST_OF_INCLUSION_CONDITIONS= “NETWORK_TYPE IE == umts” |  |

b) The information server sends back Information_response(RESPONSE_TLV), with RESPONSE_TLV containing only UMTS network information:

| Type=MIIS_RESPONSE |  |  |  | Length= Variable |
| --- | --- | --- | --- | --- |
| Type=LIST_OF_NETWORKS |  |  |  | Length= Variable |
|  | Type=NETWORK |  |  | Length= Variable |
|  | NETWORK_TYPE IE = “umts” |  |  |  |
|  | OPERATOR_IDENTIFICATION IE = “red-mobile” |  |  |  |
|  | ROAMING_PARTNERS IE = “red-partner1, red-partner2” |  |  |  |
|  | COST IE = 5 |  |  |  |
|  | NETWORK_SECURITY IE = “strong-cipher” |  |  |  |
|  | QOS IE = “class1/class2/class3” |  |  |  |
|  |  | Type=POA | Length= Variable |  |
|  |  | ADDRESS IE = mac_addr1 |  |  |
|  |  | LOCATION IE = coord_1 |  |  |
|  |  | DATA_RATE IE = 50 |  |  |
|  |  | MAC_TYPE IE = NULL |  |  |
|  |  | CHANNEL_RANGE IE = 8 |  |  |
|  |  | SUBNET_INFORMATION IE = “IPv4-192.35.0.0” |  |  |
|  |  | CABABILITIES IE = 0x4A |  |  |
|  |  | Type=POA | Length= Variable |  |
|  |  | ADDRESS IE = mac_addr2 |  |  |
|  |  | LOCATION IE = coord_2 |  |  |
|  |  | DATA_RATE IE = 50 |  |  |
|  |  | MAC_TYPE IE = NULL |  |  |
|  |  | CHANNEL_RANGE IE = 6 |  |  |
|  |  | SUBNET_INFORMATION IE = “IPv4-192.31.0.0” |  |  |
|  |  | CABABILITIES IE = 0x4C |  |  |

Example Query 3: IS client asking for PoA information owned by operator “blue-mobile”

a) The client sends Information_query(QUERY_TLV) to information server, with QUERY_TLV set to:

| Type=MIIS_QUERY | Length= Variable |
| --- | --- |
| LIST_OF_IE_TYPES_TO_RETURN= POA |  |
| LIST_OF_INCLUSION_CONDITIONS= “OPERATOR_IDENTIFICATION == blue-mobile” |  |

b) The information server sends back Information_response(RESPONSE_TLV), with RESPONSE_TLV containing only PoA information from operator “blue-mobile”:

| Type=MIIS_RESPONSE |  | Length= Variable |  |
| --- | --- | --- | --- |
|  | Type=POA |  | Length= Variable |
|  | ADDRESS IE = mac_addr4 |  |  |
|  | LOCATION IE = coord_4 |  |  |
|  | DATA_RATE IE = 95 |  |  |
|  | MAC_TYPE IE = 802.16 |  |  |
|  | CHANNEL_RANGE IE = 7 |  |  |
|  | SUBNET_INFORMATION IE = “IPv6-2002:522d:94d1::/48” |  |  |
|  | CABABILITIES IE = 0xB1 |  |  |

Example Query 4: IS client asking for subnet_information of PoAs of operator “red-mobile”

a) The client sends Information_query(QUERY_TLV) to information server, with QUERY_TLV set to:

| Type=MIIS_QUERY | Length= Variable |
| --- | --- |
| LIST_OF_IE_TYPES_TO_RETURN= SUBNET_INFORMATION |  |
| LIST_OF_INCLUSION_CONDITIONS= “OPERATOR_IDENTIFICATION == red-mobile” |  |

b) The information server sends back Information_response(RESPONSE_TLV), with RESPONSE_TLV containing only Subnet_information of PoAs from operator “red-mobile”:

| Type=MIIS_RESPONSE |  | Length= Variable |
| --- | --- | --- |
|  | SUBNET_INFORMATION IE = “IPv4-192.35.0.0” |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.31.0.0” |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.201.0.0” |  |

Note, though not shown here, each SUBNET_INFORMATION IE itself is a TLV.

Example Query 4: IS client asking for PoAs, excluding those with DATA_RATE less than 75

a) The client sends Information_query(QUERY_TLV) to information server, with QUERY_TLV set to:

| Type=MIIS_QUERY | Length= Variable |
| --- | --- |
| LIST_OF_IE_TYPES_TO_RETURN= POA |  |
| LIST_OF_EXCLUSION_CONDITIONS= “DATA_RATE < 74” |  |

b) The information server sends back Information_response(RESPONSE_TLV), with RESPONSE_TLV containing only PoAs with DATA_RATE larger or equal to 75:

| Type=MIIS_RESPONSE |  | Length= Variable |  |
| --- | --- | --- | --- |
|  | Type=POA |  | Length= Variable |
|  | ADDRESS IE = mac_addr3 |  |  |
|  | LOCATION IE = coord_3 |  |  |
|  | DATA_RATE IE = 80 |  |  |
|  | MAC_TYPE IE = 802.16 |  |  |
|  | CHANNEL_RANGE IE = 7 |  |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.201.0.0” |  |  |
|  | CABABILITIES IE = 0xB7 |  |  |
|  | Type=POA |  | Length= Variable |
|  | ADDRESS IE = mac_addr4 |  |  |
|  | LOCATION IE = coord_4 |  |  |
|  | DATA_RATE IE = 95 |  |  |
|  | MAC_TYPE IE = 802.16 |  |  |
|  | CHANNEL_RANGE IE = 7 |  |  |
|  | SUBNET_INFORMATION IE = “IPv6-2002:522d:94d1::/48” |  |  |
|  | CABABILITIES IE = 0xB1 |  |  |