| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements TLV Representation (Comment Assignment #3) |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Qiaobing Xie, Vivek Gupta, Xiaoyu Liu, Soohong Daniel Park, Eric Njedjou, Masahiro Kuroda |  |
| Re: | 21-06-0674-02-0000-IE_TLV_representation |  |
| Abstract | Representation of Information Elements |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

A) Add the following figure in section 6.3.2 after Table-6.

TYPE_IE_NETWORK_TYPE              = …….

TYPE_IE_OPERATOR_IDENTIFIER = ……..

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = UMTS

TYPE_IE_OPERATOR_IDENTIFIER = Operator_3

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = UMTS

TYPE_IE_OPERATOR_IDENTIFIER = Operator_1

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = 802.16

TYPE_IE_OPERATOR_IDENTIFIER = Operator_3

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

TYPE_IE_NETWORK_TYPE              = 802.11

TYPE_IE_OPERATOR_IDENTIFIER = Operator_2

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

802.11, Operator_1 PoAK

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_1, PoA3

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_1, PoA2

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

List of Networks

TYPE_IE_NETWORK_TYPE              = 802.11

TYPE_IE_OPERATOR_IDENTIFIER = Operator_1

General Network Information

{

TYPE_IE_ROAMING_PARTNERS

TYPE_IE_COST

TYPE_IE_SECURITY

TYPE_IE_QOS

}

802.11, Operator_1, PoA1

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANGE

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_2, PoAn

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_2, PoA2

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANG

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

802.11, Operator_2, PoA1

PoA Information

{

TYPE_IE_POA_ADDRESS

TYPE_IE_POA_LOCATION

TYPE_IE_POA_MAC_TYPE

TYPE_IE_POA_PHY_TYPE

TYPE_IE_POA_CHANNEL_RANGE

TYPE_IE_POA_DATA_RATE

TYPE_IE_POA_SUBNET_INFO

TYPE_IE_POA_CAPABILITIES

}

Figure XX – Depicting a List of Neighboring Networks with Information Elements

B) Insert the following new section before the current 6.3.4.1:

6.3.4.0. IE Containers

To effectively represent the heterogeneous neighborhood information in TLV format, three Information Element Containers are defined, namely List of Neighboring Access Networks Container, Access Network Container, and PoA Container:

- List of Neighboring Access Networks Container - contains a list of heterogeneous neighboring access networks for a given geophysical location. 

A List of Neighboring Access Networks Container shall contain at least one Access Network and may contain one or more Vender Specific IEs.

| Type= TYPE_CONTAINER_LIST_OF_NETWORKS | Length= Variable |
| --- | --- |
| Access Network Container #1 |  |
| Access Network Container #2 (optional) |  |
| … |  |
| Access Network Container #k (optional) |  |
| Vender Specific IE (optional) |  |

- Access Network Container – contains all the information depicting an access network. 

An Access Network Container shall contain at least one PoA Container and may contain one or more Vender Specific Network IEs:

| Type=TYPE_CONTAINER_NETWORK | Length= Variable |
| --- | --- |
| Network Type IE |  |
| Operator Identifier IE |  |
| Roaming Partners IE (optional) |  |
| Cost IE (optional) |  |
| Network Security IE (optional) |  |
| QoS IE (optional) |  |
| PoA Container #1 |  |
| PoA Container #2 (optional) |  |
| … |  |
| PoA Container #k (optional) |  |
| Vender Specific Network IE (optional) |  |

- PoA Container – contains all the information depicting a PoA and may include one or more optional Vender Specific PoA IEs:

| Type=TYPE_CONTAINER_POA | Length= Variable |
| --- | --- |
| PoA Location IE |  |
| PoA Data Rate IE |  |
| PoA MAC Type IE |  |
| PoA Channel Range IE |  |
| PoA Subnet Information IE |  |
| PoA Capabilities IE |  |
| PoA PHY Type IE |  |
| PoA Address IE |  |
| Vender Specific PoA IE (optional) |  |

TLVs for the component IEs containing in the Access Network Container and PoA Container are defined in current Sections 6.3.4.1 – 6.3.4.3.

C) Delete sections 6.3.4.1.1, 6.3.4.1.2, 6.3.4.1.3, and 6.3.4.2.1 (no longer needed).

D) Add the following new IE to depicting network type:

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_TYPE | Type of the access network | 1 octets | For example, RADIUS NAS-Port-Type attribute value which is a unique identifier defined in http://www.iana.org/ assignments/radius-types, e.g., 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - 1X-EV etc. |

E) Replace current 6.3.5, 6.3.6, and 6.3.7 with the following new 6.3.5:

6.3.5 Information Retrieval Using TLV Representation

To retrieve the heterogeneous neighborhood information (or a subset of it), the client sends an Information Request to the MIH PoS. The Information request contains query for Information Elements. The MIH PoS sends an Information Response which contains the response of the query back to the client.

6.3.5.1 Information Request

Information Request (from client to information server) shall carry a MIIS_TLV_QUERY TLV which contains: 1) a list of IE types to be returned; 2) an optional Querier Location TLV parameter indicating the location of the querier, and 3) an optional list of Exclusion Conditions TLVs, as shown below:

| Type=MIIS_TLV_QUERY | Length= Variable |
| --- | --- |
| Querier Location TLV [variable] (optional) |  |
| Exclusion Conditions TLV #1 [variable] (optional) |  |
| … |  |
| Exclusion Conditions TLV #k [variable] (optional) |  |
| Reporting Template TLV [variable] (optional) |  |

The TLVs that are carried in the Information Request are described below.

Querier Location TLV is defined as follows:

| Type=MIIS_TLV_QUERIER_LOCATION | Length= Variable |
| --- | --- |
| The value is defined the same as that of PoA Location IE (see 6.3.4.3.1) |  |

Querier Location, if provided in the query, can be useful for the Information Server to refine its response. If not provided (for example, a UE does not have its location measurement when making the query), the Information Server shall either get the querier location information from other means or use best estimate of the querier’s location to generate the neighboring network information.

Exclusion Conditions TLV is defined as follows:

| Type=MIIS_TLV_EXCLUSION_CONDITIONS | Length= Variable |
| --- | --- |
| IE Condition TLV #1 |  |
| … |  |
| IE Condition TLV #k |  |

where each IE Condition TLV has the following structure:

| Type=MIIS_TLV_IE_CONDITION | Length= Variable |
| --- | --- |
| IE_type_to_compare [4 octets] |  |
| Relational_operator [1 octet] |  |
| Compare_to_value [variable] |  |

Relational_operator, an integer, takes the following values:

0: equal to (==)

1: unequal to (!=)

2: greater than (>)

3: less than (<)

4: greater than or equal to (>=)

5: less than or equal to (<=)

6-7: reserved and not to be used.

Compare_to_value can be either numerical or text string, depending on the type of IE. The evaluation result of an Exclusion Conditions TLV is only TRUE when all of the contained IE Condition TLVs are TRUE.

The following table indicates for a given IE type which comparators are allowed.

| Name of IE | Comparator Support |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | == | != | > | < | >= | <= |
| List of Neighboring Access Networks Container | - | - | - | - | - | - |
| Access Network Container | - | - | - | - | - | - |
| Network Type IE | X | X | - | - | - | - |
| Operator Identifier IE | X | X | - | - | - | - |
| Roaming Partners IE | X | X | - | - | - | - |
| Cost IE | X | X | X | X | X | X |
| Network Security IE | X | X | - | - | - | - |
| QoS IE | X | X | - | - | - | - |
| PoA Container | - | - | - | - | - | - |
| PoA Location IE | X? | X? | - | - | - | - |
| PoA Data Rate IE | X | X | X | X | X | X |
| PoA MAC Type IE | X | X |  |  |  |  |
| PoA Channel Range IE | X | X | X | X | X | X |
| PoA Subnet Information IE | X | X | - | - | - | - |
| PoA Capabilities IE | X | X | - | - | - | - |
| PoA PHY Type IE | X | X | - | - | - | - |
| PoA Address IE | X | X | - | - | - | - |

Reporting Template TLV is optional and of variable size. When present, it indicates to the information server a template of the list of IEs which shall be included in the information response:

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= Variable |
| --- | --- |
| TYPE_CONTAINER_LIST_OF_NETWORKS [4 octets] (optional) |  |
| TYPE_CONTAINER_NETWORK [4 octets] (optional) |  |
| TYPE_IE_Network_Type [4 octets] (optional) |  |
| TYPE_IE_Operator_Identifier [4 octets] (optional) |  |
| TYPE_IE_Roaming_Partners [4 octets] (optional) |  |
| TYPE_IE_Cost [4 octets] (optional) |  |
| TYPE_IE_Network_Security [4 octets] (optional) |  |
| TYPE_IE_QoS [4 octets] (optional) |  |
| TYPE_ Container _PoA [4 octets] (optional) |  |
| TYPE_IE_PoA_Location [4 octets] (optional) |  |
| TYPE_IE_PoA_Data_Rate [4 octets] (optional) |  |
| TYPE_IE_PoA_MAC_Type [4 octets] (optional) |  |
| TYPE_IE_PoA_Channel_Range [4 octets] (optional) |  |
| TYPE_IE_PoA_Subnet_Information [4 octets] (optional) |  |
| TYPE_IE_PoA_Capabilities [4 octets] (optional) |  |
| TYPE_IE_PoA_PHY_Type [4 octets] (optional) |  |
| TYPE_IE_PoA_Address [4 octets] (optional) |  |

Each of the IEs in the Requested IEs TLV is indicated by its 32 bit identifier as assigned in Table yy.

Rules for using the Reporting Template TLV:

1) If the Reporting Template TLV is absent, the entire list of neighboring networks container shall be returned in the response. E.g.

| Type=MIIS_TLV_QUERY | Length= actual size of Location TLV |
| --- | --- |
| Querier Location TLV = (x, y) |  |

returns the entire List of Neighboring Networks Container at location (x, y).

2) If a container is listed without any of its component IEs, the entire container shall be returned in the response. E.g.,

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= 4 |
| --- | --- |
| TYPE_ Container _PoA [4 octets] |  |

returns a list of PoA Containers with all their component IEs.

3) if a container is listed with one or more of its component IEs, the container with only the listed component IEs shall be returned. E.g.,

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= 12 |
| --- | --- |
| TYPE_CONTAINER_NETWORK [4 octets] |  |
| TYPE_IE_Network_Type [4 octets] |  |
| TYPE_IE_Operator_Identifier [4 octets] |  |

returns a list of Network Containers with each containing only Network Type and Operator ID.

4) If a component IE is listed without its parent container, the listed component IE shall be returned as an individual IE. E.g.,

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= 8 |
| --- | --- |
| TYPE_IE_Network_Type [4 octets] |  |
| TYPE_IE_Cost [4 octets] |  |

returns a list of Network Types and a list of Costs. Note, a list of individual IEs out of their context may have very limited usefulness. This is only an example to show the flexible use of Reporting Template and does not mean to

6.3.5.2 Information Response

Information Response (from information server to client) shall carry a MIIS_TLV_RESPONSE TLV, which contains a list of returned IEs/Containers , as shown below:

| Type=MIIS_TLV_RESPONSE | Length= Variable |
| --- | --- |
| Returned IE/Container #1 TLV |  |
| Returned IE/Container #2 TLV (optional) |  |
| … |  |
| Returned IE/Container #k TLV (optional) |  |

Here, each of the Returned IE contains an IE or Container as defined in 6.3.4.

6.3.5.2.1 Rules for Generating Returned IEs

Upon receipt of the query, the information server will:

1) create the list of neighboring access network information for the given location;

2) if Exclusion Conditions are present in the query, apply the Exclusion Conditions to the list of neighboring access network information and exclude the IEs that meet any one of the Exclusion Conditions TLVs; See below for the rules for applying exclusion conditions.

3) if no Reporting Template TLV is given in the query, send the filtered list of neighboring access network information in a List of Neighboring Access Networks Container in an Information Response.

3) if a Reporting Template TLV is given in the query, extract the requested IE(s)/Containers from the filtered list of neighboring access network information using the rules from 6.3.5.1 and send them in an Information Response.

Rules for applying exclusion conditions:

- An IE shall be considered as meeting an Exclusion Conditions TLV if all of the contained IE Condition TLVs are TRUE when applied to the IE. This enables “AND” of exclusion conditions. For example, to create a combined exclusion condition of {“Network_Security < high” AND “Cost > 8”}, one could use a Exclusion Conditions TLV containing two IE Condition TLVs in the request, as illustrated below:

| Type=MIIS_TLV_QUERY |  | Length= Variable |
| --- | --- | --- |
| … |  |  |
| Exclusion Conditions TLV = {“Network_Security < high” AND “Cost > 8”} |  |  |
|  | IE Conditions TLV #1 = {Network_Security < high} |  |
|  | IE Conditions TLV #2 = {Cost > 8} |  |

- If one or more Exclusion Conditions TLV is present in a query, an IE shall be excluded in the response as long as it meets one of the Exclusion Conditions TLVs in the query. This enables “OR” of exclusion conditions. For example, to create a combined exclusion condition of {“Network_Security < high” OR “Cost > 8”}, one could use two Exclusion Conditions TLVs in the request, as illustrated below:

| Type=MIIS_TLV_QUERY | Length= Variable |
| --- | --- |
| … |  |
| Exclusion Conditions TLV #1 = {Network_Security < high} |  |
| Exclusion Conditions TLV #2 = {Cost > 8} |  |

- When more than one Exclusion Conditions TLVs are present in the query, they shall be applied in the order they appear in the query;

- If any component of an IE meets any Exclusion Conditions TLV, the IE shall be excluded. For example, if a component of a PoA IE meets an Exclusion Conditions TLV, the PoA IE is excluded; similarly, if a component of a Access Network IE meets an Exclusion Condition TLV, the Access Network IE is excluded, and so on.

Appendix X: TLV based Information Query and Response Examples

We use the following examples to illustrate how neighboring network information is represented and retrieved.

[Editorial Note: the values of the IEs used in the following examples are fictitious now but will be replaced with the actual format and more realistic values when they are finalized in the spec.]

We assume the querier is at location {x, y}, and we assume that at this location there are three neighboring networks – a UMTS network owned by “red-mobile”, a 802.16 network also owned by “red-mobile”, and another 802.16  network owned by “blue-mobile” – with the following attributes, respectively:

1. a UMTS network:

- Network Type: “umts” (Use Radius code…)

- Operator_ID: “red-mobile”

- Roaming_Partners: “red-partner1, red-partner2”

- Cost: 5 ($9.99 day pass, $39.99 monthly pass)

- Network_Security: “strong-cipher” (may need to have Cipher Suites and Authentication Mechanisms listed separately here…)

- QoS: “class1/class2/class3”

- PoA #1:

  - MAC_address: mac_addr1

  - Location: coord_1 (Latitude, Longitude…)

  - Data_rate: 50Kbps

  - MAC_type: “CSMA-CA”

  - Channel_range: 8

  - Subnet_info: “IPv4-192.35.0.0”

  - Capabilities: 0x4A

- PoA #2:

  - MAC address: mac_addr2

  - Location: coord_2

  - Data_rate: 50

  - MAC_type: unknown

  - Channel_range: 6

  - Subnet_info: “IPv4-192.31.0.0”

  - Capabilities: 0x4C

2. a 802.16 (WiMAX) network:

- Network Type: “802.16”

- Operator_ID: “red-mobile”

- Roaming_Partners: “none”

- Cost: 3

- Network_Security: “strong-cipher”

- QoS: “class1”

- PoA #1:

  - MAC address: mac_addr3

  - Location: coord_3

  - Data_rate: 80

  - MAC_type: 802.16

  - Channel_range: 7

  - Subnet_info: “IPv4-192.201.0.0”

  - Capabilities: 0xB7

3. a 802.16 (WiMax) network:

- Network Type: “802.16”

- Operator_ID: “blue-mobile”

- Roaming_Partners: “blue-partner1”

- Cost: 3

- Network_Security: “open”

- QoS: “class1/class2”

- PoA #1:

  - MAC address: mac_addr4

  - Location: coord_4

  - Data_rate: 95

  - MAC_type: 802.16

  - Channel_range: 7

  - Subnet_info: “IPv6-2002:522d:94d1::/48”

  - Capabilities: 0xB1

Example Query 1: IS client asking for all neighboring network information without providing a querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY | Length= 0 |
| --- | --- |

b) The information server sends back an Information Response, carrying a TYPE_CONAINER_LIST_OF_NETWORKS as the only Returned IE that contains information of all the three neighboring networks at location {x, y}:

| Type=MIIS_TLV_RESPONSE |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- |
| Type= TYPE_CONTAINER_LIST_OF_NETWORKS |  |  |  | Length= actual size |  |
|  | Type=TYPE_CONTAINER_NETWORK |  |  | Length= actual size |  |
|  | Network Type IE = “umts” |  |  |  |  |
|  | Operator Identifier IE = “red-mobile” |  |  |  |  |
|  | Roaming Partners IE = “red-partner1, red-partner2” |  |  |  |  |
|  | Cost IE = 5 |  |  |  |  |
|  | Network Security IE = “strong-cipher” |  |  |  |  |
|  | QoS IE = “class1/class2/class3” |  |  |  |  |
|  |  | Type=TYPE_CONTAINER_POA | Length= Variable |  |  |
|  |  | PoA Location IE = coord_1 |  |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |  |
|  |  | PoA Channel Range IE = 8 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.35.0.0” |  |  |  |
|  |  | PoA Capabilities IE = 0x4A |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr1 |  |  |  |
|  |  | Type=TYPE_CONTAINER_POA | Length= Variable |  |  |
|  |  | PoA Location IE = coord_2 |  |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |  |
|  |  | PoA Channel Range IE = 6 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.31.0.0” |  |  |  |
|  |  | PoA Capabilities IE = 0x4C |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr2 |  |  |  |
|  | Type= TYPE_CONTAINER_NETWORK |  | Length= Variable |  |  |
|  | Network Type IE = “802.16” |  |  |  |  |
|  | Operator Identifier IE = “red-mobile” |  |  |  |  |
|  | Roaming Partners IE = “none” |  |  |  |  |
|  | Cost IE = 3 |  |  |  |  |
|  | Network Security IE = “strong-cipher” |  |  |  |  |
|  | QoS IE = “class1” |  |  |  |  |
|  |  | Type= TYPE_CONTAINER_POA |  |  | Length= Variable |
|  |  | PoA Location IE = coord_3 |  |  |  |
|  |  | PoA Data Rate IE = 80 |  |  |  |
|  |  | PoA MAC Type IE = 802.16 |  |  |  |
|  |  | PoA Channel Range IE = 7 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.201.0.0” |  |  |  |
|  |  | PoA Capabilities IE = 0xB7 |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr3 |  |  |  |
|  | Type= TYPE_CONTAINER_NETWORK |  | Length= Variable |  |  |
|  | Network Type IE = “802.16” |  |  |  |  |
|  | Operator Identifier IE = “blue-mobile” |  |  |  |  |
|  | Roaming Partners IE = “blue-partner1” |  |  |  |  |
|  | Cost IE = 3 |  |  |  |  |
|  | Network Security IE = “open” |  |  |  |  |
|  | QoS IE = “class1/class2” |  |  |  |  |
|  |  | Type= TYPE_CONTAINER_POA |  |  | Length= Variable |
|  |  | PoA Location IE = coord_4 |  |  |  |
|  |  | PoA Data Rate IE = 95 |  |  |  |
|  |  | PoA MAC Type IE = 802.16 |  |  |  |
|  |  | PoA Channel Range IE = 7 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv6-2002:522d:94d1::/48” |  |  |  |
|  |  | PoA Capabilities IE = 0xB1 |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr4 |  |  |  |

Example Query 2: IS client asking for only UMTS neighboring network information, providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  | Length= actual size |
| --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |
| Type=MIIS_TLV_EXCLUSION_CONDITIONS |  | Length= actual size |  |
|  | Type=MIIS_TLV_IE_CONDITION | Length= actual size |  |
|  | IE_type_to_compare = TYPE_IE_NETWOR_TYPE |  |  |
|  | Operator = 1 (i.e., unequal to) |  |  |
|  | Compare_to_value = “umts” |  |  |

b) The information server sends back an Information Response, carrying a TYPE _CONTAINER_LIST_OF_NETWORKS Container that contains information of only the UMTS network:

| Type=MIIS_TLV_RESPONSE |  |  |  | Length= Variable |
| --- | --- | --- | --- | --- |
| Type= TYPE_CONTAINER_LIST_OF_NETWORKS |  |  |  | Length= Variable |
|  | Type=TYPE _CONTAINER_NETWORK |  |  | Length= Variable |
|  | Network Type IE = “umts” |  |  |  |
|  | Operator Identifier IE = “red-mobile” |  |  |  |
|  | Roaming Partners IE = “red-partner1, red-partner2” |  |  |  |
|  | Cost IE = 5 |  |  |  |
|  | Network Security IE = “strong-cipher” |  |  |  |
|  | QoS IE = “class1/class2/class3” |  |  |  |
|  |  | Type=TYPE _CONTAINER_POA | Length= Variable |  |
|  |  | PoA Location IE = coord_1 |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |
|  |  | PoA Channel Range IE = 8 |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.35.0.0” |  |  |
|  |  | PoA Capabilities IE = 0x4A |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |
|  |  | PoA Address IE = mac_addr1 |  |  |
|  |  | Type=TYPE _CONTAINER_POA | Length= Variable |  |
|  |  | PoA Location IE = coord_2 |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |
|  |  | PoA Channel Range IE = 6 |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.31.0.0” |  |  |
|  |  | PoA Capabilities IE = 0x4C |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |
|  |  | PoA Address IE = mac_addr2 |  |  |

Example Query 3: IS client asking for only UMTS neighboring network information and only wanting the return of “General Network” information.

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  | Length= actual size |
| --- | --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |  |
| Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |
|  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |
|  | IE_type_to_compare = TYPE_IE_NETWOR_TYPE |  |  |  |
|  | Operator = 1 (i.e., unequal to) |  |  |  |
|  | Compare_to_value = “umts” |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  | Length= 28 |  |  |
| TYPE_CONTAINER_NETWORK [4 octets] |  |  |  |  |
| TYPE_IE_Network_Type [4 octets] |  |  |  |  |
| TYPE_IE_Operator_Identifier [4 octets] |  |  |  |  |
| TYPE_IE_Roaming_Partners [4 octets] |  |  |  |  |
| TYPE_IE_Cost [4 octets] |  |  |  |  |
| TYPE_IE_Network_Security [4 octets] |  |  |  |  |
| TYPE_IE_QoS [4 octets] |  |  |  |  |

b) The information server sends back an Information Response, carrying a list of Network Containers that contains information of only the UMTS network:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |
| --- | --- | --- |
|  | Type=TYPE_CONTAINER_NETWORK | Length= actual size |
|  | Network Type IE = “umts” |  |
|  | Operator Identifier IE = “red-mobile” |  |
|  | Roaming Partners IE = “red-partner1, red-partner2” |  |
|  | Cost IE = 5 |  |
|  | Network Security IE = “strong-cipher” |  |
|  | QoS IE = “class1/class2/class3” |  |

As an alternative to using Reporting Template, the same can be achieved by using an Exclusion Condition to exclude the PoAs from the response.

Example Query 4: IS client asking for PoA information owned by operator “blue-mobile”, providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |  |  |  |
|  | Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |  |
|  |  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |  |
|  |  | IE_type_to_compare = TYPE_IE_OPERATOR_IDENTIFIER |  |  |  |  |
|  |  | Operator = 1 (i.e., unequal to) |  |  |  |  |
|  |  | Compare_to_value = “blue-mobile” |  |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  |  | Length= 4 |  |  |  |
| TYPE_ Container _PoA [4 octets] |  |  |  |  |  |  |

b) The information server sends back an Information Response, carrying all  TYPE_CONTAINER_POAs owned by operator “blue-mobile” as the Returned IEs. In this case, there is only one TYPE_CONTAINER_POA owned by “blue-mobile” in the neighborhood of location {x, y}:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |  |
| --- | --- | --- | --- |
|  | Type= TYPE_CONTAINER_POA |  | Length= actual size |
|  | Include Network Type and Operator Identifier before PoA information. PoA Location IE = coord_4 |  |  |
|  | PoA Data Rate IE = 95 |  |  |
|  | PoA MAC Type IE = 802.16 |  |  |
|  | PoA Channel Range IE = 7 |  |  |
|  | PoA Subnet Information IE = “IPv6-2002:522d:94d1::/48” |  |  |
|  | PoA Capabilities IE = 0xB1 |  |  |
|  | PoA PHY Type IE = n/a |  |  |
|  | PoA Address IE = mac_addr4 |  |  |

Example Query 5: IS client asking for only subnet information of PoAs of operator “red-mobile”, without providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |  |
|  |  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |  |
|  |  | IE_type_to_compare = TYPE_IE_OPERATOR_IDENTIFIER |  |  |  |  |
|  |  | Operator = 1 (i.e., unequal to) |  |  |  |  |
|  |  | Compare_to_value = “red-mobile” |  |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  |  | Length= 4 |  |  |  |
| TYPE_IE_PoA_Subnet_Information [4 octets] |  |  |  |  |  |  |

b) The information server sends back an Information Response, carrying the  TYPE_IE_SUBNET_INFORMATIONs of all PoAs owned by operator “red-mobile” as the Returned IEs. In this case, there are three TYPE_IE_SUBNET_INFORMATIONs that qualify:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |
| --- | --- | --- |
|  | SUBNET_INFORMATION IE = “IPv4-192.35.0.0” |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.31.0.0” |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.201.0.0” |  |

Note, though not shown here, each SUBNET_INFORMATION IE itself is a TLV. Also note, there is no contextual information returned since the query specifically asked for only returning a component IE type.

Example Query 6: IS client asking for PoAs, excluding those with DATA_RATE less than 75, providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |  |  |  |
|  | Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |  |
|  |  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |  |
|  |  | IE_type_to_compare = TYPE_IE_POA_DATA_RATE |  |  |  |  |
|  |  | Operator = 3 (i.e., less than) |  |  |  |  |
|  |  | Compare_to_value = “75” |  |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  |  | Length= 4 |  |  |  |
| TYPE_ Container _PoA [4 octets] |  |  |  |  |  |  |

b) The information server sends back an Information Response, carrying  TYPE_IE_POAs that containing the two PoAs with DATA RATE larger or equal to 75 in the neighborhood of location {x, y}:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |  |
| --- | --- | --- | --- |
|  | Type= TYPE_CONTAINER_POA |  | Length= actual size |
|  | Include Network Type and Operator Identifier here; PoA Location IE = coord_3 (show these as TLVs) |  |  |
|  | PoA Data Rate IE = 80 |  |  |
|  | PoA MAC Type IE = 802.16 |  |  |
|  | PoA Channel Range IE = 7 |  |  |
|  | PoA Subnet Information IE = “IPv4-192.201.0.0” |  |  |
|  | PoA Capabilities IE = 0xB7 |  |  |
|  | PoA PHY Type IE = n/a |  |  |
|  | PoA Address IE = mac_addr3 |  |  |
|  | Type= TYPE_CONTAINER_POA |  | Length= actual size |
|  | Include Network Type and Operator identifier here: PoA Location IE = coord_4 |  |  |
|  | PoA Data Rate IE = 95 |  |  |
|  | PoA MAC Type IE = 802.16 |  |  |
|  | PoA Channel Range IE = 7 |  |  |
|  | PoA Subnet Information IE = “IPv6-2002:522d:94d1::/48” |  |  |
|  | PoA Capabilities IE = 0xB1 |  |  |
|  | PoA PHY Type IE = n/a |  |  |
|  | PoA Address IE = mac_addr4 |  |  |