| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | LB#1 Issues 5,6,10,18,31 and 34 |  |
| Date Submitted |  |  |
| Source(s) | Srinivas Sreemanthula |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for the issues. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Issue-3 (#352, 360, 378, 380, 403)

| No | Name of Information Element | Description | Representation in reference section |
| --- | --- | --- | --- |
| General Information Elements |  |  |  |
| 1.1 | TYPE_IE_LIST_ OF_NETWORKS | List of Neighboring Access Networks. Link types of the networks that are available in a given geographical area | 6.3.4.1.1 |
| 1.2 | TYPE_IE_NUMB ER_OF_OPERAT ORS | Number of distinct operators for each available link type. There maybe multiple operators for each link type. | 6.3.4.1.2 |
| 1.3 | TYPE_IE_LIST_ OF_OPERATORS | Network Operator List for each type of link. This includes the list of network operators for each of the access networks. | 6.3.4.1.3 |
| Access Network specific information For each (Link Type and Operator) combination following information may be provided |  |  |  |
| 2.1 | TYPE_IE_NUMB ER_POA | Number of Point of Attachments (PoA) for a specific Access Network in the Neighborhood. Number of APs, BSs etc. in the vicinity of client device | 6.3.4.2.1 |
| 2.2 | TYPE_IE_OPER ATOR_IDENTIFI ER | Network Operator The operator of a network. | 6.3.4.1.4 |
| 2.3 | TYPE_IE_ROAM ING_PARTNERS | Roaming Partners. Operators with which the current network operator has direct roaming agreements. | 6.3.4.1.5 |
| 2.4 | TYPE_IE_COST | Cost Indication of cost for service or network usage | 6.3.4.1.6 |
| 2.5 | TYPE_IE_NETW ORK_SECURITY | Security characteristics of the link layer. | 6.3.4.1.7 |
| 2.6 | TYPE_IE_QOS | QoS (Quality of Service) characteristics of the link layer | 6.3.4.1.8 |
| PoA specific information For each PoA of each (Link Type + Operator) following IEs may be provided |  |  |  |
| 3.1 | TYPE_IE_POA_ ADDRESS | MAC Address of PoA | 6.3.4.3.8 |
| 3.2 | TYPE_IE_POA_L OCATION | Location of PoA Geographical location of a given PoA. Multiple location types are supported including coordinate-based location information and civic address | 6.3.4.3.1 |
| 3.3 | TYPE_IE_POA_ DATA_RATE | Data Rate The minimum and maximum value of data rate supported by the link layer of a given PoA | 6.3.4.3.2 |
| 3.4 | TYPE_IE_POA_P HY_TYPE | The media PHY type. | 6.3.4.3.9 |
| 3.5 | TYPE_IE_POA_ MAC_TYPE | The media MAC type. | 6.3.4.3.3 |
| 3.6 | TYPE_IE_POA_ CHANNEL_RAN GE | Channel Range/Parameters Spectrum range supported by the Channel for that PoA | 6.3.4.3.4 |
| Higher layer services/information per PoA |  |  |  |
| 4.1 | TYPE_IE_POA_S UBNET_INFOR MATION | Information about subnets supported by a typical PoA | 6.3.4.3.5 |
| 4.2 | TYPE_IE_POA_ CAPABILITIES | Bitmap of PoA capabilities | 6.3.4.3.6 |
| Other Information Elements |  |  |  |
| 5.1 | Vendor Specific IEs | Vendor Specific Services | These may be defined by individual vendors. |

## Section 6.3.4.1.1

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_LIST_OF_NETW ORKS | List of Neighboring Access Networks | Variable | List of different link types. 1 octets for each link (rounded to 4 bytes) 0x00 – Ethernet 0x01: Wireless - Other 0x02: Wireless - IEEE 802.11b/g 0x03.Wireless - IEEE 802.11a 0x04.Wireless - IEEE 802.16e 0x05: Wireless - CDMA2000 0x06: Wireless - WCDMA 0x07. Wireless – GSM 0x08: Wireless - 1X-EV 0xFF. padding byte |

## Add new section 6.3.4.1.5

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_ROAMING_PARTNERS | Operators with which the current network operator has direct roaming agreements. | variable | First 4 bytes define the number of different operator identifiers as defined in section 6.3.4.1.4 (in current spec) or section 6.3.4.1.5 after the proposed changes above. Following that, the operator identifiers IE are nested within this IE. Number (4 bytes); TYPE_IE_OPERATOR_IDENTIFIER #1. TYPE_IE_OPERATOR_IDENTIFIER #2; …. |

## Add new section 6.3.4.1.6

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_COST | Indication of cost | 4 bytes | List of cost types. Higher 3 bytes are reserved. 0x00 – Not available 0x01: Fixed rate - Daily 0x02: Fixed rate - Hourly 0x03. Fixed rate – Minutes 0x04. Variable rate – data usage (Mbytes) 0x05. Variable rate – QoS class based data usage 0x06. Fixed + Variable – minimum fixed rate per connection + data usage |

## Add new section 6.3.4.1.7

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_SECURITY | Security characteristics of the link layer | 4 bytes | List of supported security suites. Higher 3 bytes are reserved. 0x00 – Open 0x01: 802.11i capable 0x02: 802.11r + 802.11i capable 0x03: EAP based 0x04: cellular specific |

## Add new section 6.3.4.1.8

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_QOS | QoS characteristics of the link layer | 4 bytes | List of supported QoS types. Higher 3 bytes are reserved. 0x00 – Best Effort only 0x01: Wifi WMM 0x02: Wifi WME 0x03: IEEE 802.16 QoS 0x04: cellular specific |

## Add new section 6.3.4.3.9 (not sure if this is of much value)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_POA_PHY_TYPE | Sypported physical radio types | 4 bytes | List of supported PHY types. Higher 3 bytes are reserved. 0x00 – not available 0x01: 802.11b PHY 0x02: 802.11b/g PHY 0x03: 802.11a PHY 0x04: 802.16 PHY 0x05: cellular specific |

# Issue-10

This section needs a major rewrite.

# Issue-18 (#495)

Not clear how QoS resource query for inter-technology cases can be done via backend. UE will still have to negotiate with the target AP/BS. Backend query causes significant complexity and deployment issues. So, handover prepare and handover network information query can be deprecated.

# Issue-31 (#695)

Need to agree if MIH function discovery at L2 can be provided in our specification. L3 is provided by IETF. Remote registration section is updated by MIH registration (so it can be amended as per commenter).

# Issue-34 (#715 and 716)

It is true that the retransmitted packet can be determined only within the time window

Proposal: will provide addional info