| Title | Handover Commands Update: LB Issue- #18: Comment 495 |  |
| --- | --- | --- |
| Date Submitted | September 2006 |  |
| Source(s) | Junghoon Jee, Eunah Kim, Hongseok Jeon, Vivek Gupta, Qiaobing Xie |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for the issues related to Handover Commands |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Remedy 1: Insert following sub-section (6.2.2.1) in section 6.2.2

6.2.2.1 MIH Commands for Network Selection

The 802.21 specification facilitates both mobile-initiated and network-initiated handovers. Handovers may be initiated by changes in the wireless environment which may lead to the selection of networks that support different access technology than the serving network.

During network selection, the mobile node and the network need to exchange information about available candidate networks and select the best network. The network selection policy engine may end up selecting a different network than the current one which may necessitate an inter-technology handover. Network Selection and handover initiation are outside the scope of mobility management protocols such as MIP and SIP. Once a new network has been selected and handover has been initiated, mobility management protocols handle packet routing aspects such as address update and transfer of packet delivery to the new network.

The 802.21 specification supports a set of media independent commands which help with network selection under different conditions. These commands allow both the mobile node and the network to initiate handovers and exchange information about available networks and negotiate the best available network under different conditions. Please refer to the flow diagrams (Figs below in sections 6.2.2.2 and 6.2.2.3)  for more information. These commands do not affect packet routing aspects and can be used in conjunction with other mobility management protocols such as MIP and SIP to perform inter-technology handovers.

Remedy 2: Insert following sections 6.2.2.2 and 6.2.2.3 in section 6.2.2

6.2.2.2. Mobile-initiated Handover Procedure

The Mobile-initiated Handover Procedure operates as follows:

- The MN is connected to the serving network via the Current PoS and it has access to MIH Information Services.

- The MN queries information about neighboring networks by sending the MIH_Get_Information Request message to the Information Server. The Information Server responds with the MIH_Get_Information Response message. This information query may be attempted as soon as a MN first accesses a new serving network.

- The MN triggers a mobile-initiated handover by sending the MIH_Handover_Initiate Request message to the Serving PoS. This request message contains the information about potential candidate networks. A MN might attempt either a Handover or a network reselection, depending, amongst other things, on policy settings and general radio conditions. If a data session is established and the policy decision engine determines that a candidate network is preferred, then the MN might trigger a mobile-initiated handover request by sending the MIH_Handover_Initiate_Request message to the Serving PoS. The request specifies details on the potential candidate networks. Note that the policy decision engine might determine that a system re-selection is warranted. If this is the case the MN might simply access the new system using existing access procedures as per relevant system standard

- The Serving PoS queries the availability of resources at the candidate networks based on the suggested candidate networks from the MN by sending the MIH_Handover_Prepare Request messages to one or multiple Candidate PoSs.

- Candidate PoSs respond with MIH_Handover_Prepare Response messages. The Serving PoS notifies the MN of the resulting resource availability at the candidate networks through MIH_Handover_Initiate Response message. The MN then decides the target PoS based on the information from that message.

- The MN then indicates its intention to do a handover by sending the MIH_Handover_Commit Request message to the Serving PoS. The Serving PoS notifies the target PoS of the fact that MN is about to move to the specified network through the MIH_Handover_Commit Request message. The Serving PoS then notifies the result of the handover commit to the MN by sending the MIH_Handover_Commit Response message.

- After the completion of higher layer handover execution procedures, the MN may send the MIH_Handover_Complete Request message to the Target PoS. The Target PoS sends the MIH_Handover_Complete Request message to the Serving PoS. The Serving PoS responds by sending the MIH_Handover_Complete Response message to the Target PoS. The Target PoS may respond by sending the MIH_Handover_Complete Response message to the MN.

6.2.2.3 Network-initiated Handover Procedure

The Network-initiated Handover Procedure operates as follows,

- The Serving PoS may query the Information Server using the MIH_Get_Information Request and Response messages to get information about available neighboring networks.

- The Serving PoS triggers a network-initiated handover by sending the MIH_Handover_Initiate Request message to the MN. The MN responds through the MIH_Handover_Initiate Response message. This message contains MN’s acknowledgement about the handover and its preferred link and PoS lists.

- Based on Network Selection policies if there is a need of handover, the The Serving PoS sends the MIH_Handover_Prepare Request messages to different Candidate PoSs (can be more than one) to query the availability of the resource at candidate networks. The Candidate PoS responds by sending the MIH_Handover_Prepare Response message to the Serving PoS.

- The Serving PoS initiates the handover by sending the MIH_Handover_Initiate Request to the MN. The MN responds by sending the MIH_Handover_Initiate Response to the Serving PoS.  The Serving PoS decides the handover target based on the resource availability information of candidate networks through the received MIH_Handover_Prepare Response message. The Serving PoS requests handover commitment to the specified network type and PoA by sending the MIH_Handover_Commit Request message to the MN. The Serving PoS also notifies the target PoS of the fact that MN is about to move to the network through the MIH_Handover_Commit Request messages. The MN replies the result of the handover commitment by sending the MIH_Handover_Commit Response message to the Serving PoS.

- Once handover has been committed specific mobility management protocol procedures may be carried out and once these procedures are completed packet reception may resume at the selected PoA.

- After the higher layer handover execution, the MN may send the MIH_Handover_Complete Request message to the Target PoS. The target PoS sends the MIH_Handover_Complete Request message to the previous serving PoS. as well which in turn may respond with MIH_Handover_Complete Response message. The Target PoS may send the MIH_Handover_Complete Response message to the MN.

These flow diagrams (in section 6) are for information only.

Remedy 3: Insert Figure 1 and Figure 2 below in Annex-A1.

(These need to be updated once above sections are settled)

MN

3G

3. MIH_Liink_Detected Indication

4. MIH_Handover_Initiate Request

5. MIH_Handover_Initiate Response

8. MIH_Handover_Commit Request

9. MIH_Handover_Commit Response

Establish L2 Connection

Network Selection  Switch to WLAN

MIP Registration. Binding Update Complete. Packet Reception on New Link

13. MIH_Handover_Complete..response

CurrentLinkType    = WLAN

1. MIH_Get_Information Request

2. MIH_Get_Information Response

WLAN Available. Power on WLAN Radio. Listen to Beacon

MIH Capability = 1 (Supported)

Link Type          = WLAN

MACNewPoA   = MAC of AP(x)

CurrentLinkType    = 3G

SuggestedNewLinkType = WLAN

SuggestedNewPoA = MAC of AP(x)

Handover Mode      = Make before Break

CurrentLinkAction  = LINK_NO_ACTION

CurrentLinkType    = 3G

HandoverAck          = 1 (Initiate Handover)

PreferredLinkType = WLAN

PreferredPoA          = MAC of AP(x)

AbortReason           = 0.

CurrentLinkType    = 3G

NewLinkType        = WLAN

NewPoA                 = MAC of AP(x)

CurrentLinkAction  = LINK_NO_ACTION

CurrentLinkType    = 3G

CurrentLinkAction  = LINK_LOW_POWER

Handover Status      = 1 (Perform Handover)

Candidate PoS

Serving PoS

WLAN

Information Server

6. MIH_Handover_Prepare Request

QueryResourceList = QoS:??

7. MIH_Handover_Prepare Response

ResourceStatus = 1(Available)

AvailableResourceList = QoS:??

Figure 1. Network Initiated Handover from 3G to WLAN

10. MIH_Handover_Complete Request

CurrentLinkType    = WLAN

11. MIH_Handover_Complete Request

12. MIH_Handover_Complete Response

8’. MIH_Handover_Commit Request

9’. MIH_Handover_Commit Response

MN

3G

3. MIH_Liink_Detected Indication

6. MIH_Handover_Initiate Request

7. MIH_Handover_Initiate Response

8. MIH_Handover_Commit Request

11. MIH_Handover_Commit Response

Establish L2 Connection

Network Selection  Switch to WLAN

MIP Registration. Binding Update Complete. Packet Reception on New Link

15. MIH_Handover_Complete Response

CurrentLinkType    = WLAN

1. MIH_Get_Information Request

2. MIH_Get_Information Response

WLAN Available. Power on WLAN Radio. Listen to Beacon

MIH Capability = 1 (Supported)

Link Type          = WLAN

MACNewPoA   = MAC of AP(x)

CurrentLinkType    = 3G

SuggestedNewLinkType = WLAN

SuggestedNewPoA = MAC of AP(x)

Handover Mode      = Make before Break

CurrentLinkAction  = LINK_NO_ACTION

CurrentLinkType    = 3G

HandoverAck          = 1 (Initiate Handover)

PreferredLinkType = WLAN

PreferredPoA          = MAC of AP(x)

AbortReason           = 0.

CurrentLinkType    = 3G

NewLinkType        = WLAN

NewPoA                 = MAC of AP(x)

CurrentLinkAction  = LINK_NO_ACTION

CurrentLinkType    = 3G

CurrentLinkAction  = LINK_LOW_POWER

Handover Status      = 1 (Perform Handover)

Candidate PoS

Serving PoS

WLAN

Information Server

4. MIH_Handover_Prepare Request

QueryResourceList = QoS:??

5. MIH_Handover_Prepare Response

ResourceStatus = 1(Available)

AvailableResourceList = QoS:??

Figure 2. MN Initiated Handover from 3G to WLAN

9. MIH_Handover_Commit_Request

10. MIH_Handover_Commit_Response

12. MIH_Handover_Complete Request

CurrentLinkType    = WLAN

14. MIH_Handover_Complete Response

13. MIH_Handover_Complete Request

Remedy 4: Make following amendments (as marked) in Section 7

    - MIH Handover Initiate

      - MIH_Handover_Initiate.request

        - Function

This primitive is used by MIH Function on a mobile node or network to communicate with peer MIH Function on a network or mobile node. The primitive is used to communicate intent of handover initiation. The handover may be initiated either by the client or by the network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.request 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

SuggestedNewLinkIdentifier,

SuggestedNewPoAIdentifier,

HandoverMode,

CurrentLinkAction,

QueryResourceList,

QueryResourceFlag

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| ~snip~ Refer the D01.09 |  |  |  |
| QueryResourceList | BooleanList | N/A | List of resources to be queried at candidate networks. This filed is used in case of the mobile-initiated handover. |
| QueryResourceFlag | Boolean | N/A | Flag to query the available list of resources on the new link. This flag is used in case of the network-initiated handover. |

        - When generated

This primitive is generated by an upper layer entity such as a Handover Policy engine, L3 Mobility protocol like MIP, Application or some other entity that may want to cause a change in the current selected link. This command may be sent either to just the local MIH Function or it may be sent to the remote MIH Function as well. . In case of the mobile-initiated handover, MN uses the QueryResourceList paramenter to notify the Serving PoS of the resource list which it is interested in initiating a handover. In case of the network-initiated handover, Serving PoS requests MN to provide information about resources required to initiate a handover by setting the QueiryResourceFlag parameter.

        - Effect on receipt

The remote MIH Function responds with MIH_Handover_Initiate.response primitive. If the recipient of the command is a network MIH function, In case of the mobile-initiated handover, Serving PoS may performs a MIH_Handover_Prepare message exchanges with the MIH Function in the candidate networks utilizing the information from the received MIH_Handover_Initiate Request message.target link under consideration before sending the response message. A corresponding indication (MIH_Handover_Initiate.indication) maybe triggered to send to all registered MIH user entities in the local stack. The associated parameters for the generated indication are same as those used in command request. In case of the network-initiated handover, MN needs to provide information about resources required to handover as part of the MIH_Handover_Initiate.response primitive.

      - MIH_Handover_Initiate.response

        - Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that sent out a MIH_Handover_Initiate.request. The primitive is used to communicate the response of handover initiation request. The response may be sent either by the client or the network depending on where the request was initiated.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Initiate.response 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkIdentifier,

PreferedPoAIdentifier,

AbortReason,

AvailableResourceList

RequiredQueryResourceList

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| ~snip~ Refer the D01.09 |  |  |  |
| AbortReason | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| AvailableResourceList | List | N/A | List of available resources on candidate networks. This filed is used in case of the mobile-initiated handover. |
| QueryResourceList | List | N/A | List of resources to be queried at candidate networks. This filed is used in case of the network-initiated handover. |

        - When generated

The MIH Function responds with this primitive in response to a MIH_Handover_Initiate.request command from a peer MIH Function entity.

        - Effect on receipt

On receiving the response the entity which originally initiated the handover request may decides to carry out the handover or abort it based on the response. In case of the mobile-initiated handvoer, MN decides the tareget network based on the information from AvailableResourceList parameter. In case of the network-initated handover, Serving PoS performs a MIH_Handover_Prepare message exchanges with the MIH Function in the candidate networks utilizing the information from the received MIH_Handover_Initiate.reponse parameter.

    - MIH_Handover_Commit

      - MIH_Handover_Commit.request

        - Function

This primitive is used by a peer MIH Function to communicate with the remote MIH Function on network. The primitive is used to communicate commitment to perform handover based on selected choices for candidate network and PoA.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.request 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

NewLinkIdentifier,

MACNewPoA,

CurrentLinkAction

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Commit Request message is transferred between the mobile node and the Serving PoS. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| NewLinkIdentifier | May be one of dif ferent 802 or cel lular networks | N/A | This specifies the new network to which the connection needs to be handed over. |
| MACNewPoA | MAC Address. | N/A | MAC Address of New PoA |
| CurrentLinkAction | Bitmap | 0-4 | Specifies suggested action on old link once handover procedure has been executed: 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4: Reserved |

        - When generated

This primitive is generated when successful MIH_Handover_Initiate.response is received to allow a mobile node to begin handover and the mobile node decides to perform actual handover based on suggested choices for candidate network and PoA.

        - Effect on receipt

The recipient may identify that the mobile node starts handover to the selected network. A corresponding indication (MIH_Handover_Commit.indication) maybe triggered to send to all registered MIH user entities in the local stack. The associated parameters for the generated indication are same as those used in command request.

      - MIH_Handover_Commit.response

        - Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that has sent out the MIH_Handover_Commit.request. The primitive is used to communicate the response of a handover commit request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Commit.response 	(

DestinationIdentifier,

SourceIdentifier,

CurrentLinkIdentifier,

CurrentLinkAction,

HandoverStatus

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Commit R message is transferred between the mobile node and the Serving PoS. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| CurrentLinkAction | Bitmap | 0-4 | Specifies suggested action on old link once handover procedure has been exe cuted: 0: LINK_DISCONNECT 1: LINK_LOW_POWER 2: LINK_POWER_DOWN 3: LINK_NO_ACTION 4: Reserved |
| HandoverStatus | Boolean | N/A | 0: Abort handover 1: Perform handover |

When generated

This primitive is generated in response to MIH_Handover_Commit.request primitive.

        - Effect on receipt

Upon receipt, the old Serving PoS may start forwarding any pending or remaing packets to the new Target PoS. The client recipient may initiate handover process and begin setting up of new layer 2 connection. The network recipient may determine that the handover procedure is in progress to the intended network.

    - MIH_Handover_Complete

      - MIH_Handover_Complete.request

        - Function

This primitive is used by a peer MIH Function to communicate with the remote MIH Function. This is used to communicate the completion of handover operation.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Complete.request 	(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identi fier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIden tifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Complete Request message is transferred between the mobile node and the Serving PoS. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |

        - When generated

The MIH Function responds with this primitive when handover operations to the new network PoA have been completed.

        - Effect on receipt

Upon receipt, the receiving MIH Function on the new link shall forward the message to the MIH Function in the old link. Upon receipt, the MIH Users receiving MIH Function in the old link may start forwarding any pending or remaining packets and also to release any resources in use by the old link. A corresponding indication (MIH_Handover_Complete.indication) may be triggered to send to all registered MIH user entities in the local stack. The associated parameters for the generated indication are same as those used in command request.

      - MIH_Handover_Complete.response

        - Function

This primitive is used by a peer MIH Function to communicate with the MIH Function that has sent out the MIH_Handover_Complete.request. This primitive is used to send a response to the handover complete request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Complete.response (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier

)

Local or Remote: Both

MIHF (Mobile Node)  <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identi fier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIden tifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_Handover_Complete Response message is transferred between the mobile node and the Serving PoS. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |

        - When generated

The MIH Function responds with this primitive after processing the handover complete request.

        - Effect on receipt

Upon receipt, the client MIH Function may determine that the handover complete request was processed successfully. Upon receipt, the MIH Function in the new network may forward this message to the client.

    - MIH Handover Prepare

      - MIH_Handover_Prepare.request

        - Function

This primitive is used by a MIH function on the serving network to communicate with its peer MIH function on the candidate network. This is used to prepare the new link resource for impending handover and to query the available link resource of the candidate network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Prepare.request 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

QueryResourceList,

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

        - When generated

This primitive is generated after receiving the MIH_Handover_Initiate Request message from the MIH function on the mobile node in case of mobile-initiated handover. In case of network-initiated handover, this primitive is generated after receiving the MIH_Handover_Initiate Response message from the mobile node. may be generated by the upper layer entity such as a Handover Policy engine.

        - Effect on receipt

This peer MIH function on the candidate network identifies the link resource usage and prepares resources for the impending handover. A corresponding indication (MIH_Handover_Prepare.indication) maybe triggered to send to all registered MIH user entities in the local stack. The associated parameters for the generated indication are same as those used in the command request.

      - MIH_Handover_Prepare.response

        - Function

This primitive is used by a MIH function on the candidate network to communicate with its peer MIH function on the serving network which sent out a MIH_Handover_Prepare.request. This is used to respond the result of resource preparation of the impending handover and to notify the MIH function on the serving network of the link resource status of the candidate network

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Handover_Prepare.response 				(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

ResourceStatus,

AvailableResourceList,

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| ResourceStatus | Enumerate | N/A 0: Not Available 1: Available | Specifies whether requested resources are available or not at the new PoA. |
| AvailableResourceList | List | N/A | List of resources actually available at the new suggested network and the new net workPoA. |

        - When generated

The MIH Function on the candidate network responds with this primitive in response to a MIH_Handover_Prepare Request message from a peer MIH Function entity on the serving network.

        - Effect on receipt

In case of the mobile-initiated handover, the Serving PoS sends the MIH_Handover_Initiate Response message to the mobile node utilzing the received paramenters from the MIH_Handover_Prepare.reponse primitive. In case of the network-initiated handover, the Serving PoS decides the target of the handover based on the received paramenters from the MIH_Handover_Prepare.reponse primitive.