| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Link Event and Command IDs : LB Issue #21 (Comment 502) |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Eunah Kim, Junghoon Jee, Hongseok Jeon ETRI | {eakim, jhjee, jeonhs}@etri.re.kr |
| Re: | IEEE 802.21 Session #NN in San Diego, California |  |
| Abstract | This contribution provides a solution to the ID assignment for different events and commands to be used in Event List and Command List, respectively. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Problems

- LB issue 21 (comment 502) remarks the necessity of ID assignment for different events and commands to be used in Event List and Command List, respectively.

- Current encoding for each event or command requires 2 bytes, not one byte because Message ID is 2- byte-length

- Current encoding for commands don’t support some commands such as Link Connect, Link Disconnect, etc., because they are not Message IDs

2. Proposed Changes

- Assign Event IDs for Link Events

- Assign Command IDs for Link Commands that are copied from P802-21-D00-04

Modify subsections, 8.4.1.1 and 8.4.1.2 as indicated below:

8.4.1.1 Event List

This parameter specifies the list of events that may carried in a message. Each event is represented by an event code ID and occupies one byte.

| Type | Length | Value |
| --- | --- | --- |
| 255 | Variable | Array of event codes IDs formatted as message-id : Event Code 1, Event Code 2, Event Code 3, … Event ID is as follows : 0 : Link Up 1 : Link Down 2 : Link Going Down 3 : Link Event Rollback 4 : Link Detected 5 : Link Parameters Change 6 : Link SDU Transmit Status 7 : Link Handover Imminent 8 : Link Handover Complete 9~31 : Reserved |

8.4.1.2 Command List

This parameter specifies the list of commands that may carried in a message. Each command is represented by a command code ID and occupies one byte.

| Type | Length | Value |
| --- | --- | --- |
| 254 | Variable | Array of command codes IDs formatted as message-id : Command Code 1, Command Code 2, Command Code 3, … Command ID is as follows : 0 : Link Power Up 1 : Link Power Down 2 : Link Configure 3 : Link Connect 4 : Link Disconnect 5 : Link Sleep 6 : Link Scan 7 : Link Poll 8~31 : Reserved |