| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements TLV Retrieval without support Exclusion Conditions (Comment Assignment #3) |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Qiaobing Xie, Vivek Gupta, Xiaoyu Liu, Soohong Daniel Park, Eric Njedjou, Masahiro Kuroda |  |
| Re: | 21-06-0xxx-00-0000-IE_TLV_retrieval_no_exclusion |  |
| Abstract | TLV Retrieval of Information Elements without support for Exclusion conditions |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Replace current 6.3.5, 6.3.6, and 6.3.7 with the following new 6.3.5:

6.3.5 Information Retrieval Using TLV Representation

To retrieve the heterogeneous neighborhood information (or a subset of it), the client sends an Information Request to the MIH PoS. The Information request contains query for Information Elements. The MIH PoS sends an Information Response which contains the response of the query back to the client.

6.3.5.1 Information Request

Information Request (from client to information server) shall carry a MIIS_TLV_QUERY TLV which contains: 1) an optional Querier Location TLV parameter indicating the location of the querier, and 2) an optional Reporting Template TLV, as shown below:

| Type=MIIS_TLV_QUERY | Length= Variable |
| --- | --- |
| Querier Location TLV [variable] (optional) |  |
| Reporting Template TLV [variable] (optional) |  |

The TLVs that are carried in the Information Request are described below.

Querier Location TLV is defined as follows:

| Type=MIIS_TLV_QUERIER_LOCATION | Length= Variable |
| --- | --- |
| The value is defined the same as that of PoA Location IE (see 6.3.4.3.1) |  |

Querier Location, if provided in the query, can be useful for the Information Server to refine its response. If not provided (for example, a UE does not have its location measurement when making the query), the Information Server shall either get the querier location information from other means or use best estimate of the querier’s location to generate the neighboring network information.

Reporting Template TLV is optional and of variable size. When present, it indicates to the information server a template of the list of IEs which shall be included in the information response:

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= Variable |
| --- | --- |
| TYPE_CONTAINER_LIST_OF_NETWORKS [4 octets] (optional) |  |
| TYPE_CONTAINER_NETWORK [4 octets] (optional) |  |
| TYPE_IE_Network_Type [4 octets] (optional) |  |
| TYPE_IE_Operator_Identifier [4 octets] (optional) |  |
| TYPE_IE_Roaming_Partners [4 octets] (optional) |  |
| TYPE_IE_Cost [4 octets] (optional) |  |
| TYPE_IE_Network_Security [4 octets] (optional) |  |
| TYPE_IE_QoS [4 octets] (optional) |  |
| TYPE_ Container _PoA [4 octets] (optional) |  |
| TYPE_IE_PoA_Location [4 octets] (optional) |  |
| TYPE_IE_PoA_Data_Rate [4 octets] (optional) |  |
| TYPE_IE_PoA_MAC_Type [4 octets] (optional) |  |
| TYPE_IE_PoA_Channel_Range [4 octets] (optional) |  |
| TYPE_IE_PoA_Subnet_Information [4 octets] (optional) |  |
| TYPE_IE_PoA_Capabilities [4 octets] (optional) |  |
| TYPE_IE_PoA_PHY_Type [4 octets] (optional) |  |
| TYPE_IE_PoA_Address [4 octets] (optional) |  |

Each of the IEs in the Requested IEs TLV is indicated by its 32 bit identifier as assigned in Table yy.

Rules for using the Reporting Template TLV:

1) If the Reporting Template TLV is absent, the entire list of neighboring networks container shall be returned in the response. E.g.

| Type=MIIS_TLV_QUERY | Length= actual size of Location TLV |
| --- | --- |
| Querier Location TLV = (x, y) |  |

returns the entire List of Neighboring Networks Container at location (x, y).

2) If a container is listed without any of its component IEs, the entire container shall be returned in the response. E.g.,

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= 4 |
| --- | --- |
| TYPE_ Container _PoA [4 octets] |  |

returns a list of PoA Containers with all their component IEs.

3) if a container is listed with one or more of its component IEs, the container with only the listed component IEs shall be returned. E.g.,

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= 12 |
| --- | --- |
| TYPE_CONTAINER_NETWORK [4 octets] |  |
| TYPE_IE_Network_Type [4 octets] |  |
| TYPE_IE_Operator_Identifier [4 octets] |  |

returns a list of Network Containers with each containing only Network Type and Operator ID.

4) If a component IE is listed without its parent container, the listed component IE shall be returned as an individual IE. E.g.,

| Type=MIIS_TLV_REPORT_TEMPLATE | Length= 8 |
| --- | --- |
| TYPE_IE_Network_Type [4 octets] |  |
| TYPE_IE_Cost [4 octets] |  |

returns a list of Network Types and a list of Costs. Note, a list of individual IEs out of their context may have very limited usefulness. This is only an example to show the flexible use of Reporting Template and does not mean to

6.3.5.2 Information Response

Information Response (from information server to client) shall carry a MIIS_TLV_RESPONSE TLV, which contains a list of returned IEs/Containers , as shown below:

| Type=MIIS_TLV_RESPONSE | Length= Variable |
| --- | --- |
| Returned IE/Container #1 TLV |  |
| Returned IE/Container #2 TLV (optional) |  |
| … |  |
| Returned IE/Container #k TLV (optional) |  |

Here, each of the Returned IE contains an IE or Container as defined in 6.3.4.

6.3.5.2.1 Rules for Generating Returned IEs

Upon receipt of the query, the information server will:

1) create the list of neighboring access network information for the given location;

2) if no Reporting Template TLV is given in the query, send the list of neighboring access network information in a List of Neighboring Access Networks Container in an Information Response.

3) if a Reporting Template TLV is given in the query, extract the requested IE(s)/Containers from the list of neighboring access network information using the rules from 6.3.5.1 and send them in an Information Response.