| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements TLV Retrieval Examples (Comment Assignment #3) |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Qiaobing Xie, Vivek Gupta, Xiaoyu Liu, Soohong Daniel Park, Eric Njedjou, Masahiro Kuroda |  |
| Re: | 21-06-0xxx-00-0000-IE_TLV_retrieval_examples |  |
| Abstract | Examples for TLV Retrieval of Information Elements |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Add the following Appendix:

Appendix X: TLV based Information Query and Response Examples

We use the following examples to illustrate how neighboring network information is represented and retrieved.

[Editorial Note: the values of the IEs used in the following examples are fictitious now but will be replaced with the actual format and more realistic values when they are finalized in the spec.]

We assume the querier is at location {x, y}, and we assume that at this location there are three neighboring networks – a UMTS network owned by “red-mobile”, a 802.16 network also owned by “red-mobile”, and another 802.16  network owned by “blue-mobile” – with the following attributes, respectively:

1. a UMTS network:

- Network Type: “umts” (Use Radius code…)

- Operator_ID: “red-mobile”

- Roaming_Partners: “red-partner1, red-partner2”

- Cost: 5 ($9.99 day pass, $39.99 monthly pass)

- Network_Security: “strong-cipher” (may need to have Cipher Suites and Authentication Mechanisms listed separately here…)

- QoS: “class1/class2/class3”

- PoA #1:

  - MAC_address: mac_addr1

  - Location: coord_1 (Latitude, Longitude…)

  - Data_rate: 50Kbps

  - MAC_type: “CSMA-CA”

  - Channel_range: 8

  - Subnet_info: “IPv4-192.35.0.0”

  - Capabilities: 0x4A

- PoA #2:

  - MAC address: mac_addr2

  - Location: coord_2

  - Data_rate: 50

  - MAC_type: unknown

  - Channel_range: 6

  - Subnet_info: “IPv4-192.31.0.0”

  - Capabilities: 0x4C

2. a 802.16 (WiMAX) network:

- Network Type: “802.16”

- Operator_ID: “red-mobile”

- Roaming_Partners: “none”

- Cost: 3

- Network_Security: “strong-cipher”

- QoS: “class1”

- PoA #1:

  - MAC address: mac_addr3

  - Location: coord_3

  - Data_rate: 80

  - MAC_type: 802.16

  - Channel_range: 7

  - Subnet_info: “IPv4-192.201.0.0”

  - Capabilities: 0xB7

3. a 802.16 (WiMax) network:

- Network Type: “802.16”

- Operator_ID: “blue-mobile”

- Roaming_Partners: “blue-partner1”

- Cost: 3

- Network_Security: “open”

- QoS: “class1/class2”

- PoA #1:

  - MAC address: mac_addr4

  - Location: coord_4

  - Data_rate: 95

  - MAC_type: 802.16

  - Channel_range: 7

  - Subnet_info: “IPv6-2002:522d:94d1::/48”

  - Capabilities: 0xB1

Example Query 1: IS client asking for all neighboring network information without providing a querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY | Length= 0 |
| --- | --- |

b) The information server sends back an Information Response, carrying a TYPE_CONAINER_LIST_OF_NETWORKS as the only Returned IE that contains information of all the three neighboring networks at location {x, y}:

| Type=MIIS_TLV_RESPONSE |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- |
| Type= TYPE_CONTAINER_LIST_OF_NETWORKS |  |  |  | Length= actual size |  |
|  | Type=TYPE_CONTAINER_NETWORK |  |  | Length= actual size |  |
|  | Network Type IE = “umts” |  |  |  |  |
|  | Operator Identifier IE = “red-mobile” |  |  |  |  |
|  | Roaming Partners IE = “red-partner1, red-partner2” |  |  |  |  |
|  | Cost IE = 5 |  |  |  |  |
|  | Network Security IE = “strong-cipher” |  |  |  |  |
|  | QoS IE = “class1/class2/class3” |  |  |  |  |
|  |  | Type=TYPE_CONTAINER_POA | Length= Variable |  |  |
|  |  | PoA Location IE = coord_1 |  |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |  |
|  |  | PoA Channel Range IE = 8 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.35.0.0” |  |  |  |
|  |  | PoA Capabilities IE = 0x4A |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr1 |  |  |  |
|  |  | Type=TYPE_CONTAINER_POA | Length= Variable |  |  |
|  |  | PoA Location IE = coord_2 |  |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |  |
|  |  | PoA Channel Range IE = 6 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.31.0.0” |  |  |  |
|  |  | PoA Capabilities IE = 0x4C |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr2 |  |  |  |
|  | Type= TYPE_CONTAINER_NETWORK |  | Length= Variable |  |  |
|  | Network Type IE = “802.16” |  |  |  |  |
|  | Operator Identifier IE = “red-mobile” |  |  |  |  |
|  | Roaming Partners IE = “none” |  |  |  |  |
|  | Cost IE = 3 |  |  |  |  |
|  | Network Security IE = “strong-cipher” |  |  |  |  |
|  | QoS IE = “class1” |  |  |  |  |
|  |  | Type= TYPE_CONTAINER_POA |  |  | Length= Variable |
|  |  | PoA Location IE = coord_3 |  |  |  |
|  |  | PoA Data Rate IE = 80 |  |  |  |
|  |  | PoA MAC Type IE = 802.16 |  |  |  |
|  |  | PoA Channel Range IE = 7 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.201.0.0” |  |  |  |
|  |  | PoA Capabilities IE = 0xB7 |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr3 |  |  |  |
|  | Type= TYPE_CONTAINER_NETWORK |  | Length= Variable |  |  |
|  | Network Type IE = “802.16” |  |  |  |  |
|  | Operator Identifier IE = “blue-mobile” |  |  |  |  |
|  | Roaming Partners IE = “blue-partner1” |  |  |  |  |
|  | Cost IE = 3 |  |  |  |  |
|  | Network Security IE = “open” |  |  |  |  |
|  | QoS IE = “class1/class2” |  |  |  |  |
|  |  | Type= TYPE_CONTAINER_POA |  |  | Length= Variable |
|  |  | PoA Location IE = coord_4 |  |  |  |
|  |  | PoA Data Rate IE = 95 |  |  |  |
|  |  | PoA MAC Type IE = 802.16 |  |  |  |
|  |  | PoA Channel Range IE = 7 |  |  |  |
|  |  | PoA Subnet Information IE = “IPv6-2002:522d:94d1::/48” |  |  |  |
|  |  | PoA Capabilities IE = 0xB1 |  |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |  |
|  |  | PoA Address IE = mac_addr4 |  |  |  |

Example Query 2: IS client asking for only UMTS neighboring network information, providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  | Length= actual size |
| --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |
| Type=MIIS_TLV_EXCLUSION_CONDITIONS |  | Length= actual size |  |
|  | Type=MIIS_TLV_IE_CONDITION | Length= actual size |  |
|  | IE_type_to_compare = TYPE_IE_NETWOR_TYPE |  |  |
|  | Operator = 1 (i.e., unequal to) |  |  |
|  | Compare_to_value = “umts” |  |  |

b) The information server sends back an Information Response, carrying a TYPE _CONTAINER_LIST_OF_NETWORKS Container that contains information of only the UMTS network:

| Type=MIIS_TLV_RESPONSE |  |  |  | Length= Variable |
| --- | --- | --- | --- | --- |
| Type= TYPE_CONTAINER_LIST_OF_NETWORKS |  |  |  | Length= Variable |
|  | Type=TYPE _CONTAINER_NETWORK |  |  | Length= Variable |
|  | Network Type IE = “umts” |  |  |  |
|  | Operator Identifier IE = “red-mobile” |  |  |  |
|  | Roaming Partners IE = “red-partner1, red-partner2” |  |  |  |
|  | Cost IE = 5 |  |  |  |
|  | Network Security IE = “strong-cipher” |  |  |  |
|  | QoS IE = “class1/class2/class3” |  |  |  |
|  |  | Type=TYPE _CONTAINER_POA | Length= Variable |  |
|  |  | PoA Location IE = coord_1 |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |
|  |  | PoA Channel Range IE = 8 |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.35.0.0” |  |  |
|  |  | PoA Capabilities IE = 0x4A |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |
|  |  | PoA Address IE = mac_addr1 |  |  |
|  |  | Type=TYPE _CONTAINER_POA | Length= Variable |  |
|  |  | PoA Location IE = coord_2 |  |  |
|  |  | PoA Data Rate IE = 50 |  |  |
|  |  | PoA MAC Type IE = NULL |  |  |
|  |  | PoA Channel Range IE = 6 |  |  |
|  |  | PoA Subnet Information IE = “IPv4-192.31.0.0” |  |  |
|  |  | PoA Capabilities IE = 0x4C |  |  |
|  |  | PoA PHY Type IE = n/a |  |  |
|  |  | PoA Address IE = mac_addr2 |  |  |

Example Query 3: IS client asking for only UMTS neighboring network information and only wanting the return of “General Network” information.

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  | Length= actual size |
| --- | --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |  |
| Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |
|  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |
|  | IE_type_to_compare = TYPE_IE_NETWOR_TYPE |  |  |  |
|  | Operator = 1 (i.e., unequal to) |  |  |  |
|  | Compare_to_value = “umts” |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  | Length= 28 |  |  |
| TYPE_CONTAINER_NETWORK [4 octets] |  |  |  |  |
| TYPE_IE_Network_Type [4 octets] |  |  |  |  |
| TYPE_IE_Operator_Identifier [4 octets] |  |  |  |  |
| TYPE_IE_Roaming_Partners [4 octets] |  |  |  |  |
| TYPE_IE_Cost [4 octets] |  |  |  |  |
| TYPE_IE_Network_Security [4 octets] |  |  |  |  |
| TYPE_IE_QoS [4 octets] |  |  |  |  |

b) The information server sends back an Information Response, carrying a list of Network Containers that contains information of only the UMTS network:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |
| --- | --- | --- |
|  | Type=TYPE_CONTAINER_NETWORK | Length= actual size |
|  | Network Type IE = “umts” |  |
|  | Operator Identifier IE = “red-mobile” |  |
|  | Roaming Partners IE = “red-partner1, red-partner2” |  |
|  | Cost IE = 5 |  |
|  | Network Security IE = “strong-cipher” |  |
|  | QoS IE = “class1/class2/class3” |  |

As an alternative to using Reporting Template, the same can be achieved by using an Exclusion Condition to exclude the PoAs from the response.

Example Query 4: IS client asking for PoA information owned by operator “blue-mobile”, providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |  |  |  |
|  | Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |  |
|  |  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |  |
|  |  | IE_type_to_compare = TYPE_IE_OPERATOR_IDENTIFIER |  |  |  |  |
|  |  | Operator = 1 (i.e., unequal to) |  |  |  |  |
|  |  | Compare_to_value = “blue-mobile” |  |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  |  | Length= 4 |  |  |  |
| TYPE_ Container _PoA [4 octets] |  |  |  |  |  |  |

b) The information server sends back an Information Response, carrying all  TYPE_CONTAINER_POAs owned by operator “blue-mobile” as the Returned IEs. In this case, there is only one TYPE_CONTAINER_POA owned by “blue-mobile” in the neighborhood of location {x, y}:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |  |
| --- | --- | --- | --- |
|  | Type= TYPE_CONTAINER_POA |  | Length= actual size |
|  | Include Network Type and Operator Identifier before PoA information. PoA Location IE = coord_4 |  |  |
|  | PoA Data Rate IE = 95 |  |  |
|  | PoA MAC Type IE = 802.16 |  |  |
|  | PoA Channel Range IE = 7 |  |  |
|  | PoA Subnet Information IE = “IPv6-2002:522d:94d1::/48” |  |  |
|  | PoA Capabilities IE = 0xB1 |  |  |
|  | PoA PHY Type IE = n/a |  |  |
|  | PoA Address IE = mac_addr4 |  |  |

Example Query 5: IS client asking for only subnet information of PoAs of operator “red-mobile”, without providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |  |
|  |  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |  |
|  |  | IE_type_to_compare = TYPE_IE_OPERATOR_IDENTIFIER |  |  |  |  |
|  |  | Operator = 1 (i.e., unequal to) |  |  |  |  |
|  |  | Compare_to_value = “red-mobile” |  |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  |  | Length= 4 |  |  |  |
| TYPE_IE_PoA_Subnet_Information [4 octets] |  |  |  |  |  |  |

b) The information server sends back an Information Response, carrying the  TYPE_IE_SUBNET_INFORMATIONs of all PoAs owned by operator “red-mobile” as the Returned IEs. In this case, there are three TYPE_IE_SUBNET_INFORMATIONs that qualify:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |
| --- | --- | --- |
|  | SUBNET_INFORMATION IE = “IPv4-192.35.0.0” |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.31.0.0” |  |
|  | SUBNET_INFORMATION IE = “IPv4-192.201.0.0” |  |

Note, though not shown here, each SUBNET_INFORMATION IE itself is a TLV. Also note, there is no contextual information returned since the query specifically asked for only returning a component IE type.

Example Query 6: IS client asking for PoAs, excluding those with DATA_RATE less than 75, providing querier location

a) The client sends an Information Request to the information server, with MIIS_TLV_QUERY TLV set to:

| Type=MIIS_TLV_QUERY |  |  |  |  | Length= actual size |  |
| --- | --- | --- | --- | --- | --- | --- |
| Querier Location TLV = {x, y} |  |  |  |  |  |  |
|  | Type=MIIS_TLV_EXCLUSION_CONDITIONS |  |  | Length= actual size |  |  |
|  |  | Type=MIIS_TLV_IE_CONDITION |  | Length= actual size |  |  |
|  |  | IE_type_to_compare = TYPE_IE_POA_DATA_RATE |  |  |  |  |
|  |  | Operator = 3 (i.e., less than) |  |  |  |  |
|  |  | Compare_to_value = “75” |  |  |  |  |
| Type=MIIS_TLV_REPORT_TEMPLATE |  |  | Length= 4 |  |  |  |
| TYPE_ Container _PoA [4 octets] |  |  |  |  |  |  |

b) The information server sends back an Information Response, carrying  TYPE_IE_POAs that containing the two PoAs with DATA RATE larger or equal to 75 in the neighborhood of location {x, y}:

| Type=MIIS_TLV_RESPONSE |  | Length= actual size |  |
| --- | --- | --- | --- |
|  | Type= TYPE_CONTAINER_POA |  | Length= actual size |
|  | Include Network Type and Operator Identifier here; PoA Location IE = coord_3 (show these as TLVs) |  |  |
|  | PoA Data Rate IE = 80 |  |  |
|  | PoA MAC Type IE = 802.16 |  |  |
|  | PoA Channel Range IE = 7 |  |  |
|  | PoA Subnet Information IE = “IPv4-192.201.0.0” |  |  |
|  | PoA Capabilities IE = 0xB7 |  |  |
|  | PoA PHY Type IE = n/a |  |  |
|  | PoA Address IE = mac_addr3 |  |  |
|  | Type= TYPE_CONTAINER_POA |  | Length= actual size |
|  | Include Network Type and Operator identifier here: PoA Location IE = coord_4 |  |  |
|  | PoA Data Rate IE = 95 |  |  |
|  | PoA MAC Type IE = 802.16 |  |  |
|  | PoA Channel Range IE = 7 |  |  |
|  | PoA Subnet Information IE = “IPv6-2002:522d:94d1::/48” |  |  |
|  | PoA Capabilities IE = 0xB1 |  |  |
|  | PoA PHY Type IE = n/a |  |  |
|  | PoA Address IE = mac_addr4 |  |  |