| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_Get_Information primitives |  |
| Date Submitted | September 13, 2006 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #16 in September 2006 |  |
| Abstract | This document proposes text for LB1 comment # 661 (Issue 50). |  |
| Purpose | LB#1 comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Issue 45 (Comment #661)

  - Does every device have to support every query type?

  - How is the format between two peers decided?

  - What happens if there is a mismatch?

  - Additional behaviour for the Get_Information primitives needs to be defined.

- Proposal for D01-09

Proposal #1: Change Section 7.4.15.1.2 as follows:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InfoQueryType | An integer value corresponding to one of the following types: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA | N/A | The type of query that is specified. It is expected that this field contain one of the query types that are known to be supported by the communicating peers via MIH capability discovery. |
| InfoQueryParameters | Query type specific parameters | N/A | Query type specific parameters which indicate the type of information the client may be interested in. |

Proposal #2: Change Section 7.4.15.2.2 as follows:

| Name | TypeValid Ran | ge | Description |
| --- | --- | --- | --- |
| InfoQueryType | An integer value corresponding to one of the following types: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA | N/A | The type of query that is specified. |
| MIH_REPORT | String | N/A | Report consisting of information requested by the MIH User. |
| Status | Enumerate | 0: success 1: failure 2: rejected | Query type specific parameters which indicate the type of information the client may be interested in. Status == 2 indicates an error due to a mismatch in the supported query type. Status == 1 is used for other errors. Status == 0 indicates the query is successful. MIH_REPORT contains meaningful data only when Status == 0. |